# 3GPP TSG-RAN WG4 Meeting #112	R4-2411771

# Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	8.8.2.1

Source: 	CMCC

Title: 	co-existence study for NTN

Document for:	Discussion

# 1. Introduction

In RAN plenary #103 meeting, a new WID[1] of enhanced requirements and test methodology for NR and IoT NTN is approved with following RAN4 lead RF part objectives:

High power UE (HPUE) for NTN - Conduct the coexistence analysis based on TR 38.863 for the above example bands and corresponding power classes - Evaluate the impact on the uplink performance of TN (Terrestrial network) - Example bands include bands n256 and 256, n255 and 255 - PC2, PC1.5 (for NR-NTN only) and PC1 will be evaluated for ACLR and ACS - For NR-NTN and IoT-NTN: coexistence studies shall be carried out under the same conditions, including geographical separation between NTN UE and Terrestrial Networks (TN), as the co-existence studies in Rel-17 and Rel-18 - Specify high power UE (HPUE) for NR-NTN (Non-Terrestrial Networks) and IoT-NTN (NB-IoT and eMTC based NTN) in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario - Support UE with power class 2 (PC2, 26 dBm) that applies to both handheld and non-handheld based on the co-existence study. - Support other power classes based on the co-existence and feasibility study - Non-hand held UE is prioritized - Specify RF requirements for all considered power classes for NR-NTN and IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands - Necessary Tx requirements, at least including - Investigate and if necessary, specify the solution to address potential SAR (Specific Absorption Rate restriction) requirements for handheld UE - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary - Necessary Rx requirements, at least including - Specify the RSD requirements on the example bands, if necessary - Specify the necessary signaling to support the above objectives

- This is the first meeting of this NTN WID, we only focus on simulation parameter discussion and mainly show the difference from TR 38.863 PC3 UE NTN co-existence analysis.

- Discussion

- Following lists the simulation scenarios that are listed in TR 38.863 for information.

The aggressor and victim combinations are listed in Table 6.1-2.

Table 6.1-2: Aggressor and victim

| No. | Combination | Aggressor | Victim | Notes | Study Phase |
| --- | --- | --- | --- | --- | --- |
| 1 | TN with NTN | TN DL | NTN DL | Applicable for satellite operating in e.g. S-band, for e.g. coexistence with n1 FDD. | Phase 1 |
| 2 | TN with NTN | TN UL | NTN UL | Applicable for satellite operating in e.g. S-band, for e.g. coexistence with n1 FDD. | Phase 1 |
| 3 | TN with NTN | NTN DL | TN DL | Applicable for satellite operating in e.g. S-band, for e.g. coexistence with n1 FDD. | Phase 1 |
| 4 | TN with NTN | NTN UL | TN UL | Applicable for satellite operating in e.g. S-band, for e.g. coexistence with n1 FDD. | Phase 1 |
| 5 | TN with NTN | NTN UL | TN DL | Applicable for satellite operating in S-band, for e.g. coexistence with n34 TDD. | Phase 1 |
| 6 | TN with NTN | TN DL | NTN UL | Applicable for satellite operating in S-band, for e.g. coexistence with n34 TDD. | Phase 1 |
| 7 | NTN with NTN | NTN DL | NTN DL | HAPS-HAPS | Phase 2 |
|  |  | NTN UL | NTN UL | HAPS-HAPS | Phase 2 |

- According to the WID objective, RAN4 will only focus to evaluate the impact on the uplink performance of TN (terrestrial network), we can only focus on above scenario 4 that aggressor is NTN UL and victim is TN UL.

- Proposal 1: it’s suggested to focus on scenario 4 from NTN UL to TN UL as in TR 38.863.

- For the power class, now PC2, PC1.5 (NR-NTN only) and PC1 are all in the scope, they are suggested as the same high priority.

- Detailed ACLR/ACS assumption of TN system are listed as below

| - TN |  | - NR | - NB-IoT |
| --- | --- | --- | --- |
| - BS | - ACLR | - 45dB | - 40dB |
|  | - ACS | - 46dB | - 46dB |

- Proposal 2: for the TN BS ACLR and ACS are listed as above.

- For other simulation parameters of NR NTN, TN, IoT NTN, it’s suggested to use following as baseline.

- For co-existence between NR NTN and TN, use the parameter as in TR 38.863 for baseline

- For co-existence between IoT NTN and TN, use the parameter as in R4-2217473 for baseline

- Proposal 3: it’s suggested to use following parameter as baseline.

- For co-existence between NR NTN and TN, use the parameter as in TR 38.863 for baseline

- For co-existence between IoT NTN and TN, use the parameter as in R4-2217473 for baseline

- For the simulation in TR 38.863, there is isolation distance assumption and details are listed as below. And according to the WID, the coexistence studies shall be carried out under the same conditions, including geographical separation between NTN UE and Terrestrial Networks (TN), as the co-existence studies in Rel-17 and Rel-18.

NOTE: The NTN UE(s) shall be dropped at the edge of the “central 19 TN cells (cluster)”. For Case 1 (Urban scenario), an Isolation distance of 1500m as 2*ISD is considered to reflect the NTN-TN selection algorithm at the border, assuming TN connection will always be prioritized over NTN. As defined in Figure 6.2.1.1-1, isolation distance is the distance between the blue-dotted line which represents TN cell boarder and the red line. No UEs deployed in the isolation region is assumed to reduce the calculation complexity.

- Considering most simulation parameters are not changed compared with PC3, based on the TN co-existence analysis experience, it seems the the enhancement of NTN HPUE will not be that large and we can use 1dB as step to evaluate ACLR of NTN HPUE.

- Observation 1: we can use 1dB as ACLR enhancement step for NTN and IoT NTN HPUE.

# 3. Conclusions

In this contribution, co-existence parameter for NTN and IoT NTN are discussed with following observations and proposals.

- Proposal 1: it’s suggested to focus on scenario 4 from NTN UL to TN UL as in TR 38.863.

| - TN |  | - NR | - NB-IoT |
| --- | --- | --- | --- |
| - BS | - ACLR | - 45dB | - 40dB |
|  | - ACS | - 46dB | - 46dB |

- Proposal 2: for the TN BS ACLR and ACS are listed as above.

- Proposal 3: it’s suggested to use following parameter as baseline.

- For co-existence between NR NTN and TN, use the parameter as in TR 38.863 for baseline

- For co-existence between IoT NTN and TN, use the parameter as in R4-2217473 for baseline

- Observation 1: we can use 1dB as ACLR enhancement step for NTN and IoT NTN HPUE.

# 4. Reference

- RP-240857, New WID: Enhanced requirements and test methodology for NR and IoT NTN, RAN4 chair (Huawei)