3GPP TSG-RAN WG4 #112	R4-2411772

Maastricht, Netherlands, August 19-23, 2024

Agenda item:	5.19.2 Advanced receiver to cancel inter-user interference for MU-MIMO              demodulation requirements

Source: 	Qualcomm Incorporated

Title: 	MU-MIMO enhancement simulation results

Document for:	Discussion

# Introduction

In RAN4 #111, several companies provided alignment and impairment results for the MU-MIMO interference mitigation advanced receiver that are summarized in [1]. In this contribution, we provide our simulation results.

# Discussion

## Advanced MU-MIMO Receiver Simulations and Impairment Results

Our alignment and impairment for the MU-MIMO interference mitigation advanced receiver results are listed in the following:

| Case Number (New) | Duplex Mode and SCS | CHBW | Rank for target + Co-UE | MIMO | Precoder selection | Channel Model | Antenna correlation | MCS for the target UE (MCS Table 1) | Modulation order for the co-scheduled UE | DCI Index | Alignment |  | Impairment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | FDD 15KHz SCS | 10 MHz | 1+1 | 2T2R | Orthogonal | TDLC300-100 | ULA Medium | MCS 13 | QPSK | Index 1/2 | 13.8 | 16.3 |  |
| 2 |  |  |  | 2T4R |  |  |  |  |  |  | 13.7 | 16.6 |  |
| 3 |  |  | 2+2 | 4T4R |  | TDLA30-10 | ULA Low | MCS 17 | 16QAM |  | 16.7 | 19.2 |  |
| 4 | TDD 30KHz | 40 MHz | 1+1 | 2T2R |  | TDLC300-100 | ULA Medium | MCS 13 | QPSK |  | 14.6 | 17.1 |  |
| 5 |  |  |  | 2T4R |  |  |  |  |  |  | 14.2 | 17 |  |
| 6 |  |  | 2+2 | 4T4R |  | TDLA30-10 | ULA Low | MCS 17 | 16QAM |  | 16.5 | 19 |  |
| 9 | FDD 15 KHz SCS | 10MHz | 1+1 | 2T2R | Orthogonal | TDLC300-100 | ULA medium | MCS 17 | 16QAM | Index 6 | 21.1 | 23.6 |  |
| 10 |  |  |  | 2T4R |  |  |  |  |  |  | 21.0 | 23.5 |  |
| 13 |  |  | 2+2 | 4T4R |  | TDLA30-10 | ULA Low | MCS 13 | QPSK |  | 11.3 | 13.8 |  |
| 17 | TDD 30KHz SCS | 40 MHz | 1+1 | 2T2R | Orthogonal | TDLC300-100 | ULA medium | MCS 17 | 16QAM |  | 22.7 | 25.5 |  |
| 18 |  |  |  | 2T4R |  |  |  |  |  |  | 22.5 | 25.1 |  |
| 21 |  |  | 2+2 | 4T4R |  | TDLA30-10 | ULA Low | MCS 13 | QPSK |  | 12 | 14.5 |  |

# Conclusions

In this paper we presented our alignment and impairment for the MU-MIMO interference mitigation advanced receiver.

# References

- R4-2407250, Simulation result collection for MU-MIMO with advanced receiver, RAN4 #111, May 2024