3GPP TSG-RAN WG4 Meeting # 112																    R4-2411773

Maastricht, Netherlands, 19th – 23th Aug, 2024

Agenda Item: 8.1.1.3.2

Source: MediaTek Inc.

Title: Feasibility evaluation of MIMO layer for 6Rx UE

Document for: Discussion

# Introduction

The WI NR_ENDC_RF_Ph4 [1] was approved in RP#103 meeting with one objective “6Rx for handheld and FWA UE” within which one task to RAN4 is to study the gain and feasibility of 6 MIMO layers, targeting at support of handheld UEs.

6Rx for handheld and FWA UE - Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario - Example bands: n41, n77/n78, n79, n104 - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers - Specify the Rx requirements including reference sensitivity requirements for support 6Rx - Note: the specified requirements can be applicable to both handheld UE and FWA devices - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, t4r6 depending on UE capability - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution.

In RAN4#111 meeting, the discussions started. An WF [2] was agreed with the relevant part captured as below.

# Topic 3: MIMO layer evaluation for 6Rx UE ## Sub-topic 3-1: General considerations for MIMO layer evaluation for 6Rx UE Issue 3-1-1: Tightening BS EVM requirement Way Forward: RAN4 to further discuss if the BS EVM requirement needs to be tightened when discussing the feasibility of 6-Layer MIMO for 6Rx UEs. Issue 3-1-2: 6-Layer Performance Evaluation Assumptions Way Forward: RAN4 to further discuss the 6-Layer MIMO performance evaluation assumptions considering realistic antenna correlation assumptions and deployment scenarios. Issue 3-1-3: Whether to use same requirement for handheld UE and FWA Way Forward: RAN4 to defer decision after evaluation of 6 MIMO layer performance for handheld and FWA devices. ## Sub-topic 3-2: 6-layer Support Issue 3-2-1: 6-layer Support Agreement: - Decision on support of 6 MIMO layers for handheld is deferred until performance level differences in comparison with 4 MIMO layers are known based on evaluations considering realistic antenna correlation assumptions and deployment scenarios Issue 3-2-2: 6-layer Support as optional feature Way Forward: RAN4 to further discuss if 6-Layer support should be considered an optional feature. Issue 3-2-3: Release independence of 6-layer Support Way Forward: Follow RAN4 Chair guidance and defer any release independence discussions until the requirements are stable.

In this paper, we provided our views to above open issues as well as some observations based on system-level simulations.

# General issues

As indicated in the WID, this 6Rx objective focuses on handheld UE. In our understanding, the WID is written is this way because from 3GPP perspective we already have 8Rx CPE requirements defined in Rel-18. Therefore, it is not urgent for 3GPP to further pursue the requirement definitions of 6Rx CPEs. Instead, the current smartphone has only up to 4Rx. It seems more motivated to understand the benefit of 6Rx smartphone, rather than 6Rx CPE.

Proposal 1: Following the guidance of WID, the feasibility study of 6-layer MIMO for 6Rx should focus on handheld UEs rather than CPE devices.

# System-level simulation

In this section, we provide the system-level simulation results. The simulation assumptions are provided in Table 4 below.

Table 4

| Configuration | Setting | Note |
| --- | --- | --- |
| Scenarios | - UMa with ISD 500m - 7 sites (21 sectors), 10 UEs in each sector |  |
| UE distribution | - 80% indoor UE, UE speed 3km/h - 20% indoor UE, UE speed 30km/h | Further related analysis on channel aging effect in section 4.2. |
| Carrier freq | - 3.5GHz |  |
| Traffic Model | - FTP traffic model with resource utilizations (RU) rates - 30% - 70% | This leads to different interference level. Further analysis in section 4.1. |
| BS Tx setting | - Antenna number (M,N,P,Mg,Ng,Mp,Np) = (8,8,2,1,1, 2,8) for 32TXRU - BS EVM 0% | Other EVM values will be considered in section 4.5 |
| UE Rx setting | - UE antenna distance (dV, dH) = (0.3, 0.3) - MMSE receiver, w/o CE error | 0.3 for smartphone due to font-size limitation. Other value will be considered in section 4.4 |
| Transmission scheme | - Max MIMO layer: - Option 1) 4 - Option 2) 6 - SU-MIMO, 256QAM |  |
| PDSCH DMRS | - 1-symbol overhead for MIMO layer | With DMRS Type 1, 5 & 6 layers requires double DMRS. This issue will be discussed in section 4.3 |
| CSI | - CSI period 30ms, SRS period 10ms - Enable OLLA with target BLER=0.1 | Rank and PMI selection are based on SRS |

Based on above simulation assumptions, the results can be found in Table 5 with whole CDF curves of user throughput provided in Figure 4.

Table 5

| User throughput | RU 30% |  | RU 70% |  |
| --- | --- | --- | --- | --- |
|  | Up to 4L | Up to 6L | Up to 4L | Up to 6L |
| Average of all UEs | 5.01 | 4.85 (97%) | 3.46 | 3.22 (93%) |
| 5%-ile CDF (cell-edge UEs) | 1.39 | 1.35 (97%) | 0.9 | 0.85 (94%) |
| 50%-ile CDF | 4.21 | 4.03 (96%) | 2.81 | 2.65 (94%) |
| 95%-ile CDF (cell-center UEs) | 10.56 | 10.65 (101%) | 7.87 | 7.37 (94%) |

Figure 4. CDF curves for UE throughputs for different max MIMO layers and different RUs

From the results, we can see that allowing 6-layer DL transmission for 6Rx UEs does not provide obvious gain. In most of the cases, degradations can even be observed. We understand this may not look intuitive at the first glance. Therefore, we also conducted some further analyses below to better understand the reason of this observation.

For simplicity, we will use 4L and 6L to denote 4-layer MIMO and 6-layer MIMO, respectively. Also, we will use RU 30% for the remaining analyses.

## 4.1 Inter-site interference

First test we conducted is to remove all the inter-site interference, in order to know how interference affects the throughput performance of UEs with up to 4 and 6 layers. The results are shown in Table 6 and Figure 5 in which we compare the results with and without considering inter-site interference in both reporting CSI and determining the BLER of PDSCH. The simulation without inter-site interference is conducted based on a single-site simulation. When interference is considered, we can see that the 4L & 6L CDF curves are closed to each other. After removing the impact of interference, we can see up to 6L outperforms up to 4L in some cases. The reason is not only due to the interference lowers the SINR level but also the mismatch of the interference level between the time of CSI estimation and PDSCH decoding. With this test, we can observe that up to 6L is more sensitive to the dynamic of the interference.

Table 6

| User throughput | w/ interference (7 BS) |  | w/o interference (Single BS) |  |
| --- | --- | --- | --- | --- |
|  | Up to 4L | Up to 6L | Up to 4L | Up to 6L |
| Average of all UEs | 5.01 | 4.85(97%) | 6.58 | 6.65 (101%) |
| 5%-ile CDF (cell-edge UEs) | 1.39 | 1.35(97%) | 1.02 | 1.18 (116%) |
| 50%-ile CDF | 4.21 | 4.03(96%) | 6.19 | 6.09 (98%) |
| 95%-ile CDF (cell-center UEs) | 10.56 | 10.65(101%) | 13.1 | 14.34 (109%) |

Figure 5. CDF curves for UE throughputs for up to 4L and 6L, with and without considering the impact of inter-site interference

## 4.2 Channel aging effect

Second test we conducted is to understand whether 6 layer is more sensitive to channel aging effect. For comparison, we simply make the channels fixed once they are generated (i.e., Doppler = 0Hz). In this way, the channel UE used for CSI reporting will always be the same as the channel UE sees when decoding PDSCH. The results are provided in Table 7 and Figure 6. The observation is a bit similar to the previous test. Larger gain can be observed for up to 6L if the channels become fixed. With this test, we can observe that up to 6L is more sensitive to the channel aging effect.

Table 7

| User throughput | Time-varying channel |  | Fixed channel |  |
| --- | --- | --- | --- | --- |
|  | Up to 4L | Up to 6L | Up to 4L | Up to 6L |
| Average of all UEs | 5.01 | 4.85(97%) | 7.21 | 7.34 (102%) |
| 5%-ile CDF (cell-edge UEs) | 1.39 | 1.35(97%) | 2.46 | 2.48 (101%) |
| 50%-ile CDF | 4.21 | 4.03(96%) | 6.62 | 6.49 (98%) |
| 95%-ile CDF (cell-center UEs) | 10.56 | 10.65(101%) | 12.87 | 14.1 (110%) |

Figure 6. CDF curves for UE throughputs for up to 4L and 6L, time-varying channel and fixed channel

## 4.3 PDSCH DMRS overhead

Based on TS 38.211, the DMRS overheads are different for PDSCH with different MIMO layers. Taking Type-1 DMRS (which is the only type used in all RAN4 Demod test cases) as an example, as provided in Table 7.4.1.1.2-5 (v15.10.0) below, supporting 5 & 6 layers for PDSCH requires double-symbol DMRS, while supporting 1 to 4 layers requires only single-symbol DMRS. Considering 2 symbols occupied by PDCCH and 1 symbol occupied by single-symbol DMRS, there are 11 symbols left for PDSCH. With additional double-symbol DMRS for supporting 5 & 6 layer, only 10 symbols remaining for PDSCH. In summary it means extra 9% overhead for transmitting 5 & 6 layers. Figure 7 is plotted to illustrate the issue of this additional overhead.

Table 7.4.1.1.2-5: PDSCH DM-RS time index  and antenna ports .

| Single or double symbol DM-RS |  | Supported antenna ports |  |
| --- | --- | --- | --- |
|  |  | Configuration type 1 | Configuration type 2 |
| single | 0 | 1000 – 1003 | 1000 – 1005 |
| double | 0, 1 | 1000 – 1007 | 1000 – 1011 |

Figure 7. illustration of Type 1 DMRS overhead for MIMO layer ≤ 4 and >4.

The system-level simulation results are provided in Table 8 and Figure 8. After considering 2 DMRS symbols, we can see the throughput for up to 6L is further reduced (roughly 80% of the 4L throughput). Therefore, we can conclude that with Type-1 DMRS, it is even more difficult to see the benefit of 6L MIMO for 6Rx UEs.

Table 8

| User throughput | 1 DMRS symbol |  | 2 DMRS symbol |
| --- | --- | --- | --- |
|  | Up to 4L | Up to 6L | Up to 6L |
| Average of all UEs | 5.01 | 4.85(97%) | 4.13 (82%) |
| 5%-ile CDF (cell-edge UEs) | 1.39 | 1.35(97%) | 1.12 (81%) |
| 50%-ile CDF | 4.21 | 4.03(96%) | 3.43 (81%) |
| 95%-ile CDF (cell-center UEs) | 10.56 | 10.65(101%) | 9.05 (86%) |

Figure 8. CDF curves for UE throughputs for up to 4L with 1 DMRS symbol overhead and 6L with 1 or 2 DMRS symbol overhead

Note that here we did not consider any additional DMRS in the evaluation. If additional DMRS is considered, the number of available DMRS symbols will be reduced from 10 (for single DMRS) to 8 (for double DMRS). The extra overhead becomes 20%.

## 4.4 UE antenna distance

In previous sections, dH=0.3λ was used. Here, we tried dH=0.5λ to see if the performance can be improved. This trail may be treated as a related evaluation to UE antenna correlation, but not 100% the same. Table 9 and Figure 9 provide the corresponding results. With larger UE antenna distance, we can see throughput improvement in both 4L and 6L cases. However, the relative performance difference between 4L and 6L remains the same, even if UE antenna distance increases.

Table 9

| User throughput | dH=0.3λ |  | dH=0.5λ |  |
| --- | --- | --- | --- | --- |
|  | Up to 4 L | Up to 6L | Up to 4L | Up to 6L |
| Average of all UEs | 5.01 | 4.85(97%) | 6.82 | 6.57 (96%) |
| 5%-ile CDF (cell-edge UEs) | 1.39 | 1.35(97%) | 2.38 | 2.29 (96%) |
| 50%-ile CDF | 4.21 | 4.03(96%) | 6.11 | 5.61 (92%) |
| 95%-ile CDF (cell-center UEs) | 10.56 | 10.65(101%) | 12.62 | 13.2 (105%) |

Figure 9. CDF curves for UE throughputs for up to 4L and 6L with different antenna distance

## 4.5 BS EVM

In this section, we compare the throughput with BS EVM 0% and 3.5%. After considering more realistic BS EVM assumption, we can see some throughput degradation for both 4L and 6L (but not big). However, the relative performance difference between 4L and 6L remains the same. In summary, BS EVM 0% and 3.5% have similar performance. The main reason could be that majority of the UEs operate at SNR<30dB.

| User throughput | BS EVM 0% |  | BS EVM 3.5% |  |
| --- | --- | --- | --- | --- |
|  | Up to 4L | Up to 6L | Up to 4L | Up to 6L |
| Average of all UEs | 5.01 | 4.85(97%) | 4.88 | 4.64 (97%) |
| 5%-ile CDF (cell-edge UEs) | 1.39 | 1.35(97%) | 1.36 | 1.35 (99%) |
| 50%-ile CDF | 4.21 | 4.03(96%) | 4.15 | 3.91 (94%) |
| 95%-ile CDF (cell-center UEs) | 10.56 | 10.65(101%) | 10.27 | 9.97 (97%) |

## 4.5 Conclusion of system-level simulations

From all the evaluations above, we provide the following summary of observations:

- From system-level perspective, allowing max 6-layer MIMO for 6Rx UE does not bring expected gain over the baseline with max 4-layer MIMO.

- Max 6-layer MIMO suffers more degradation to the dynamic inter-site interference than max 4-layer MIMO

- Max 6-layer MIMO suffers more degradation to channel aging effect than max 4-layer MIMO

- Considering Type-1 DMRS, max 6-layer MIMO requires higher DMRS overhead compared to max 4-layer MIMO, leading to up to 18% lower throughput than max 4-layer MIMO.

- Changing UE antenna distance from 0.3λ to 0.5λ does not make max 6-layer MIMO outperform max 4-layer MIMO.

- No big performance difference between BS EVM 0% and 3.5%, because the majority of the UEs operate under SNR<30dB.

Observation 1: Observations from system-level simulation

- Allowing max 6-layer MIMO for 6Rx UE does not bring expected gain over the baseline with max 4-layer MIMO.

- Max 6-layer MIMO suffers more degradation to the dynamic inter-site interference than max 4-layer MIMO

- Max 6-layer MIMO suffers more degradation to channel aging effect than max 4-layer MIMO

- Considering Type-1 DMRS, max 6-layer MIMO requires higher DMRS overhead compared to max 4-layer MIMO, l leading to up to 18% lower throughput than max 4-layer MIMO.

- Changing UE antenna distance from 0.3λ to 0.5λ does not make max 6-layer MIMO outperform max 4-layer MIMO

- No big performance difference between BS EVM 0% and 3.5%, because the majority of the UEs operate under SNR<30dB

There could be some other scenarios or assumptions to be further considered in the system-level simulation. We are open to discuss other options and further align the simulation assumptions/results with companies.

Proposal 2: RAN4 to further discuss the system-level simulation assumptions for the feasibility study of 6-layer MIMO for 6Rx handheld UEs.

# Summary

In this paper, we provided our view on the feasibility of MIMO layer for 6Rx UE. We have the following observations and proposal.

Proposal 1: Following the guidance of WID, the feasibility study of 6-layer MIMO for 6Rx should focus on handheld UEs rather than CPE devices.

Observation 1: Observations from system-level simulation

- Allowing max 6-layer MIMO for 6Rx UE does not bring expected gain over the baseline with max 4-layer MIMO.

- Max 6-layer MIMO suffers more degradation to the dynamic inter-site interference than max 4-layer MIMO

- Max 6-layer MIMO suffers more degradation to channel aging effect than max 4-layer MIMO

- Considering Type-1 DMRS, max 6-layer MIMO requires higher DMRS overhead compared to max 4-layer MIMO, leading to up to 18% lower throughput than max 4-layer MIMO.

- Changing UE antenna distance from 0.3λ to 0.5λ does not make max 6-layer MIMO outperform max 4-layer MIMO

- No big performance difference between BS EVM 0% and 3.5%, because the majority of the UEs operate under SNR<30dB

Proposal 2: RAN4 to further discuss the system-level simulation assumptions for the feasibility study of 6-layer MIMO for 6Rx handheld UEs.

# Reference

[1] RP-240828, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, RAN4 chair (Huawei)

[2] R4-2410751, WF on requirements for 6Rx, AT&T

[3] R4-080620, High Dimension E-UTRA MIMO Correlation matrixes, Ericsson