3GPP TSG-RAN WG4 Meeting # 112								    R4-2411774

Maastricht, Netherlands, 19th – 23th Aug, 2024

Agenda Item: 8.1.1.3.4

Source: MediaTek Inc.

Title: UE SRS IL imbalance issue for 6Rx UE

Document for: Discussion

# 1	Introduction

In RANP#103, the WI [1] was agreed with one objective for 6Rx. One of the objectives is to study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution.

6Rx for handheld and FWA UE - Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario - Example bands: n41, n77/n78, n79, n104 - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers - Specify the Rx requirements including reference sensitivity requirements for support 6Rx - Note: the specified requirements can be applicable to both handheld UE and FWA devices - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, t4r6 depending on UE capability - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution.

In RAN4#111 meeting, the issue of 6Rx insertion loss (IL) was discussed. A WF [2] was agreed with the relevant parts captured below:

# Topic 4: SRS IL imbalance issue ## Sub-topic 4-1: General considerations for SRS IL imbalance issue Issue 4-1-2: Initial Considerations for SRS IL imbalance issue Way forward: Given the different views amongst companies, RAN4 to further discuss the set of initial considerations which will allow companies to have a common understanding for the study including existing UE behavior for SRS transmissions in case of SRS IL. - Companies are encouraged to bring analysis on the existing UE behavior and achievable power imbalance for SRS transmissions based on current specification and UE implementations in case of SRS IL. - Companies are encouraged to analyse the impact of SRS IL imbalance on NW performance degradation. ## Sub-topic 4-2: SRS IL imbalance issue solutions Issue 4-2-1: Candidate solutions for the SRS IL imbalance issue Way forward: After determining the set of initial considerations for the study, discuss candidate solutions considering the UE behaviour in terms of the IL imbalance compensation and whether the UE can compensate the IL imbalance for AS-SRS for all power levels and whether NW and UE have consistent understanding for the possible compensation.

In this paper, we provide our views for above issues.

# 2	Discussion

## 2.1	Impact of different compensation schemes to be BS SRS channel estimation quality

TS38.213 does not capture any IL in the power control equation. One interpretation is that equal transmit power is to be used on each SRS antenna port within an SRS resource set. On the other hand, TS38.101-1 specifies ∆TRxSRS for configured maximum output power requirements to allow for some deviation from the maximum configured output power for the UE when transmitting SRS. Although the 2 specs seem a bit contradicting, our understanding is that they allow UE to conduct self-compensation on the transmission for individual SRS antenna port.

The additional IL, if not compensated, may lead to inaccuracy in gNB channel estimation on the SRS, leading to the degradation in the DL performance due to sub-optimal DL precoder selection. For example, as shown in Figure 1, if UE’s transmit power is reduced by 3dB due to IL, it means gNB’s channel estimation needs to be carried out under a 3dB worse SNR condition, resulting to larger channel estimation error.

Figure 1. Effect of IL to the SRS channel estimation at BS

Observation 1: Additional UE transmit power reduction due to IL, if not compensated, will degrade DL performance due to lower SNR in gNB channel estimation on SRS for determining DL precoder.

To mitigate the degradation, many solutions were suggested by companies, e.g., pre/post-compensation by UE or gNB.

- Pre-compensation by UE is more effective, because it can increase the SNR at gNB directly. gNB can estimate the channel at a higher SNR condition and make more accurate decision in DL precoder selection. For example in Figure 2(a), if UE can fully pre-compensate the IL, the channel estimation error can be reduced (compared to Figure 1). Note that even if UE cannot fully compensate the IL as in Figure 2(b) because of Tx power limit, the amount that UE compensated can still help to increase the SNR at gNB.

Figure 2. Effect of UE pre-compensation to the SRS channel estimation error at BS

- Post-compensation by gNB means that gNB still needs to conduct the channel estimation under worse SNR condition. Therefore, the estimated channel error is already worse than the main antenna. After the channel estimation is done, gNB up-scales the estimated channel to post-compensate the loss. This up-scaling will further amplify the errors in the channel estimation, which may not help much in making the DL precoder selection, as illustrated in Figure 3.

Figure 3. Effect of BS post-compensation to the SRS channel estimation error at BS

As mentioned by several companies, compensation by UE may not work when UE already reached its power limit. Nevertheless, this solution is still a more effectively approach to help mitigating the DL performance degradation. Based on above analysis, we suggest taking UE-side pre-compensation as the baseline approach when UE haven’t reached its Tx power limit.

Observation 2: Pre-compensation by UE can directly improve the SNR condition at gNB, while post-compensation by gNB may amplify the errors in the channel estimation.

Proposal 1: To handle the SRS IL issue, RAN4 should take UE pre-compensation as the baseline solution, when UE haven’t reached its Tx power limit.

With Proposal 1, the remaining issue is to discuss whether and what to do when UE cannot fully pre-compensate the IL. We are open to discuss if any solutions are needed. In our view, some SLS work is needed to justify the gain of different proposals. In the next section, we provide some evaluations as an example.

Proposal 2: RAN4 to further discuss whether a solution is needed when UE cannot fully pre-compensate the SRS IL.

## 2.2	Performance analysis (new)

We have performed system-level performance to compare the DL throughput performance for different schemes. The assumption for IL values are [0, 3, 4.5, 4.5, 7.5, 7.5] for individual SRS ports. The following cases are studied:

- no IL: Zero IL, as the performance upper bound for benchmarking

- w/ IL: With IL but doing nothing, as the performance lower bound for benchmarking

- w/ IL (UE-c): With IL and UE pre-compensation

- w/ IL (BS-c): With IL and BS post-compensation

- w/ IL (UE/BS-c): With IL and both UE and BS compensation

- Type-I: Rank, precoder and MCS decided by Type-I CSI feedback, rather than SRS.

Simulation assumptions can be found in the Annex of this document, and the results are shown in the Figures 4 and 5 for resource utilization (RU) rate 30% and RU 70%, respectively. Note that in each figure,

- All throughputs are normalized to ‘no IL’

- Besides the overall average throughput, the throughputs for different %-ile UEs are also provided. The intention is to understand the potentially different observations for cell-edge UEs (5%-ile), mid-range UEs (50%-ile) and cell-center UEs (95%-ile), respectively.

Figure 4. Throughput performance for different cases under RU 30%

Figure 4. Throughput performance for different cases under RU 70%

From Figures 4 and 5, the following can be observed:

- No solution seems able to fully recover the ideal “no IL” performance.

- In general, we can see that UE pre-compensation is always better than BS post-compensation in all cases. In our understanding, the main reason is that UE pre-compensation can minimize the channel estimation error at BS.

- Even with compensation at both UE and BS (i.e., UE pre-compensate first, and BS post-compensate the remaining IL), the performance is not sufficiently better than UE pre-compensation only.

- At cell edge (UE is operating at close to maximum output power), all compensation-based methods have degradation, comparing to ‘no IL’. Instead, it is better to use “Type I CSI”, instead of “SRS-based CSI”.

Observation 3: From the system-level simulation results, UE pre-compensation is always better than BS post-compensation in all cases.

Observation 4: From the system-level simulation results, using ‘Type-I CSI’ can achieve better performance than ‘SRS-based CSI’ for cell edge UEs.

# 3	Conclusion

The following observations and proposals are made:

Observation 1: Additional UE transmit power reduction due to IL, if not compensated, will degrade DL performance due to lower SNR in gNB channel estimation on SRS for determining DL precoder.

Observation 2: Pre-compensation by UE can directly improve the SNR condition at gNB, while post-compensation by gNB may amplify the errors in the channel estimation.

Proposal 1: To handle the SRS IL issue, RAN4 should take UE pre-compensation as the baseline solution, when UE haven’t reached its Tx power limit.

Proposal 2: RAN4 to further discuss whether a solution is needed when UE cannot fully pre-compensate the SRS IL.

Observation 3: From the system-level simulation results, UE pre-compensation is always better than BS post-compensation in all cases.

Observation 4: From the system-level simulation results, using ‘Type-I CSI’ can achieve better performance than ‘SRS-based CSI’ for cell edge UEs.

# 4	References

RP-240828, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, RAN4 chair (Huawei)

R4-2410751, WF on requirements for 6Rx, AT&T

R4-2407699, UE SRS IL imbalance for 6Rx UE, MediaTek inc.

# Annex: Simulation assumptions

The differences to our previous contribution [3] are highlighted.

| Parameter | Value |
| --- | --- |
| Evaluation configuration | Configuration A from IMT-2020 evaluations |
| Channel model | UMa_A |
| ISD | 500 m |
| Frame structure | DSUDD |
| Carrier Frequency | 3.5 GHz |
| System bandwidth | 20MHz |
| Subcarrier spacing | 30KHz |
| Number of TXRU per TRxP | 32 TXRUs |
| Number of antenna elements per TRxP | (M,N,P,Mg,Ng,Mp,Np) = (8,8,2,1,1, 2,8) for 32TXRU |
| Transmit power per TRxP | 43 dBm |
| TRxP number per site | 3 |
| Mechanic tilt | 90deg in GCS (pointing to the horizontal direction) |
| Electronic tilt | 102deg in LCS |
| UE power class | PC2 26dBm |
| Number of antenna elements per UE | 6Rx, 0°,90° polarization antenna (dH, dV) = (0.3λ, 0.3λ) (M,N,P,Mg,Ng,Mp,Np) = (1,3,2,1,1, 1,3) for 6Rx |
| UT attachment | Based on RSRP (Eq. (8.1-1) in TR 36.873) from port 0 |
| Scheduling | PF scheduler |
| ACK/NACK delay | Next available UL slot |
| MIMO mode | SU-MIMO with rank 4 adaptation and OLLA per user |
| BS/MS receiver type | MMSE-IRC |
| Channel estimation | realistic |