3GPP TSG-RAN WG4 Meeting # 112								    R4-2411775

Maastricht, Netherlands, 19th – 23th Aug, 2024

Agenda Item: 8.2.4.1

Source: MediaTek Inc.

Title: Coexistence study for 14800 to 15350 MHz

Document for: Discussion

# 1	Introduction

Follow the Rel-19 SI [1], RAN4 continues the evaluation of deployment parameters for 14800 to 15350 MHz. In RAN4#111 meeting, a WF [2] was agreed with the relevant agreements captured below:

# 4 IMT parameters for 14800 to 15350 MHz frequency range Issue 3-1: Simulation scenarios - 1st priority urban macro - 2nd priority indoor - 3rd priority dense urban Issue 3-3: ISD - For indoor, agree 20m - For outdoor, consider both 450m (1st priority) and 350 (2nd priority) until August. - Also other ISD not precluded as 3rd priority than 450 and 350. Issue 3-4: Percentage indoor users for urban macro - Keep both options Issue 3-5: Co-ordinated and un-coordinated for outdoor - Both co-ordinated and un-coordinated Issue 3-6: Co-ordinated and un-coordinated for indoor - Only co-ordinated Issue 3-7: Pathloss model - Follow 38.901 Issue 3-6: BS antenna array size Per polarization, for simulation only: - 2048 (Sub Array size 8) - 1536 (Sub Array size 4) - Array size and Sub Array size for the response will be decided later, taking into account feasibility Issue 3-7: BS antenna array other parameters - 0.7 spacing in vertical domain is for simulation assumptions, double check for reply Issue 3-8: UE type - For simulations, consider both options. - FR1 like - RX diversity gain: [0 or 5] dB assuming 4RX - TX: 0dBi omnidirectional - Max Power: 23 dBm, 26dBm - FR2 like - Two panels (one in each direction) 2x2 antenna - 5dBi element gain. Array gain comes on top - Power: 23 dBm as max TRP Issue 3-9: BS output power - Assume 43dBm BS power for simulations Issue 3-10: Bandwidth - For number of UEs per slot: - 1 UE per slot as 1st priority - 3 UE per slot as 2nd priority Issue 3-11: BS noise factor - For simulations, assume 8dB Issue 3-11: UE noise factor - 11dB for simulations. Actual noise factor for reply will be decided based on feasibility. Issue 3-12: UL SNR target - 15dB

In this paper, we focus on the coexistence study for 14800 to 15350 MHz based on the [2] and TR 38.922.

# 2	Simulation assumption

For the coexistence study, we follow the simulation step in clause 5.3 of TR 38.803 with extended AAS model for BS (which is also defined in clause 6.1.2.3.2.4 of TR 38.922).

At the UE side, as captured in WF [2], two different assumptions (FR1-like and FR2-like) were both agreed for further evaluations. For FR2-like case, we can simply follow the current clause 6.1.2.3.3.2 in TR 38.922 for the single antenna element pattern with 5dBi max directional gain and then combine this into a composite array radiation pattern for 4Tx or 4Rx UE according to Table 6.1.2.3.2.4 of TR 38.922. The horizontal radiation pattern at θ″=90° and vertical radiation pattern at ϕ″=0° are provided in Figure 1 for quick reference. Same radiation pattern formulations are used for both DL reception and UL transmission.

(a)                                       (b)

Figure 1. FR2-like UE composite array radiation pattern: (a) horizontal radiation pattern at θ″=90° and (b) vertical radiation pattern at ϕ″=0°

However, some more discussions are needed for FR1-like case.

## 2.1	FR1-like UE antenna patterns

For FR1-like architecture, firstly we assume omnidirectional antennas. Therefore, the gains to be applied for desired signal can be assumed the same for all angle of arrivals (AoAs). Same rule can also be applicable to undesired signals, i.e., co-channel interference or adjacent channel interference.

For the exact gain to be applied, current WF assumes [0 to 5] dB gain for Rx diversity with 4Rx. In our understanding, how the digital Rx beamforming is done at the UE side is assumed to be similar to estimate the channel coefficients based on the reference signals on individual receive antennas, and then multiply the received signals to be decoded with the conjugates of the estimated channel coefficients (aka max ratio combing, MRC). Theoretically, we need to obtain the small-scale fading coefficients to determine the exact gain. However, since coexistence study focuses on a simplified approach which mainly considers only large-scale fading effect, we think it is fair to consider a 6dB long-term average MRC gain. After considering some implementation margin, we will adopt 5dB in our later evaluations.

Note that MRC coefficients are only calculated based on the desired signals. As for co-channel and adjacent channel interference, this 5dB gain should not be applied. The exact gain also depends on small-scale fading coefficients. Therefore, to simplify the evaluation, we will assume 0dB gain for co-channel and adjacent channel interference for now.

In summary, for DL-side 5dB gain is considered for UE receive signals from its serving cell regardless the AoA, while 0dB is considered for signals from all non-serving co-channel and adjacent-channel neighbouring cells regardless the AoA.

Same rule can also be used for Tx transmission. The conjugates of the channel coefficient estimated from DL reference signals can also be used as the precoder coefficients for UE’s UL transmission. One potential points is whether to consider additional margin for this beam correspondence implementation. At this moment, for the coexistence study, we simply assume the same values as DL. In summary, for UL-side 5dB gain is considered for UE transmit signals toward its serving cell regardless the AoA, while 0dB is considered for signals toward all non-serving co-channel and adjacent-channel neighbouring cells regardless the AoA

Proposal 1: FFS to consider the UE FR1-like antenna radiation pattern:

- DL 4Rx: 5dB gain for UE receive signals from its serving cell regardless the AoA, and 0dB for signals from all non-serving co-channel and adjacent-channel neighbouring cells regardless the AoA

- UL 4Tx: 5dB gain for UE transmit signals toward its serving cell regardless the AoA, and 0dB for signals toward all non-serving co-channel and adjacent-channel neighbouring cells regardless the AoA.

## 2.2	Inconsistent assumption in TR 38.922

In this section, we point out some inconsistencies in the current TR 38.922. Discussions are needed to further align the parameters in TR if RAN4 wants to further align the simulation results.

- In Table 6.1.2.3.2.4-3 the element gain for Indoor case is 5dBi, while in Table 6.1.2.3.2.3-1 the gain is 5.5dBi.

| Parameter | Indoor | Urban macro | Dense urban |
| --- | --- | --- | --- |
| Element gain (dBi) (Note 2) | 5 | 6.4 | 6.4 |

| Parameter | Values |
| --- | --- |
| Maximum directional gain of an antenna element GE,max | 5.5 dBi |

- In Table 6.1.2.3.2.4-3 the front-to-back ratio for indoor is 30dB, while in Table 6.1.2.3.2.3-1 the values are 25dB.

| Parameter | Indoor | Urban macro | Dense urban |
| --- | --- | --- | --- |
| Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V | 30 for both H/V | 30 for both H/V |

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) | AE,V(θ″)=-min12θ″-90°θ3dB2,SLAV,θ3dB=90°,SLAV=25dB |
| Antenna element horizontal radiation pattern (dB) | AE,H(ϕ″)=-min12ϕ″ϕ3dB2,Am,ϕ3dB=90°,Am=25dB |

- In Table 6.1.2.3.2.4-3 the 3dB beamwidth are 90o for H and 65o for V for Urban marco, while in Table 6.1.2.3.2.1-1 and Table 6.1.2.3.2.2-1 they are 90o for V and 65o for H.

| Parameter | Indoor | Urban macro | Dense urban |
| --- | --- | --- | --- |
| Horizontal/vertical 3 dB beam width of single element (degree) | 90º for H 90º for V | 90º for H 65º for V | 90º for H 65º for V |

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) | AE,V(θ″)=-min12θ″-90°θ3dB2,SLAV,θ3dB=90°,SLAV=30dB |
| Antenna element horizontal radiation pattern (dB) | AE,H(ϕ″)=-min12ϕ″ϕ3dB2,Am,ϕ3dB=65°,Am=30dB |

- In Table 6.1.2.3.2.4-3 the notation used for “Antenna sub-array configuration (Row × Column)” does not match the description in Note 4.

  - In the row of Antenna sub-array configuration, the unit is ‘element’.

  - In Note 4, the unit is ‘sub-array’.

| Parameter | Indoor | Urban macro | Dense urban |
| --- | --- | --- | --- |
| Antenna sub-array configuration (Row × Column) (Note 4) | [8 x 8 / 4 x 4] elements | 64 x 24 / 64 x 32 elements | 16 x 24 / 64 x 32 elements |
| Note 4: 64 × 24/32 means there are 64 vertical and 24/32 horizontal radiating sub-arrays. |  |  |  |

We suggest RAN4 to discuss the above-mentioned parameters and align them in the TR.

Proposal 2: RAN4 to further align the inconsistency in TR38.922

  - Antenna element gain (dBi) for Indoor case in Table 6.1.2.3.2.4-3 and Table 6.1.2.3.2.3-1

  - Front-to-back ratio of H and V for Indoor case in Table 6.1.2.3.2.4-3 and Table 6.1.2.3.2.3-1

  - 3dB beamwidths of H and V in Table 6.1.2.3.2.4-3 is different from those in Table 6.1.2.3.2.1-1 and Table 6.1.2.3.2.2-1 for or Urban marco and Dense Urban, respectively

  - The unit used in Antenna sub-array configuration and Note 4 in Table 6.1.2.3.2.4-3

# 3	Coexistence simulation results

In this section, we provide our preliminary evaluation results for UMa and Indoor.

## 3.1 UMa

The evaluation results for throughput loss with different ACIR levels for UMa are provided first in below tables.

| UMa, ISD450m, Coordinated Network, BS ANT# 16x24x4x2 (1536x2) |  |  |
| --- | --- | --- |
| DL | Cell average | ACIR [dB]; 5; 10; 15; 20; 25; 30; 35; 40. FR2-like; 3.21; 1.80; 0.98; 0.50; 0.20; 0.05; 0.02; 0.01. FR1-like; 2.23; 1.18; 0.59; 0.24; 0.08; 0.03; 0.01; 0.0. |
|  | DL 5%-tile | ACIR [dB]; 5; 10; 15; 20; 25; 30; 35; 40. FR2-like; 10.28; 4.60; 3.43; 2.22; 1.28; 0.91; 0.44; 0.02. FR1-like; 12.94; 4.42; 0.17; 0.15; 0.14; 0.05; 0.03; 0.0. |
| UL | Cell average | ACIR [dB]; 5; 10; 15; 20; 25; 30; 35; 40. FR2-like; 1.30; 0.65; 0.28; 0.11; 0.04; 0.01; 0.0; 0.0. FR1-like; 0.75; 0.32; 0.12; 0.04; 0.02; 0.0; 0.0; 0.0. |
|  | UL 5%-tile | ACIR [dB]; 5; 10; 15; 20; 25; 30; 35; 40. FR2-like; 4.18; 1.92; 0.86; 0.36; 0.07; 0.0; 0.0; 0.0. FR1-like; 4.70; 2.26; 0.05; 0.04; 0.0; 0.0; 0.0; 0.0. |

| UMa, ISD450m, Coordinated Network, BS ANT# 8x32x8x2 (2048x2) |  |  |
| --- | --- | --- |
| DL | Cell average | ACIR [dB]; 5; 10; 15; 20; 25; 30; 35; 40. FR2-like; 3.21; 1.80; 0.98; 0.50; 0.20; 0.05; 0.02; 0.01. FR1-like; 2.27; 1.21; 0.61; 0.25; 0.08; 0.03; 0.01; 0.0. |
|  | DL 5%-tile | ACIR [dB]; 5; 10; 15; 20; 25; 30; 35; 40. FR2-like; 10.28; 4.60; 3.43; 2.22; 1.28; 0.91; 0.44; 0.02. FR1-like; 3.74; 1.47; 0.81; 0.06; 0.01; 0.0; 0.0; 0.0. |
| UL | Cell average | ACIR [dB]; 5; 10; 15; 20; 25; 30; 35; 40. FR2-like; 1.30; 0.65; 0.28; 0.11; 0.04; 0.01; 0.0; 0.0. FR1-like; 0.75; 0.32; 0.12; 0.04; 0.02; 0.0; 0.0; 0.0. |
|  | UL 5%-tile | ACIR [dB]; 5; 10; 15; 20; 25; 30; 35; 40. FR2-like; 4.18; 1.92; 0.86; 0.36; 0.07; 0.0; 0.0; 0.0. FR1-like; 2.17; 0.21; 0.01; 0.0; 0.0; 0.0; 0.0; 0.0. |

The corresponding CDF curves of the cell throughput loss are also provided in below figures for reference.

| DL | Cell average |  |
| --- | --- | --- |
|  | DL 5%-tile |  |
| UL | Cell average |  |
|  | UL 5%-tile |  |

## 3.2 Indoor

The evaluation results for throughput loss with different ACIR levels for Indoor are provided first in below tables.

| Indoor, ISD20m, Coordinated Network, BS ANT# 4x4x2 |  |  |
| --- | --- | --- |
| DL | Cell average | ACIR [dB]; 5; 10; 15; 20; 25; 30; 35; 40. FR2-like; 26.97; 16.99; 9.68; 4.88; 2.04; 0.58; 0.15; 0.05. FR1-like; 20.47; 11.38; 5.56; 2.34; 0.78; 0.24; 0.07; 0.02. |
|  | DL 5%-tile | ACIR [dB]; 5; 10; 15; 20; 25; 30; 35; 40. FR2-like; 30.16; 8.02; 2.26; 0.52; 0.11; 0.04; 0.03; 0.01. FR1-like; 11.85; 3.05; 0.80; 0.23; 0.09; 0.03; 0.01; 0.0. |
| UL | Cell average | ACIR [dB]; 5; 10; 15; 20; 25; 30; 35; 40. FR2-like; 15.60; 7.96; 3.46; 1.30; 0.44; 0.15; 0.05; 0.01. FR1-like; 9.85; 4.20; 1.56; 0.53; 0.17; 0.05; 0.02; 0.01. |
|  | UL 5%-tile | ACIR [dB]; 5; 10; 15; 20; 25; 30; 35; 40. FR2-like; 26.97; 6.06; 1.37; 0.20; 0.03; 0.0; 0.0; 0.0. FR1-like; 13.26; 3.41; 1.21; 0.43; 0.15; 0.05; 0.01; 0.0. |

| Indoor, ISD20m, Coordinated Network, BS ANT# 8x8x2 |  |  |
| --- | --- | --- |
| DL | Cell average | ACIR [dB]; 5; 10; 15; 20; 25; 30; 35; 40. FR2-like; 15.15; 9.27; 5.24; 2.76; 1.15; 0.26; 0.06; 0.02. FR1-like; 12.85; 7.11; 3.55; 1.45; 0.39; 0.10; 0.03; 0.01. |
|  | DL 5%-tile | ACIR [dB]; 5; 10; 15; 20; 25; 30; 35; 40. FR2-like; 41.87; 17.60; 2.84; 0.99; 0.12; 0.02; 0.02; 0.02. FR1-like; 15.35; 3.83; 0.81; 0.10; 0.05; 0.02; 0.02; 0.01. |
| UL | Cell average | ACIR [dB]; 5; 10; 15; 20; 25; 30; 35; 40. FR2-like; 7.68; 3.99; 1.77; 0.68; 0.23; 0.08; 0.02; 0.01. FR1-like; 5.09; 2.20; 0.83; 0.09; 0.03; 0.01; 0.0; 0.0. |
|  | UL 5%-tile | ACIR [dB]; 5; 10; 15; 20; 25; 30; 35; 40. FR2-like; 35.47; 12.02; 1.65; 0.06; 0.03; 0.0; 0.0; 0.0. FR1-like; 11.89; 3.20; 0.99; 0.33; 0.11; 0.08; 0.01; 0.0. |

The corresponding CDF curves of the cell throughput loss are also provided in below figures for reference.

| DL | Cell average |  |
| --- | --- | --- |
|  | DL 5%-tile |  |
| UL | Cell average |  |
|  | UL 5%-tile |  |

# 4	Conclusion

In this paper, we provide our views on the assumptions for FR1-like UE antenna pattern, point out some inconsistency in current TR 38.922 and also our preliminary coexistence evaluation results. The following observations and proposals are made:

Proposal 1: FFS to consider the UE FR1-like antenna radiation pattern:

- DL 4Rx: 5dB gain for UE receive signals from its serving cell regardless the AoA, and 0dB for signals from all non-serving co-channel and adjacent-channel neighbouring cells regardless the AoA

- UL 4Tx: 5dB gain for UE transmit signals toward its serving cell regardless the AoA, and 0dB for signals toward all non-serving co-channel and adjacent-channel neighbouring cells regardless the AoA.

Proposal 2: RAN4 to further align the inconsistency in TR38.922

  - Antenna element gain (dBi) for Indoor case in Table 6.1.2.3.2.4-3 and Table 6.1.2.3.2.3-1

  - Front-to-back ratio of H and V for Indoor case in Table 6.1.2.3.2.4-3 and Table 6.1.2.3.2.3-1

  - 3dB beamwidths of H and V in Table 6.1.2.3.2.4-3 is different from those in Table 6.1.2.3.2.1-1 and Table 6.1.2.3.2.2-1 for or Urban marco and Dense Urban, respectively

  - The unit used in Antenna sub-array configuration and Note 4 in Table 6.1.2.3.2.4-3

# 5	References

RP-241326, Revised SID: Study on IMT parameters for NR for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz, Ericsson

R4-2410741, WF on IMT parameter study, Ericsson