3GPP TSG-RAN WG4 Meeting # 112								    R4-2411776

Maastricht, Netherlands, 19th – 23th Aug, 2024

Agenda Item: 8.2.4.2

Source: MediaTek Inc.

Title: Coverage study for 14800 to 15350 MHz

Document for: Discussion

# 1	Introduction

Follow the Rel-19 SI [1], RAN4 continues the evaluation of deployment parameters for 14800 to 15350 MHz. In RAN4#111 meeting, a WF [2] was agreed with the relevant agreements captured below:

# 4 IMT parameters for 14800 to 15350 MHz frequency range Issue 3-1: Simulation scenarios - 1st priority urban macro - 2nd priority indoor - 3rd priority dense urban Issue 3-3: ISD - For indoor, agree 20m - For outdoor, consider both 450m (1st priority) and 350 (2nd priority) until August. - Also other ISD not precluded as 3rd priority than 450 and 350. Issue 3-4: Percentage indoor users for urban macro - Keep both options Issue 3-5: Co-ordinated and un-coordinated for outdoor - Both co-ordinated and un-coordinated Issue 3-6: Co-ordinated and un-coordinated for indoor - Only co-ordinated Issue 3-7: Pathloss model - Follow 38.901 Issue 3-6: BS antenna array size Per polarization, for simulation only: - 2048 (Sub Array size 8) - 1536 (Sub Array size 4) - Array size and Sub Array size for the response will be decided later, taking into account feasibility Issue 3-7: BS antenna array other parameters - 0.7 spacing in vertical domain is for simulation assumptions, double check for reply Issue 3-8: UE type - For simulations, consider both options. - FR1 like - RX diversity gain: [0 or 5] dB assuming 4RX - TX: 0dBi omnidirectional - Max Power: 23 dBm, 26dBm - FR2 like - Two panels (one in each direction) 2x2 antenna - 5dBi element gain. Array gain comes on top - Power: 23 dBm as max TRP Issue 3-9: BS output power - Assume 43dBm BS power for simulations Issue 3-10: Bandwidth - For number of UEs per slot: - 1 UE per slot as 1st priority - 3 UE per slot as 2nd priority Issue 3-11: BS noise factor - For simulations, assume 8dB Issue 3-11: UE noise factor - 11dB for simulations. Actual noise factor for reply will be decided based on feasibility. Issue 3-12: UL SNR target - 15dB

In this paper, we focus on the coverage study for 14800 to 15350 MHz based on the [2] and TR 38.922. The purpose is to determine the basic deployment parameters in the response to ITU.

# 2	System-level Simulation Assumption

One of the main purposes to conduct the system level simulation is to determine some key deployment parameters, such as the number of BS antenna elements (AE), the BS down-tilt angle and UE antenna assumptions (e.g., FR1-like or FR2-like). Therefore, in the follow sub-sections, we try to compare the spectrum efficiency (SE) of different deployment settings for 15GHz with the spectrum efficiency of typical deployment of FR1 at 4GHz. From these evaluations, we want to know whether and how we can achieve a comparable system-level throughput of 15GHz, when comparing with FR1 4GHz.

The common simulation assumptions are provided in Table 1. Note that

- We take spectrum efficiency as the KPI for comparison to avoid considering the effect of different channel BW in FR1 and 15GHz. Anyone can easily scales the results if he/she wants to consider channel BW difference.

- In the simulation, only 60KHz SCS to reduce the number of different parameters between FR1 and 15GHz simulations. We are open to consider 15KHz or 30KHz SCS, if needed.

- Only DL SE is evaluated.

- Only full buffer case is considered. It is kind of the worst case in terms of the interference level. We are also open to consider non-full buffer cases, if needed.

Table I. General SLS assumptions

| Parameters | Settings |  |
| --- | --- | --- |
| Scenario | Urban Macro | Dense Urban |
| Link direction | Downlink | Downlink |
| Channel model | UMa | UMa |
| Inter-site distance | 450m/350m | 200m |
| Indoor UE ratio | 20% | 80% |
| Network layout | 21 cells with wraparound | 21 cells with wraparound |
| Number of UE per cell | 10 | 10 |
| Carrier frequency | 4GHz/15GHz | 4GHz/15GHz |
| Channel bandwidth | 100MHz | 100MHz |
| SCS | 60kHz | 60kHz |
| BS tx power | 43dBm | 43dBm |
| Scheduling | MU-PF | MU-PF |
| Traffic model | Full buffer | Full buffer |
| Receiver type | MMSE | MMSE |
| MIMO mode | MU-MIMO | MU-MIMO |
| Precoding method | SVD-based | SVD-based |

We adopt the following BS antenna assumptions [2] for FR1:

- 192 AEs with 3 AEs vertically placed in one subarray for urban macro scenario

- 128 AEs with 4 AEs vertically placed in one subarray for dense urban scenario

When simulating 15GHz scenarios, we will increase the number of AEs at BS, while keeping the same subarray structure.

On the UE side, we always assume 4Rx UE in both FR1 and 15GHz. Particularly for 15GHz, we consider both FR1-like and FR2-like UE antenna implementation, as follows.

- FR1-like UE antenna: X-pol with isotropic antenna pattern (0dBi). Support up to 4-port transmission/reception

- FR2-like UE antenna: 2 panels with panel selection (1 active at a time). 4 AE per panel (i.e., 2 AE per polarization) with X-pol antenna element radiation pattern in table 6.1.2.3.3.2-1 of TR38.922. Note the only up to 2-port transmission/reception is possible.

# 3	Simulation results

## 3.1 Results for FR1-like and FR2-like UE antennas

Figure 1 presents the SE of of 4GHz and 15GHz with FR1-like and FR2-like UE antenna architectures in UMa. In the cell average SE, FR1-like slightly outperforms FR2-like in 15GHz, regardless of the BS AE number. Look a bit closer:

- At cell center (95%-tile SE), FR1-like is worse than FR2-like. It is because from the simulation assumption in TR 38.901, it is more likely to see light-of-sight (LOS) channel when UE is closer to the BS. With LOS, there would not be sufficient AoAs to provide UE the opportunity for higher layer MIMO. Instead, FR2-like can achieve a higher beamforming gain than FR1-like.

- At mid-range (50%-tile SE), FR1-like UEs outperform FR2-like UEs, because it is more likely to see NLOS channels which gives the opportunity for >2 MIMO layer, while FR2-like is upper bounded by 2-layer MIMO.

- At cell edge (5%-tile SE), FR1-like provides higher diversity gain with 4-port reception, while FR2-like can only have up to 2-port for reception.

Figure 1. DL SE performance of 4GHz and 15GHz with FR1-like and FR2-like UE antenna architectures.

Observation 1: In UMa scenario, FR1-like UE outperforms FR2-like UE in most cases

Based on above observations, in the later sub-sections we will take FR1-like UEs as the baseline to conduct more analyses.

## 3.2 Results for different ISDs

Figure 2 presents the SE performance of 4GHz and 15GHz with different numbers of BS AEs in UMa ISD 450m. Results for UMa ISD 350m is shown in Figure 3, while results for dense urban with ISD 200m is given in Figure 4. With ISD 450m, we can achieve similar cell average SE as FR1 by increasing the BS AE number. However, it is difficult to maintain similar SE as FR1 at cell edge due to the large pathloss. This situation is eased by decreasing ISD to 350m in Figure 3. For dense urban scenario in Figure 4, due to larger link budget, 1k BS antenna number at 15GHz can achieve comparable SE to FR1 across all cases.

Figure 2. DL SE performance of 4GHz and 15GHz with different numbers of BS AEs in UMa ISD 450m

Figure 3. DL SE performance of 4GHz and 15GHz with different numbers of BS AEs in UMa ISD 350m

Figure 4. DL SE performance of 4GHz and 15GHz with different numbers of BS AEs in dense urban ISD 200m

Observation 2: To achieve similar spectrum efficiency as FR1 deployment, 2K BS antenna elements need to be considered for UMa scenario in 15GHz.

Observation 3: With reduced ISD size, the number of required BS AEs in 15GHz also reduces to achieve comparable spectrum efficiency as FR1 deployment.

## 3.3 Study of BS down-tilt angle

In this section, we discuss the issue of the BS down-tilt angle as it plays a crucial role in optimizing network performance. An improperly selected down-tilt angle can result in high interference level or insufficient serving power to UEs, leading to network performance degradation.

We evaluated the SE performance with various tilting angles, including average cell SE and 5%-tile UE SE, in a UMa scenario with an ISD of 450m in the 15GHz frequency range. To establish a baseline, we compared the tilting angle assumptions in RAN4. RAN4 assumes a 3-degree electrical tilting angle and a 6-degree mechanical tilting angle, resulting in an effective down-tilt angle of 9 degrees.

Our analyses indicate that, for average cell SE, a tilting angle of less than 15 degrees yields relatively better performance in general. However, at the cell edge, a down-tilt angle below 9 degrees may result in strong interference from neighbouring cells. Conversely, if the down-tilt angle is too large (e.g., exceeding 12 or 15 degrees), the cell-edge UE may experience reduced serving power, leading to poor SE performance.

BS Down-tilt angle:

Figure 5. DL SE performance of 15GHz with different BS antenna down-tilt angles in UMa ISD 450m

Observation 4: For a UMa scenario with 450m ISD in 15GHz, a BS down-tilt angle between 9 to 15 degrees offers relatively better SE performance.

Proposal 1: Capture observations 1, 2, 3 and 4 in the TR.

# 4	UE noise factor

In the last RAN4 meeting, RAN4 reached an agreement on the UE NF, deciding to use 11dB for simulations, while the actual noise factor for the response will be determined based on feasibility. From our perspective, a UE NF of 12 dB was assumed during the band n104 discussions. Considering the higher implementation loss at 15GHz, we believe the UE NF could be worse. We will provide a more detailed analysis at the next meeting. Therefore, from a feasibility standpoint, the UE NF could be worse than 11dB.

Proposal 2: From a feasibility standpoint, the UE NF could be worse than 11dB.

# 5	Conclusion

In this paper, we provided our system-level evaluation results for comparing DL spectrum efficiency between FR1 and 15GHz deployments. We covered the comparison including FR1-like and FR2-like UE architecture, different ISD size and different BS antenna down-tilt angles. We have the following observations:

Observation 1: In UMa scenario, FR1-like UE outperforms FR2-like UE in most cases

Observation 2: To achieve similar spectrum efficiency as FR1 deployment, 2K BS antenna elements need to be considered for UMa scenario in 15GHz.

Observation 3: With reduced ISD size, the number of required BS AEs in 15GHz also reduces to achieve comparable spectrum efficiency as FR1 deployment.

Observation 4: For a UMa scenario with 450m ISD in 15GHz, a BS down-tilt angle between 9 to 15 degrees offers relatively better SE performance.

Proposal 1: Capture observations 1, 2, 3 and 4 in the TR.

Proposal 2: From a feasibility standpoint, the UE NF could be worse than 11dB.

# 5	References

RP-241326, Revised SID: Study on IMT parameters for NR for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz, Ericsson

R4-2410741, WF on IMT parameter study, Ericsson