3GPP TSG-RAN WG4 Meeting # 112								    R4-2411777

Maastricht, Netherlands, 19th – 23th Aug, 2024

Agenda Item: 8.9.3

Source: MediaTek Inc.

Title: Discussions on system parameters of Ku Band for NR NTN

Document for: Discussion

# 1	Introduction

In RP#104, a new WI [1] for Ku band was approved with objectives asking RAN4 to discuss the system parameters, including FR1/FR2, SCS and channel BW. The relevant objectives are captured as below.

- Priority 1: for all ITU regions, define Ku band #1a and #1b for the downlink block and uplink block B to enable the future possibility of supporting satellite services in 125 MHz (UL) and 250 MHz (DL) aggregated bandwidths based on existing channel bandwidths. Potential approaches for achieving this shall be studied, from which one shall be selected including but not limited to: - Ku band #1a and #1b as FR1-NTN supporting 20 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing - Ku band #1a and #1b as FR2-NTN supporting 50 MHz, 100 MHz, 200 MHz, 400 MHz (120kHz SCS only) channel bandwidths with 120 kHz subcarrier spacing - RAN4 will select which one of the above approaches for DL and UL will be pursued by RAN #106. - Priority 2: for all ITU regions excluding the US in region 2, using the approach in Priority 1, define Ku band #2a and #2b as appropriate for the downlink block and uplink block A to support satellite services in 125 MHz (UL) and 250 MHz (DL) aggregated bandwidths.

In this paper, we provide our views on this topic.

# 2	Discussions

Due to the large pathloss, communications between ground UEs and satellites require more advanced antenna technology, e.g., beamforming. This issue is further impacted by the channel BW from the consideration of link budget. Note that unlike TN links which typically are interference-limit, NTN links are usually noise-limited. Larger channel BW brings higher sensitivity levels in the receivers and lower power spectrum density in the transmitters. Each of the consequences leads a lower receive signal power and/or lower SNR level at the receiver side. To address these drawbacks, even more antennas for higher beamforming gain are need to be implemented at both UEs and satellites. From UE perspective, large antenna system would increase the cost and put the constraint in the implementations, e.g., on vehicles. This will eventually impact how Ku band service penetrates the market. Therefore, we suggest RAN4 to carefully discuss the need of larger channel BWs. If needed, large BW can be considered as optional.

Observation 1: Larger channel BW would lead to higher sensitivity levels in the receiver and lower power spectrum density in the transmitter. These may need even more antennas to mitigate, putting extra cost and implementation constraint to UEs.

Proposal 1: RAN4 to carefully discuss the need of larger channel BWs. If needed, large BWs can be considered as optional.

On the issue of FR1 and FR2 down-selection, the same arguments apply because FR2 BWs are typically larger than FR1 BWs. Furthermore, the shorter CP length of FR2 make it very challenging for implementation due to very tight UE Tx transmit timing accuracy requirements. Therefore, we suggest considering prioritizing FR1 system parameters over FR2.

Observation 2: Beside large bandwidth, the shorter CP length at FR2 make it very challenging for implementation due to very tight UE Tx transmit timing accuracy requirements.

Proposal 2: RAN4 to prioritize FR1 over FR2 in Ku band system parameter discussions.

# 4	Summary

In this paper, we provide our views on the system parameters for Ku band. We have the following observations and proposals.

Observation 1: Larger channel BW would lead to higher sensitivity levels in the receiver and lower power spectrum density in the transmitter. These may need even more antennas to mitigate, putting extra cost and implementation constraint to UEs.

Proposal 1: RAN4 to carefully discuss the need of larger channel BWs. If needed, large BWs can be considered as optional.

Observation 2: Beside large bandwidth, the shorter CP length at FR2 make it very challenging for implementation due to very tight UE Tx transmit timing accuracy requirements.

Proposal 2: RAN4 to prioritize FR1 over FR2 in Ku band system parameter discussions.

# 5	References

RP-241690, New WID on Introduction of Ku Band for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat