3GPP TSG-RAN WG4 Meeting # 112	R4-2411780

Maastricht, Netherlands, Aug 19 – Aug 23, 2024

Agenda Item: 5.13.1

Source: MediaTek Inc.

Title: Discussion on core part maintenance for multi-Rx UEs

Document for: Discussion

# Introduction

According to WF [1], there’re some core part requirement discussed without conclusions. In this paper, we provide our view on these issues, particularly on the following topics.

- The condition about faster beam sweeping.

- Measurement restriction

# Discussion

## 2.1 The condition about faster beam sweeping

In last meeting, RAN4 discussed how to capture UE is in multi-RX operation for faster beam switching in SSB based L1-RSRP measurement. Note the current feature list only considers the UEs which require to turn on multiple panels to achieve faster beam sweeping. Therefore, the discussions in this paper also consider this kind of UEs only.

Issue 1-1-1: When is UE considered to be in multi-rx operation - Proposals - Proposal 1: Keep the agreed definition - Given UE indication of its preference of multi-RX/single-RX operation has been allowed, it is OK to take the current definition of multi-RX operation. - Proposal 2: Add following additional condition - P2a: - UE sent a recent valid Rel-17 group-based beam reporting (GBBR). - If UE recently reported ‘Not valid’ for one of the RSRP for a beam pair, this means UE is allow to fallback to single panel for the later reception QCL-ed to that beam pair. - P2b: Rel-17 group-based beam reporting (GBBR) is activated/triggered by the network. - Recommended WF: - Further discuss.

Currently, RAN4 only considers the condition that UE is configured with Rel-17 GBBR for faster beam sweeping. This will lead to power consumption issue from UE perspective. We further provide our views under below 2 cases.

- Case A: Rel-17 GBBR is configured with semi-persistent (SP) or aperiodic (AP) reporting.

- Case B: Rel-17 GBBR is configured with periodic reporting.

For case A, UE needs to turn on 2 panels as long as SP or AP reporting is configured, even if they are never triggered by MAC CE or DCI. Since RRC configuration is semi-static, UE has no chance to turn off any panels unless the RRC configurations are released. This seems not reasonable and will lead to unnecessary UE power waste.

For case B, UE sends L1-RSRP GBBR report periodically, but it does not mean UE is currently in a good condition for multi-Rx operation. For example, if UE reports a ‘Not valid’ for one of the RSRP for a beam pair, it is obvious that the current channel condition is not suitable for multiple Rx operation. (e.g., one antenna panel is blocked by human hand). In this case RAN4 should allow UE to still turn on one panel at a time for measurement, rather than forcing UE to always activate 2 panels.

For case A and case B, we should refrain from wasting UE power.

Observation 1: It will lead to unnecessary UE power consumption if UE is mandated to operate in multi-RX mode as long as UE is configured with group-based beam reporting (GBBR) report.

One consequence of this decision is that it may impact the Rel-19 NR_RRM_Ph5 WI in which some discussions for the condition to enable faster beam sweeping for L3 measurements are ongoing. It is highly likely that the Rel-19 WI will directly inherit the condition(s) agreed in Rel-18 mRx WI. Since L3 measurement is kind of background behaviour that UE has to perform all the time, the power consumption issue needs to be carefully addressed. Besides, it is very important to note that even if some UEs may not need simultaneous multiple panels to achieve faster L1 Rx beam sweeping (e.g., L3 measurements were used for panel selection), it is very likely all UEs needs simultaneous multiple panels to achieve faster L3 Rx beam sweeping. To avoid negative power consumption impact to Rel-19, we need to be very careful in making such a decision.

Based on above observation, RAN4 at least to consider the condition that UE sent valid Rel-17 GBBR.

After UE has reported valid Rel-17 GBBR, NW knows that UE is in a good condition for multiple panels operation to support enhancements like faster beam sweeping or simultaneous L1 measurement. From the UE side, it is also reasonable to assume that UE is responsible of enabling these enhancements from now on, because the valid GBBR is sent by UE.

Since this issue was discussed in last meeting without conclusion, one potential compromised proposal is to define a leaving condition, e.g., the condition that the UE is allow to fallback to single-panel operation. For example, if UE recently reported ‘Not valid’ for one of the RSRP for a beam pair, this means UE is allow to fallback to single panel for the later reception QCL-ed to that beam pair.

Proposal 1: If UE recently reported ‘Not valid’ for one of the RSRP for a beam pair, this means UE is allow to fallback to single panel for the later reception QCL-ed to that beam pair.

## 2.2 Measurement restriction

In previous meetings, there’s an issue discussed without conclusion for the condition of measurement restriction.

#### TS 38.133 #### 9.5.5.2 Measurement restriction for CSI-RS based L1-RSRP For FR2-1, when the CSI-RS for L1-RSRP measurement on the one CC is in the same OFDM symbol as another CSI-RS for RLM, BFD, CBD or L1-RSRP measurement on the same CC, UE supporting [multi-Rx capability] according to the conditions described in clause 3.6.y shall be able to measure the CSI-RS for L1-RSRP measurement without restriction when the following conditions are met: - Both the CSI-RS for L1-RSRP and the other CSI-RS are not in any CSI-RS resource set configured with repetition ON, and - Resources of the active TCI states for two PDSCHs have been reported as a resource group in Rel-17 group-based RSRP report, and - [The two CSI-RSs and both PDSCHs are overlapped on the same OFDM symbol, and] - The CSI-RS for L1-RSRP has same QCL source as the active TCI state of one [PDSCH], and the other CSI-RS has same QCL source as the active TCI state of the other [PDSCH], and Editor’s note 1: FFS the condition CSI-RS(s) and PDSCH(s) are overlapped on the same OFDM symbol(s). Editor’s note 2: FFS how to capture UE is activated with multi-Rx operation. Issue 1-3-2: measurement restriction for sDCI and mDCI Moderator recommendation - Same slot scheduling or scheduling offset less than timeDurationForQCL - Option 1: Remove the measurement restrictions (i.e., remove the following conditions) - The two CSI-RS resources and both PDSCHs are overlapped on the same OFDM symbol - The CSI-RS for L1-RSRP has same QCL source as the active TCI state of one PDSCH, and the other CSI-RS has same QCL source as the active TCI state of the other PDSCH - Option 2: Modify the measurement restrictions - the measurement restriction relaxation can be modified as follows: - Both the CSI-RS for L1-RSRP and the other CSI-RS are not in any CSI-RS resource set configured with repetition ON, and - Resources of the active TCI states or default QCL assumption as defined in TS38.214 for two PDSCHs have been reported as a resource group in Rel-17 group-based RSRP report, and - [The two CSI-RSs and both PDSCHs are overlapped on the same OFDM symbol, and] - The CSI-RS for L1-RSRP has same QCL source as the active TCI state/or default QCL assumption of one [PDSCH], and the other CSI-RS has same QCL source as the active TCI state / or default QCL assumption of the other [PDSCH] - Cross slot scheduling when scheduling offset is larger than timeDurationForQCL - Option 1: Remove the measurement restrictions (i.e., remove the following conditions) - The two CSI-RS resources and both PDSCHs are overlapped on the same OFDM symbol - The CSI-RS for L1-RSRP has same QCL source as the active TCI state of one PDSCH, and the other CSI-RS has same QCL source as the active TCI state of the other PDSCH - Option 2: Modify the measurement restrictions - the measurement restriction relaxation can be modified as follows: - Both the CSI-RS for L1-RSRP and the other CSI-RS are not in any CSI-RS resource set configured with repetition ON, and - Resources of the active TCI states or default QCL assumption as defined in TS38.214 for two PDSCHs have been reported as a resource group in Rel-17 group-based RSRP report, and - [The two CSI-RSs and both PDSCHs are overlapped on the same OFDM symbol, and] - The CSI-RS for L1-RSRP has same QCL source as the active TCI state/or default QCL assumption of one [PDSCH], and the other CSI-RS has same QCL source as the active TCI state / or default QCL assumption of the other [PDSCH

The issue is divided into two scenarios:

(1) same slot scheduling or scheduling offset less than timeDurationForQCL

(2) cross slot scheduling when scheduling offset is larger than timeDurationForQCL.

No matter for scenarios 1 or scenarios 2, UE should already know the RX beam used for the CSI-RS L1-RSRP measurement based on current measurement restriction defined in TS 38.133 9.5.5.2

- Resources of the active TCI states for two PDSCHs have been reported as a resource group in Rel-17 group-based RSRP report.

- The CSI-RS for L1-RSRP has same QCL source as the active TCI state of PDSCH.

In our understanding, we don’t see the reason why RAN4 need to mention the default QCL assumption which is for PDSCH under measurement restriction. We do not think the measurement restriction should only be limited to the case when the two CSI-RS resources are overlapped with both PDSCHs.

- Measurement restriction is only about 2 RS. There is nothing to do with the PDSCH scheduling. In legacy, we don’t have requirement assuming that the reference signals and data reception should be overlapped for measurement restriction.

- UE’s scheduling of L1 measurement (i.e., how to select Rx beam to measure for each SSB occasion) is usually static based on NW’s configuration. This scheduling is not expected to be changed dynamically due to whether PDSCH is scheduled or not in a slot. Even if the CSI-RS resources are not overlapped with PDSCHs, UE should perform both CSI-RS measurements at the same time when feasible.

- UE needs a certain processing time to decode PDCCH to realize whether there’s a PDSCH to receive in the same slot. Before UE can successfully decode PDCCH (as shown in Figure 1), UE does not know whether the PDSCH is intended to it. What UE can do is only to buffer all signals with a certain Rx beam assumption during the PDCCH processing time. This means that UE has no way to re-decide which Rx beam pair to be used for CSI-RS measurement because the CSI-RS symbol is already received. Therefore, the condition could cause problem in UE’s DL processing and should be removed.

There were some discussions on this issue. One concern is that removing the overlapping condition means that UE will never have the chance to sweep beams on the CSI-RS, because UE needs to always use the best Rx beam for PDSCH reception to receive CSI-RS. This is a correct understanding. To accommodate this problem, it is necessary that network also needs to provide other QCL source (e.g., SSB) for UE to perform beam sweeping.

Proposal 2: Remove the following condition of measurement restriction relaxation for CSI-RS based L1 measurements.

- [The two CSI-RS resources and both PDSCHs are overlapped on the same OFDM symbol].

Figure 1. Relation between UE processing time for PDCCH and the reception of CSI-RS for measurement.

# Summary

In this paper, the discussion of R18 multi-Rx chains is provided. We have the following proposal:

Observation 1: It will lead to unnecessary UE power consumption if UE is mandated to operate in multi-RX mode as long as UE is configured with group-based beam reporting (GBBR) report

Proposal 1: If UE recently reported ‘Not valid’ for one of the RSRP for a beam pair, this means UE is allow to fallback to single panel for the later reception QCL-ed to that beam pair

Proposal 2: Remove the following condition of measurement restriction relaxation for CSI-RS based L1 measurements.

- [The two CSI-RS resources and both PDSCHs are overlapped on the same OFDM symbol].

# Reference

[1] R4-2410356 WF on core maintenance and performance part for multi-Rx, Ericsson RAN4#111