3GPP TSG-RAN WG4 #112	R4-2411784

Maastricht, Netherlands, August 19-23, 2024

Agenda item:	8.18.3		RRM Core Requirements

Source: 	Qualcomm Incorporated

Title: 	Views on the RRM impact of NR MIMO Phase 5

Document for:	Discussion

# Introduction

In RAN4 #112 RAN4 will start working on the RAN1-led WI for NR MIMO Phase 5. The objectives of the WI as defined in [1] are as follows:

- Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management

  - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching

  - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting

- Specify CSI support for up to 128 CSI-RS ports, targeting FR1

  - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks

  - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s)

  - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design

- Specify UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD

- Inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH

- Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource

Note: UL full power transmission mode 1 and 2 are not supported.

- Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2

  - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP.

The status of the work item is reported in [4] and [5]. In this paper, we share our views on the major topics of NR MIMO Phase 5 and a preliminary assessment of the RRM impact.

# Discussion

The work item has five main topics. In the table below we summarize our high-level view whether the topic has RRM impact and which specification document should cover the corresponding requirements.

| Topic | RRM impact | Specification |
| --- | --- | --- |
| UE-initiated/event-driven beam management, targeting FR2 | RRM impact (core) | TS 38.133 |
| 128 CSI-RS ports, targeting FR1 | Demod impact only | TS 38.101-4 |
| reporting enhancement for CJT deployments | Performance impact | TS 38.133 (RRM) or TS 38.101-4 (demod) |
| non-coherent UL codebook for three antenna ports | No RRM impact | 38.104? |
| enhancement for asymmetric DL sTRP/UL mTRP deployment | RRM impact (core) | TS 38.133 |

In our view, 128 CSI-RS ports and non-coherent uplink codebook for three antenna ports do not have RRM impact and do not need to be treated in the RAN4 RRM sessions.

Observation 1: 128 CSI-RS ports and non-coherent uplink codebook for three antenna ports does not have RRM impact.

## UE-initiated/Event-driven Beam Management

The motivation for the UE-initiated/event-driven beam management is to reduce overhead and/or latency. For the event detection for the beam quality metric, L1-RSRP as been agreed in RAN1 #116. Since this is an existing measurement, accuracy requirements should not be impacted and no impact to the specification is foreseen to define new accuracy requirements. This may change in case RAN1 decides to introduce new non-existing measurements for the beam reports.

Proposal 1: No new accuracy requirements for existing L1 measurements are required.

Since the L1 UE beam reporting has similarities to the L3 event triggered reporting for mobility, similar requirements should be defined for event triggered beam management.	The L3 measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. Similar definition could be applied for L1 beam reporting.

Proposal 2: RAN4 should study how the reporting delay for UE-initiated/event-driven beam management can be defined. The definition of L3 measurement reporting delay could serve as a starting point.

So far, RAN1 has agreed to support two modes for the beam report transmission procedure, namely mode A and mode B as shown in Figure 2-1 below. Mode A requires that uplink resources for the beam report are dynamically scheduled by the gNB, while Mode B is a applies preconfigured uplink resources. One approach could be to define the L1 beam reporting delay has the time between the triggering event and the transmission of the first PUCCH. However, this does not allow to distinguish in the reporting delay between Mode A and Mode B. The definition of the reporting delay in UE-initiated/event-triggered beam management should be further studied in RAN4.

Figure 2-1: Illustration of Beam Report Mode A and Mode B

Observation 2: One possibility to define the L1 reporting delay could be the time between the triggering event and the transmission of the 1st PUCCH. However, this does not allow to distinguish between the reporting modes Mode A and Mode B that have been defined in RAN1 so far.

It should be investigated whether additional delay requirements can be defined that allow to distinguish between Mode A and Mode B. In Mode A, the overall delay for the beam report depends on how quickly the gNB allocates UL resources. The time between step 2 (allocation of uplink resources) and step 3 (beam report) in Figure 2-1 depends on the network-configured K2 value and the PUSCH preparation given by N2 as defined in [3]. In Mode B, however, uplink resources a preconfigured and the K2 value does not apply. It should be investigated in RAN4 whether it is needed to define additional delay requirements between the individual steps of Mode A and Mode B

Proposal 3: RAN4 should investigate whether it is needed to define additional delay requirements between the individual steps of Mode A and Mode B beam report.

The second goal in the work item is to reduce the latency of the beam switch. RAN1 has agreed that the current beam is a beam corresponding to the indicated TCI state in the Rel-17 unified TCI framework, i.e., the current serving beam of the PDCCH/PDSCH. For the new beam(s) to be measured it has been agreed that they will be explicitly configured but details are to be down selected in RAN1 #118. If the quality, e.g., the L1-RSRP of at least one new beam becomes a threshold value better than the current beam (Event-2), an event is triggered. The action to be taken by the network is left open, however it can be expected that the network triggers a TCI state switch to a new target TCI. Figure 2-2 illustrates the definition of TCI state switch delay in [2]. It is the delay between the reception of the TCI switch command and the time when PDCCH with the new target TCI is receivable.

Figure 2-2: Illustration of TCI State Switch Delay

It is likely that the network will configure the reference signals of the new beam(s) in such a way that also TCI states that are not yet activated may be measured. If such a new beam corresponding to a TCI state that is not yet activated leads to an event trigger, the network has to switch the TCI state with a MAC CE. Depending on whether the target TCI state is known, or unknown, different TCI state switch delays are defined in [2]. It should be studied in RAN4 whether any new required TCI state switch delays need to be defined or existing requirements are sufficient.

Proposal 4: RAN4 should investigate whether any new required TCI state switch delays need to be defined or existing requirements are sufficient.

## Reporting Enhancement for CJT Deployments

Reporting enhancements for coherent joint transmission deployments under non-ideal synchronization and backhaul deal with reporting of additional information by the UE for delay offset compensation, per-TRP frequency offset compensation and per TRP DL/UL Rx-Tx phase compensation to enhance reciprocity. These reporting enhancements do not impact the core requirements of MIMO Phase 5 but rather impact the performance part of this work item. Since the reporting enhancements impact performance, it needs to be discussed whether these performance aspects should be treated under RRM performance or under demodulation performance.

In our view, these reporting enhancements are similar to the time domain channel properties (TDCP) in Rel-18. We think that the main specification impact is do define accuracy requirements for this additional reporting but how to apply this information in the base station to enhance performance is in our opinion out of the scope of the 3GPP specification. Hence, no PDSCH throughput tests should be defined for these reporting enhancements. Since RRM performance specification deals with accuracy requirements for measurement reports, specification impact of reporting enhancement for CJT deployments should be covered in TS 38.133 and not in TS 38.101-4.

Observation 3: Reporting enhancements for CJT deployments impact performance requirements but not the RRM core requirements.

Observation 4: Performance impact by reporting enhancements for CJT deployments can be appropriately covered by defining accuracy requirements. Defining PDSCH throughput requirements for CJT reporting enhancements is beyond the scope of 3GPP.

Proposal 5: Specification impact of reporting enhancement for CJT deployments should be covered in TS 38.133 under RRM performance and not in 38.101-4 under demodulation performance.

According to the TU allocation of the work item, work on performance aspects for MIMO Phase 5 starts in RAN4 #115 in May ’25. Hence, we propose to postpone working on defining requirements for reporting enhancements for CJT till the RAN4 #115 meeting.

Proposal 6: The work on defining requirements for reporting enhancements for CJT deployments under non-ideal synchronization and backhaul should start in RAN4 #115 in May ’25.

## Enhancement for Asymmetric DL sTRP/UL mTRP Deployments

Figure 2-3 shows an example of an asymmetric DL sTRP/UL mTRP deployment as considered in the work item. The downlink is coming from the macro gNB while the uplink is either transmitted to the macro gNB or to a non-collocated micro node, called UL-TRP in the figure.

Figure 2-3: Asymmetric DL sTRP/UL mTRP Deployment

The pathloss reference signal is defined with respect to the macro gNB. It is not expected that the pathloss reference signal switch delay is impacted by the new deployment since the pathloss reference signal is always defined with respect to the macro gNB.

Observation 6: It is not expected that the pathloss reference signal switch delay is impacted by the new deployment.

A pathloss offset can be associated with a UL TCI state to allow the UE calculating the Tx power of the PUSCH/PUCCH/SRS for transmission to the UL-TRP. The pathloss offset is not only changed when the UL TCI to the UL-TRP changes but can be updated independently. New requirements should be defined how quickly the pathloss offset can be applied by the UE in case it is changed together with the UL TCI and in case it changes independently from the UL TCI.

Observation 7: The pathloss offset towards the UL-TRP may change together with the UL TCI but may also be changed independently. This may imply that new requirements should be defined how quickly the UE is able to transmit an uplink signal applying the new path loss offset.

Proposal 7: RAN4 should study whether the uplink TCI state switch delay is impacted if the target TCI is associated with a pathloss offset.

Proposal 8: RAN4 should study whether new requirements should be defined if the pathloss offset changes independently from the uplink TCI.

# Conclusions

In this paper we provided our views on the major topics of NR MIMO Phase 5 and a preliminary assessment of the RRM impact. Our observations and proposals are summarized as follows:

Observation 1: 128 CSI-RS ports and non-coherent uplink codebook for three antenna ports does not have RRM impact.

UE-initiated/Event-driven Beam Management

Proposal 1: No new accuracy requirements for existing L1 measurements are required.

Proposal 2: RAN4 should study how the reporting delay for UE-initiated/event-driven beam management can be defined. The definition of L3 measurement reporting delay could serve as a starting point.

Observation 2: One possibility to define the L1 reporting delay could be the time between the triggering event and the transmission of the 1st PUCCH. However, this does not allow to distinguish between the reporting modes Mode A and Mode B that have been defined in RAN1 so far.

Proposal 3: RAN4 should investigate whether it is needed to define additional delay requirements between the individual steps of Mode A and Mode B beam report.

Proposal 4: RAN4 should investigate whether any new required TCI state switch delays need to be defined or existing requirements are sufficient.

Reporting Enhancement for CJT Deployments

Observation 3: Reporting enhancements for CJT deployments impact performance requirements but not the RRM core requirements.

Observation 4: Performance impact by reporting enhancements for CJT deployments can be appropriately covered by defining accuracy requirements. Defining PDSCH throughput requirements for CJT reporting enhancements is beyond the scope of 3GPP.

Proposal 5: Specification impact of reporting enhancement for CJT deployments should be covered in TS 38.133 under RRM performance and not in 38.101-4 under demodulation performance.

Proposal 6: The work on defining requirements for reporting enhancements for CJT deployments under non-ideal synchronization and backhaul should start in RAN4 #115 in May ’25.

Enhancement for Asymmetric DL sTRP/UL mTRP Deployments

Observation 6: It is not expected that the pathloss reference signal switch delay is impacted by the new deployment.

Observation 7: The pathloss offset towards the UL-TRP may change together with the UL TCI but may also be changed independently. This may imply that new requirements should be defined how quickly the UE is able to transmit an uplink signal applying the new path loss offset.

Proposal 7: RAN4 should study whether the uplink TCI state switch delay is impacted if the target TCI is associated with a pathloss offset.

Proposal 8: RAN4 should study whether new requirements should be defined if the pathloss offset changes independently from the uplink TCI.

# References

- RP-240087, WID revision: NR MIMO Phase 5, RAN #103, March 2024

- TS 38.133, Requirements for support of radio resource management, v18.6.0, June 2024

- TS 38.214, Physical layer procedures for data, v18.3.0, June 2024

- RP-240086, Status Report NR MIMO Phase 5, RAN #103, March 2024

- RP-241051, Status Report NR MIMO Phase 5, RAN #104, June 2024