3GPP TSG-RAN WG4 #112	R4-2411785

Maastricht, Netherlands, August 19-23, 2024

Agenda item:	10.1.2.1	Specification improvement in R19 timeframe

Source: 	Qualcomm Incorporated

Title: 	On improvements of Rel-19 RRM specifications

Document for:	Discussion

# Introduction

In RAN4 #111, good progress has been made regarding the RRM specification improvement [1]. The following agreements were achieved [2].

- Agreement:

  - Issue1: [], “TBD” and “FFS” clean up

    - The case, which is not an obvious editorial revision and subject to the technical discussion, should be listed separately and excluded from this exercise.

  - Issue2: Terminology/style inconsistency, incorrect notation/symbols/abbreviation, undefined abbreviations, redundant information/notes clean up

    - If the revision results in an update in Section 3, TS38.133, it should be indicated separately.

    - The case, which is not an obvious editorial revision and subject to the technical discussion, should be excluded from this exercise.

  - Issue3: Empty test cases clean up

    - Empty test cases in Annex A of TS 38.133 can be made void

In this contribution we provide our views and additional proposals to improve the quality of the RRM spec.

# Discussion

Agreement:

- The summary of other identified issues, where RAN4 has not yet decided if, when and how to address, is listed as below. Depending on the discussion in both RAN4 and RAN, these issues and the associated guideline for drafting rules, can be revisited.

  - Hierarchy of indent

  - Suffix alignment

  - Unused test configurations

  - Duplicated requirements

  - Modal verbs

Several other issues were identified for improvement where RAN4 has not decided if, when and how to address them [2]:

In this contribution we want to focus on the first bullet hierarchy of indent and duplicated requirements. As an example, we consider the activation delay requirement for deactivated Scell and show how difficult it is to extract the appropriate requirements.

## Text Analysis Example – SCell Activation Delay Requirement for Deactivated SCell

The following two text boxes show an example of the requirement uncertainty in TS 38.133 that come from the fact that many requirements are written in text form instead of pseudo-code. The two text boxes below are copies from statements in section 8.3.2 of TS 38.133.

In the first text box, the SCell activation time is defined for unknown SCell in FR1 to be TFirstSSB_MAX + TSMTC_MAX + Trs + 5ms under various conditions marked with different highlighting colors. Four sub-bullets need to be fulfilled marked in pink, green, blue and yellow in addition to the text written in blue. The text in red states that with respect to a contiguous active serving cell, the RTD needs to be less than 260 ns and the Rx power difference needs to be less than 6 dB. Since all four sub-bullets are combined with an AND condition as shown in bold, it means that the activation delay becomes TFirstSSB_MAX + TSMTC_MAX + 2*Trs + 5ms if the RTD is larger than 260 ns or the Rx power difference is larger than 6 dB relative to a contiguous active serving cell.

The second text box defines when no requirements for SCell activation delay are defined. The same color coding is used for text highlighting as for the first text box when the side conditions are identical. It is seen that all four side conditions marked in pink, green, blue and yellow are repeated. The text in blue mentions the same side conditions as the text in red in the first text box. The second text box states that no requirement is defined if the RTD is larger than 260 ns or the Rx power difference is larger than 6 dB relative to a contiguous active serving cell.

Both text boxes seem to contradict each other. It is unclear from the reading whether the requirement is TFirstSSB_MAX + TSMTC_MAX + 2*Trs + 5ms or whether no requirement is defined for the case that the SCell to be activated does not fulfill the RTD and Rx power criterion with respected to an already active contiguous serving cell.

If the SCell is unknown and belongs to FR1, and if one of the following conditions is met

- ‘ssb-PositionInBurst’ indicates only one SSB is being actually transmitted, or

- ‘ssb-PositionInBurst’ indicates multiple SSBs and TCI indication is provided in same MAC PDU with SCell activation,

provided that the side condition Ês/Iot ≥ -2dB is fulfilled, Tactivation_time is:

- TFirstSSB_MAX + TSMTC_MAX + Trs + 5ms, if the following conditions are met,

  - the SCell is contiguous to an active serving cell in the same band, and

  - its ssb-PositionInBurst is same as the one of contiguous FR1 active serving cell, and

  - its SMTC offset is same as the one of contiguous FR1 active serving cell, and

  - its RTD with contiguous FR1 active serving cell is smaller than or equal to 260ns with respect to the to-be-activated SCell’s SSB numerology, and its reception power difference with contiguous FR1 active serving cell is smaller than or equal to 6dB;

- TFirstSSB_MAX + TSMTC_MAX + 2*Trs + 5ms, otherwise.

However, when the following conditions are fulfilled, no activation requirement will be applied for this unknown SCell:

- the SCell is contiguous to an active serving cell in the same band, and

- A single SSB is used in the unknown SCell; or multiple SSBs are used in the SCell and TCI state indication for PDCCH is provided by the same MAC PDU used for SCell activation; and

- its ssb-PositionInBurst is same as the one of contiguous FR1 active serving cell, and

- its SMTC offset is same as the one of contiguous FR1 active serving cell

- its RTD with contiguous FR1 active serving cell is larger than 260ns with respect to the to-be activated SCell’s SSB numerology, or its reception power difference with contiguous FR1 active serving cell is larger than 6dB;

The uncertainty in the requirement seems to be caused by repeating text for the side condition and by not properly following a hierarchy of indent. Hence, we repeat our proposal from [4]:

Proposal 1: RAN4 to develop guidelines for drafting requirements with complex logic, including adopting a pseudo-code approach (e.g. similar to the way RAN2 procedures are specified).

Observation 1: The hierarchy of indent could be indicated by the same approach as in TS 38.321, i.e., 1>, 2>, 3>, etc. for the indent level.

Following proposal 1, the next subsection the SCell activation delay requirements in section 8.3.2 of TS 38.133 are restructured applying the methodology of TS 38.321. The wording of the original text has not been changed but was just copied and reorganized. Hence, potential errors and inconsistencies are still included in the new version.

In our view the requirements for SCell activation delay in subsection 2.1.1 are much easier to read after simple text reshuffling and using proper indent level. In addition, the text becomes shorter and also allows easier maintenance.

Observation 2: Significant improvement of the readability in TS 38.133 can be achieved by applying proper indentation and reorganizing of the text without the need to rewrite it.

The text marked in pink in the subsection 2.1.1 indicates an example where the side conditions are duplicated. In our view such duplication should be avoided in a clean text but that would require more effort and considerable re-writing of the existing text.

Observation 3: Further optimization beyond proper indentation and text reorganization is possible.

However, such an additional optimization may not be feasible for an existing and complicated specification like TS 38.133. It is also not needed in a first step since significant improvements in readability could already be achieved by simple means.

Proposal 2: RAN4 should select a few example sections of TS 38.133 and try to improve readability by proper indentation and restructuring of the existing text.

Potential sections to start with are all the sections for SCell activation delay and the sections on handover. But also, others have sufficient room for improvement.

### Indentation and Restructuring of Section 8.3.2 – SCell Activation Delay Requirement for Deactivated SCell

Upon receiving SCell activation command in slot n, the UE shall be capable to transmit valid CSI report and apply actions related to the activation command for the SCell being activated no later than in slot n+THARQ+Tactivation time+TCSI reportingNR slot length. THARQ and TCSI reporting are defined as follows:

- THARQ in ms is the timing between DL data transmission and acknowledgement as specified in TS 38.213

- TCSI reporting is the delay in ms including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331

Tactivation time is the SCell activation delay in ms. Its value depends on the side conditions defined by the pseudo-code below.

- If the SCell to be activated belongs to FR1

  - If the SCell is known

    - If the measurement period of the SCell is equal to or smaller than 2400ms

      - Tactivation time=TFirstSSB+5ms

    - Otherwise

      - Tactivation time=TFirstSSB_Max+Trs+5ms

  - If the SCell is unknown and EsIot≥-2 dB

    - If ssb-PositionsinBurst indicates one SSB only or

    - If ssb-PositionsinBurst indicates multiple SSBs and TCI indication is provided in the same MAC CE as SCell activation

      - The SCell is contiguous to an active serving cell in the same band, and

      - If its ssb-PositionsInBurst is the same as the one of the active serving cell, and

      - If its SMTC offset is the same as the one of the contiguous active serving cell, and

      - If its RTD with the active serving cell is smaller than or equal to 260ns and

      - If its reception power difference with the contiguous active serving cell is smaller than or equal to 6dB

        - Tactivation time=TFirstSSB_Max+TSMTC_MAX+Trs+5ms

      - Otherwise

        - Tactivation time=TFirstSSB_Max+TSMTC_MAX+2Trs+5ms

    - Otherwise

      - If semi-persistent CSI-RS is used for CSI reporting

        - Tactivation time=6ms+TFirstSSB_Max+TSMTC_MAX+Trs+TL1-RSRP,measure+TL1-RSRP,report+THARQ+maxTuncertainty_MAC+TFineTuning+2ms,Tuncertainty_SP

      - If periodic CSI-RS is used for CSI-reporting

        - Tactivation time=3ms+TFirstSSB_Max+TSMTC_MAX+Trs+TL1-RSRP,measure+TL1-RSRP,report+maxTHARQ+Tuncertainty_MAC+TFineTuning+5ms,Tuncertainty_RRC+TRRC_delay

  - If there is at least one active serving cell contiguous in the same band

    - If the UE is not provided a SSB configuration and the UE supports scellwithoutSSB

      - If its RTD with the active serving cell is smaller than or equal to 260ns and

      - If the difference of the reception power with the active serving cell is less than or equal to 6 dB and

      - If the RS(s) of the SCell are QCL-TypeA with its TRS(s) and its TRS(s) are QCL-TypeC with the SSB(s) of the active serving cell

        - Tactivation time=3ms

- If the SCell to be activated belongs to FR2

  - If the SCell is known

    - If semi-persistent CSI-RS is used for CSI reporting

      - Tactivation time=3ms+maxTuncertainty_MAC+TFineTuning+2ms,Tuncertainty_SP

    - If periodic CSI-RS is used for CSI reporting

      - Tactivation time=maxTuncertainty_MAC+TFineTuning+5ms,Tuncertainty_RRC+TRRC_delay-THARQ

  - If the SCell is unknown and EsIot≥-2 dB

    - If semi-persistent CSI-RS is used for CSI reporting

      - Tactivation time=6ms+TFirstSSB_Max+15TSMTC_MAX+8Trs+TL1-RSRP,measure+TL1-RSRP,report+THARQ+maxTuncertainty_MAC+TFineTuning+2ms,Tuncertainty_SP

    - If periodic CSI-RS is used for CSI reporting

      - Tactivation time=3ms+TFirstSSB_Max+15TSMTC_MAX+8Trs+TL1-RSRP,measure+TL1-RSRP,report+maxTHARQ+Tuncertainty_MAC+TFineTuning+5ms,Tuncertainty_RRC+TRRC_delay

  - If there is at least once active serving cell contiguous in the same band

    - If the UE is provided SMTC configuration

      - If the SSB(s) in the SCell and in the active serving cell fulfil the condition defined in clause 3.6.3 and

      - If its ssb-PositionsInBurst is the same as the one of the active serving cell and

      - If SSB is in the same half frame on the SCell and the active serving cell and

        - Tactivation time=TFirstSSB+5ms

    - If the UE is not provided a SMTC configuration and the UE supports scellwithoutSSB

      - If the RS(s) of the SCell are QCL-TypeD with the RS(s) of the active serving cell

        - Tactivation time=3ms

## Additional Comments

Similar improvements by proper indentation and reshuffling of existing text could help not only for clarifying the requirements but also improve readability of e.g., the parameters used to define the requirements

The text box below shows the definition of TSMTC_MAX, Trs, and TFirstSSB. Three different styles are used to define the parameters. While TSMTC_MAX and TFirstSSB show some clarity, the definition of Trs is difficult to follow since it goes across seven lines. Proper indentation could improve readability here as well. In particular, the phrase There are no requirements if the SSB transmission periodicity is not 5ms in the definition of Trs is unclear since Trs is not a requirement.

Proposal 3: RAN4 should develop guidelines how parameters are defined in TS 38.133.

TSMTC_MAX:

- In FR1, in case of intra-band SCell activation, TSMTC_MAX is the longer SMTC periodicity between active serving cells and SCell being activated provided the cell specific reference signals from the active serving cells and the SCells being activated or released are available in the same slot; in case of inter-band SCell activation, TSMTC_MAX is the SMTC periodicity of SCell being activated.

- In FR2, in case of intra-band SCell activation, TSMTC_MAX is the longer SMTC periodicity between active serving cells and SCell being activated provided that in Rel-15 only support FR2 intra-band CA; in case of FR2 inter-band SCell activation, TSMTC_MAX is the SMTC periodicity of SCell being activated.

- TSMTC_MAX is bounded to a minimum value of 10ms.

Trs is the SMTC periodicity of the SCell being activated if the UE has been provided with an SMTC configuration for the SCell in SCell addition message, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If the measObjectNRs having the same SSB frequency and subcarrier spacing configured by MN and SN have different SMTC, Trs is the periodicity of one of the SMTC which is up to UE implementation. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement which involves Trs is applied with Trs = 5ms assuming the SSB transmission periodicity is 5ms. There are no requirements if the SSB transmission periodicity is not 5ms

TFirstSSB: is the time to the end of the first complete SSB burst indicated by the SMTC, or within 5ms if SMTC is not configured, after slot n+THARQ+3msNR slot length.

# Conclusions

In this paper we provided our views on improvements of the RRM specification for Rel-19. Our observations and proposals are summarized as follows:

Proposal 1: RAN4 to develop guidelines for drafting requirements with complex logic, including adopting a pseudo-code approach (e.g. similar to the way RAN2 procedures are specified).

Observation 1: The hierarchy of indent could be indicated by the same approach as in TS 38.321, i.e., 1>, 2>, 3>, etc. for the indent level.

Observation 2: Significant improvement of the readability in TS 38.133 can be achieved by applying proper indentation and reorganizing of the text without the need to rewrite it.

Observation 3: Further optimization beyond proper indentation and text reorganization is possible.

Proposal 2: RAN4 should select a few example sections of TS 38.133 and try to improve readability by proper indentation and restructuring of the existing text.

Proposal 3: RAN4 should develop guidelines how parameters are defined in TS 38.133.

# References

- R4-2408951, Topic summary for [111][140] RRM_Spec_Improvement, RAN4 #111, May 2024

- R4-2410715, WF on [111][140] RRM_Spec_Improvement, RAN4 #111, May 2024

- TS 38.133, TS 38.133, Requirements for support of radio resource management, v18.6.0, June 2024

- R4-2405674, On RRM specification improvements, Qualcomm RAN4 #110bis, April 2024

- R4-2407976, On Rel-19 RRM specification improvements, Qualcomm, RAN4 #111, May 2024