3GPP TSG-RAN WG4 Meeting #112				  R4-2411786

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	8.17.4

Source: 	Moderator(Qualcomm Incorporated)

Title: 	Summary of e-mail discussion on AI/ML Model for CSI testing Option 3 feasibility study

Document for:	Approval

# Introduction

This document captures the e-mail discussion on the AI/ML model that RAN4 will use for the Option 3 feasibility study. The discussion took place on the RAN4 email exploder in June 2024.

# Discussion

## Proposed Models and Parameters

In RAN4#111 it was agreed to focus on two AI/ML models for the CSI decoder, MLP and CNN. The following models and preliminary set of parameters:

MLP model parameters

| Decoder model type | MLP-Mixer |
| --- | --- |
| Embedding dimension | 64 |
| Number of mixer block | 2 |
| Size of message passed across layers | (13, 64) |
| Token mixer hidden dimension | 104 |
| Channel mixer hidden dimension | 512 |

CNN model parameters:

| Model architecture parameters (convolutional) | Model type | CNN (Resnet like design) |
| --- | --- | --- |
|  | Num of Resnet blocks | 6 |
|  | Layer before resnet blocks | 2 layers (one FC layer, one Conv layer) |
|  | Layer after resnet blocks | 1 layers (one conv layer) |
|  | Layer within a resnet block | 2 Conv layer + 1 layer for combination |
|  | Kernel/Channel size within a resnet block | (3,3,128) |

## Discussion – Round 1

### AI/ML Model

Companies are invited to provide their opinions and arguments in the table below:

| Company | Model parameters proposal: | Model preference and pros/cons |
| --- | --- | --- |
| vivo | MLP: Proposal 1 of MLP: Both reference encoder and reference decoder are needed, for both Option 3 and Option 4. It has been agreed in the last meeting that for Option 3, a test decoder needs to be captured in the specs, a reference encoder might be needed, where the reference decoder is the test decoder. For Option 4, it has been agreed that there might be a need to have reference encoder and/or reference decoder. The reference encoder can be used to derive the test decoder and/or requirements and/or to validate the test decoder implementation in the TE. Thus, for better alignment between TE and DUT, both reference encoder and reference decoder are needed, for both Option 3 and Option 4. Proposal 2 of MLP: Details of MLP-Mixer would be needed, e.g., detailed model diagrams or python code. Current model hyper-parameters seem not so clear enough for companies to write the code. |  |
|  | CNN: Proposal 1 of CNN: Both reference encoder and reference decoder are needed, for both Option 3 and Option 4. Proposal 2 of CNN: Detailed model diagrams of CNN are suggested in the below figures to reduce the misalignment between companies, based on the CNN model parameters in WF, including both encoder and decoder. Current model hyper-parameters seem not so clear enough for companies to write the code. So detailed model diagrams are suggested to reduce the misalignment between companies, based on the CNN model parameters in WF. Other parameters of neurons (e.g., conv layers) are default value of Pytorch. The FLOPs and the model size of the encoder is 47M and 112K, respectively, and the FLOPs and the model size of the decoder is 740M and 1.8M, respectively. Fig 1. Detailed model diagrams of encoder. Fig 2. Detailed model diagrams of decoder. The size of message (feature map) passed across layers is also important in reference models and is suggested to add in the table of CNN model parameters. Typical size of feature map is (13,32), which is adopted here. The CNN model parameters for encoder is also provided in below Table 2, where encoder and decoder are symmetric. Since encoder is simpler to decoder, the channel size of encoder could be reduced from 128 to 32. Table 1. CNN model parameters for decoder. Model architecture parameters for decoder (convolutional); Model type; CNN (Resnet like design). Num of Resnet blocks; 6. Layer before resnet blocks; 2 layers (one FC layer, one Conv layer). Layer after resnet blocks; 1 layers (one conv layer). Layer within a resnet block; 2 Conv layer + 1 layer for combination. Kernel/Channel size within a resnet block; (3,3,128). Size of message (feature map) passed across layers; (13,32). Table 2. CNN model parameters for encoder. Model architecture parameters for encoder (convolutional); Model type; CNN (Resnet like design). Num of Resnet blocks; 6. Layer after before resnet blocks; 2 layers (one FC layer, one Conv layer). Layer before after resnet blocks; 1 layers (one conv layer). Layer within a resnet block; 2 Conv layer + 1 layer for combination. Kernel/Channel size within a resnet block; (3,3,32128). Size of message (feature map) passed across layers; (13,32). |  |
| Nokia | MLP: In our understanding, the MLP-Mixer architecture is originated from the following paper: https://arxiv.org/pdf/2105.01601. However, in this paper, the architecture is used for image processing, whereas, in our case, the input for the decoder is the vector (i.e., the compressed CSI/codeword of size K). Therefore, we agree with the view of Apple and Vivo that a scheme of the model is needed for clarity. Below, we describe MLP Mixer decoder architecture for reference: Since the size of FC-Embedding layer is not defined by the parameters explicitly, we put it as 64 x 64. Additionally, we assume the size of CSI feedback is 13 (sub-bands) x 2 x 32 = 13x64. We also support the proposal to define the parameters not only for the decoder but also for the encoder. We can use the same architecture as for the decoder (inversed) in the encoder. However, if any of the companies have better proposal, we are open to discuss. |  |
|  | CNN: In general, the proposal by Vivo is reasonable, except for the following differences (also reflected in the proposed architecture below): - The input size (latent space) does not need to be fixed, unless we have a clear agreement (see below, in Section 2.2.2). We can just use K bits as a generic input size. - It more straightforward to use only single type of scalar quantization, i.e., only 2-bit (instead of the mixture of 2-bit and 3-bit). - We prefer to use Leaky ReLU slightly more, than just ReLU. - Indeed, the parameterization allows different options for Size of message (feature map). We used (13,64) dimensions, which aligns with the MLP-mixer-based structure. - What “feature extractor layer” in decoder stands for in the proposal by Vivo? - We can also define the channel size of the convolutional layer before the resnet blocks in the decoder, and we used 32 in the following example. The illustration of the decoder design, is shown below: Similarly, to the MLP Mixer, we also think that encoder architecture should be agreed for reference and for better alignment of the accuracy in between the companies. Similarly to the decoder, (13,64) can be used for Size of message (feature map) passed across layers. |  |
| QC | We prefer a simpler model that can be implemented by different vendors, and also agreeable to all the companies. |  |
| Xiaomi | CNN model parameters for decoder： Model architecture parameters for encoder (convolutional); Model type; CNN(Resnet like design). Num of Resnet blocks; 4. Layer after resnet blocks; 2 layers（1 avgpooling layer,1 FC layer）. Layer before resnet blocks; 3layers(1 Conv layer, 1 batchnormalization layer, 1 maxpooling layer). Layer within a resnet block; 2 Conv layer + 1 layer for combination. Kernel/Channel size within a resnet block; (3,3,64)/ (3,3,128)/ (3,3,256)/ (3,3,512). Size of message (feature map) passed across layers; (13,32). For encoder, the similar structure is used. |  |
| ZTE | Regarding CNN model, the above all proposals are based on the classic CNN framework. In general, we are fine with this framework. And we still have some different views from our side . Firstly, regarding scalar quantization, we prefer 2bit scalar quantization. The model parameters are as below. Model architecture parameters (convolutional); Model type; CNN (Resnet like design). Num of Resnet blocks; 6. Layer before resnet blocks; 2 layers (one Conv layer,one pooling layer). Layer after resnet blocks; 1 layers (one conv layer, one FC layer). Layer within a resnet block; 2 Conv layer + 1 layer for combination. Kernel/Channel size within a resnet block; (3,3,128). |  |
| CAICT | We also propose reference encoder is provided to make cross check. |  |
| Intel | - Agree with vivo proposal that both reference encoder and reference decoder are needed. Also, agree that it is desirable to agree on detailed diagrams rather than general assumptions only. - To minimize divergence and simplify model down-selection in the next meeting we propose to select single model based on majority preference. From our point of view CNN model can be a reasonable choice. - CNN models proposed by vivo and Nokia can be used as a starting point for the discussion in the next round with the following considerations: a) similar assumptions can be considered for encoder/decoder; b) the Kernel/Channel size within a resnet block can be adjusted ((3,3,12832 or 64) |  |
| CATT | The reference diagrams and detailed parameters proposed by companies for CNN are reasonable. We also have a similar CNN structure. In order to simplify the evaluation as much as possible and to align the understandings, we made the following changes and clarifications: Model architecture parameters (convolutional); Model type; CNN (Resnet like design). Num of Resnet blocks; 6, 4 (But okay to keep 6). Layer before resnet blocks; 2 layers (one FC layer, one Conv layer). Layer after resnet blocks; 1 layers (one conv layer). Layer within a resnet block; 2 Conv layer + 1 layer for combination. Kernel/Channel size within a resnet block; (3,3,128, 64). Size of feature map; (13, 32). Other details: Input size: For the input size which should be aligned with the simulation assumptions, the size is (13, 32, 2) [(subband number, port number, Re. and Im. parts)] for 10M BW with SCS 15kHz. Activation function: Regarding the activation function, since we are not assessing the performance, we are open to use either ReLU or leakyReLU. Also, to simplify the evaluation, we prefer to use ReLU at the beginning. Quantization: 2 bit scalar quantization is preferred. Latent size: We are okay to follow the rule: [X: below 80bits, Y: 100 – 140 bits]. MIMO layer: Follow the simulation assumptions: 1 layer is baseline (preferred), 2 is optional. Evaluation metrics: At least the SGCS should be reported by companies. Reference models: Regarding the reference encoder and decoder, we think they are needed, at least for defining performance requirements. |  |
| CMCC | We share similar view that both decoder and encoder are needed to have better alignment. For size of message (feature map), we use (13,64). |  |
| Huawei, Hisilicon | Regarding to the MLP-mixer, the model structures of encoder and decoder are provided as follows. The complexity of MLP is much lower than that of CNN. Note that eType-II requires less than 2M FLOPs, while the encoder with CNN requires more than 40M FLOPs according to companies’ analysis. Our suggestion is to take account of the complexity for model selection. |  |
| Apple | 1) General: In our view, the goal of the first round of simulations is to align simulation results across companies and therefore it is important to specify most of the parameters and dimensions for both the encoder and decoder. This preliminary architectural specification does not preclude the possibility that different implementations for the encoder/decoder may be investigated in the future to achieve performance/complexity trade-offs that could impact the final specification for the test decoder. 2) CNN architecture: We agree with Vivo’s architecture as a starting point (with the change to Leaky RELU). We prefer to use the notation of (N,C,H,W) as the 4D data tensor that flows through the network. In Vivo’s architecture C=2, H=13, W=32. We prefer this parameterization compared to C=1, H=13, W=64. The parameter N is the mini batch size, and we propose that each company reports N along with the simulation results. 3) Batch Normalization: During training this layer keeps running estimates of its computed mean and variance, which are then used for normalization during evaluation. The running estimates are kept with a value of momentum. Moreover, this layer can be enabled to track the running mean and variance, and when it is disabled, this module does not track such statistics. We propose that each company reports the exact functionality of batch normalization (enabled/disabled tracking and momentum value) | ) |

### Other Evaluation Parameters

| Company | Evaluation parameters and value Other considerations | Arguments to support the proposal |
| --- | --- | --- |
| vivo | Proposal 1: In this e-mail discussion, the evaluation parameters, the derived reference model(s), and the feasibility evaluation can be used for both Option 3 and Option 4. Proposal 2: RAN4 may select an encoder and decoder pair based the simulation results, and then check whether companies can train encoder based on the selected decoder, or can train decoder based on the selected encoder, without obvious performance loss compared to the encoder and decoder pair. Proposal 3: There are two sets of simulation assumptions, where one set is for 2GHz and the other set is for 4GHz. RAN4 may choose one set, or mix the dataset with both two sets, for further alignment. | For Proposal 1, it has been discussed above that both reference encoder and reference decoder are needed, for both Option 3 and Option 4. Thus, the evaluation parameters, the derived reference model(s), and the feasibility evaluation in this e-mail discussion can be used for both Option 3 and Option 4. For Proposal 2, moderator suggested to check whether we can converge on a model and whether it would be feasible for other companies to train its own model to pair with the aligned model. Considering the alignment of both reference encoder and reference decoder, we have Proposal 2. For Proposal 3, currently, there are two sets of simulation assumptions, where one set is for 2GHz and the other set is for 4GHz. From Subsection 6.2.2.2 in TR 38.843, the generalization performance of different carrier frequencies may be good. But for better alignment between companies, it is suggested that RAN4 may choose one set of simulation assumptions, or mix the dataset with both two sets. |
| Ericsson | Latent space X 64 bits, Y 112 bits | 1. The goal of this evaluation is to have convergence/same understanding of the performance of a certain model. To have such understanding, it is better to have an one agreed size, inside a category. The values can be selected based on the payload sizes required for the legacy eTypeII, e.g., (PC1, PC3). I.e., for category X, 64 bits can be used. For category Y, 112 bits can be used. 2. For the case of eigenvector used as the input/output type, reporting size (in bits) comes from the latent space size multiplied by the quantization size. I.e., one reporting size may come from different latent space size and quantization size combinations. For easier alignment, it is better to agree on the latent space size(s) and the quantization size(s). For example, 4-bit quantization can be used. I.e., the latent space size for cat. X and cat. Y will be 16 and 28, respectively, if 64 bits and 112 bits are agreed to be used. 3. In addition, it needs to be further clarified how these different latent space sizes can be obtained. I.e., whether two models will be evaluated (one model for each latent space size), or whether one common model will be used, and the different latent space size is achieved by, e.g., puncturing. As the mechanism to adaptively change the latent space size (e.g., via puncturing) may take another discussion, having a model for each latent space size may serve as a starting point. Note that this should not hinder discussion on having a more general model in the future, as the basic assumption should be that the number of the models should be as low as possible. |
| Ericsson | Number of models for 2 layers and rank 2 Propose per layer model assumption | The assumptions have 2 layers as optional. It would be good to clarify whether to assume the same model for both layers or per layer model. |
| Nokia | We also think that it will be beneficial to select the concrete values of Latent /reporting size out of [X: below 80bits, Y: 100 – 140 bits]. Proposal 1: RAN4 to use in the study two Latent /reporting sizes: 64 (for cat. X) and 128 (for cat. Y) bits. 112 bit for cat. Y is also acceptable to us. It seems that we do not have an explicit agreement on the size of uncompressed CSI and accuracy/performance metric to be reported. Therefore, we propose, additionally: Proposal 2: RAN4 to assume that the size of CSI feedback (uncompressed) is 13 (sub-bands) x 2 x 32 = 13x64. Proposal 3: Companies to report at least SGCS for the trained encoder-decoder pair. Additionally, in our view the most straightforward approach would be to start with a single encoder-decoder pair trained for each default parametrization, i.e., if we keep two Model types and two Reporting sizes, it will be 4 models in total. |  |
| ZTE | Latent space size: 64 and 128. Further clarification of the test metric. | Regarding latent space, it’s better to define a value for different candidate options of [X: below 80bits, Y: 100 – 140 bits]. . Different latent sizes could have different potential information, and will lead to different performance. Also test metric is still not explicit. If RAN4 wants to align results or achieve some agreements. Test metric should be further clarification |
| CAICT | We are fine to have concreted values of latent/reporting size, e.g., X:64, Y:112. Simulation bandwidth: CSI feedback Sub-bands number=13 could be added. For simulation frequency: 2GHz (baseline), 4GHz (optional) |  |
| Intel | 1) General: the goal of the task for the next meetings is to understand feasibility. Therefore, we think it is important to downselect some reasonable subset of parameters for evaluations rather than trying to cover a multitude of scenarios. It is preferable to downselect specific options for parameters, where we have several alternatives. 2) Latent /reporting sizes: Propose to agree on a single value instead of going 2 different settings, which will double the number of simulations (e.g. 104 bits) 3) Frequency Range: prefer to down-select a single value or at least make one of values as baseline and the other as optional (e.g., 2GHz as baseline) 4) Traffic load: Does not have impact on training and can be removed. The interf. structure on data and CSI-RS is different and load is not expected to have impact on the compression performance. 5) CSI feedback assumption: We do not expect impact on training unless we consider prediction. Recommend to remove from the table. 6) Channel estimation: Prefer to keep “ideal” only to minimize divergence of results. Otherwise, the specific configuration for CSI-RS as well as noise modelling assumptions in SLS need to be discussed/agreed. 7) Number of MIMO layers: as the overall goal is to understand feasibility of testing, it is important to minimize divergence of assumptions. Recommend 1 MIMO layer only. 8) Metrics: companies are recommended to report NMSE (baseline) and SGCS (optional) |  |
| CMCC | For latent/reporting size, X: 60 bits, Y: 120 bits. We are also fine with X:64, Y:112. For metrics, FLOPS and SGCS with comparable overhead are suggested. |  |
| Apple | 1) Latent space: We suggest agreeing on a single value (64 bits) for latent space. Supporting 2 values raises the question of having a different model per latent dimension or how to adapt 2) Number of MIMO layers: To converge on the number of assumptions to evaluate the feasibility of testing Option 3, Option 4, we propose to focus on 1 MIMO layer only. 3) Metrics: We propose to use a loss/evaluation function the NMSE (baseline). We can have the SGCS as an optional evaluation metric 4) Batch Normalization: During training this layer keeps running estimates of its computed mean and variance, which are then used for normalization during evaluation. The running estimates are kept with a value of momentum. Moreover, this layer can be enabled to track the running mean and variance, and when it is disabled, this module does not track such statistics. We propose that each company reports the exact functionality of batch normalization (enabled/disabled tracking and momentum value) 5) Hyperparameters: We need to agree on another set of training options for aiding the performance simulation alignment. The set of training options can include: - Solver (Adam, RMS prop, SGD) - Regularization (L2, early stoping etc) - MiniBatch Size - Initial Learning range - Max Epochs, validation patience Proposal (1): Companies to report the details of training options such as: batch size, solver type, regularization technique, number of training epochs, details on batch normalization along with their simulation results |  |

## Discussion – Round 2

### AI/ML Model proposed parameters

Large majority of companies agree to take CNN as the basis for the AI/ML model and also commented that encoder and decoder structures could be the same for simplicity. Vivo had a comprehensive proposal and most companies seem to support it so it is proposed to take this as the basis for further discussion. Based on the inputs from Round 1, the following set of parameters is proposed:

Take CNN as the basic model for encoder and decoder

Fig 1. Detailed model diagrams of encoder.      Fig 2. Detailed model diagrams of decoder.

Table 1. CNN model parameters for encoder.

| Model architecture parameters for encoder (convolutional) | Model type | CNN (Resnet like design) |
| --- | --- | --- |
|  | FLOPs | 47M (vs. 1.6M for eTypeII) |
|  | Model size | 448Kbyte |
|  | Num of Resnet blocks | 6 |
|  | Layer after resnet blocks | 2 layers (one FC layer, one Conv layer) |
|  | Layer before resnet blocks | 1 layer (one conv layer) |
|  | Layer within a resnet block | 2 Conv layer and direct combination |
|  | CNN kernel parameters | - dilation =1 (which is the default parameter) - padding type = zero padding (which is the default parameter) - The value of groups =1 (which is default parameter) |
|  | Kernel/Channel size within a resnet block | (3,3,32) |
|  | Size of message (feature map) passed across layers | (13,32) |

Table 2. CNN model parameters for decoder.

| Model architecture parameters for decoder (convolutional) | Model type | CNN (Resnet like design) |
| --- | --- | --- |
|  | FLOPs | 740M |
|  | Model size | 7.2Mbyte |
|  | Num of Resnet blocks | 6 |
|  | Layer before resnet blocks | 2 layers (one FC layer, one Conv layer) |
|  | Layer after resnet blocks | 1 layer (one conv layer) |
|  | Layer within a resnet block | 2 Conv layer and direct combination |
|  | CNN kernel parameters | - dilation =1 (which is the default parameter) - padding type = zero padding (which is the default parameter) - The value of groups =1 (which is default parameter) |
|  | Kernel/Channel size within a resnet block | (3,3,128) |
|  | Size of message (feature map) passed across layers | (13,32) |

Other model related parameters:

| Activation function | 1:ReLU or 2:LeakyReLU |
| --- | --- |
| Latent message size | 32 float values |
| Payload size, bits | 64 per layer. |
| Quantization | 2 bits |

System parameters:

| Frequency range | 2GHz(4GHz optional) |
| --- | --- |
| MIMO Layers | 1 layer |
| Sub-bands | 13 |
| Channel estimation | ideal |
| Metrics | NMSE and SGCS of AI CSI feedback and eTypeII CSI feedback, both relative to ideal CSI. |

Training hyperparameters (not for alignment, but for companies’ reporting)

TBA

### Companies comments

| Company name | Comments |
| --- | --- |
| Huawei, Hisilicon | To have a full picture of the model under exercise, the FLOPs and model size have to be captured. For the payload size, companies proposed to select a value based on the payload sizes required for the legacy eTypeII, e.g., (PC1, PC3). For the latent message size (before quantization), according to the diagram, the value is 28. Since 64 is selected as the payload size, the latent message size is supposed to be 32 instead. For the metric, according to companies’ assumption of the model input format, eigenvector is used instead of the raw channel. Therefore, it is not reasonable to use NMSE. For the benchmark of the metric, since we directly borrow the system-level simulation assumption from RAN1, eTypeII has also to be reused as the benchmark. Another benefit to reuse the benchmark is that if companies cannot align with each other regarding to the AIML, we can check if the training/testing dataset is not aligned (as reflected by the performance of eTypeII) at the very beginning. |
| vivo | Proposal 1: Detailed model diagrams of CNN are updated in the below figures, based on the moderator proposal, including both encoder and decoder. We support moderator proposal on the model parameters and simulation system parameters. It is noted that there are some differences between the proposed detailed model diagrams of CNN and other model related parameters. To facilitate the alignment between companies, detailed model diagrams of CNN are updated based on the moderator proposal. The main changes are: 1. The quantization part is updated to 64 latent size and 2-bit quantization. In previous figure, 3-bit quantization is used to match up the odd number of latent size. Since the latent size is even, 2-bit quantization is enough. 2. Thanks for Nokia’s comment on “feature extractor layer”. This was used for Resnet blocks. It has been deleted in the updated version. 3. We are OK for both ReLU and LeakyReLU, since they have the same performance. In the below figures, “ReLU” represents both ReLU and LeakyReLU. Fig 1. Detailed model diagrams of encoder. Fig 2. Detailed model diagrams of decoder. Proposal 2: The feasibility conclusion through the simulation campaign of test options can be draw for both Option 3 and Option 4. For feasibility studies of AI/ML model for test options, we believe the conclusion can be drawn for both Option 3 and Option 4. The difference between Option 3 and Option 4 is how the AI/ML model(s) is captured in the spec. The evaluation should be same in terms of performance. We understand the evaluation focus on Option 3, but it is important to apply the same conclusion to Option 4, unless essential issue is identified. Otherwise, the entire progress of testability for test options will be delayed. |
| Nokia | Our original proposal was to have two payload sizes for a single layer: X: 64 and Y: 128. Now, the latest version states that we use 64 bits for one layer and 112 for two layers. We are OK to consider only a single payload size for one layer (i.e., 64bits) but why shall we use a different payload per layer? Our proposal is to agree just on a single payload per layer, i.e., 64 bits. We acknowledge that some difference in the baseline performance is possible in between the companies. Reporting fore reference of SGCS for the eTypeII would be useful. Therefore, we prefer to report SGCS of eTypeII CSI feedback in addition to SGCS of AI CSI, both relative to the ideal CSI. Thanks, Vivo for updating the decoder/encoder diagrams! Look OK to us. Additionally, the assumption to use ideal channel estimate should be reflected in the System-level simulation parameters table below, in Section 3 to avoid comfusion. |
| Xiaomi | Generally, we are fine with CNN model proposed in 2nd round. For Num of Resnet blocks, fine with 6. For activation function, fine with both Leaky ReLU or ReLU. 2bit scalar quantization is OK. For latent message size, fine with 32 or 64. |
| Samsung | For the above CNN structure and parameters for 2nd round alignment, we have the following comments: 1. Okay for both ReLU or Leaky ReLU for activation function used, but in theory for Leaky ReLU, the parameter alpha needs to be fixed for aligned results. For simplicity and no big difference in practice for these two activation functions, suggest to use ReLU directly. The current figure is okay. 2. For CNN kernel, we would like to clarify more to make sure the companies’ understandings are aligned for some parameters not mentioned, which are added to the table for clarification: - The parameter dilation =1 (which is the default parameter) and our understanding is we can directly use the default value. - The method of padding (i.e., padding type) = zero padding (which is the default parameter), and our understanding is we can directly use the default value. - The value of groups =1 (which is default parameter), and our understanding is we can directly use the default value. 3. The description “2 Conv layer + 1 layer for combination” for Resnet block is not clear enough, since no additional upscaling operation is needed here, so strictly speaking there is no “1 layer” for combination. Suggest to change to “2 Conv layer and direct combination”, which is more precise. 4. Other than the above comments, the illustrated CNN structure is clear, while we have not gotten the logic behind the upscaling Conv layer used for both encoder and decoder. Normally for upscaling purpose, kernel 1*1 could be okay. For performance alignment, we don’t deny an aligned structure works, while to further clarify this could be better. For other parameters: 1. MIMO layer: Since it is the common understanding, the works here are just to check “the feasibility of deriving a test decoder” under some proposed parameters agreed only for this feasibility study. Then, the simpler for parameter is preferred. From that perspective, we also prefer to only have MIMO layer=1. 2. The evaluation metrics: we are okay to consider only SGCS, while also okay to report both. 3. The training hyperparameters are suggested to be reported (which is not yet captured), for which Apple gives the good list, based on that, we suggest the following by highlighting the differences: - Float type used in training/inference (Float32, Float16, combined Float32 and Float16) - Solver (Adam, RMS prop, SGD) - Regularization (L2, early stoping, etc) - MiniBatch Size - Initial Learning rate, strategy - Max Epochs, validation patience - Drop out for each layer, if non-default value is used. |
| Ericsson | Regarding the activation function, appreciate that ReLU and leaky ReLu are likely to give the same performance, but then it would be good to tie down a common assumption to eliminate the risk of having to check on divergence later on. As Samsung suggest, ReLU may be best to assume as it is simplest. We would encourage companies to consider 2 layers too to check if that impacts convergence, but 1 layer as baseline. We should also clarify that we are assuming 1 model per layer in the 2 layer case. We think it could also be useful to have some common assumption, at least to an order of magnitude on the training set size. RAN1 has considered 100k-800k. Propose we assume 300-400k for this exercise. |
| Intel | Payload size: Agree with Nokia comments and 64 bits look fine SGCS metric: SGCS shall be relative to ideal CSI and Nokia updates are fine for us. eTypeII CSI feedback SGCS metric does not bring much value for feasibility analysis and can be considered as optional. AI/ML model: we are fine with moderator proposal to use CNN model and model updates proposed by vivo. |
| ZTE | CNN structure and parameters are fine for us. Payload size: We are agree with 64bit for one layer. Test metric: SGCS is fine for us. Also we can compare performance gain between AI SGCS and eTypeII CSI feedback SGCS. |

# Final proposal

Based on the two rounds of discussions, the following parameters are proposed:

Fig 1. Detailed model diagrams of encoder.

Fig 2. Detailed model diagrams of decoder.

Table 1. CNN model parameters for encoder.

| Model architecture parameters for encoder (convolutional) | Model type | CNN (Resnet like design) |
| --- | --- | --- |
|  | FLOPs | 47M (vs. 1.6M for eTypeII) |
|  | Model size | 448Kbyte |
|  | Num of Resnet blocks | 6 |
|  | Layer after resnet blocks | 2 layers (one FC layer, one Conv layer) |
|  | Layer before resnet blocks | 1 layer (one conv layer) |
|  | Layer within a resnet block | 2 Conv layer and direct combination |
|  | CNN kernel parameters | - dilation =1 (which is the default parameter) - padding type = zero padding (which is the default parameter) - The value of groups =1 (which is default parameter) |
|  | Kernel/Channel size within a resnet block | (3,3,32) |
|  | Size of message (feature map) passed across layers | (13,32) |

Table 2. CNN model parameters for decoder.

| Model architecture parameters for decoder (convolutional) | Model type | CNN (Resnet like design) |
| --- | --- | --- |
|  | FLOPs | 740M |
|  | Model size | 7.2Mbyte |
|  | Num of Resnet blocks | 6 |
|  | Layer before resnet blocks | 2 layers (one FC layer, one Conv layer) |
|  | Layer after resnet blocks | 1 layer (one conv layer) |
|  | Layer within a resnet block | 2 Conv layer and direct combination |
|  | CNN kernel parameters | - dilation =1 (which is the default parameter) - padding type = zero padding (which is the default parameter) - The value of groups =1 (which is default parameter) |
|  | Kernel/Channel size within a resnet block | (3,3,128) |
|  | Size of message (feature map) passed across layers | (13,32) |

Other model related parameters:

| Activation function | ReLU |
| --- | --- |
| Latent message size | 32 float values |
| Payload size, bits | 64 per layer. |
| Quantization | 2 bits |

System parameters:

| Frequency range | 2GHz (4GHz optional) |
| --- | --- |
| MIMO Layers | 1 layer (2 layers optional, companies to |
| Sub-bands | 13 |
| Channel estimation | ideal |
| Metrics | SGCS of AI CSI feedback and eTypeII CSI feedback, both relative to ideal CSI. |

Companies are also invited to provide the training parameters used.

# Conclusion

No further comments were received after the final proposal so interested companies are invited to provide simulations results based on the parameters shown in Section 3.

# References:

#### Agreements about the parameters used in the simulations for checking the feasibility of testing options for CSI compression and prediction:

Agreement:

Parameters agreed are just for the feasibility study of testing options.

If/when RAN4 discusses requirement definition, RAN4 will define a new test decoder which may or may not reuse any of the parameters agreed in the feasibility study.

#### Agreements regarding the process for checking the feasibility of deriving a test decoder for Option 3

The following options were discussed:

Test decoder derivation procedure:

Option 1

companies bring encoder + decoder set based on agreed parameters. RAN4 chooses one of the decoders and interested companies further check if an encoder can be trained with this decoder to obtain similar performance/complexity (or other evaluation criteria)

Option 2

companies bring training data for encoder + decoder pair, interested companies train an encoder + decoder pair based on the aggregated dataset from all companies to check decoder derivation feasibility and performance/complexity (or other evaluation criteria)

RAN4 agrees to work on option 1.

Interested companies are invited to bring further feasibility analysis for Option 2

#### Simulation parameters for verifying the feasibility of aligning model among companies

System-level simulation parameters (from TR):

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD, OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) |
| Frequency Range |  | FR1 only, [2GHz, 4GHz] |
| Inter-BS distance |  | 200m |
| Channel model |  | According to TR 38.901 |
| Antenna setup and port layouts at gNB |  | Companies need to report which option(s) are used between - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for (rank 1-4) |
| BS Tx power |  | 44dBm for 20MHz |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | Baseline: 15kHz for 2GHz; Optional: 30kHz for 4GHz |
| Simulation bandwidth |  | Baseline: 10 MHz for 15kHz Optional: 20 MHz for 30kHz |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | SU-MIMO |
| MIMO layers |  | Baseline: 1 Optional: 2 |
| CSI feedback |  | Feedback assumption at least for baseline scheme - CSI feedback periodicity (full CSI feedback): 5 ms (baseline) - Scheduling delay (from CSI feedback to time to apply in scheduling): 4 ms |
| Traffic load (Resource utilization) |  | Baseline: 50% Optional: 20/70% |
| UE distribution |  | CSI compression: 80% indoor (3 km/h), 20% outdoor (30 km/h) |
| UE receiver |  | MMSE-IRC as the baseline receiver |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Realistic or ideal channel estimation |

Additional parameters of the encoder/decoder and feedback based on RAN1 simulation parameters by companies in RAN1 (from Excel):

| Assumptions |  | Value |
| --- | --- | --- |
| CSI generation part | AI/ML model backbone | MLP, CNN, Transformer |
|  | Pre-processing | SVD to get channel eigenvectors |
| CSI reconstruction part | AI/ML model backbone | [MLP, CNN, Transformer] |
| Common description | Input type | Eigenvectors of channel matrix |
|  | Output type | Eigenvectors of channel matrix |
|  | Quantization /dequantization method | Scalar quantization |
| Dataset description |
|  | Train/Test | 10% of the training dataset as baseline |
|  | Ground-truth CSI quantization method (including scalar/codebook based quantization, and the parameters) | Floating point (float 32) |
| Other parameters | Laten/reporting size | [X: below 80bits, Y: 100 – 140 bits] |