3GPP TSG-RAN4 Meeting #112	R4-2411791

# Maastricht, Netherlands,  Aug 19 – 23, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draftCR | rev |  | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR – Test cases for RSTD measurement delay and accuracy with eDRX > 10.24s in RRC_IDLE, Sets 10-4, 10-10, 11-4, 11-10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm, Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Perf |  |  |  |  |  | Date: |  |  | 8/9/2024 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Add test cases to verify RSTD measurement delay and accuracy requirements with eDRX > 10.24s in RRC_IDLE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the following test cases: - RSTD measurement reporting delay with eDRX > 10.24s in RRC_IDLE state for non-RedCap UE in FR2 (clause A.7.X.Y.2) - RSTD without FH measurement reporting delay with eDRX > 10.24s in RRC_IDLE state for RedCap UE in FR2 (clause A.17.X.C.2) - RSTD measurement accuracy with eDRX > 10.24s in RRC_IDLE state for non-RedCap UE in FR2 (clause A.7.X1.Y.2) - RSTD without FH measurement accuracy with eDRX > 10.24s in RRC_IDLE state for RedCap UE in FR2 (clause A.17.X1.C.2) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases to verify measurement delay and accuracy requirements for PRS measurements in RRC_IDLE would be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.7.X.Y.2, A.17.X.C.2, A.7.X1.Y.2, A.17.X1.C.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------START OF CHANGE----------------------------

#### A.7.X.Y.2	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA in RRC_IDLE state with eDRX > 10.24s

##### A.7.X.Y.2.1	Test purpose and environment

The purpose of the test is to verify the measurement requirements specified in clause 4.5.2.5 for RSTD measurements in RRC_IDLE with eDRX. The tests are conducted under AWGN propagation condition with the UE operating in FR2 stand-alone mode and configured to perform RSTD measurements on a single positioning frequency layer (PFL) in FR2.

The supported test configurations are listed in Table A.7.X.Y.2.1-1.

Table A.7.X.Y.2.1-1: Supported test configurations

| PCell configuration | Description |
| --- | --- |
| 1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

There are three cells in the test: Cell 1 (PCell and RSTD reference cell), Cell 2 (neighbor cell) and Cell 3 (neighbor cell). All cells are on the same RF channel in FR2.

The test consists of two consecutive time intervals, with duration of T1 and T2. The UE shall be in RRC_CONNECTED state during T1 and released to RRC_IDLE state before the start of T2. All cells transmit PRS only during the second time interval of duration T2.

Note: The information on when PRS is muted is conveyed to the UE using PRS muting information.

The NR-DL-TDOA-ProvideAssistanceData and NR-DL-TDOA-RequestLocationInformation as defined in TS 37.355 [34, clause 6.5.12], shall be provided to the UE during T1. The last TTI of the last message shall be provided to the UE at least T ms before the start of T2, where T = 50 ms is the maximum processing time of the DL-TDOA assistance data and location information request.

The beginning of the time interval T2 shall be aligned with the start of the first PRS resource instance received after the UE has transitioned to RRC_IDLE.

The general test parameters and cell specific test parameters are as given in Table A.7.X.Y.2.1-2 and Table A.7.X.Y.2.1-3 respectively.

Table A.7.X.Y.2.1-2: General test parameters

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Reference cell |  | 1 | Cell 1 | Reference cell is the cell in the DL-TDOA assistance data with respect to which the RSTD measurement is defined, as specified in TS 38.215 [4] and TS 37.355 [34]. The reference cell is the PCell in this test case. |
| Neighbour cells |  | 1 | Cell 2 and Cell 3 | Cell 2 and Cell 3 appear at the first and second places in the neighbour cell list in the DL-TDOA assistance data. |
| RF Channel Number |  | 1 | 1 | For Cell 1, Cell 2 and Cell 3 |
| BWchannel | MHz | 1 | 100: NRB,c = 66 |  |
| SSB configuration |  | 1 | SSB.3 FR2 |  |
| SMTC configuration |  | 1 | SMTC.1 |  |
| CP length |  | 1 | Normal |  |
| DRX | s | 1 | 0.64 |  |
| eDRX cycle length (for both RAN and CN) | s | 1 | 40.96 |  |
| PTW length (for both RAN and CN) | s | 1 | 1.28 |  |
| Time offset between serving and neighbour cells | s | 1 | Cell 2 to Cell 1: 0 Cell 3 to Cell 1: 3 | Synchronous cells |
| Expected RSTD | s | 1 | Cell 2: 3 Cell 3: 3 Other neighbour cells: randomly between -3 and 3 | The expected RSTD is what is expected at the receiver. The corresponding parameter in the DL-TDOA assistance data specified in TS 37.355 [34] is the expectedRSTD indicator |
| Expected RSTD uncertainty | s | 1 | 5 |  |
| Number of cells provided in DL-TDOA assistance data |  | 1 | 4 | Including the reference cell |
| PRS muting info |  | 1 | Cell 1: ‘10’ Cell 2: ‘01’ Cell 3: ‘10’ | Correponds to NR-MutingPattern-r16 in DL-PRS-MutingOption1-r16 defined in TS 37.355 [24] |
| PRS resource RE offset |  | 1 | Cell 1: 0 Cell 2: 0 Cell 3: 1 | Cell 1 and Cell 3 are configured with different resource offsets |
| T1 | s | 1 | 5 |  |
| T2 | s | 1 | [41] |  |

Table A.7.X.Y.2.1-3: Cell specific test parameters

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |  | T1 | T2 |
| AoA setup |  | 1 | Setup 1 as specified in clause A.3.15 |  |  |  |  |  |  |
| Beam AssumptionNote 5 |  | 1 | Rough |  | Rough |  |  | Rough |  |
| TDD configuration |  | 1 | TDDConf.3.1 |  | TDDConf.3.1 |  |  | TDDConf.3.1 |  |
| PDSCH RMC configuration |  | 1 | SR.3.1 TDD |  | N/A |  |  | N/A |  |
| RMSI CORESET RMC configuration |  | 1 | CR.3.1 TDD |  | N/A |  |  | N/A |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.3.1 TDD |  | N/A |  |  | N/A |  |
| OCNG PatternsNote 1 |  | 1 | OP.1 |  | OP.1 |  |  | OP.1 |  |
| EPRE ratio of PSS to SSS | dB | 1 | 0 |  | 0 |  |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1, 4 |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1, 4 |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PRS to SSS |  |  |  |  |  |  |  |  |  |
| TRS Configuration |  | 1 | TRS.2.1 TDD |  | N/A |  |  | N/A |  |
| Initial BWP configuration |  | 1 | DLBWP.0.1 ULBWP.0.1 |  | N/A |  |  | N/A |  |
| Active DL BWP configuration |  | 1 | DLBWP.1.1 |  | N/A |  |  | N/A |  |
| Active UL BWP configuration |  | 1 | ULBWP.1.1 |  | N/A |  |  | N/A |  |
| PRS configuration |  | 1 | PRS.1.1 FR2 |  | PRS.1.1 FR2 |  |  | PRS.1.1 FR2 |  |
| PRS muting info (dl-PRS-MutingOption1) |  | 1 | ‘10’ |  | ‘01’ |  |  | ‘10’ |  |
| Note 2 | dBm/SCS | 1 | -89 |  |  |  |  |  |  |
| PRS | dB | 1 | -Infinity | -6 | -Infinity | -13 |  | -Infinity | -13 |
| PRS | dB | 1 | -Infinity | -6 | -Infinity | -13 |  | -Infinity | -13 |
| PRP Note 3 | dBm/SCS kHz | 1 | -Infinity | -95 | -Infinity | -102 |  | -Infinity | -102 |
| Io Note 3 (on symbols where PRS is not allocated) | dBm/95.04 MHz | 1 | N/A | -58.7 | N/A | -58.7 |  | N/A | -58.7 |
| Propagation Condition |  | 1 | AWGN |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that active cell (Cell 1) is fully allocated and a constant total transmitted power spectral density is achieved on all OFDM symbols except those in which PRS is allocated. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: PRP and Io levels have been derived from other parameters and they are provided for information only. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 5: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation Note 6: Calculation of Es/Iot includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1dB for UE multi-band relaxation factor ΔMBP from TS 38.101-2 [19] Table 6.2.1.3-4. [Note 7: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone] [Note 8: As observed with 0 dBi gain antenna at the centre of the quiet zone] |  |  |  |  |  |  |  |  |  |

##### A.7.X.Y.2.2	Test requirements

The RSTD measurement time shall fulfill the requirements specified in clause 4.5.2.5.

The UE shall perform and report the RSTD measurements for Cell 1, Cell 2 and Cell 3 within the specified measurement period duration starting from the beginning of time interval T2.

NOTE:	The actual overall delays measured in the test may be higher than the time duration above because of the uncertainty in acquiring the first available PRACH occasion to transition to RRC_CONNECTED state to report the measurements.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the time duration above because of TTI insertion uncertainty of the measurement report in DCCH.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported RSTD measurement for each correct event shall be within the RSTD reporting range specified in clause 10.1.23.3.

----------------------END OF CHANGE----------------------------

----------------------START OF CHANGE----------------------------

#### A.17.X.C.2	NR RSTD without FH measurement reporting delay test case for single positioning frequency layer in FR2 SA in RRC_IDLE state with eDRX > 10.24s

##### A.17.X.C.2.1	Test purpose and environment

The purpose of the test is to verify the measurement requirements specified in clause 4.6.2.5 for RSTD measurements in RRC_IDLE with eDRX. Refer to clause A.7.X.Y.2.1 for test configuration and procedure.

##### A.17.X.C.2.2	Test requirements

The RSTD measurement time shall fulfill the requirements specified in clause 4.6.2.5.

The UE shall perform and report the RSTD measurements for Cell 1, Cell 2 and Cell 3 within the specified measurement period duration starting from the beginning of time interval T2.

NOTE:	The actual overall delays measured in the test may be higher than the time duration above because of the uncertainty in acquiring the first available PRACH occasion to transition to RRC_CONNECTED state to report the measurements.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the time duration above because of TTI insertion uncertainty of the measurement report in DCCH.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported RSTD measurement for each correct event shall be within the RSTD reporting range specified in clause 10.1.23.3.

----------------------END OF CHANGE----------------------------

----------------------START OF CHANGE----------------------------

#### A.7.X1.Y.2	RSTD measurement accuracy test case for single positioning frequency layer in FR2 SA in RRC_IDLE state with eDRX > 10.24s

##### A.7.X1.Y.2.1	Test purpose and environment

The purpose of this test is to verify that RSTD measurements performed in RRC_IDLE with eDRX satisfy the measurement accuracy requirements specified in clause 10.1.23.2. The tests are conducted under AWGN propagation condition with the UE operating in FR2 stand-alone mode and configured to perform RSTD measurements on a single positioning frequency layer (PFL) in FR2.

The supported test configurations are listed in Table A.7.X1.Y.2.1-1.

Table A.7.X1.Y.2.1-1: Supported test configurations

| PCell configuration | Description |
| --- | --- |
| 1 | 120 kHz SSB SCS, 200 MHz bandwidth, TDD duplex mode |

There are two synchronous cells in the test: Cell 1 and Cell 2. Cell 1 is the reference as well as the PCell. Cell 2 is a neighbour cell. Both cells are on the same NR RF channel in FR2.

The NR-TDOA-ProvideAssistanceData and NR-TDOA-RequestLocationInformation message as defined in TS 37.355 shall be provided to the UE before the start of the test. The test duration should be larger than the UE measurement period as defined in clause 4.5.2.5.

The general test parameters and cell specific test parameters are as given in Table A.7.X1.Y.2.1-2 and Table A.7.X1.Y.2.1-3 respectively.

Table A.7.X1.Y.2.1-2: General test parameters

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Reference cell |  | 1 | Cell 1 |  |
| Neighbour cell |  | 1 | Cell 2 |  |
| RF Channel Number |  | 1 | 1 | For Cell 1 and Cell 2 |
| BWchannel | MHz | 1 | 200: NRB,c = 132 |  |
| SSB configuration |  | 1 | SSB.3 FR2 |  |
| SMTC configuration |  | 1 | SMTC.1 |  |
| CP length |  | 1 | Normal |  |
| DRX | s | 1 | 0.64 |  |
| eDRX cycle length | s | 1 | 40.96 |  |
| PTW length | s | 1 | 1.28 |  |
| Time offset between serving and neighbour cell | s | 1 | 3 | Synchronous cells |
| Expected RSTD | s | 1 | 3 |  |
| Expected RSTD uncertainty | s | 1 | 5 |  |

Table A.7.X1.Y.2.1-3: Cell specific test parameters

| Parameter | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| AoA setup |  | Setup 1 as specified in clause A.3.15 |  |  |  |
| Beam AssumptionNote 4 |  | Rough |  | Rough |  |
| TDD configuration |  | TDDConf.3.1 |  | TDDConf.3.1 |  |
| PDSCH RMC configuration |  | SR.3.1 TDD | - | SR.3.1 TDD | - |
| RMSI CORESET RMC configuration |  | CR.3.1 TDD | - | CR.3.1 TDD | - |
| Dedicated CORESET RMC configuration |  | CR.3.1 TDD | - | CR.3.1 TDD | - |
| Control channel RMC |  | CCR.3.1 TDD | - | CCR.3.1 TDD | - |
| OCNG Patterns |  | OP.1 | OP.1 | OP.1 | OP.1 |
| EPRE ratio of PSS to SSS | dB | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| TRS configuration |  | TRS.2.1 TDD | - | TRS.2.1 TDD | - |
| Initial BWP configuration |  | DLBWP.0.1 ULBWP.0.1 | - | DLBWP.0.1 ULBWP.0.1 | - |
| Dedicated BWP configuration |  | DLBWP.1.1 ULBWP.1.1 | - | DLBWP.1.1 ULBWP.1.1 | - |
| PRS configuration |  | PRS.1.1 FR2 | PRS.1.1 FR2 | PRS.1.2 FR2 | PRS.1.2 FR2 |
| PRS Resource slot offset | slot | 0 | 4 | 0 | 4 |
| Note 2 | dBm/SCS | -89 |  |  |  |
| PRS | dB | -6 | -13 | -6 | -13 |
| PRS | dB | -6 | -13 | -6 | -13 |
| PRP Note 3 | dBm/SCS kHz | -95 | -102 | -95 | -102 |
| Io Note 3 (on symbols where PRS is not allocated) | dBm/190.08 MHz | -55.86 | -55.86 | -55.86 | -55.86 |
| Propagation conditions |  | AWGN |  |  |  |
| Antenna configuration |  | 1x2 | 1x2 | 1x2 | 1x2 |
| Note 1: OCNG shall be used such that active cell (Cell 1) is fully allocated and a constant total transmitted power spectral density is achieved on all OFDM symbols except those in which PRS is allocated. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: PRP and Io levels have been derived from other parameters and they are provided for information only. They are not settable parameters themselves. Note 4: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation Note 5: Calculation of Es/Iot includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1dB for UE multi-band relaxation factor ΔMBP from TS 38.101-2 [19] Table 6.2.1.3-4. [Note 6: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone] [Note 7: As observed with 0 dBi gain antenna at the centre of the quiet zone] |  |  |  |  |  |

##### A.7.X1.Y.2.2	Test requirements

The reported RSTD measurements shall fulfill the absolute accuracy requirements specified in clause 10.1.23.2.

----------------------END OF CHANGE----------------------------

----------------------START OF CHANGE----------------------------

#### A.17.X1.C.2	RSTD without FH measurement accuracy test case for single positioning frequency layer in FR2 SA in RRC_IDLE state with eDRX > 10.24s

##### A.17.X1.C.2.1	Test purpose and environment

The purpose of this test is to verify that RSTD measurements performed in RRC_IDLE with eDRX satisfy the measurement accuracy requirements specified in clause 10.1A.X.2.1. The tests are conducted under AWGN propagation condition with the UE operating in FR2 stand-alone mode and configured to perform RSTD measurements on a single positioning frequency layer (PFL) in FR2.

The supported test configurations are listed in Table A.17.X1.C.2.1-1.

Table A.17.X1.C.2.1-1: Supported test configurations

| PCell configuration | Description |
| --- | --- |
| 1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

There are two synchronous cells in the test: Cell 1 and Cell 2. Cell 1 is the reference as well as the PCell. Cell 2 is a neighbour cell. Both cells are on the same NR RF channel in FR2.

The NR-TDOA-ProvideAssistanceData and NR-TDOA-RequestLocationInformation message as defined in TS 37.355 shall be provided to the UE before the start of the test. The test duration should be larger than the UE measurement period as defined in clause 4.5.2.5.

The general test parameters and cell specific test parameters are as given in Table A.17.X1.C.2.1-2 and Table A.17.X1.C.2.1-3 respectively.

Table A.17.X1.C.2.1-2: General test parameters

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Reference cell |  | 1 | Cell 1 |  |
| Neighbour cell |  | 1 | Cell 2 |  |
| RF Channel Number |  | 1 | 1 | For Cell 1 and Cell 2 |
| BWchannel | MHz | 1 | 100: NRB,c = 66 |  |
| SSB configuration |  | 1 | SSB.3 FR2 |  |
| SMTC configuration |  | 1 | SMTC.1 |  |
| CP length |  | 1 | Normal |  |
| DRX | s | 1 | 0.64 |  |
| eDRX cycle length | s | 1 | 40.96 |  |
| PTW length | s | 1 | 1.28 |  |
| Time offset between serving and neighbour cell | s | 1 | 3 | Synchronous cells |
| Expected RSTD | s | 1 | 3 |  |
| Expected RSTD uncertainty | s | 1 | 5 |  |

Table A.17.X1.C.2.1-3: Cell specific test parameters

| Parameter | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| AoA setup |  | Setup 1 as specified in clause A.3.15 |  |  |  |
| Beam AssumptionNote 4 |  | Rough |  | Rough |  |
| TDD configuration |  | TDDConf.3.1 |  | TDDConf.3.1 |  |
| PDSCH RMC configuration |  | SR.3.1 TDD | - | SR.3.1 TDD | - |
| RMSI CORESET RMC configuration |  | CR.3.1 TDD | - | CR.3.1 TDD | - |
| Dedicated CORESET RMC configuration |  | CR.3.1 TDD | - | CR.3.1 TDD | - |
| Control channel RMC |  | CCR.3.1 TDD | - | CCR.3.1 TDD | - |
| OCNG Patterns |  | OP.1 | OP.1 | OP.1 | OP.1 |
| EPRE ratio of PSS to SSS | dB | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| TRS configuration |  | TRS.2.1 TDD | - | TRS.2.1 TDD | - |
| Initial BWP configuration |  | DLBWP.0.1 ULBWP.0.1 | - | DLBWP.0.1 ULBWP.0.1 | - |
| Dedicated BWP configuration |  | DLBWP.1.1 ULBWP.1.1 | - | DLBWP.1.1 ULBWP.1.1 | - |
| PRS configuration |  | PRS.1.1 FR2 | PRS.1.1 FR2 | PRS.1.2 FR2 | PRS.1.2 FR2 |
| PRS num RB |  | 32 | 32 | 64 | 64 |
| PRS Resource slot offset | slot | 0 | 4 | 0 | 4 |
| Note 2 | dBm/SCS | -89 |  |  |  |
| PRS | dB | -6 | -13 | -6 | -13 |
| PRS | dB | -6 | -13 | -6 | -13 |
| PRP Note 3 | dBm/SCS kHz | -95 | -102 | -95 | -102 |
| Io Note 3 (on symbols where PRS is not allocated) | dBm/95.04 MHz | -58.7 | -58.7 | -58.7 | -58.7 |
| Propagation conditions |  | AWGN |  |  |  |
| Antenna configuration |  | 1x2 | 1x2 | 1x2 | 1x2 |
| Note 1: OCNG shall be used such that active cell (Cell 1) is fully allocated and a constant total transmitted power spectral density is achieved on all OFDM symbols except those in which PRS is allocated. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: PRP and Io levels have been derived from other parameters and they are provided for information only. They are not settable parameters themselves. Note 4: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation Note 5: Calculation of Es/Iot includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1dB for UE multi-band relaxation factor ΔMBP from TS 38.101-2 [19] Table 6.2.1.3-4. [Note 6: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone] [Note 7: As observed with 0 dBi gain antenna at the centre of the quiet zone] |  |  |  |  |  |

##### A.17.X1.C.2.2	Test requirements

The reported RSTD measurements shall fulfill the absolute accuracy requirements specified in clause 10.1A.X.2.1.

----------------------END OF CHANGE----------------------------