3GPP TSG-RAN WG4 Meeting # 112	R4-2411833

# Maastricht, Netherlands,  August 19 – 23, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 2401 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_CADC_R18_3BDL_xBUL-Core) CR for TS 38.101-1 on UE configured power relaxation for special component bands (R18) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_R18_3BDL_xBUL-Core, NR_CADC_R18_yBDL_xBUL-Core, LTE_NR_Simult_RxTx_R18-Core |  |  |  |  |  | Date: |  |  | 2024-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For allowed maximum configured ouput power relaxation, the component bands which are special bands such as SDL bands, immediately close band, band combination with overlapping band, non-zero values are not allowed for such bands in the delta TIB tables. If uplink is not supported on a constituted band of the DC/CA band combination, "N/A" is used when deriving the delta TIB requirements for that constituted band of the band combination. Furthermore, the note for CA_n39-n40-n41 in Table 6.2A.4.2.4-1 is missing for UE supporting inter-band CA with simultaneous Rx/Tx capacility. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct the delta TIB values for the band combinations with component SDL band and immediately close band. - Add note for CA_n39-n40-n41 in Table 6.2A.4.2.4-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The delta TIB values for component SDL band and immediately close band will be inaccurate and the note for CA_n39-n40-n41 will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.4.2.4, 6.2A.4.2.5, 6.2A.4.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... CR ... 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes >>

6.2A.4.2.4	ΔTIB,c for Inter-band CA (three bands)

Table 6.2A.4.2.4-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)8 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |  |
| CA_n1-n3-n5 | 0.3 | 0.3 | 0.3 |
| CA_n1-n3-n7 | 0.6 | 0.6 | 0.6 |
| CA_n1-n3-n8 | 0.3 | 0.3 | 0.3 |
| CA_n1-n3-n18 | 0.3 | 0.3 | 0.3 |
| CA_n1-n3-n20 | 0.3 | 0.3 | 0.3 |
| CA_n1-n3-n26 | 0.3 | 0.3 | 0.3 |
| CA_n1-n3-n28 | 0.3 | 0.3 | 0.6 |
| CA_n1-n3-n38 | 0.5 | 0.5 | 0.3 |
| CA_n1-n3-n40 | 0.5 | 0.5 | 0.5 |
| CA_n1-n3-n41 | 0.5 | 0.5 | 0.35 / 0.86 |
| CA_n1-n3-n75 | 0.3 | 0.3 | N/A |
| CA_n1-n3-n77 | 0.6 | 0.6 | 0.8 |
| CA_n1-n3-n78 | 0.6 | 0.6 | 0.8 |
| CA_n1-n3-n79 | 0.3 | 0.3 | 0.8 |
| CA_n1-n3-n105 | 0.3 | 0.3 | 0.6 |
| CA_n1-n5-n7 | 0.5 | 0.3 | 0.6 |
| CA_n1-n5-n28 | 0.3 | 0.6 | 0.6 |
| CA_n1-n5-n40 | 0.6 | 0.3 | 0.5 |
| CA_n1-n5-n78 | 0.6 | 0.6 | 0.8 |
| CA_n1-n5-n79 | 0.6 | 0.6 | 0.8 |
| CA_n1-n5-n105 | 0.3 | 0.6 | 0.6 |
| CA_n1-n7-n8 | 0.5 | 0.6 | 0.6 |
| CA_n1-n7-n26 | 0.5 | 0.6 | 0.3 |
| CA_n1-n7-n28 | 0.5 | 0.6 | 0.6 |
| CA_n1-n7-n38 | 0.5 | N/A | N/A |
| CA_n1-n7-n40 | 0.6 | 0.8 | 0.9 |
| CA_n1-n7-n67 | 0.5 | 0.5 | N/A |
| CA_n1-n7-n75 | 0.5 | 0.6 | N/A |
| CA_n1-n7-n78 | 0.6 | 0.6 | 0.8 |
| CA_n1-n7-n79 | 0.6 | 0.6 | 0.8 |
| CA_n1-n7-n105 | 0.5 | 0.6 | 0.6 |
| CA_n1-n8-n28 | 0.3 | 0.6 | 0.6 |
| CA_n1-n8-n40 | 0.3 | 0.3 | 0.5 |
| CA_n1-n8-n77 | 0.3 | 0.6 | 0.8 |
| CA_n1-n8-n78 | 0.3 | 0.6 | 0.8 |
| CA_n1-n8-n79 | 0.3 | 0.6 | 0.8 |
| CA_n1-n18-n28 | 0.3 | 0.5 | 0.5 |
| CA_n1-n18-n41 | 0.5 | 0.3 | 0.5 |
| CA_n1-n18-n77 | 0.3 | 0.3 | 0.8 |
| CA_n1-n20-n67 | 0.5 | 0.6 | N/A |
| CA_n1-n20-n78 | 0.3 | 0.6 | 0.8 |
| CA_n1-n26-n78 | 0.6 | 0.6 | 0.8 |
| CA_n1-n28-n38 | 0.5 | 0.6 | 0.5 |
| CA_n1-n28-n40 | 0.6 | 0.3 | 0.5 |
| CA_n1-n28-n41 | 0.5 | 0.6 | 0.6 |
| CA_n1-n28-n46 | 0.3 | 0.6 | - |
| CA_n1-n28-n75 | 0.3 | 0.6 | N/A |
| CA_n1-n28-n77 | 0.6 | 0.6 | 0.8 |
| CA_n1-n28-n78 | 0.3 | 0.6 | 0.8 |
| CA_n1-n28-n79 | - | 0.2 | 0.5 |
| CA_n1-n28-n102 | 0.6 | 0.6 | 0.8 |
| CA_n1-n38-n78 | 0.5 | 0.5 | 0.8 |
| CA_n1-n40-n77 | 0.3 | 0.5 | 0.8 |
| CA_n1-n40-n78 | 0.3 | 0.5 | 0.8 |
| CA_n1-n40-n105 | 0.5 | 0.5 | 0.6 |
| CA_n1-n41-n77 | 0.5 | 0.5 | 0.8 |
| CA_n1-n41-n79 | 0.5 | 0.5 | 0.8 |
| CA_n1-n46-n78 | 0.3 | - | 0.8 |
| CA_n1-n67-n78 | 0.3 | N/A | 0.8 |
| CA_n1-n75-n78 | 0.3 | N/A | 0.8 |
| CA_n1-n77-n79 | 0.6 | 0.8 | 0.5 |
| CA_n1-n78-n79 | 0.3 | 0.8 / 1.57 | 0.5 / 1.57 |
| CA_n1-n78-n102 | 0.6 | 1.5 | 1.5 |
| CA_n1-n78-n105 | 0.3 | 0.8 | 0.6 |
| CA_n2-n5-n30 | 0.5 | 0.3 | 0.3 |
| CA_n2-n5-n41 | 0.5 | 0.6 | 0.45 / 0.96 |
| CA_n2-n5-n48 | 0.6 | 0.3 | 0.8 |
| CA_n2-n5-n66 | 0.5 | 0.3 | 0.5 |
| CA_n2-n5-n77 | 0.6 | 0.8 | 0.8 |
| CA_n2-n7-n12 | 0.5 | 0.5 | 0.3 |
| CA_n2-n7-n71 | 0.5 | 0.5 | 0.6 |
| CA_n2-n7-n66 | 0.5 | 0.5 | 0.5 |
| CA_n2-n7-n77 | 0.6 | 0.5 | 0.8 |
| CA_n2-n12-n30 | 0.5 | 0.3 | 0.3 |
| CA_n2-n12-n41 | 0.5 | 0.3 | 0.45 / 0.96 |
| CA_n2-n12-n66 | 0.5 | 0.8 | 0.5 |
| CA_n2-n12-n71 | 0.3 | 1 | 1 |
| CA_n2-n12-n77 | 0.6 | 0.3 | 0.8 |
| CA_n2-n14-n30 | 0.5 | 0.3 | 0.5 |
| CA_n2-n14-n66 | 0.5 | 0.3 | 0.5 |
| CA_n2-n14-n77 | 0.5 | 0.3 | 0.8 |
| CA_n2-n29-n30 | 0.5 | N/A | 0.3 |
| CA_n2-n29-n66 | 0.5 | N/A | 0.5 |
| CA_n2-n29-n77 | 0.6 | N/A | 0.8 |
| CA_n2-n30-n66 | 0.5 | 0.3 | 0.5 |
| CA_n2-n30-n77 | 0.6 | 0.3 | 0.8 |
| CA_n2-n41-n66 | 0.5 | 0.86 / 1.37 | 0.5 |
| CA_n2-n41-n71 | 0.5 | 0.45/0.96 | 0.6 |
| CA_n2-n48-n66 | 0.6 | 0.8 | 0.6 |
| CA_n2-n48-n77 | 0.6 | 0.8 | 0.8 |
| CA_n2-n66-n71 | 0.5 | 0.5 | 0.3 |
| CA_n2-n66-n77 | 0.6 | 0.6 | 0.8 |
| CA_n2-n66-n78 | 0.6 | 0.6 | 0.8 |
| CA_n2-n71-n77 | 0.6 | 0.3 | 0.8 |
| CA_n2-n71-n78 | 0.6 | 0.6 | 0.8 |
| CA_n3-n5-n7 | 0.5 | 0.3 | 0.5 |
| CA_n3-n5-n28 | 0.3 | 0.6 | 0.5 |
| CA_n3-n5-n78 | 0.6 | 0.6 | 0.8 |
| CA_n3-n5-n79 | 0.3 | 0.5 | 0.8 |
| CA_n3-n7-n8 | 0.5 | 0.5 | 0.6 |
| CA_n3-n7-n20 | 0.5 | 0.5 | 0.3 |
| CA_n3-n7-n26 | 0.5 | 0.5 | 0.3 |
| CA_n3-n7-n28 | 0.5 | 0.5 | 0.3 |
| CA_n3-n7-n38 | 0.5 | N/A | N/A |
| CA_n3-n7-n40 | 0.6 | 0.6 | 0.6 |
| CA_n3-n7-n67 | 0.5 | 0.5 | N/A |
| CA_n3-n7-n75 | 0.7 | 0.7 | N/A |
| CA_n3-n7-n78 | 0.6 | 0.6 | 0.8 |
| CA_n3-n7-n79 | 0.5 | 0.5 | 0.8 |
| CA_n3-n7-n105 | 0.5 | 0.5 | 0.6 |
| CA_n3-n8-n28 | 0.3 | 0.6 | 0.5 |
| CA_n3-n8-n39 | 0.5 | 0.3 | 0.5 |
| CA_n3-n8-n40 | 0.5 | 0.3 | 0.5 |
| CA_n3-n8-n41 | 0.5 | 0.3 | 0.31 / 0.82 |
| CA_n3-n8-n77 | 0.6 | 0.6 | 0.8 |
| CA_n3-n8-n78 | 0.6 | 0.6 | 0.8 |
| CA_n3-n8-n79 | 0.3 | 0.3 | 0.5 |
| CA_n3-n18-n28 | 0.3 | 0.5 | 0.3 |
| CA_n3-n18-n41 | 0.5 | 0.3 | 0.31 / 0.82 |
| CA_n3-n18-n77 | 0.6 | 0.3 | 0.8 |
| CA_n3-n20-n28 | 0.3 | 0.5 | 0.5 |
| CA_n3-n20-n67 | 0.3 | 0.5 | N/A |
| CA_n3-n20-n78 | 0.5 | 0.3 | 0.8 |
| CA_n3-n26-n78 | 0.6 | 0.6 | 0.8 |
| CA_n3-n28-n38 | 0.5 | 0.5 | 0.3 |
| CA_n3-n28-n40 | 0.5 | 0.3 | 0.5 |
| CA_n3-n28-n41 | 0.5 | 0.3 | 0.31 / 0.82 |
| CA_n3-n28-n77 | 0.6 | 0.5 | 0.8 |
| CA_n3-n28-n78 | 0.5 | 0.3 | 0.8 |
| CA_n3-n28-n79 | 0.3 | 0.5 | 0.8 |
| CA_n3-n38-n40 | 0.5 | 0.51,3 | 0.5 |
| CA_n3-n39-n41 | 0.5 | 0.5 | 0.54 / 1.05 |
| CA_n3-n39-n79 | 0.5 | 0.5 | 0.8 |
| CA_n3-n40-n78 | 0.6 | 0.5 | 0.8 |
| CA_n3-n40-n105 | 0.5 | 0.5 | 0.6 |
| CA_n3-n67-n78 | 0.5 | N/A | 0.8 |
| CA_n3-n75-n78 | 0.6 | N/A | 0.8 |
| CA_n3-n77-n79 | 0.6 | 0.8 | - |
| CA_n3-n78-n79 | 0.6 | 0.8 | 0.8 |
| CA_n3-n78-n105 | 0.6 | 0.8 | 0.6 |
| CA_n3-n40-n41 | 0.5 | 0.5 | 0.51 / 0.823 |
| CA_n3-n40-n77 | 0.6 | 0.5 | 0.8 |
| CA_n3-n41-n77 | 0.6 | 0.31 / 0.82 | 0.8 |
| CA_n3-n41-n78 | 0.6 | 0.31 / 0.82 | 0.8 |
| CA_n3-n41-n79 | 0.3 | 0.31 / 0.82 | 0.8 |
| CA_n5-n7-n25 | 0.3 | 0.5 | 0.5 |
| CA_n5-n7-n28 | 0.5 | 0.3 | 0.6 |
| CA_n5-n7-n40 | 0.3 | 0.5 | 0.6 |
| CA_n5-n7-n66 | 0.3 | 0.5 | 0.5 |
| CA_n5-n7-n77 | 0.6 | 0.6 | 0.8 |
| CA_n5-n7-n78 | 0.6 | 0.6 | 0.8 |
| CA_n5-n7-n105 | 0.5 | 0.3 | 0.6 |
| CA_n5-n12-n77 | 0.8 | 0.4 | 0.5 |
| CA_n5-n14-n77 | 0.5 | 0.3 | 0.8 |
| CA_n5-n25-n29 | 0.5 | 0.3 | N/A |
| CA_n5-n25-n41 | 0.6 | 0.5 | 0.45 / 0.96 |
| CA_n5-n25-n66 | 0.3 | 0.5 | 0.5 |
| CA_n5-n25-n77 | 0.6 | 0.6 | 0.8 |
| CA_n5-n25-n78 | 0.6 | 0.6 | 0.8 |
| CA_n5-n28-n105 | 0.7 | 1.0 | 1.0 |
| CA_n5-n28-n78 | 0.7 | 0.7 | 0.8 |
| CA_n5-n28-n79 | 0.7 | 0.7 | 0.8 |
| CA_n5-n29-n66 | 0.5 | N/A | 0.3 |
| CA_n5-n29-n77 | 0.8 | N/A | 0.5 |
| CA_n5-n30-n66 | 0.3 | 0.3 | 0.5 |
| CA_n5-n30-n77 | 0.6 | 0.3 | 0.8 |
| CA_n5-n40-n78 | 0.6 | 0.5 | 0.8 |
| CA_n5-n40-n105 | 0.6 | 0.9 | 0.6 |
| CA_n5-n41-n66 | 0.6 | 0.85 / 1.36 | 0.5 |
| CA_n5-n41-n77 | 0.6 | 0.3 | 0.8 |
| CA_n5-n48-n66 | 0.3 | 0.8 | 0.6 |
| CA_n5-n48-n77 | 0.6 | 0.8 | 0.8 |
| CA_n5-n66-n77 | 0.6 | 0.6 | 0.8 |
| CA_n5-n66-n78 | 0.6 | 0.6 | 0.8 |
| CA_n5_n78-n79 | 0.5 | 0.8 / 1.57 | 0.5 / 1.57 |
| CA_n5-n78-n105 | 0.7 | 0.8 | 0.7 |
| CA_n7-n8-n28 | 0.3 | 0.6 | 0.5 |
| CA_n7-n8-n40 | 0.5 | 0.6 | 0.6 |
| CA_n7-n8-n78 | 0.5 | 0.6 | 0.8 |
| CA_n7-n12-n25 | 0.5 | 0.3 | 0.5 |
| CA_n7-n12-n66 | 0.5 | 0.3 | 0.5 |
| CA_n7-n12-n71 | 0.3 | 1 | 1 |
| CA_n7-n12-n77 | 0.5 | 0.5 | 0.8 |
| CA_n7-n20-n67 | 0.3 | 0.3 | N/A |
| CA_n7-n20-n78 | 0.3 | 0.3 | 0.8 |
| CA_n7-n25-n66 | 0.5 | 0.5 | 0.5 |
| CA_n7-n25-n71 | 0.5 | 0.5 | 0.3 |
| CA_n7-n25-n77 | 0.5 | 0.6 | 0.8 |
| CA_n7-n25-n78 | 0.5 | 0.6 | 0.8 |
| CA_n7-n26-n78 | 0.6 | 0.6 | 0.8 |
| CA_n7-n28-n38 | N/A | 0.3 | N/A |
| CA_n7-n28-n40 | 0.5 | 0.3 | 0.6 |
| CA_n7-n28-n78 | 0.3 | 0.3 | 0.8 |
| CA_n7-n40-n105 | 0.5 | 0.6 | 0.6 |
| CA_n7-n46-n78 | 0.5 | - | 0.8 |
| CA_n7-n66-n71 | 0.5 | 0.5 | 0.5 |
| CA_n7-n66-n77 | 0.5 | 0.6 | 0.8 |
| CA_n7-n66-n78 | 0.5 | 0.6 | 0.8 |
| CA_n7-n67-n78 | 0.5 | N/A | 0.8 |
| CA_n7-n71-n77 | 0.3 | 0.5 | 0.8 |
| CA_n7-n75-n78 | 0.7 | N/A | 0.8 |
| CA_n7-n78-n102 | 0.5 | 1.5 | 1.5 |
| CA_n7-n78-n105 | 0.3 | 0.8 | 0.5 |
| CA_n8-n20-n28 | 0.8 | 0.7 | 0.7 |
| CA_n8-n20-n75 | 0.4 | 0.4 | N/A |
| CA_n8-n28-n40 | 0.6 | 0.5 | 0.3 |
| CA_n8-n28-n75 | 0.6 | 0.5 | N/A |
| CA_n8-n28-n77 | 0.6 | 0.5 | 0.8 |
| CA_n8-n28-n78 | 0.6 | 0.5 | 0.8 |
| CA_n8-n38-n40 | 0.3 | 0.3 | 0.3 |
| CA_n8-n39-n40 | 0.3 | 0.3 | 0.3 |
| CA_n8-n39-n41 | 0.6 | 0.5 | 0.5 |
| CA_n8-n39-n79 | 0.3 | 0.3 | - |
| CA_n8-n40-n41 | 0.3 | 0.3 | 0.3 |
| CA_n8-n40-n77 | 0.6 | 0.3 | 0.8 |
| CA_n8-n40-n78 | 0.6 | 0.3 | 0.8 |
| CA_n8-n40-n79 | 0.3 | 0.3 | 0.8 |
| CA_n8-n41-n79 | 0.6 | 0.3 | 0.8 |
| CA_n8-n78-n79 | 0.6 | 0.8 | 0.8 |
| CA_n12-n25-n41 | 0.3 | 0.5 | 0.45 / 0.96 |
| CA_n12-n25-n66 | 0.8 | 0.5 | 0.5 |
| CA_n12-n30-n66 | 0.8 | 0.3 | 0.5 |
| CA_n12-n30-n77 | 0.5 | 0.3 | 0.5 |
| CA_n12-n41-n66 | 0.5 | 0.5 | 0.5 |
| CA_n12-n41-n77 | 0.5 | 0.5 | 0.8 |
| CA_n12-n66-n77 | 0.8 | 0.6 | 0.8 |
| CA_n12-n71-n77 | 1 | 1 | 0.8 |
| CA_n13-n25-n66 | 0.3 | 0.5 | 0.5 |
| CA_n13-n25-n77 | 0.3 | 0.6 | 0.8 |
| CA_n13-n66-n77 | 0.5 | 0.6 | 0.8 |
| CA_n14-n30-n66 | 0.3 | 0.3 | 0.5 |
| CA_n14-n30-n77 | 0.5 | 0.3 | 0.8 |
| CA_n14-n66-n77 | 0.6 | 0.6 | 0.8 |
| CA_n18-n28-n41 | 0.4 | 0.4 | 0.3 |
| CA_n18-n28-n77 | 0.5 | 0.5 | 0.8 |
| CA_n18-n41-n77 | 0.3 | 0.3 | 0.8 |
| CA_n20-n28-n75 | 0.5 | 0.5 | N/A |
| CA_n20-n28-n78 | 0.6 | 0.5 | 0.8 |
| CA_n20-n67-n78 | 0.6 | N/A | 0.8 |
| CA_n24-n41-n48 | 0.6 | 0.41 / 0.92 | 0.8 |
| CA_n24-n41-n77 | 0.6 | 0.45 / 0.96 | 0.8 |
| CA_n24-n48-n77 | 0.6 | 0.8 | 0.8 |
| CA_n25-n29-n66 | 0.5 | N/A | 0.5 |
| CA_n25-n38-n78 | 0.5 | 0.4 | 0.8 |
| CA_n25-n41-n66 | 0.5 | 0.85 / 1.36 | 0.5 |
| CA_n25-n41-n71 | 0.5 | 0.5 | 0.6 |
| CA_n25-n41-n77 | 0.5 | 0.5 | 0.6 |
| CA_n25-n41-n78 | 0.6 | 0.5 | 0.8 |
| CA_n25-n41-n85 | 0.5 | 0.5 | 0.3 |
| CA_n25-n48-n66 | 0.6 | 0.8 | 0.6 |
| CA_n25-n66-n71 | 0.5 | 0.5 | 0.6 |
| CA_n25-n66-n77 | 0.6 | 0.6 | 0.8 |
| CA_n25-n66-n78 | 0.6 | 0.6 | 0.8 |
| CA_n25-n66-n85 | 0.5 | 0.5 | 0.8 |
| CA_n25-n71-n77 | 0.6 | 0.6 | 0.8 |
| CA_n25-n71-n78 | 0.6 | 0.6 | 0.8 |
| CA_n25-n71-n85 | 0.3 | 1 | 1 |
| CA_n25-n77-n85 | 0.6 | 0.8 | 0.3 |
| CA_n26-n29-n66 | 0.5 | N/A | 0.3 |
| CA_n26-n29-n70 | 0.5 | N/A | 0.3 |
| CA_n26-n48-n66 | 0.3 | 0.8 | 0.6 |
| CA_n26-n48-n70 | 0.3 | 0.8 | 0.6 |
| CA_n26-n66-n70 | 0.3 | 0.5 | 0.5 |
| CA_n26-n66-n71 | 0.5 | 0.3 | 0.5 |
| CA_n26-n66-n77 | 0.6 | 0.6 | 0.8 |
| CA_n26-n70-n71 | 0.5 | 0.3 | 0.6 |
| CA_n26-n70-n77 | 0.6 | 0.6 | 0.8 |
| CA_n28-n38-n78 | 0.5 | 0.3 | 0.8 |
| CA_n28-n39-n40 | 0.3 | 0.3 | 0.3 |
| CA_n28-n39-n41 | 0.3 | 0.5 | 0.5 |
| CA_n28-n39-n79 | 0.5 | 0.3 | 0.8 |
| CA_n28-n40-n41 | 0.3 | 0.5 | 0.5 |
| CA_n28-n40-n77 | 0.5 | 0.3 | 0.8 |
| CA_n28-n40-n78 | 0.5 | 0.3 | 0.8 |
| CA_n28-n40-n79 | 0.5 | 0.3 | 0.8 |
| CA_n28-n41-n79 | 0.5 | 0.3 | 0.8 |
| CA_n28-n41-n77 | 0.5 | 0.3 | 0.8 |
| CA_n28-n41-n78 | 0.5 | 0.3 | 0.8 |
| CA_n28-n46-n78 | 0.5 | - | 0.8 |
| CA_n28-n75-n78 | 0.3 | N/A | 0.8 |
| CA_n28-n77-n79 | 0.5 | 0.8 | 0.5 |
| CA_n28-n78-n79 | 0.5 | 0.8 / 1.57 | 0.5 / 1.57 |
| CA_n28-n78-n102 | 0.5 | 1.5 | 1.5 |
| CA_n29-n30-n66 | N/A | 0.3 | 0.5 |
| CA_n29-n30-n77 | N/A | 0.3 | 0.5 |
| CA_n29-n66-n70 | N/A | 0.5 | 0.5 |
| CA_n29-n66-n71 | N/A | 0.3 | 0.5 |
| CA_n29-n66-n77 | N/A | 0.6 | 0.8 |
| CA_n29-n70-n71 | N/A | 0.3 | 0.6 |
| CA_n30-n66-n77 | 0.3 | 0.6 | 0.8 |
| CA_n34-n39-n41 | 0.3 | 0.5 | 0.5 |
| CA_n34-n40-n41 | 0.3 | 0.5 | 0.5 |
| CA_n34-n41-n79 | 0.3 | 0.5 | 0.8 |
| CA_n38-n66-n78 | 0.5 | 0.5 | 0.8 |
| CA_n39-n40-n41 | 0.3 | 0.3 / 0.610 | 0.3 / 0.610 |
| CA_n39-n40-n79 | 0.3 | - | 0.8 |
| CA_n39-n41-n79 | 0.3 | 0.3 | 0.8 |
| CA_n40-n41-n79 | 0.5 | 0.5 | 0.8 |
| CA_n40-n78-n105 | 0.3 | 0.8 | 0.5 |
| CA_n41-n66-n71 | 0.8 / 1.36 | 0.5 | 0.3 |
| CA_n41-n66-n77 | 0.5 | 0.6 | 0.8 |
| CA_n41-n66-n78 | 0.5 | 0.6 | 0.8 |
| CA_n41-n66-n85 | 0.81 / 1.32 | 0.5 | 0.6 |
| CA_n41-n70-n78 | 0.6 | 0.6 | 0.8 |
| CA_n41-n71-n77 | 0.3 | 0.5 | 0.8 |
| CA_n41-n71-n78 | 0.3 | 0.5 | 0.8 |
| CA_n41-n71-n85 | 0.3 | 1 | 1 |
| CA_n41-n77-n79 | 0.3 | 0.8 | 0.8 |
| CA_n41-n77-n85 | 0.6 | 0.8 | 0.8 |
| CA_n46-n48-n96 | 0.5 | 0.8 | 0.6 |
| CA_n46-n78-n102 | - | 1.5 | 1.5 |
| CA_n48-n66-n70 | 0.8 | 0.6 | 0.6 |
| CA_n48-n66-n71 | 0.5 | 0.5 | 0.3 |
| CA_n48-n66-n77 | 0.8 | 0.6 | 0.8 |
| CA_n48-n70-n71 | 0.5 | 0.5 | 0.3 |
| CA_n48-n70-n77 | 0.8 | 0.6 | 0.8 |
| CA_n48-n71-n77 | 0.8 | 0.6 | 0.8 |
| CA_n66-n70-n71 | 0.5 | 0.5 | 0.6 |
| CA_n66-n70-n77 | 0.6 | 0.6 | 0.8 |
| CA_n66-n71-n77 | 0.6 | 0.6 | 0.8 |
| CA_n66-n71-n78 | 0.6 | 0.5 | 0.8 |
| CA_n66-n71-n85 | 0.8 | 1 | 1 |
| CA_n66-n77-n85 | 0.6 | 0.8 | 0.8 |
| CA_n70-n71-n77 | 0.6 | 0.3 | 0.8 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2515-2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2515 MHz. NOTE 3: Void. NOTE 4: Void. NOTE 5: The requirement is applied for UE transmitting on the frequency range of 2545 - 2690 MHz. NOTE 6: The requirement is applied for UE transmitting on the frequency range of 2496 - 2545 MHz. NOTE 7: The requirements only apply for UE supporting inter-band carrier aggregation with simultaneous Rx/Tx capability, and NR UL carrier frequencies are confined to 3700 MHz-3800MHz for n78 and 4400 MHz-4500MHz for n79. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. NOTE 8: “-” denotes ΔTIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. NOTE 10: The requirements only apply for UE supporting inter-band carrier aggregation with simultaneous Rx/Tx capability. |  |  |  |

6.2A.4.2.5	ΔTIB,c for Inter-band CA (four bands)

Table 6.2A.4.2.5-1: ΔTIB,c due to NR CA (four bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)5 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration6 |  |  |  |
| CA_n1-n3-n5-n7 | 0.6 | 0.6 | 0.3 | - |
| CA_n1-n3-n5-n28 | 0.3 | 0.3 | 0.7 | 0.7 |
| CA_n1-n3-n5-n78 | 0.6 | 0.6 | 0.3 | 0.8 |
| CA_n1-n3-n7-n8 | 0.6 | 0.6 | 0.6 | 0.6 |
| CA_n1-n3-n7-n26 | 0.6 | 0.6 | 0.6 | 0.6 |
| CA_n1-n3-n7-n28 | 0.6 | 0.6 | 0.6 | 0.6 |
| CA_n1-n3-n7-n38 | 0.6 | 0.6 | N/A | N/A |
| CA_n1-n3-n7-n40 | 0.6 | 0.6 | 0.6 | 0.6 |
| CA_n1-n3-n7-n67 | 0.6 | 0.6 | 0.6 | N/A |
| CA_n1-n3-n7-n75 | 0.6 | 0.6 | 0.6 | N/A |
| CA_n1-n3-n7-n78 | 0.7 | 0.7 | 0.7 | 0.8 |
| CA_n1-n3-n7-n79 | 0.6 | 0.6 | 0.6 | 0.8 |
| CA_n1-n3-n7-n105 | 0.6 | 0.6 | 0.6 | 0.6 |
| CA_n1-n3-n8-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| CA_n1-n3-n8-n77 | 0.6 | 0.6 | 0.6 | 0.8 |
| CA_n1-n3-n18-n28 | 0.3 | 0.3 | 0.5 | 0.5 |
| CA_n1-n3-n18-n41 | 0.5 | 0.5 | 0.3 | 0.33 / 0.84 |
| CA_n1-n3-n18-n77 | 0.6 | 0.6 | 0.3 | 0.8 |
| CA_n1-n3-n20-n67 | 0.3 | 0.3 | 0.3 | N/A |
| CA_n1-n3-n26-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| CA_n1-n3-n28-n38 | 0.6 | 0.6 | 0.6 | 0.6 |
| CA_n1-n3-n28-n41 | 0.5 | 0.5 | 0.5 | 0.33 / 0.84 |
| CA_n1-n3-n28-n77 | 0.6 | 0.6 | 0.6 | 0.8 |
| CA_n1-n3-n28-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| CA_n1-n3-n28-n79 | 0.3 | 0.3 | 0.6 | 0.8 |
| CA_n1-n3-n40-n77 | 0.7 | 0.7 | 0.7 | 0.8 |
| CA_n1-n3-n40-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| CA_n1-n3-n40-n105 | 0.7 | 0.7 | 0.7 | 0.5 |
| CA_n1-n3-n41-n77 | 0.6 | 0.6 | 0.33 / 0.84 | 0.8 |
| CA_n1-n3-n41-n79 | 0.5 | 0.5 | 0.53 / 0.84 | 0.8 |
| CA_n1-n3-n67-n78 | 0.6 | 0.6 | N/A | 0.8 |
| CA_n1-n3-n75-n78 | 0.6 | 0.6 | N/A | 0.8 |
| CA_n1-n3-n77-n79 | 0.6 | 0.6 | 0.8 | 0.8 |
| CA_n1-n5-n7-n40 | 0.5 | 0.3 | 0.6 | 0.5 |
| CA_n1-n5-n7-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| CA_n1-n5-n7-n105 | 0.5 | 0.3 | 0.6 | 0.6 |
| CA_n1-n5-n28-n78 | 0.3 | 0.7 | 0.7 | 0.8 |
| CA_n1-n5-n28-n79 | 0.3 | 0.7 | 0.7 | 0.8 |
| CA_n1-n5-n40-n78 | 0.6 | 0.6 | 0.5 | 0.8 |
| CA_n1-n5-n40-n105 | 0.5 | 0.3 | 0.5 | 0.6 |
| CA_n1-n5-n78-n79 | 0.6 | 0.6 | 0.8 | 0.5 |
| CA_n1-n5-n78-n105 | 0.3 | 0.6 | 0.8 | 0.6 |
| CA_n1-n7-n8-n40 | 0.6 | 0.8 | 0.6 | 0.9 |
| CA_n1-n7-n8-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| CA_n1-n7-n26-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| CA_n1-n7-n28-n38 | 0.5 | N/A | 0.6 | N/A |
| CA_n1-n7-n28-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| CA_n1-n7-n40-n78 | 0.6 | 0.5 | 0.5 | 0.8 |
| CA_n1-n7-n40-n105 | 0.6 | 0.5 | 0.5 | 0.5 |
| CA_n1-n7-n67-n78 | 0.6 | 0.6 | N/A | 0.8 |
| CA_n1-n7-n75-n78 | 0.6 | 0.6 | N/A | 0.8 |
| CA_n1-n7-n78-n105 | 0.6 | 0.6 | 0.8 | 0.5 |
| CA_n1-n8-n40-n78 | 0.5 | 0.3 | 0.5 | 0.8 |
| CA_n1-n8-n78-n79 | 0.3 | 0.6 | 0.8 | 0.5 |
| CA_n1-n18-n28-n41 | 0.6 | 0.5 | 0.6 | 0.5 |
| CA_n1-n18-n28-n77 | 0.6 | 0.5 | 0.6 | 0.8 |
| CA_n1-n18-n41-n77 | 0.5 | 0.5 | 0.5 | 0.8 |
| CA_n1-n28-n38-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| CA_n1-n28-n40-n77 | 0.3 | 0.6 | 0.5 | 0.8 |
| CA_n1-n28-n40-n78 | 0.3 | 0.6 | 0.5 | 0.8 |
| CA_n1-n28-n41-n77 | 0.6 | 0.6 | 0.6 | 0.8 |
| CA_n1-n28-n41-n79 | 0.5 | 0.6 | 0.6 | 0.8 |
| CA_n1-n28-n75-n78 | 0.5 | 0.6 | N/A | 0.8 |
| CA_n1-n28-n77-n79 | 0.6 | 0.6 | 0.8 | 0.8 |
| CA_n1-n28-n78-n79 | 0.6 | 0.6 | 0.8 | 0.8 |
| CA_n1-n41-n77-n79 | 0.6 | 0.5 | 0.8 | 0.8 |
| CA_n2-n5-n30-n66 | 0.5 | 0.3 | 0.3 | 0.5 |
| CA_n2-n5-n30-n77 | 0.6 | 0.6 | 0.3 | 0.8 |
| CA_n2-n5-n48-n66 | 0.6 | 0.3 | 0.8 | 0.6 |
| CA_n2-n5-n48-n77 | 0.6 | 0.3 | 0.8 | 0.8 |
| CA_n2-n5-n66-n77 | 0.5 | 0.3 | 0.5 | 0.8 |
| CA_n2-n12-n30-n66 | 0.5 | 0.8 | 0.3 | 0.5 |
| CA_n2-n12-n30-n77 | 0.6 | 0.5 | 0.3 | 0.8 |
| CA_n2-n12-n66-n77 | 0.6 | 0.8 | 0.6 | 0.8 |
| CA_n2-n14-n30-n66 | 0.5 | 0.3 | 0.3 | 0.5 |
| CA_n2-n14-n30-n77 | 0.6 | 0.5 | 0.3 | 0.8 |
| CA_n2-n14-n66-n77 | 0.6 | 0.6 | 0.6 | 0.8 |
| CA_n2-n29-n30-n66 | 0.5 | N/A | 0.3 | 0.5 |
| CA_n2-n29-n30-n77 | 0.6 | N/A | 0.3 | 0.8 |
| CA_n2-n29-n66-n77 | 0.6 | N/A | 0.6 | 0.8 |
| CA_n2-n30-n66-n77 | 0.6 | 0.3 | 0.6 | 0.8 |
| CA_n2-n41-n66-n71 | 0.5 | 0.83 / 1.34 | 0.5 | 0.6 |
| CA_n2-n48-n66-n77 | 0.6 | 0.8 | 0.6 | 0.8 |
| CA_n2-n66-n71-n77 | 0.5 | 0.5 | 0.3 | 0.5 |
| CA_n2-n66-n71-n78 | 0.5 | 0.5 | 0.3 | 0.5 |
| CA_n3-n5-n7-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| CA_n3-n5-n28-n78 | 0.6 | 0.7 | 0.7 | 0.8 |
| CA_n3-n5-n28-n79 | 0.6 | 0.7 | 0.7 | 0.8 |
| CA_n3-n7-n8-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| CA_n3-n7-n20-n67 | 0.5 | 0.5 | 0.3 | N/A |
| CA_n3-n7-n20-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| CA_n3-n7-n26-n78 | 0.6 | 0.6 | 0.6 | 0.6 |
| CA_n3-n7-n28-n38 | 0.5 | N/A | 0.3 | N/A |
| CA_n3-n7-n28-n78 | 0.6 | 0.6 | 0.6 | 0.6 |
| CA_n3-n7-n40-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| CA_n3-n7-n40-n105 | 0.6 | 0.6 | 0.6 | 0.5 |
| CA_n3-n7-n67-n78 | 0.6 | 0.6 | N/A | 0.6 |
| CA_n3-n7-n75-n78 | 0.6 | 0.6 | N/A | 0.6 |
| CA_n3-n7-n78-n105 | 0.6 | 0.6 | 0.8 | 0.5 |
| CA_n3-n8-n41-n79 | 0.5 | 0.5 | 0.31 / 0.82 | 0.8 |
| CA_n3-n18-n28-n41 | 0.5 | 0.4 | 0.4 | 0.33 / 0.84 |
| CA_n3-n18-n28-n77 | 0.6 | 0.5 | 0.5 | 0.8 |
| CA_n3-n18-n41-n77 | 0.6 | 0.4 | 0.33 / 0.84 | 0.8 |
| CA_n3-n20-n67-n78 | 0.5 | 0.5 | N/A | 0.8 |
| CA_n3-n28-n40-n77 | 0.6 | 0.6 | 0.6 | 0.8 |
| CA_n3-n28-n41-n77 | 1 | 0.5 | 0.31 / 0.82 | 0.8 |
| CA_n3-n28-n41-n78 | 1 | 0.5 | 0.31 / 0.82 | 0.8 |
| CA_n3-n28-n41-n79 | 0.5 | 0.5 | 0.31 / 0.82 | 0.8 |
| CA_n3-n28-n77-n79 | 0.6 | 0.5 | 0.8 | 0.8 |
| CA_n3-n41-n77-n79 | 0.6 | 0.31 / 0.82 | 0.8 | 0.8 |
| CA_n5-n7-n40-n78 | 0.6 | 0.6 | 0.5 | 0.8 |
| CA_n5-n7-n40-n105 | 0.3 | 0.3 | 0.6 | 0.6 |
| CA_n5-n7-n66-n77 | 0.6 | 0.6 | 0.6 | 0.8 |
| CA_n5-n7-n78-n105 | 0.6 | 0.3 | 0.8 | 0.5 |
| CA_n5-n25-n29-n66 | 0.5 | 0.5 | N/A | 0.5 |
| CA_n5-n25-n66-n77 | 0.6 | 0.6 | 0.6 | 0.8 |
| CA_n5-n25-n66-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| CA_n5-n28-n78-n79 | 0.7 | 0.7 | 0.8 | 0.8 |
| CA_n5-n30-n66-n77 | 0.6 | 0.3 | 0.6 | 0.8 |
| CA_n5-n40-n78-n105 | 0.6 | 0.3 | 0.8 | 0.5 |
| CA_n5-n48-n66-n77 | 0.6 | 0.8 | 0.6 | 0.8 |
| CA_n7-n8-n40-n78 | 0.5 | 0.3 | 0.5 | 0.8 |
| CA_n7-n12-n25-n66 | 0.5 | 0.3 | 0.5 | 0.5 |
| CA_n7-n20-n67-n78 | 0.6 | 0.6 | N/A | 0.8 |
| CA_n7-n25-n66-n71 | 0.5 | 0.5 | 0.5 | 0.6 |
| CA_n7-n25-n66-n77 | 0.5 | 0.6 | 0.6 | 0.8 |
| CA_n7-n25-n66-n78 | 0.5 | 0.6 | 0.6 | 0.8 |
| CA_n7-n40-n78-n105 | 0.5 | 0.6 | 0.8 | 0.5 |
| CA_n7-n66-n71-n77 | 0.5 | 0.6 | 0.6 | 0.8 |
| CA_n8-n20-n28-n75 | 0.8 | 0.7 | 0.7 | N/A |
| CA_n8-n39-n41-n79 | 0.5 | 0.5 | 0.31 / 0.82 | 0.8 |
| CA_n12-n30-n66-n77 | 0.8 | 0.3 | 0.6 | 0.8 |
| CA_n13-n25-n66-n77 | 0.5 | 0.6 | 0.6 | 0.8 |
| CA_n14-n30-n66-n77 | 0.6 | 0.3 | 0.6 | 0.8 |
| CA_n18-n28-n41-n77 | 0.5 | 0.5 | 0.33 / 0.84 | 0.8 |
| CA_n25-n38-n66-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| CA_n25-n41-n66-n71 | 0.5 | 0.5 | 0.5 | 0.3 |
| CA_n25-n41-n66-n77 | 0.5 | 0.83 / 1.34 | 0.5 | 0.8 |
| CA_n25-n41-n66-n78 | 0.5 | 0.83 / 1.34 | 0.5 | 0.8 |
| CA_n25-n41-n66-n85 | 0.5 | 0.5 | 0.5 | 0.3 |
| CA_n25-n41-n71-n77 | 0.5 | 0.5 | 0.6 | 0.8 |
| CA_n25-n41-n71-n78 | 0.5 | 0.5 | 0.6 | 0.8 |
| CA_n25-n41-n71-n85 | 0.5 | 0.5 | 0.5 | 0.5 |
| CA_n25-n41-n77-n85 | 0.5 | 0.5 | 0.8 | 0.6 |
| CA_n25-n66-n71-n77 | 0.5 | 0.5 | 0.6 | 0.8 |
| CA_n25-n66-n71-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| CA_n25-n66-n71-n85 | 0.5 | 0.5 | 1 | 1 |
| CA_n25-n66-n77-n85 | 0.6 | 0.6 | 0.8 | 0.8 |
| CA_n28-n41-n77-n79 | 0.5 | 0.3 | 0.8 | 0.8 |
| CA_n29-n30-n66-n77 | N/A | 0.3 | 0.6 | 0.8 |
| CA_n29-n66-n70-n71 | N/A | 0.5 | 0.5 | 0.6 |
| CA_n41-n66-n70-n78 | 0.5 | 0.6 | 0.6 | 0.8 |
| CA_n41-n66-n71-n77 | 0.33 / 0.84 | 1 | 0.5 | 0.8 |
| CA_n41-n66-n71-n78 | 0.33 / 0.84 | 1 | 0.5 | 0.8 |
| CA_n41-n66-n71-n85 | 0.5 | 0.5 | 0.5 | 0.5 |
| CA_n41-n66-n77-n85 | 0.33 / 0.84 | 1 | 0.8 | 0.5 |
| CA_n48-n66-n70-n77 | 0.8 | 0.6 | 0.6 | 0.8 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. NOTE 3: The requirement is applied for UE transmitting on the frequency range of 2545 - 2690 MHz. NOTE 4: The requirement is applied for UE transmitting on the frequency range of 2496 - 2545 MHz. NOTE 5: “-” denotes ΔTIB,c = 0. NOTE 6: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5-n78 the band order from left to right is n1, n3, n5 and n78. |  |  |  |  |

6.2A.4.2.6	ΔTIB,c for Inter-band CA (five bands)

Table 6.2A.4.2.6-1: ΔTIB,c due to NR CA (five bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration2 |  |  |  |  |
| CA_n1-n3-n5-n7-n78 | 0.6 | 0.6 | 0.6 | 0.6 | 0.8 |
| CA_n1-n3-n5-n28-n78 | 0.6 | 0.6 | 0.7 | 0.7 | 0.8 |
| CA_n1-n3-n7-n8-n78 | 0.7 | 0.7 | 0.7 | 0.6 | 0.8 |
| CA_n1-n3-n7-n26-n78 | 0.7 | 0.7 | 0.7 | 0.6 | 0.8 |
| CA_n1-n3-n7-n28-n38 | 0.6 | 0.6 | N/A | 0.6 | N/A |
| CA_n1-n3-n7-n28-n78 | 0.7 | 0.7 | 0.7 | 0.6 | 0.8 |
| CA_n1-n3-n7-n40-n78 | 0.6 | 0.6 | 0.6 | 0.6 | 0.8 |
| CA_n1-n3-n7-n40-n105 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 |
| CA_n1-n3-n7-n67-n78 | 0.7 | 0.7 | 0.7 | N/A | 0.8 |
| CA_n1-n3-n7-n75-n78 | 0.6 | 0.6 | 0.6 | N/A | 0.8 |
| CA_n1-n3-n28-n41-n77 | 0.6 | 1 | 0.6 | 0.63/0.84 | 0.8 |
| CA_n1-n3-n28-n41-n79 | 0.5 | 0.5 | 0.6 | 0.63/0.84 | 0.8 |
| CA_n1-n3-n28-n77-n79 | 0.6 | 0.6 | 0.6 | 0.8 | 0.8 |
| CA_n1-n3-n40-n78-n105 | 0.6 | 0.6 | 0.6 | 0.8 | 0.6 |
| CA_n1-n3-n41-n77-n79 | 0.6 | 0.6 | 0.53/0.84 | 0.8 | 0.8 |
| CA_n1-n5-n7-n40-n78 | 0.5 | 0.6 | 0.6 | 0.5 | 0.8 |
| CA_n1-n5-n7-n40-n105 | 0.5 | 0.3 | 0.6 | 0.5 | 0.6 |
| CA_n1-n5-n7-n78-n105 | 0.5 | 0.6 | 0.6 | 0.8 | 0.6 |
| CA_n1-n5-n28-n78-n79 | 0.6 | 0.7 | 0.7 | 0.8 | 0.8 |
| CA_n1-n5-n40-n78-n105 | 0.5 | 0.6 | 0.5 | 0.8 | 0.6 |
| CA_n1-n7-n40-n78-n105 | 0.6 | 0.6 | 0.6 | 0.8 | 0.6 |
| CA_n1-n28-n41-n77-n79 | 0.6 | 0.6 | 0.6 | 0.8 | 0.8 |
| CA_n2-n5-n30-n66-n77 | 0.6 | 0.6 | 0.3 | 0.6 | 0.8 |
| CA_n2-n5-n48-n66-n77 | 0.6 | 0.3 | 0.8 | 0.6 | 0.8 |
| CA_n2-n12-n30-n66-n77 | 0.6 | 0.8 | 0.3 | 0.6 | 0.8 |
| CA_n2-n14-n30-n66-n77 | 0.6 | 0.6 | 0.3 | 0.6 | 0.8 |
| CA_n2-n29-n30-n66-n77 | 0.6 | N/A | 0.3 | 0.6 | 0.8 |
| CA_n3-n7-n20-n67-n78 | 0.6 | 0.6 | 0.6 | N/A | 0.8 |
| CA_n3-n7-n40-n78-n105 | 0.6 | 0.8 | 0.6 | 0.8 | 0.6 |
| CA_n3-n28-n41-n77-n79 | 1 | 0.5 | 0.8 | 0.8 | 0.8 |
| CA_n5-n7-n40-n78-n105 | 0.6 | 0.6 | 0.5 | 0.8 | 0.6 |
| NOTE 1: “-” denotes ΔTIB,c = 0. NOTE 2: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5-n7-n78 the band order from left to right is n1, n3, n5, n7 and n78. NOTE 3: The requirement is applied for UE transmitting on the frequency range of 2545 - 2690 MHz NOTE 4: The requirement is applied for UE transmitting on the frequency range of 2496 - 2545 MHz |  |  |  |  |  |

6.2A.4.2.7	ΔTIB,c for Inter-band CA (six bands)

Table 6.2A.4.2.7-1: ΔTIB,c due to NR CA (six bands)

| Inter-band CA combination |  | ΔTIB,c for NR bands (dB)1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Component band in order of bands in configuration2 |  |  |  |  |
| CA_n1-n3-n7-n28-n38-n78 | 0.7 | 0.7 | N/A | 0.6 | N/A | 0.8 |
| CA_n1-n3-n7-n40-n78-n105 | 0.6 | 0.6 | 0.6 | 0.6 | 0.8 | 0.6 |
| CA_n1-n5-n7-n40-n78-n105 | 0.5 | 0.6 | 0.6 | 0.5 | 0.8 | 0.6 |
| NOTE 1: “-” denotes ΔTIB,c = 0. NOTE 2: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5-n7-n78 the band order from left to right is n1, n3, n5, n7 and n78. |  |  |  |  |  |  |

<< End of changes >>