3GPP TSG-RAN WG4 Meeting #112	R4-2411838

# Maastricht, Netherlands, August 19 – 23, 2024

Title:	Considerations on specification improvement for table modification

Source:	ZTE Corporation, Sanechips

Agenda item:	10.1.1.1

Document for:	Approval

- 1.	Introduction

Sub-topic 1-1 Specification table simplification

Issue 1-1-1: Simplification of Table 5.2A.2.1-1: Inter-band CA operating bands involving FR1 (two bands) (from R4-2404448)

Issue 1-1-2: Simplification of Table 5.5B.5.1-1: Inter-band EN-DC configurations including FR2 (two bands) (from proposal by [R4-2404896)

<Way forward/Agreement>:

Agreement:

  - Do not consider the simplification for the tables of combinations of operating bands for 5G

  - Further discuss the simplification for configuration tables for EN-DC

  - Reducing the redundancy of band combination tables with the readability being ensured can be done for the future generation

In last RAN4#111 meeting, the issues of further improving the RAN4 spec quality were discussed and a WF was approved in [1] for Table modification part as below.

In this proposal, we further discuss the simplification for configuration tables for EN-DC including FR2 band.

- 2.	Discussion

While looking at the current specs in TS 38.101-2 and 38.101-3 with inter-band CA configurations having FR2 constituent bands, the grouping rules in the uplink configurations have been applied. The delimiter “/” is used for the FR2 part of the uplink configuration, such as for CA_nxA-nyA/B/C denotes CA_nxA-nyA, CA_nxA-nyB and CA_nxA-nyC, where nx and ny are two NR bands, ny is a FR2 band and A, B and C are the corresponding bandwidth classes respectively (See Table 1). The reason why FR2 constituent band is chosen to be simplified is that FR2 bands have larger bandwidth and more CA BW classes than FR1 bands (See Table 2), in which FBG#2 and FBG#3 both having 7 BW classes, and FBG#5 even having 11 BW classes.

Table 1: Examples for inter-band CA with “/” in FR2 uplink constituent band

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) | BCS |
| --- | --- | --- | --- | --- |
| … | … | … | … | … |
| CA_n1A-n257G | CA_n257G CA_n1A-n257A/G | n1 | 5, 10, 15, 20 | 0 |
|  |  | n257 | CA_n257G |  |
| CA_n1A-n257H | CA_n257G/H CA_n1A-n257A/G/H | n1 | 5, 10, 15, 20 | 0 |
|  |  | n257 | CA_n257H |  |
| CA_n1A-n257I | CA_n257G/H/I CA_n1A-n257A/G/H/I | n1 | 5, 10, 15, 20 | 0 |
|  |  | n257 | CA_n257I |  |
| CA_n1A-n257J | CA_n257G/H/I/J CA_n1A-n257A/G/H/I/J | n1 | 5, 10, 15, 20 | 0 |
|  |  | n257 | CA_n257J |  |
| CA_n1A-n257K | CA_n257G/H/I/J/K CA_n1A-n257A/G/H/I/J/K | n1 | 5, 10, 15, 20 | 0 |
|  |  | n257 | CA_n257K |  |
| CA_n1A-n257L | CA_n257G/H/I/J/K CA_n1A-n257A/G/H/I/J/K | n1 | 5, 10, 15, 20 | 0 |
|  |  | n257 | CA_n257L |  |
| CA_n1A-n257M | CA_n257G/H/I/J/K CA_n1A-n257A/G/H/I/J/K | n1 | 5, 10, 15, 20 | 0 |
|  |  | n257 | CA_n257M |  |

Table 2: FR2 CA bandwidth classes

| NR CA bandwidth class | Aggregated channel bandwidth | Number of contiguous CC | Fallback group |
| --- | --- | --- | --- |
| A | BWChannel ≤ 400 MHz | 1 | 1,2,3,4,5 |
| B | 400 MHz < BWChannel_CA ≤ 800 MHz | 2 | 1 |
| C | 800 MHz < BWChannel_CA ≤ 1200 MHz | 3 |  |
| V (Note 4) | 1200 MHz < BWChannel_CA ≤ 1600 MHz | 4 |  |
| W (Note 4) | 1600 MHz < BWChannel_CA ≤ 2000 MHz | 5 |  |
| D | 200 MHz < BWChannel_CA ≤ 400 MHz | 2 | 2 |
| E | 400 MHz < BWChannel_CA ≤ 600 MHz | 3 |  |
| F | 600 MHz < BWChannel_CA ≤ 800 MHz | 4 |  |
| R | 800 MHz < BWChannel_CA ≤ 1000 MHz | 5 | 7 classes |
| S | 1000 MHz < BWChannel_CA ≤ 1200 MHz | 6 |  |
| T | 1200 MHz < BWChannel_CA ≤ 1400 MHz | 7 |  |
| U | 1400 MHz < BWChannel_CA ≤ 1600 MHz | 8 |  |
| G | 100 MHz < BWChannel_CA ≤ 200 MHz | 2 | 3 |
| H | 200 MHz < BWChannel_CA ≤ 300 MHz | 3 |  |
| I | 300 MHz < BWChannel_CA ≤ 400 MHz | 4 |  |
| J | 400 MHz < BWChannel_CA ≤ 500 MHz | 5 | 7 classes |
| K | 500 MHz < BWChannel_CA ≤ 600 MHz | 6 |  |
| L | 600 MHz < BWChannel_CA ≤ 700 MHz | 7 |  |
| M | 700 MHz < BWChannel_CA ≤ 800 MHz | 8 |  |
| O | 100 MHz ≤ BWChannel_CA ≤ 200 MHz | 2 | 4 |
| P | 150 MHz ≤ BWChannel_CA ≤ 300 MHz | 3 |  |
| Q | 200 MHz ≤ BWChannel_CA ≤ 400 MHz | 4 |  |
| R2 | 200 MHz ≤ BWChannel_CA ≤ 400 MHz | 2 | 5 |
| R3 | 300 MHz ≤ BWChannel_CA ≤ 600 MHz | 3 |  |
| R4 | 400 MHz ≤ BWChannel_CA ≤ 800 MHz | 4 |  |
| R5 | 500 MHz ≤ BWChannel_CA ≤ 1000 MHz | 5 |  |
| R6 | 600 MHz ≤ BWChannel_CA ≤ 1200 MHz | 6 |  |
| R7 | 700 MHz ≤ BWChannel_CA ≤ 1400 MHz | 7 | 11 classes |
| R8 | 800 MHz ≤ BWChannel_CA ≤ 1600 MHz | 8 |  |
| R9 | 900 MHz ≤ BWChannel_CA ≤ 1800 MHz | 9 |  |
| R10 | 1000 MHz ≤ BWChannel_CA ≤ 2000 MHz | 10 |  |
| R11 | 1100 MHz ≤ BWChannel_CA ≤ 2200 MHz | 11 |  |
| R12 | 1200 MHz ≤ BWChannel_CA ≤ 2400 MHz | 12 |  |

Observation 1.  It is observed that due to the large number of CA BW classes in FR2, the inter-band CA configuration table has been greatly optimized by using the grouping rules with delimiter “/” in uplink CA. The retrieval of a certain band combination in the configuration table has no impact since the downlink CA remains no change.

Regarding to the inter-band EN-DC configuration tables, it is noticed that the downlink configurations are grouped with the configurations having the same component frequency bands in the current RAN4 spec. If multiple UL DC configurations are listed for multiple DL DC configurations, valid uplink configurations are such that uplink does not have more carriers than downlink. However, the delimiter “/” has not been introduced in the EN-DC configuration tables. Fig. 1 shows the comparison between the pages of inter-band DC configurations having FR2 band and without FR2 band as constituent band. It is beneficial if we can find a solution to optimize the configuration table having FR2 constituent band.

Fig 1   Inter-band DC configurations pages in TS 38.101-3 v18.6.0

In [2], the same solution as inter-band CA in uplink configuration by using the delimiter “/” to separate the different FR2 bandwidth classes are proposed.

Proposal 1.  For inter-band DC configurations with FR2 part of the uplink configurations, it is proposed to optimize the configuration tables with the following grouping rules.

- For inter-band DC configurations, the delimiter “/” could be used for the FR2 part of the uplink configurations, such as DC_xA_nyA/B/C, where x and ny are E-UTRA band and FR2 NR band, and A, B and C are the corresponding bandwidth classes respectively.

- For inter-band NR-DC configurations between FR1 and FR2, the delimiter “/” could be used for the FR2 part of the uplink configurations, such as DC_nxA_nyA/B/C, where nx and ny are FR1 NR band and FR2 NR band, and A, B and C are the corresponding bandwidth classes respectively.

Observation 2.  With Proposal 1, it is obvious that optimization to inter-band DC configuration table has the following benefits.

- More concise and readable uplink configurations.

- Same grouping rules as inter-band CA configurations for uplink.

- Reduced configuration table size.

- No information lost in the optimization.

- Example 1  Optimized EN-DC configuration table

Table 5.5B.6.5-1: Inter-band EN-DC configurations including FR1 and FR2 (six bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| … | … |
| DC_1A-3A-7A-28A_n38A-n257A DC_1A-3A-7A-28A_n38A-n257G DC_1A-3A-7A-28A_n38A-n257H DC_1A-3A-7A-28A_n38A-n257I | DC_1A_n257A/G/H/I DC_3A_n257A/G/H/I DC_28A_n257A/G/H/I |
| DC_1A-3A-7A-28A_n78A-n257A DC_1A-3A-7A-28A_n78A-n257G DC_1A-3A-7A-28A_n78A-n257H DC_1A-3A-7A-28A_n78A-n257I | DC_1A_n78A DC_1A_n257A/G/H/I DC_3A_n78A DC_3A_n257A/G/H/I DC_7A_n78A DC_7A_n257A/G/H/I DC_28A_n78A DC_28A_n257A/G/H/I |
| … | … |

- Example 2  Optimized NR-DC configuration table

Table 5.5B.7-4: Inter-band NR-DC configurations between FR1 and FR2 (five bands)

| Downlink NR DC configuration | Uplink NR DC configuration |
| --- | --- |
| DC_n3A-n28A-n77A-n79A-n257A DC_n3A-n28A-n77A-n79A-n257G DC_n3A-n28A-n77A-n79A-n257H DC_n3A-n28A-n77A-n79A-n257I | DC_n3A-n257A/G/H/I DC_n28A-n257A/G/H/I DC_n77A-n257A/G/H/I DC_n79A-n257A/G/H/I |
| DC_n3A-n28A-n77(2A)-n79A-n257A DC_n3A-n28A-n77(2A)-n79A-n257G DC_n3A-n28A-n77(2A)-n79A-n257H DC_n3A-n28A-n77(2A)-n79A-n257I | DC_n3A-n257A/G/H/I DC_n28A-n257A/G/H/I DC_n77A-n257A/G/H/I DC_n79A-n257A/G/H/I |

In last RAN4 meeting, there is another candidate table format for EN-DC configuration as proposed in [3] shown as below.

Candidate DC table optimization

5.5B.5.1-1: Inter-band EN-DC configurations including FR2 (two bands): Inter-band CA operating bands involving FR1 (three bands)

| EN-DC configuration |  | Uplink EN-DC configuration (NOTE 1) |
| --- | --- | --- |
| Configuration | Possible values of X for each configuration |  |
| DC_2A_n258”X” | A, D, H, I, J, K, L, M, O, P, Q | A, D, H, I, J, K, L, M, O, P, Q |

The main concern with regards to this candidate configuration table format is that the downlink configurations are much different with the current spec. If we want to retrieve a certain band combination in the spec, it is difficult to use the wildcard “X”. Step back, if we use the wildcard for DC_2A_n258”X” in the downlink, using delimiter “/” such as DC_2A-n258A/D/H/I/J/K/L/M/O/P/Q will be more intuitive. According to the agreement in last meeting, RAN4 shall avoid to have big specification change for NR specification at this late stage.

Observation 3.  The candidate DC table optimization in [3] has big change to the current configuration table and will have big impact on combo retrieval.

With the above analysis, we suggest to use Proposal 1 as the way to optimize inter-band DC configuration tables.

Proposal 2.  It is proposed to use the solution as in proposed in Proposal 1 to optimize the inter-band DC configurations.

- The optimization could be applied starting from Rel-19 and the draft CRs be ready before Dec, 2024.

- 3. Conclusion

In this paper, we provide our considerations on specification improvement for table modification. The following observations and proposals are provided.

Observation 1.  It is observed that due to the large number of CA BW classes in FR2, the inter-band CA configuration table has been greatly optimized by using the grouping rules with delimiter “/” in uplink CA. The retrieval of a certain band combination in the configuration table has no impact since the downlink CA remains no change.

Proposal 1.  For inter-band DC configurations with FR2 part of the uplink configurations, it is proposed to optimize the configuration tables with the following grouping rules.

- For inter-band DC configurations, the delimiter “/” could be used for the FR2 part of the uplink configurations, such as DC_xA_nyA/B/C, where x and ny are E-UTRA band and FR2 NR band, and A, B and C are the corresponding bandwidth classes respectively.

- For inter-band NR-DC configurations between FR1 and FR2, the delimiter “/” could be used for the FR2 part of the uplink configurations, such as DC_nxA_nyA/B/C, where nx and ny are FR1 NR band and FR2 NR band, and A, B and C are the corresponding bandwidth classes respectively.

Observation 2.  With Proposal 1, it is obvious that optimization to inter-band DC configuration table has the following benefits.

- More concise and readable uplink configurations.

- Same grouping rules as inter-band CA configurations for uplink.

- Reduced configuration table size.

- No information lost in the optimization.

Observation 3.  The candidate DC table optimization in [3] has big change to the current configuration table and will have big impact on combo retrieval.

Proposal 2.  It is proposed to use the solution as in proposed in Proposal 1 to optimize the inter-band DC configurations.

- The optimization could be applied starting from Rel-19 and the draft CRs be ready before Dec, 2024.

# 4. References

- R4-2410714, WF on for [111][139] UERF_Spec_Improvement, Qualcomm Incorporated, RAN4#111.

- R4-2409510, Further considerations on band combination configuration tables with FR2 component bands, ZTE, RAN4#111.

- R4-2407688, On table modifications and technical wording ambiguities, Huawei, HiSilicon, RAN4#111.