# 3GPP TSG- RAN WG4 Meeting # 112	R4-2411843

# Maastricht, Netherlands, 19th–23rd Aug, 2024

# Source: 	ZTE Corporation, Sanechips

# Title: 	Discussion on SAN RF requirements for NR-NTN S-band

# Agenda item:	7.17.3

# Document for:	Approval

# 1 Introduction

In RAN#104 meeting, a new Work Item [1] was approved on introduction of NR-NTN S-band with the following RAN4 objective. In this contribution, we want to share some initial views about NR-NTN S-band from SAN RF requirements perspective.

The objective of the core part is to: - Specify a new NR-NTN FDD band with a UE transmitting at 2000 – 2020 MHz and SAN transmitting at 2180-2200 MHz; - Perform necessary co-existence analysis, for example UE-UE coexistence, for adjacent band per RAN4 scope; - Consider whether assumptions for coexistence should be re-evaluated for the NTN scenario - Support channel bandwidths and SCS as presented in Table 4.1-1 below; - Mandatory support of channel raster points at step of 10 kHz; - Support UE Power class 3 (+23dBm); - Introduce the corresponding SAN and UE RF core requirements; - Introduce the corresponding RRM requirements. Table 4.1-1: Channel bandwidth and SCS system parameters. SCS (kHz); Channel bandwidth (MHz). 15; 5; 10; 15; 20. 30; 10; 15; 20. 60; 10; 15; 20. The specification work of this WI shall leverage the studies and requirements for NR NTN n256, n255, n254, where applicable. All UE requirements specified as part of this WI shall be Release-independent from Rel-17.

# 2 Discussion

The following SAN RF requirements need to be defined for the new NR-NTN S-band.

Table 1: SAN RF requirements for the new NR-NTN S-band

| NR BS RF Tx/Rx requirement | Proposal for the new NR-NTN S-band |
| --- | --- |
| 6.2 Satellite Access Node output power | Not band specific --> No specification impact. |
| 6.3.2 RE power control dynamic range | Not band specific --> No specification impact. |
| 6.3.3 Total power dynamic range | Not band specific --> No specification impact. |
| 6.4 Transmit ON/OFF power | Not applicable to SAN --> No specification impact. |
| 6.5 Transmitted signal quality | Not band specific --> No specification impact. |
| 6.6.2 Occupied bandwidth | Not band specific --> No specification impact. |
| 6.6.3 Adjacent Channel Leakage Power Ratio | Not band specific --> No specification impact. |
| 6.6.4 Out-of-band emissions | Band specific --> Specification impact. Consider define out-of-band emissions requirement similar to band n254, n255, n256. |
| 6.6.5 Transmitter spurious emissions | No specification impact. |
| 6.7 Transmitter intermodulation | Not applicable to SAN --> No specification impact. |
| 7.2 Reference sensitivity level | Not band specific --> No specification impact. |
| 7.3 Dynamic range | Not band specific --> No specification impact. |
| 7.4.1 Adjacent Channel Selectivity (ACS) | Not band specific --> No specification impact. |
| 7.4.2 In-band blocking | Not applicable for SAN --> No specification impact. |
| 7.5 Out-of-band blocking | Not band specific --> No specification impact. |
| 7.6 Receiver spurious emissions | Not applicable for SAN --> No specification impact. |
| 7.7 Receiver intermodulation | Not applicable for SAN --> No specification impact. |
| 7.8 In-channel selectivity | Not band specific --> No specification impact. |

Proposal 1: To use the proposals in Table 1 for SAN RF requirements for the new NR-NTN FDD band.

# 3 Conclusion

In this contribution, we shared some views on SAN RF requirements for the new NR-NTN S-band and the proposals are made as follows:

Proposal 1: To use the proposals in Table 1 for SAN RF requirements for the new NR-NTN S-band.

Table 1: SAN RF requirements for the new NR-NTN S-band

| NR BS RF Tx/Rx requirement | Proposal for the new NR-NTN S-band |
| --- | --- |
| 6.2 Satellite Access Node output power | Not band specific --> No specification impact. |
| 6.3.2 RE power control dynamic range | Not band specific --> No specification impact. |
| 6.3.3 Total power dynamic range | Not band specific --> No specification impact. |
| 6.4 Transmit ON/OFF power | Not applicable to SAN --> No specification impact. |
| 6.5 Transmitted signal quality | Not band specific --> No specification impact. |
| 6.6.2 Occupied bandwidth | Not band specific --> No specification impact. |
| 6.6.3 Adjacent Channel Leakage Power Ratio | Not band specific --> No specification impact. |
| 6.6.4 Out-of-band emissions | Band specific --> Specification impact. Consider define out-of-band emissions requirement similar to band n254, n255, n256. |
| 6.6.5 Transmitter spurious emissions | No specification impact. |
| 6.7 Transmitter intermodulation | Not applicable to SAN --> No specification impact. |
| 7.2 Reference sensitivity level | Not band specific --> No specification impact. |
| 7.3 Dynamic range | Not band specific --> No specification impact. |
| 7.4.1 Adjacent Channel Selectivity (ACS) | Not band specific --> No specification impact. |
| 7.4.2 In-band blocking | Not applicable for SAN --> No specification impact. |
| 7.5 Out-of-band blocking | Not band specific --> No specification impact. |
| 7.6 Receiver spurious emissions | Not applicable for SAN --> No specification impact. |
| 7.7 Receiver intermodulation | Not applicable for SAN --> No specification impact. |
| 7.8 In-channel selectivity | Not band specific --> No specification impact. |

# 4 Reference

- RP-241661, New WID on Introduction of NR-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL) , DISH Network et al.