# 3GPP TSG- RAN WG4 Meeting # 112	R4-2411845

# Maastricht, Netherlands, 19th–23rd Aug, 2024

# Source: 	ZTE Corporation, Sanechips

# Title: 	Discussion on system parameters and UE RF requirements for IoT-NTN S-band

# Agenda item:	7.18.2

# Document for:	Approval

# 1 Introduction

In RAN#104 meeting, a new Work Item [1] was approved on introduction of IoT-NTN S-band with the following RAN4 objective. In this contribution, we want to share some initial views about IoT-NTN S-band from system parameters and UE RF requirements perspective.

The objective of the core part is to: - Specify a new E-UTRA NTN FDD band for NB-IoT/eMTC NTN operation, with a UE transmitting at 2000 – 2020 MHz and SAN transmitting at 2180-2200 MHz; - Perform necessary co-existence analysis, for example UE-UE coexistence, for adjacent band per RAN4 scope; - Consider whether assumptions for coexistence should be re-evaluated for the NTN scenario; - Support UE Categories NB1, NB2, M1; - Support UE Power Class 3 (+23dBm) and Power Class 5 (+20dBm); - Introduce the corresponding SAN and UE RF core requirements; - Introduce the corresponding RRM requirements. The specification work of this WI shall leverage the studies and requirements for NR/LTE NTN n256/B256, n255/B255, n254/B254, where applicable. All UE requirements specified as part of this WI shall be Release-independent from Rel-17.

# 2 Discussion

  - 2.1 Operating band and band numbering

This WID is to introduce a new E-UTRA NTN FDD band for NB-IoT/eMTC NTN operation, with a UE transmitting at 2000 - 2020 MHz and SAN transmitting at 2180 - 2200 MHz. For the band number, RAN4 has defined a principle of “first come first served”, and the corresponding operating band and band number can be defined as Table 2.1-1. In addition, it supports UE categories NB1, NB2, M1.

Figure 2.1-1: New E-UTRA NTN FDD band with UL at 2000 - 2020 MHz and DL at 2180 - 2200 MHz

Table 2.1-1: E-UTRA operating bands for satellite access

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| [252] | 2000 MHz | – | 2020 MHz | 2180 MHz | – | 2200 MHz | FDD |

Proposal 1: Operating band and band numbering for the new IoT-NTN FDD band can be defined as Table 2.1-1, and it supports UE categories NB1, NB2, M1.

  - 2.2 Channel raster

Based on the definition of EARFCN for category M1, EARFCN for category M1 for IoT-NTN band [252] can be defined as follows.

Table 2.2-1: E-UTRA channel numbers

| E-UTRA Operating Band | ΔFRaster (kHz) | Downlink |  |  | Uplink |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | FDL_low (MHz) | NOffs-DL | Range of NDL (First – <Step size> – Last) | FUL_low (MHz) | NOffs-UL | Range of NUL (First – <Step size> – Last) |
| [252] | 100 | 2180 | 228301 | 228301 –<1>- 228500 | 2000 | 261069 | 261069 –<1>- 261268 |

Proposal 2: EARFCN for category M1 for IoT-NTN band [252] can be defined as Table 2.2-1.

  - 2.3 TX-RX frequency separation

For TX-RX frequency separation, similar as LTE band 23, which supports default UE TX-RX frequency separation of 180 MHz, it should be defined as 180 MHz for IoT-NTN band [252] as Table 2.3-1.

Table 2.3-1: Default UE TX-RX frequency separation

| NTN Satellite Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| [252] | 180 MHz |

Proposal 3: The default TX-RX frequency separation for IoT-NTN band [252] should be defined as Table 2.3-1.

  - 2.4 UE RF requirements

The following UE RF requirements need to be defined for the new IoT-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL).

Table 2.4-1 UE RF requirements for the new IoT-NTN S-band

| NR-NTN UE Tx/Rx requirement | Proposals for the new new IoT-NTN S-band |
| --- | --- |
| - 6.2A.1 UE maximum output power for category M1 | Band specific --> Specification impact. Support UE Power Class 3 (+23dBm) and Power Class 5 (+20dBm). |
| - 6.2B.1 UE maximum output power for category NB1 and NB2 |  |
| - 6.2A.2 UE maximum output power reduction for category M1 | Not band specific --> No specification impact. |
| - 6.2B.2 UE maximum output power reduction for category NB1 and NB2 |  |
| - 6.2A.3 A-MPR for category M1 UE | Band specific --> Specification impact. A-MPR requirement for B23 can be reused for the new IoT-NTN S-band. |
| - 6.2B.3 A-MPR for category NB1 and NB2 UE |  |
| - 6.2A.4 Configured transmitted Power for category M1 | Not band specific --> No specification impact. |
| - 6.2B.4 Configured transmitted Power for category NB1 and NB2 |  |
| 6.3A Output power dynamics for category M1 | No specification impact. |
| 6.3B Output power dynamics for category NB1 and NB2 |  |
| 6.4A Transmit signal quality for category M1 | Not band specific --> No specification impact. |
| 6.4B Transmit signal quality for category NB1 and NB2 |  |
| 6.4.2 Transmit modulation quality | Not band specific --> No specification impact. |
| 6.5A.2 Occupied bandwidth for category M1 | Not band specific --> No specification impact. |
| 6.5B.2 Occupied bandwidth for category NB1 and NB2 |  |
| 6.5A.3.2 Spectrum emission mask | Not band specific --> No specification impact. |
| 6.5B.3.2 Spectrum emission mask |  |
| 6.5A.3.3 Additional Spectrum Emission Mask for category M1 | Not applicable. |
| 6.5B.3.3 Additional Spectrum Emission Mask for category NB1 and NB2 |  |
| 6.5A.3.4 Adjacent Channel Leakage Ratio for category M1 | Not band specific --> No specification impact. |
| 6.5B.3.4 Adjacent Channel Leakage Ratio for category NB1 and NB2 |  |
| 6.5A.4.2 General spurious emissions | Not band specific --> No specification impact. |
| 6.5B.4.2 General spurious emissions |  |
| 6.5A.4.3 Spurious emission band UE co-existence | Specification impact. Band [252] needs to be added in Table 6.5A.4.3-1. |
| 6.5B.4.3 Spurious emission band UE co-existence |  |
| 6.5A.4.4 Additional spurious emissions | Specification impact. Additional spurious emissions requirement for B23 can be reused for the new IoT-NTN S-band. |
| 6.5B.4.4 Additional spurious emissions |  |
| 6.6A Transmit intermodulation for category M1 | Not applicable. |
| 6.6B Transmit intermodulation for category NB1 and NB2 | No specification impact. |
| 7.3A Reference sensitivity power level for UE category M1 | Band specific --> Specification impact. |
| 7.3B Reference sensitivity power level for UE category NB1 and NB2 | Band specific. REFSENS requirement for B23 can be reused for the new IoT-NTN S-band. |
| 7.4A Maximum input level for category M1 | Not band specific --> No specification impact. |
| 7.4B Maximum input level for category NB1 and NB2 |  |
| 7.5A Adjacent Channel Selectivity for category M1 | Not band specific --> No specification impact. |
| 7.5B Adjacent Channel Selectivity for category NB1 and NB2 |  |
| 7.6A.2 In-band blocking requirements for category M1 | Specification impact --> Considering define in-band blocking requirement similar to band 253, 254, 255, 256. |
| 7.6B.2 In-band blocking requirements for category NB1 and NB2 | No specification impact. |
| 7.6A.3 Out-of-band blocking requirements for category M1 | Specification impact --> Considering define out-of-band blocking requirement similar to band 253, 254, 255. |
| 7.6B.3 Out-of-band blocking requirements for category NB1 and NB2 |  |
| 7.6A.4 Narrow band blocking for category M1 | No specification impact. |
| 7.6B.4 Narrow band blocking for category NB1 and NB2 | Not applicable. |
| 7.7A Spurious response for category M1 | Not band specific --> No specification impact. |
| 7.7B Spurious response for category NB1 and NB2 |  |
| 7.8A Intermodulation characteristics for category M1 | Not band specific --> No specification impact. |
| 7.8B Intermodulation characteristics for category NB1 and NB2 |  |
| 7.9 Spurious emissions | Not band specific --> No specification impact. |

Proposal 4: To use the proposals in Table 2.4-1 for UE RF requirements for the new IoT-NTN S-band.

# 3 Conclusion

In this contribution, we shared some views on system parameters and UE RF requirements for the new IoT-NTN S-band and the proposals are made as follows:

Proposal 1: Operating band and band numbering for the new IoT-NTN FDD band can be defined as Table 2.1-1, and it supports UE categories NB1, NB2, M1.

Table 2.1-1: E-UTRA operating bands for satellite access

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| [252] | 2000 MHz | – | 2020 MHz | 2180 MHz | – | 2200 MHz | FDD |

Proposal 2: EARFCN for category M1 for IoT-NTN band [252] can be defined as Table 2.2-1.

Table 2.2-1: E-UTRA channel numbers

| E-UTRA Operating Band | ΔFRaster (kHz) | Downlink |  |  | Uplink |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | FDL_low (MHz) | NOffs-DL | Range of NDL (First – <Step size> – Last) | FUL_low (MHz) | NOffs-UL | Range of NUL (First – <Step size> – Last) |
| [252] | 100 | 2180 | 228301 | 228301 –<1>- 228500 | 2000 | 261069 | 261069 –<1>- 261268 |

Proposal 3: The default TX-RX frequency separation for IoT-NTN band [252] should be defined as Table 2.3-1.

Table 2.3-1: Default UE TX-RX frequency separation

| NTN Satellite Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| [252] | 180 MHz |

Proposal 4: To use the proposals in Table 2.4-1 for UE RF requirements for the new IoT-NTN S-band.

Table 2.4-1 UE RF requirements for the new IoT-NTN S-band

| NR-NTN UE Tx/Rx requirement | Proposals for the new new IoT-NTN S-band |
| --- | --- |
| - 6.2A.1 UE maximum output power for category M1 | Band specific --> Specification impact. Support UE Power Class 3 (+23dBm) and Power Class 5 (+20dBm). |
| - 6.2B.1 UE maximum output power for category NB1 and NB2 |  |
| - 6.2A.2 UE maximum output power reduction for category M1 | Not band specific --> No specification impact. |
| - 6.2B.2 UE maximum output power reduction for category NB1 and NB2 |  |
| - 6.2A.3 A-MPR for category M1 UE | Band specific --> Specification impact. A-MPR requirement for B23 can be reused for the new IoT-NTN S-band. |
| - 6.2B.3 A-MPR for category NB1 and NB2 UE |  |
| - 6.2A.4 Configured transmitted Power for category M1 | Not band specific --> No specification impact. |
| - 6.2B.4 Configured transmitted Power for category NB1 and NB2 |  |
| 6.3A Output power dynamics for category M1 | No specification impact. |
| 6.3B Output power dynamics for category NB1 and NB2 |  |
| 6.4A Transmit signal quality for category M1 | Not band specific --> No specification impact. |
| 6.4B Transmit signal quality for category NB1 and NB2 |  |
| 6.4.2 Transmit modulation quality | Not band specific --> No specification impact. |
| 6.5A.2 Occupied bandwidth for category M1 | Not band specific --> No specification impact. |
| 6.5B.2 Occupied bandwidth for category NB1 and NB2 |  |
| 6.5A.3.2 Spectrum emission mask | Not band specific --> No specification impact. |
| 6.5B.3.2 Spectrum emission mask |  |
| 6.5A.3.3 Additional Spectrum Emission Mask for category M1 | Not applicable. |
| 6.5B.3.3 Additional Spectrum Emission Mask for category NB1 and NB2 |  |
| 6.5A.3.4 Adjacent Channel Leakage Ratio for category M1 | Not band specific --> No specification impact. |
| 6.5B.3.4 Adjacent Channel Leakage Ratio for category NB1 and NB2 |  |
| 6.5A.4.2 General spurious emissions | Not band specific --> No specification impact. |
| 6.5B.4.2 General spurious emissions |  |
| 6.5A.4.3 Spurious emission band UE co-existence | Specification impact. Band [252] needs to be added in Table 6.5A.4.3-1. |
| 6.5B.4.3 Spurious emission band UE co-existence |  |
| 6.5A.4.4 Additional spurious emissions | Specification impact. Additional spurious emissions requirement for B23 can be reused for the new IoT-NTN S-band. |
| 6.5B.4.4 Additional spurious emissions |  |
| 6.6A Transmit intermodulation for category M1 | Not applicable. |
| 6.6B Transmit intermodulation for category NB1 and NB2 | No specification impact. |
| 7.3A Reference sensitivity power level for UE category M1 | Band specific --> Specification impact. |
| 7.3B Reference sensitivity power level for UE category NB1 and NB2 | Band specific. REFSENS requirement for B23 can be reused for the new IoT-NTN S-band. |
| 7.4A Maximum input level for category M1 | Not band specific --> No specification impact. |
| 7.4B Maximum input level for category NB1 and NB2 |  |
| 7.5A Adjacent Channel Selectivity for category M1 | Not band specific --> No specification impact. |
| 7.5B Adjacent Channel Selectivity for category NB1 and NB2 |  |
| 7.6A.2 In-band blocking requirements for category M1 | Specification impact --> Considering define in-band blocking requirement similar to band 253, 254, 255, 256. |
| 7.6B.2 In-band blocking requirements for category NB1 and NB2 | No specification impact. |
| 7.6A.3 Out-of-band blocking requirements for category M1 | Specification impact --> Considering define out-of-band blocking requirement similar to band 253, 254, 255. |
| 7.6B.3 Out-of-band blocking requirements for category NB1 and NB2 |  |
| 7.6A.4 Narrow band blocking for category M1 | No specification impact. |
| 7.6B.4 Narrow band blocking for category NB1 and NB2 | Not applicable. |
| 7.7A Spurious response for category M1 | Not band specific --> No specification impact. |
| 7.7B Spurious response for category NB1 and NB2 |  |
| 7.8A Intermodulation characteristics for category M1 | Not band specific --> No specification impact. |
| 7.8B Intermodulation characteristics for category NB1 and NB2 |  |
| 7.9 Spurious emissions | Not band specific --> No specification impact. |

# 4 Reference

- RP-241662, New WID on Introduction of IoT-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL), DISH Network et al.