# 3GPP TSG- RAN WG4 Meeting # 112	R4-2411846

# Maastricht, Netherlands, 19th–23rd Aug, 2024

# Source: 	ZTE Corporation, Sanechips

# Title: 	Discussion on SAN RF requirements for IoT-NTN S-band

# Agenda item:	7.18.3

# Document for:	Approval

# 1 Introduction

In RAN#104 meeting, a new Work Item [1] was approved on introduction of IoT-NTN S-band with the following RAN4 objective. In this contribution, we want to share some initial views about IoT-NTN S-band from SAN RF requirements perspective.

The objective of the core part is to: - Specify a new E-UTRA NTN FDD band for NB-IoT/eMTC NTN operation, with a UE transmitting at 2000 – 2020 MHz and SAN transmitting at 2180-2200 MHz; - Perform necessary co-existence analysis, for example UE-UE coexistence, for adjacent band per RAN4 scope; - Consider whether assumptions for coexistence should be re-evaluated for the NTN scenario; - Support UE Categories NB1, NB2, M1; - Support UE Power Class 3 (+23dBm) and Power Class 5 (+20dBm); - Introduce the corresponding SAN and UE RF core requirements; - Introduce the corresponding RRM requirements. The specification work of this WI shall leverage the studies and requirements for NR/LTE NTN n256/B256, n255/B255, n254/B254, where applicable. All UE requirements specified as part of this WI shall be Release-independent from Rel-17.

# 2 Discussion

The following SAN RF requirements need to be defined for the new IoT-NTN FDD band.

Table 1: SAN RF requirements for the new IoT-NTN FDD band

| NR BS RF Tx/Rx requirement | Proposal for the new IoT-NTN FDD band |
| --- | --- |
| 6.2 Satellite Access Node output power | Not band specific --> No specification impact. |
| 6.3.2 RE power control dynamic range | Not band specific --> No specification impact. |
| 6.3.3 Total power dynamic range | Not band specific --> No specification impact. |
| 6.4 Transmit ON/OFF power | Not applicable to SAN. |
| 6.5 Transmitted signal quality | Not band specific --> No specification impact. |
| 6.6.2 Occupied bandwidth | Not band specific --> No specification impact. |
| 6.6.3 Adjacent Channel Leakage Power Ratio | Not band specific --> No specification impact. |
| 6.6.4 Out-of-band emissions | Band specific --> Specification impact. Consider define out-of-band emissions requirement similar as other bands. |
| 6.6.5 Transmitter spurious emissions | No specification impact. |
| 6.7 Transmitter intermodulation | Not applicable to SAN. |
| 7.2 Reference sensitivity level | Not band specific --> No specification impact. |
| 7.3 Dynamic range | Not band specific --> No specification impact. |
| 7.4.1 Adjacent Channel Selectivity (ACS) | Not band specific --> No specification impact. |
| 7.4.2 In-band blocking | Not applicable for SAN. |
| 7.5 Out-of-band blocking | Not band specific --> No specification impact. |
| 7.6 Receiver spurious emissions | Not applicable for SAN. |
| 7.7 Receiver intermodulation | Not applicable for SAN --> No specification impact. |
| 7.8 In-channel selectivity | Not band specific --> No specification impact. |

Proposal 1: To use the proposals in Table 1 for SAN RF requirements for the new IoT-NTN FDD band.

# 3 Conclusion

In this contribution, we shared some views on SAN RF requirements for the new IoT-NTN FDD band and the proposals are made as follows:

Proposal 1: To use the proposals in Table 1 for SAN RF requirements for the new IoT-NTN FDD band.

Table 1: SAN RF requirements for the new IoT-NTN FDD band

| NR BS RF Tx/Rx requirement | Proposal for the new IoT-NTN FDD band |
| --- | --- |
| 6.2 Satellite Access Node output power | Not band specific --> No specification impact. |
| 6.3.2 RE power control dynamic range | Not band specific --> No specification impact. |
| 6.3.3 Total power dynamic range | Not band specific --> No specification impact. |
| 6.4 Transmit ON/OFF power | Not applicable to SAN. |
| 6.5 Transmitted signal quality | Not band specific --> No specification impact. |
| 6.6.2 Occupied bandwidth | Not band specific --> No specification impact. |
| 6.6.3 Adjacent Channel Leakage Power Ratio | Not band specific --> No specification impact. |
| 6.6.4 Out-of-band emissions | Band specific --> Specification impact. Consider define out-of-band emissions requirement similar as other bands. |
| 6.6.5 Transmitter spurious emissions | No specification impact. |
| 6.7 Transmitter intermodulation | Not applicable to SAN. |
| 7.2 Reference sensitivity level | Not band specific --> No specification impact. |
| 7.3 Dynamic range | Not band specific --> No specification impact. |
| 7.4.1 Adjacent Channel Selectivity (ACS) | Not band specific --> No specification impact. |
| 7.4.2 In-band blocking | Not applicable for SAN. |
| 7.5 Out-of-band blocking | Not band specific --> No specification impact. |
| 7.6 Receiver spurious emissions | Not applicable for SAN. |
| 7.7 Receiver intermodulation | Not applicable for SAN --> No specification impact. |
| 7.8 In-channel selectivity | Not band specific --> No specification impact. |

# 4 Reference

- RP-241662, New WID on Introduction of IoT-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL), DISH Network et al.