# 3GPP TSG- RAN WG4 Meeting # 112	R4-2411848

# Maastricht, Netherlands, 19th–23rd Aug, 2024

# Source: 	ZTE Corporation, Sanechips

# Title: 	Discussion on system parameters and UE RF requirements for NR-NTN combined L-band

# Agenda item:	7.19.2

# Document for:	Approval

# 1 Introduction

In RAN#104 meeting, a new Work Item [1] was approved on introduction of new NR NTN bands to support the Extended L-band (UL 1668-1675MHz, DL 1518-1525MHz) and the combined MSS L-band and Extended L-band ranges (DL 1518-1559 MHz, UL 1626.5-1660.5 MHz and 1668-1675 MHz) with the following RAN4 objective. In this contribution, we want to share some initial views from system parameters and UE RF requirements perspective.

The objectives of the core part are as follows: - Specify a new NR NTN FDD band with a UE transmitting at 1668 – 1675 MHz and SAN transmitting at 1518 – 1525 MHz; - Support 5MHz channel bandwidth with 15kHz SCS - Specify new NR NTN FDD band(s) with SAN transmitting at 1518 – 1559 MHz and UE transmitting at 1626.5 – 1660.5 MHz or 1668 – 1675 MHz; - Support channel bandwidths and SCS as presented in Table 4.1-1 below: Table 4.1-1: DL Channel bandwidth and SCS system parameters NTN satellite band; SCS kHz; DL Channel bandwidth (MHz). 5; 10; 15; 20; 25; 30; 35; 40. 15; 5; 10; 15; 20; 25; 30; 35; 40. [n25x]; 30; 10; 15; 20; 25; 30; 35; 40. 60; 10; 15; 20; 25; 30; 35; 40. - Support Asymmetric UE TX-RX Channel BW configurations to allow leveraging the 41 MHz contiguous DL allocation with a smaller UL allocation. NOTE: UL channel allocations spanning across the 1660.5-1668 MHz gap shall not be allowed - For these bands, the following shall be specified: - Support Flexible TX-RX Separation - Mandatory support for Enhanced Channel Raster with raster points at step of 10kHz - Support UE Power class 3 (+23 dBm) as a starting point - Introduce the corresponding SAN and UE RF core requirements; - Introduce the corresponding RRM requirements. - Requirements for protection of the Radio Astronomy in the 1660-1670 MHz range shall be captured - Any relevant additional Regional requirements and restrictions will also be captured. - NOTE1: The introduction of ETSI-specific requirements is expected to be addressed at a later stage, due to ongoing discussions within ETSI in respect to NTN - NOTE2: The 1668-1675 MHz range is presently not available for MSS in the continental United States. NOTE: The specification work in this WI shall leverage the studies and requirements for NR NTN n255, and the corresponding IoT NTN Work Item (IoT_NTN_extLband) for Band 253, where applicable. All UE requirements specified as part of this WI shall be Release-independent for the UE starting from Release-17.

# 2 Discussion

  - 2.1 Operating band and band numbering

This WID is to introduce new NR NTN bands to support the Extended L-band (UL 1668-1675MHz, DL 1518-1525MHz) and the combined MSS L-band and Extended L-band ranges (DL 1518-1559 MHz, UL 1626.5-1660.5 MHz and 1668-1675 MHz). For the band number, RAN4 has defined a principle of “first come first served”. And there is another approved WID of introducing a new NR NTN bands to support the extended L-band, whose tdoc number is before this WID. Hence, we can treat these new NR-NTN bands as the latter and the corresponding operating bands and band number can be defined as Table 2.1-1.

Figure 2.1-1: Extended L-band (UL 1668-1675MHz, DL 1518-1525MHz) and combined MSS L-band and Extended L-band ranges (DL 1518-1559 MHz, UL 1626.5-1660.5 MHz and 1668-1675 MHz)

Table 2.1-1: NTN satellite bands in FR1-NTN

| Satellite operating band | Uplink (UL) operating band Satellite Access Node receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band Satellite Access Node transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| [n251] | 1668 MHz - 1675 MHz | 1518 MHz - 1525 MHz | FDD |
| [n250] | 1626.5 MHz - 1660.5 MHz | 1518 MHz - 1559 MHz | FDD |
| [n249] | 1668 MHz - 1675 MHz | 1518 MHz - 1559 MHz | FDD |

Proposal 1: Operating band and band numbering for the new NR-NTN bands supporting the Extended L-band (UL 1668-1675MHz, DL 1518-1525MHz) and the combined MSS L-band and Extended L-band ranges (DL 1518-1559 MHz, UL 1626.5-1660.5 MHz and 1668-1675 MHz) can be defined as Table 2.1-1.

  - 2.2 Channel bandwidth

According to the objective of this WID, new NR NTN FDD band(s) with SAN transmitting at 1518–1559 MHz and UE transmitting at 1626.5–1660.5 MHz or 1668–1675 MHz need to support channel bandwidth up to 40MHz. However, there are only 5, 10, 15, 20 MHz channel bandwidth and 30 MHz CBW without related RF requirements supported in current spec. Firstly, supported channel bandwidths and SCS can be defined as Table 2.2-1. Whether to support 25, 35, 40 MHz channel bandwidth needs further study.

Table 2.2-1: Channel bandwidths for each NTN satellite band in FR1-NTN

| NTN satellite band | SCS kHz | UE Channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 | 30 (NOTE) |
|  | 15 | 5 |  |  |  |  |
| [n251] | 30 |  |  |  |  |  |
|  | 60 |  |  |  |  |  |
|  | 15 | 5 | 10 | 15 | 20 | 30 |
| [n250] | 30 |  | 10 | 15 | 20 | 30 |
|  | 60 |  | 10 | 15 | 20 | 30 |
|  | 15 | 5 |  |  |  |  |
| [n249] | 30 |  |  |  |  |  |
|  | 60 |  |  |  |  |  |

Proposal 2: Firstly, supported channel bandwidths and SCS can be defined as Table 2.2-1. Whether to support 25, 35, 40 MHz channel bandwidth needs further study.

  - 2.3 Asymmetric channel bandwidths

The objective of this WID states new NR NTN FDD band(s) with SAN transmitting at 1518–1559 MHz and UE transmitting at 1626.5–1660.5 MHz or 1668–1675 MHz can support asymmetric channel bandwidths.

Proposal 3: New NTN bands [n250] and [n249] can support asymmetric channel bandwidths.

  - 2.4 Channel raster and sync raster

The objective of this WID has specified that mandatory support of channel raster points at step of 10 kHz. The new NR-NTN bands need to support 100 kHz and 10 kHz channel raster. For NR-ARFCN, we propose to use the similar descending manner to define as following:

Table 2.4-1: Applicable NR-ARFCN per operating band in FR1-NTN with 100 kHz channel rater

| SAN operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| [n251] | 100 | 333600– <20> – 335000 | 303600 – <20> – 305000 |
| [n250] | 100 | 325300– <20> – 332100 | 303600 – <20> – 311800 |
| [n249] | 100 | 333600– <20> – 335000 | 303600 – <20> – 311800 |

Table 2.4-2: Applicable NR-ARFCN per operating band in FR1-NTN with 10 kHz channel rater

| NTN satellite operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) | Mandatory support |
| --- | --- | --- | --- | --- |
| [n251] | 10 | 333600– <2> – 335000 | 303600 – <2> – 305000 | Yes |
| [n250] | 10 | 325300– <2> – 332100 | 303600 – <2> – 311800 | Yes |
| [n249] | 10 | 333600– <2> – 335000 | 303600 – <2> – 311800 | Yes |

Proposal 4: NR-ARFCN for the new NR-NTN bands supporting 100 kHz and 10 kHz channel raster can be defined as Table 2.4-1 and Table 2.4-2, respectively.

In addition, band [n251] only support 5MHz channel bandwidth, so it can only support case A SSB pattern.  Downlink of 1518 - 1559 MHz for band [n250] and [n249] partially overlapping with band n254 DL, and n255 supports case A and case B SSB pattern. So, we propose that band [n250] and [n249] support case A and case B SSB pattern. Sync raster and GSCN for the new NR-NTN bands should follow the current design of sync raster in TS 38.101-5 and defined as Table 2.4-3.

Table 2.4-3: Applicable SS raster entries per operating band (FR1-NTN)

| NTN satellite operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| [n251] | 15 kHz | Case A | 3800 – <1> – 3807 |
| [n250] | 15 kHz | Case A | 3800 – <1> – 3892 |
|  | 30 kHz | Case B | 3806 – <1> – 3886 |
| [n249] | 15 kHz | Case A | 3800 – <1> – 3892 |

Proposal 5: We propose that band [n251] and [n249] support case A SSB pattern, and band [n250] supports case A and case B SSB pattern. Sync raster and GSCN for the these bands can be defined as Table 2.4-3.

  - 2.5 TX-RX frequency separation

The objective of this WID states these new bands can support flexible TX-RX Separation.

Table 2.5-1: UE TX-RX frequency separation (FR1-NTN)

| NTN Satellite Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| [n251] | 148 – 152 MHz |
| [n250] | 72.5 – 137.5 MHz |
| [n249] | 114 – 152 MHz |

Proposal 6: The flexible TX-RX frequency separation for for the new NR-NTN bands should be defined as Table 2.5-1.

  - 2.6 UE RF requirements

The following UE RF requirements need to be defined for the new NR-NTN bands.

Table 2.6-1 UE RF requirements for the new NR-NTN bands

| NR-NTN UE Tx/Rx requirement | Proposals for the new NR-NTN bands |
| --- | --- |
| 6.2.1 UE maximum output power | Band specific --> Specification impact. Support UE Power class 3 (+23dBm). |
| 6.2.2 MPR | Not band specific --> No specification impact. |
| 6.2.3 A-MPR | Band specific --> Specification impact. A-MPR requirement needs further study. |
| 6.2.4Configured transmitted power | Not band specific --> No specification impact. |
| 6.3.1 Minimum output power | Not band specific --> No specification impact. |
| 6.3.2 Transmit OFF power | Not band specific --> No specification impact. |
| 6.3.3 Transmit ON/OFF time mask | Not band specific --> No specification impact. |
| 6.3.4 Power control | Not band specific --> No specification impact. |
| 6.4.1 Frequency error | Not band specific --> No specification impact. |
| 6.4.2 Transmit modulation quality | Not band specific --> No specification impact. |
| 6.5.1 Occupied bandwidth | Not band specific --> No specification impact. |
| 6.5.2.2 Spectrum emission mask | Not band specific --> No specification impact. |
| 6.5.2.3 Additional Spectrum emission mask | Specification impact. Requirements for protection of the Radio Astronomy in the 1660-1670 MHz range shall be captured. |
| 6.5.2.4.1 NR ACLR | Not band specific --> No specification impact. |
| 6.5.2.4.2 UTRA ACLR | Not band specific --> No specification impact. |
| 6.5.3.1 General spurious emissions | Not band specific --> No specification impact. |
| 6.5.3.2 Spurious emissions for UE co-existence | Specification impact. Bands [n251], [n250] and [n249] need to be added in Table 6.5.3.2-1. |
| 6.5.3.3 Additional spurious emissions | Specification impact. Requirements for protection of the Radio Astronomy in the 1660-1670 MHz range shall be captured. |
| 6.5.4 Transmit intermodulation | Not band specific --> No specification impact. |
| 7.3.2 Reference sensitivity | Band specific --> Specification impact. REFSENS requirement needs to be defined for bands [n251], [n250] and [n249]. |
| 7.4 Max input level | Not band specific --> No specification impact. |
| 7.5 Adjacent channel selectivity | Not band specific --> No specification impact. |
| 7.6.2 In-band blocking | Specification impact --> Define in-band blocking requirement similar to band n254, n255, n256. |
| 7.6.3 Out of band blocking | Specification impact --> Out-of-band blocking requirement needs to be defined for bands [n251], [n250] and [n249]. |
| 7.6.4 Narrow band blocking | Specification impact --> Define narrow band blocking requirement similar to band n254, n255, n256. |
| 7.7 Spurious response | Not band specific --> No specification impact. |
| 7.8 Intermodulation characteristics | Not band specific --> No specification impact. |
| 7.9 Spurious emissions | Not band specific --> No specification impact. |

Proposal 7: To use the proposals in Table 2.6-1 for UE RF requirements for the new NR-NTN bands.

# 3 Conclusion

In this contribution, we shared some views on system parameters and UE RF requirements for the new NR-NTN FDD band and the proposals are made as follows:

Proposal 1: Operating band and band numbering for the new NR-NTN bands supporting the Extended L-band (UL 1668-1675MHz, DL 1518-1525MHz) and the combined MSS L-band and Extended L-band ranges (DL 1518-1559 MHz, UL 1626.5-1660.5 MHz and 1668-1675 MHz) can be defined as Table 2.1-1.

Table 2.1-1: NTN satellite bands in FR1-NTN

| Satellite operating band | Uplink (UL) operating band Satellite Access Node receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band Satellite Access Node transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| [n251] | 1668 MHz - 1675 MHz | 1518 MHz - 1525 MHz | FDD |
| [n250] | 1626.5 MHz - 1660.5 MHz | 1518 MHz - 1559 MHz | FDD |
| [n249] | 1668 MHz - 1675 MHz | 1518 MHz - 1559 MHz | FDD |

Proposal 2: Firstly, supported channel bandwidths and SCS can be defined as Table 2.2-1. Whether to support 25, 35, 40 MHz channel bandwidth needs further study.

Table 2.2-1: Channel bandwidths for each NTN satellite band in FR1-NTN

| NTN satellite band | SCS kHz | UE Channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 | 30 (NOTE) |
|  | 15 | 5 |  |  |  |  |
| [n251] | 30 |  |  |  |  |  |
|  | 60 |  |  |  |  |  |
|  | 15 | 5 | 10 | 15 | 20 | 30 |
| [n250] | 30 |  | 10 | 15 | 20 | 30 |
|  | 60 |  | 10 | 15 | 20 | 30 |
|  | 15 | 5 |  |  |  |  |
| [n249] | 30 |  |  |  |  |  |
|  | 60 |  |  |  |  |  |

Proposal 3: New NTN bands [n250] and [n249] can support asymmetric channel bandwidths.

Proposal 4: NR-ARFCN for the new NR-NTN bands supporting 100 kHz and 10 kHz channel raster can be defined as Table 2.4-1 and Table 2.4-2, respectively.

Table 2.4-1: Applicable NR-ARFCN per operating band in FR1-NTN with 100 kHz channel rater

| SAN operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| [n251] | 100 | 333600– <20> – 335000 | 303600 – <20> – 305000 |
| [n250] | 100 | 325300– <20> – 332100 | 303600 – <20> – 311800 |
| [n249] | 100 | 333600– <20> – 335000 | 303600 – <20> – 311800 |

Table 2.4-2: Applicable NR-ARFCN per operating band in FR1-NTN with 10 kHz channel rater

| NTN satellite operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) | Mandatory support |
| --- | --- | --- | --- | --- |
| [n251] | 10 | 333600– <2> – 335000 | 303600 – <2> – 305000 | Yes |
| [n250] | 10 | 325300– <2> – 332100 | 303600 – <2> – 311800 | Yes |
| [n249] | 10 | 333600– <2> – 335000 | 303600 – <2> – 311800 | Yes |

Proposal 5: We propose that band [n251] and [n249] support case A SSB pattern, and band [n250] supports case A and case B SSB pattern. Sync raster and GSCN for the these bands can be defined as Table 2.4-3.

Table 2.4-3: Applicable SS raster entries per operating band (FR1-NTN)

| NTN satellite operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| [n251] | 15 kHz | Case A | 3800 – <1> – 3807 |
| [n250] | 15 kHz | Case A | 3800 – <1> – 3892 |
|  | 30 kHz | Case B | 3806 – <1> – 3886 |
| [n249] | 15 kHz | Case A | 3800 – <1> – 3892 |

Proposal 6: The default TX-RX frequency separation for for the new NR-NTN bands should be defined as Table 2.5-1.

Table 2.5-1: UE TX-RX frequency separation (FR1-NTN)

| NTN Satellite Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| [n251] | 148 – 152 MHz |
| [n250] | 72.5 – 137.5 MHz |
| [n249] | 114 – 152 MHz |

Proposal 7: To use the proposals in Table 2.6-1 for UE RF requirements for the new NR-NTN bands.

Table 2.6-1 UE RF requirements for the new NR-NTN bands

| NR-NTN UE Tx/Rx requirement | Proposals for the new NR-NTN bands |
| --- | --- |
| 6.2.1 UE maximum output power | Band specific --> Specification impact. Support UE Power class 3 (+23dBm). |
| 6.2.2 MPR | Not band specific --> No specification impact. |
| 6.2.3 A-MPR | Band specific --> Specification impact. A-MPR requirement needs further study. |
| 6.2.4Configured transmitted power | Not band specific --> No specification impact. |
| 6.3.1 Minimum output power | Not band specific --> No specification impact. |
| 6.3.2 Transmit OFF power | Not band specific --> No specification impact. |
| 6.3.3 Transmit ON/OFF time mask | Not band specific --> No specification impact. |
| 6.3.4 Power control | Not band specific --> No specification impact. |
| 6.4.1 Frequency error | Not band specific --> No specification impact. |
| 6.4.2 Transmit modulation quality | Not band specific --> No specification impact. |
| 6.5.1 Occupied bandwidth | Not band specific --> No specification impact. |
| 6.5.2.2 Spectrum emission mask | Not band specific --> No specification impact. |
| 6.5.2.3 Additional Spectrum emission mask | Specification impact. Requirements for protection of the Radio Astronomy in the 1660-1670 MHz range shall be captured. |
| 6.5.2.4.1 NR ACLR | Not band specific --> No specification impact. |
| 6.5.2.4.2 UTRA ACLR | Not band specific --> No specification impact. |
| 6.5.3.1 General spurious emissions | Not band specific --> No specification impact. |
| 6.5.3.2 Spurious emissions for UE co-existence | Specification impact. Bands [n251], [n250] and [n249] need to be added in Table 6.5.3.2-1. |
| 6.5.3.3 Additional spurious emissions | Specification impact. Requirements for protection of the Radio Astronomy in the 1660-1670 MHz range shall be captured. |
| 6.5.4 Transmit intermodulation | Not band specific --> No specification impact. |
| 7.3.2 Reference sensitivity | Band specific --> Specification impact. REFSENS requirement needs to be defined for bands [n251], [n250] and [n249]. |
| 7.4 Max input level | Not band specific --> No specification impact. |
| 7.5 Adjacent channel selectivity | Not band specific --> No specification impact. |
| 7.6.2 In-band blocking | Specification impact --> Define in-band blocking requirement similar to band n254, n255, n256. |
| 7.6.3 Out of band blocking | Specification impact --> Out-of-band blocking requirement needs to be defined for bands [n251], [n250] and [n249]. |
| 7.6.4 Narrow band blocking | Specification impact --> Define narrow band blocking requirement similar to band n254, n255, n256. |
| 7.7 Spurious response | Not band specific --> No specification impact. |
| 7.8 Intermodulation characteristics | Not band specific --> No specification impact. |
| 7.9 Spurious emissions | Not band specific --> No specification impact. |

# 4 Reference

- RP-241689, New WID proposal: Introduction of new NR NTN bands to support the Extended L-band (UL 1668-1675MHz, DL 1518-1525MHz) and the combined MSS L-band and Extended L-band ranges (DL 1518-1559 MHz, UL 1626.5-1660.5 MHz and 1668-1675 MHz) , Inmarsat et al.