# 3GPP TSG- RAN WG4 Meeting # 112	R4-2411851

# Maastricht, Netherlands, 19th–23rd Aug, 2024

# Source: 	ZTE Corporation, Sanechips

# Title: 	Discussion on power domain enhancements for single carrier

# Agenda item:	8.1.1.2.1

# Document for:	Approval

# 1 Introduction

In RAN#102 meeting, a new Work Item [1] was approved on UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, one of the objectives is about power boosting and/or MPR reduction as follows. In this contribution, we want to share some further views on power domain enhancements for single carrier.

- Specify power domain enhancement, e.g., MPR reduction for NR single carrier and NR intra-band UL CA - Study the scenarios, and if feasible, specify the power domain enhancement, e.g., MPR reduction, for PC2 and PC3 with applicable ACLR/SEM/spurious emission modification with BS indication for NR FR1 on a single UL carrier - Include the following scenarios: - when there is no adjacent in-band/out-of-band co-existence issue - when a UE uses a narrower channel bandwidth within a wider BS bandwidth - Include both (e)RedCap UE (only PC3) and non-RedCap UE - Limited to QSPK and 16QAM - Specify MPR applicability based on the UL CCs with activated cells for NR intra-band UL CA configuration - Include both intra-band UL contiguous CA and intra-band non-contiguous UL CA for FR1 - Include intra-band UL contiguous CA and intra-band DL contiguous CA with single UL for FR2 - MPR requirement is not applicable until the SCell is activated - Necessary signaling to support the above objectives

# 2 Discussion

  - 2.1 Consideration on relaxation of requirements for identified scenarios

- 1.1 Consideration on relaxation of requirements for identified scenarios Agreement in main session: - No relaxation of level of spurious emission requirements - FFS on whether to change the spurious emission boundary

In last meeting, an agreement was reached that the level of spurious emission requirements can’t be relaxed, but whether to relax the spurious emission boundary needs further discussion. Spurious emission requirements are emissions which are caused by unwanted transmitter effects such as harmonics emission, parasitic emissions, intermodulation products and frequency conversion products, but exclude out of band emissions. It typically originates from regulatory body such as ITU-R recommendation SM.329. According to TS 36.803, the general spurious emission requirements and the corresponding reference bandwidths are taken from ITU-SM.329 for both the UE and the BS. The stricter Category B requirements were selected for the UE, to allow for global circulation of terminals. Regarding OOB boundary, it is defined as BWchannel +5 MHz between NR out of band and general spurious emission domain in TS 38.101-1. While, ITU-SM.329 and ITU-R SM.1541 specify the boundary between the OOB and spurious domains is the frequency separated from the centre frequency of the emission by 250% of the necessary bandwidth of the emission for normal case, and 150% of the necessary bandwidth plus the upper threshold of the necessary bandwidth for wideband case. The frequency separation between the centre frequency and the boundary of the spurious domain are listed in Table 2.1-1, which is partially copied from ITU-R SM.1539 TABLE 2. BN means the necessary bandwidth corresponding to BWchannel and BU means the upper threshold of the necessary bandwidth. fc is the centre frequency of the necessary bandwidth of an emission. And we turn it to the frequency separation between the channel edge and the boundary of the spurious domain as Table 2.1-2 for facilitating to compare the difference of the OOB boundary definition between TS 38.101-1 and ITU regulations.

Table 2.1-1: Frequency separation between the centre frequency and the boundary of the spurious domain (ITU-R SM.1539 TABLE 2)

| Frequency range | Normal | Wideband case (BN > BU) |  |
| --- | --- | --- | --- |
|  | Separation | BU | Separation |
| 30 MHz < fc < 1 GHz | 2.5 BN | 10MHz | 1.5 BN + 10 MHz |
| 1 GHz < fc < 3 GHz | 2.5 BN | 50MHz | 1.5 BN + 50 MHz |
| 3 GHz < fc < 10 GHz | 2.5 BN | 100MHz | 1.5 BN + 100 MHz |

Table 2.1-2: Frequency separation between the channel edge and the boundary of the spurious domain

| Frequency range | Normal | Wideband case (BN > BU) |  |
| --- | --- | --- | --- |
|  | Separation | BU | Separation |
| 30 MHz < fc < 1 GHz | 2*BWchannel | 10MHz | BWchannel + 10 MHz |
| 1 GHz < fc < 3 GHz | 2*BWchannel | 50MHz | BWchannel + 50 MHz |
| 3 GHz < fc < 10 GHz | 2*BWchannel | 100MHz | BWchannel + 100 MHz |

Comparing BWchannel +5 with separation in the above table, we can conclude that OOB boundary defined in 3GPP is stricter than ITU. For example, when 30 MHz < fc < 1 GHz, if BWchannel≥10MHz, OOB boundary defined in ITU is 5MHz larger than that defined in 3GPP. When fc > 1 GHz, if BWchannel≥10MHz, OOB boundary defined in ITU is 1/2*BWchannel and even more larger than that defined in 3GPP. From this perspective, for BWchannel≥10MHz with 30 MHz < fc < 1 GHz, there are 5MHz of OOB boundary can be relaxed, and for BWchannel≥10MHz with fc > 1 GHz, there are 1/2*BWchannel and even more can be relaxed.

Observation 1: By comparing the difference of OOB boundary between 3GPP and ITU, it can be observed that OOB boundary defined in 3GPP is stricter than ITU.

  - 2.2 Mechanisms for enabling MPR reduction and/or power boosting

- 1.2. Mechanisms for enabling MPR reduction and/or power boosting Way forward: - Study the approach to convert full RB allocation in UE CBW to “inner RB allocation” with an extended UE BW, e.g. each side of the UE BW is equal to ½ of the UE BW, inside a larger BS CBW as starting point with consideration of the following aspects - The edge of the extended UE BW, i.e. the UE CBW plus the shifted frequency symmetrically at each side of the UE CBW, should be inside BS CBW or at least aligned with the BS CBW edge - ① FFS feasibility of case where extended UE CBW edge exceeds the BS CBW edge, i.e. gap between edges of UE CBW and BS CBW < 1/2 UE CBW - ② FFS whether IBE is used between edges of UE CBW and BS CBW or between edges of UE CBW and extended UE CBW - ③ FFS ACLR and SEM are applicable from the edge of extended UE CBW or edge of BS CBW, i.e. the start point of ΔfOOB - ④ FFS integral region and boundary of OOBE is based on extended UE CBW or UE CBW - Other mechanisms are not precluded

For the first FFS, it can be divided into two cases. One is that there is no adjacent in-band/out-of-band co-existence issue, in which case, it is feasible that extended UE CBW edge exceeds the BS CBW edge. The other is that there exists adjacent in-band/out-of-band co-existence issue, and extended UE CBW edge cannot exceed the BS CBW edge.

Proposal 1: For the first FFS, it can be divided into two cases. One is that there is no adjacent in-band/out-of-band co-existence issue, in which case, it is feasible that extended UE CBW edge exceeds the BS CBW edge. The other is that there exists adjacent in-band/out-of-band co-existence issue, and extended UE CBW edge cannot exceed the BS CBW edge.

For the second FFS, the key of the mechanism lies in converting outer RB allocation into ‘inner RB allocation’ and utilizing MPR for inner RB allocation instead of outer RB allocation. The region between the edge of UE CBW and the edge of extended UE CBW can be regarded as in-band of extended UE CBW, where IBE could be used.

Proposal 2: The region between the edge of UE CBW and the edge of extended UE CBW can be regarded as in-band of extended UE CBW, where IBE could be used.

For the third FFS, considering one of the conditions of inner RB allocations is LCRB ≤ ceil(NRB/2), each side of UE BW increases 1/2 UE CBW may be enough to obtain MPR of inner RB allocations. ACLR and SEM is applied from the edge of UE CBW, so ACLR and SEM should be applicable from the edge of extended UE CBW instead of edge of BS CBW. In another perspective, for SE requirement, for BWchannel≥10MHz with fc > 1 GHz, there are 1/2*BWchannel and even more can be relaxed, and the worst case is only 1/2*BWchannel can be relaxed. If more than 1/2*BWchannel, it may not meet SE requirement, which will cause interference.

Proposal 3: ACLR and SEM should be applicable from the edge of extended UE CBW instead of edge of BS CBW.

For the fourth FFS, if we use the SEM and spurious emission requirement based on extended UE CBW instead of UE CBW, it means the level of SEM and spurious emission requirement is relaxed. However, relaxing the level of spurious emission is not feasible.

In addition, according to the principle stated in ITU-SM.329, the spurious domain generally consists of frequencies separated from the centre frequency of the emission by 250% or more of the necessary bandwidth of the emission. If integral region and boundary of OOBE is based on extended UE CBW, which means OOB boundary will be extended to 3/2 UE CBW larger than 1/2 UE CBW, it will be looser than the requirement defined in ITU. So integral region and boundary of OOBE should be based on UE CBW.

Proposal 4: Integral region and boundary of OOBE should be based on UE CBW.

# 3 Conclusion

In this contribution, we shared some views on power boosting and/or MPR reduction and the proposals are made as follows:

Observation 1: By comparing the difference of OOB boundary between 3GPP and ITU, it can be observed that OOB boundary defined in 3GPP is stricter than ITU.

Proposal 1: For the first FFS, it can be divided into two cases. One is that there is no adjacent in-band/out-of-band co-existence issue, in which case, it is feasible that extended UE CBW edge exceeds the BS CBW edge. The other is that there exists adjacent in-band/out-of-band co-existence issue, and extended UE CBW edge cannot exceed the BS CBW edge.

Proposal 2: The region between the edge of UE CBW and the edge of extended UE CBW can be regarded as in-band of extended UE CBW, where IBE could be used.

Proposal 3: ACLR and SEM should be applicable from the edge of extended UE CBW instead of edge of BS CBW.

Proposal 4: Integral region and boundary of OOBE should be based on UE CBW.

# 4 Reference

- RP-230828, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, Huawei

- RP-240453, MediaTek Views on RAN4 Rel-19: RF, MediaTek

- R4-2217745, WF on enhancements to reduce MPR/PAR, Nokia, Nokia Shanghai Bell