# 3GPP TSG- RAN WG4 Meeting # 112	R4-2411852

# Maastricht, Netherlands, 19th–23rd Aug, 2024

# Source: 	ZTE Corporation, Sanechips

# Title: 	Discussion on MPR applicability for FR1 intra-band UL CA

# Agenda item:	8.1.1.2.2

# Document for:	Approval

# 1 Introduction

In RAN#102 meeting, a new Work Item [1] was approved on UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, one of the objectives is about power boosting and/or MPR reduction as follows. In this contribution, we want to share some further views on MPR applicability for FR1 intra-band UL CA.

- Specify power domain enhancement, e.g., MPR reduction for NR single carrier and NR intra-band UL CA - Study the scenarios, and if feasible, specify the power domain enhancement, e.g., MPR reduction, for PC2 and PC3 with applicable ACLR/SEM/spurious emission modification with BS indication for NR FR1 on a single UL carrier - Include the following scenarios: - when there is no adjacent in-band/out-of-band co-existence issue - when a UE uses a narrower channel bandwidth within a wider BS bandwidth - Include both (e)RedCap UE (only PC3) and non-RedCap UE - Limited to QSPK and 16QAM - Specify MPR applicability based on the UL CCs with activated cells for NR intra-band UL CA configuration - Include both intra-band UL contiguous CA and intra-band non-contiguous UL CA for FR1 - Include intra-band UL contiguous CA and intra-band DL contiguous CA with single UL for FR2 - MPR requirement is not applicable until the SCell is activated - Necessary signaling to support the above objectives

# 2 Discussion

- 2.1. MPR applicability for FR1 intra-band contiguous UL CA Way forward: - For PC3 and PC2 contiguous UL CA use the corresponding single CC MPR when only 1 CC is activated. - FFS: MPR defined in Table 6.2.2-1 applies for UE power class 3 CA bandwidth classes B and C. - FFS: MPR defined in Table 6.2D.2-1 applies for power class 2 CA bandwidth classes B and C when TxD capability is indicated. - FFS: MPR defined in Table 6.2.2-2 applies for power class 2 CA bandwidth classes B and C when TxD capability is absent. - FFS whether spurious emissions/ACLR/SEM are kept for aggregated CBW for CA - 2.2. MPR applicability for FR1 intra-band UL non-contiguous CA Way forward: Enhancements not aligned with the WID will not be considered. - Further clarify the scope for NC CA in next meeting based on the WID objectives.

For MPR applicability for FR1 intra-band contiguous UL CA, there are two questions need to be discussed. The first is whether contiguous UL CA need to support power boosting when only 1 CC is activated because Table 6.2.2-1,Table 6.2D.2-1 and Table 6.2.2-2 not only include MPR but also support power boosting, which is defined for inner RB allocations of single carrier for DFT-s-OFDM with Pi/2 or Pi/2 BPSK w Pi/2 BPSK DMRS or QPSK. According to [4], it stated that [powerBoostRel18] and [powerBoostTSRel18] are per FS capability and only applicable to FR1. When only 1CC is activated, if the operating band of the activated CC supports UE capability of power boosting, power boosting should be supported by the activated CC. Hence, for PC3 and PC2 contiguous UL CA use the corresponding single CC MPR including power boosting when only 1 CC is activated.

Proposal 1: For PC3 and PC2 contiguous UL CA use the corresponding single CC MPR including power boosting when only 1 CC is activated.

The second is whether spurious emissions/ACLR/SEM are kept for aggregated CBW for CA or use ACLR/SEM/SE defined for single carrier when contiguous UL CA use the corresponding single CC MPR when only 1 CC is activated. From our understanding, using single carrier MPR instead of CA MPR means larger transmission power, and ACLR/SEM/SE requirement needs to meet the corresponding single carrier requirement.

Proposal 2: Using single carrier MPR instead of CA MPR means larger transmission power, and ACLR/SEM/SE requirement needs to meet the corresponding single carrier requirement.

According to the analysis of [3], the technique reason for large MPR was defined for the exception cases in NC CA is considering the excessive emissions caused by the 3rd order intermodulation product between the TX allocation and the LO leakage/Image when the LO is not at the center of the channel BW in the NC-ULCA case for 1 LO architecture. When the LO is not at the center of the channel BW, depending on the NC-ULCA configuration, the spurious limit requirement is -30dBm/MHz as opposed to -13dBm/MHz as in the single CC fall back case.

Consider this UE implementation, we think there is no need to redefine the MPR for the exception cases. For PC3 and PC2 intra-band non-contiguous CA as the standard already accounts for the use of the single CC MPR tables when only 1 CC is scheduled no further changes to the standard are required.

Proposal 3: For PC3 and PC2 intra-band non-contiguous CA as the standard already accounts for the use of the single CC MPR tables when only 1 CC is scheduled no further changes to the standard are required.

# 3 Conclusion

In this contribution, we shared some views on power boosting and/or MPR reduction and the proposals are made as follows:

Proposal 1: For PC3 and PC2 contiguous UL CA use the corresponding single CC MPR including power boosting when only 1 CC is activated.

Proposal 2: Using single carrier MPR instead of CA MPR means larger transmission power, and ACLR/SEM/SE requirement needs to meet the corresponding single carrier requirement.

Proposal 3: For PC3 and PC2 intra-band non-contiguous CA as the standard already accounts for the use of the single CC MPR tables when only 1 CC is scheduled no further changes to the standard are required.

# 4 Reference

- RP-230828, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, Huawei

- R4-2321960, LS on UE capabilities for MPR reduction, Nokia

- R4-2204225, 1CC fall-back MPR for NC UL CA with 1LO architecture, Qualcomm Incorporated, RAN4#102.