# 3GPP TSG- RAN WG4 Meeting # 112	R4-2411853

# Maastricht, Netherlands, 19th–23rd Aug, 2024

# Source: 	ZTE Corporation, Sanechips

# Title: 	Discussion on MPR applicability for FR2

# Agenda item:	8.1.1.2.3

# Document for:	Approval

# 1 Introduction

In RAN#102 meeting, a new Work Item [1] was approved on UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, one of the objectives is about power boosting and/or MPR reduction as follows. In this contribution, we want to share some further views on MPR applicability for FR2.

- Specify power domain enhancement, e.g., MPR reduction for NR single carrier and NR intra-band UL CA - Study the scenarios, and if feasible, specify the power domain enhancement, e.g., MPR reduction, for PC2 and PC3 with applicable ACLR/SEM/spurious emission modification with BS indication for NR FR1 on a single UL carrier - Include the following scenarios: - when there is no adjacent in-band/out-of-band co-existence issue - when a UE uses a narrower channel bandwidth within a wider BS bandwidth - Include both (e)RedCap UE (only PC3) and non-RedCap UE - Limited to QSPK and 16QAM - Specify MPR applicability based on the UL CCs with activated cells for NR intra-band UL CA configuration - Include both intra-band UL contiguous CA and intra-band non-contiguous UL CA for FR1 - Include intra-band UL contiguous CA and intra-band DL contiguous CA with single UL for FR2 - MPR requirement is not applicable until the SCell is activated - Necessary signaling to support the above objectives

# 2 Discussion

- 2.3. Applicable MPR for FR2 single carrier UL with DL intra band CA Way forward: - For the case of single carrier UL with DL intra band CA, the MPR requirements of single carrier case in clause 6.2.2 of TS 38.101-2 applies with UE indication of new capability for MPR improvement. - 2.4. Applicable MPR for FR2 UL CA with DL intra band CA Way forward: - MPR based on UL BWchannel_CA applies instead that based on cumulative aggregated channel BW (CABW) with UE indication of new capability for MPR improvement - If only 1 UL CC is activated, the MPR requirements of single carrier could be reused - FFS whether new MPR requirement could be defined for CABW < 400 MHz (e.g., 200 MHz).

In last meeting, we tended to reach an agreement that decouple UL and DL to define MPR relying on UL BWchannel instead of CABW with UE indication of new capability for MPR improvement. For the new UE capability, we need to consider all possible UE implementations, including UE with fast LO switching, or with dedicated LOs for Tx and Rx paths. Considering the new UE capability is related to UE implementation, this new capability shall be defined as optional and per UE, and only applicable to FR2. In addition, it could be release independent.

Proposal 1: We propose to consider all possible UE implementations, including UE with fast LO switching, or with dedicated LOs for Tx and Rx paths for the new UE capability.

Proposal 2: The new capability shall be defined as optional and per UE, and only applicable to FR2. In addition, it could be release independent.

Regarding whether new MPR requirement could be defined for CABW < 400 MHz (e.g., 200 MHz), according to the objective of the WID, it states that specify MPR applicability based on the UL CCs with activated cells for NR intra-band UL CA configuration. Hence, only MPR applicability needs to be discussed in this WID, and defining new MPR requirement is out of scope.

Proposal 3: Only MPR applicability needs to be discussed in this WID, and defining new MPR requirement is out of scope.

# 3 Conclusion

In this contribution, we shared some views on power boosting and/or MPR reduction and the proposals are made as follows:

Proposal 1: We propose to consider all possible UE implementations, including UE with fast LO switching, or with dedicated LOs for Tx and Rx paths for the new UE capability.

Proposal 2: The new capability shall be defined as optional and per UE, and only applicable to FR2. In addition, it could be release independent.

Proposal 3: Only MPR applicability needs to be discussed in this WID, and defining new MPR requirement is out of scope.

# 4 Reference

- RP-230828, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, Huawei