# 3GPP TSG- RAN WG4 Meeting # 112	R4-2411854

# Maastricht, Netherlands, 19th–23rd Aug, 2024

# Source: 	ZTE Corporation, Sanechips

# Title: 	Discussion on UE RF requirements for inter-band NR CA/DC with 3MHz CBW

# Agenda item:	8.4.2

# Document for:	Approval

# 1 Introduction

In RAN#103 meeting, a new Work Item [1] was approved on NR channel BW less than 5MHz for FR1 Phase 2 with the following RAN4 objective. In this contribution, we want to share some further views on UE RF requirements for inter-band NR CA/DC with 3MHz CBW.

- Define common co-located and non-co-located inter-band NR CA/DC UE RF requirements with 3MHz CBW in the one band and 5MHz or 10MHz CBW in the other band - Example band combination: CA/DC of 3MHz in band n100 and 5MHz or 10MHz in band n101 - Define RRM requirements for inter-band CA and DC for combinations introduced in RF part Note: other band combinations than example band combinations can be specified in basket WIs after the above generic requirements are specified.

# 2 Discussion

- Issue 1-6: UE capability and network signalling - FFS whether any updates to Rel-18 UE capability signalling are needed to enable less than 5 MHz CBW operation for CA/DC. - Further potential enhancements can be further discussed for SCell with less than 5MHz. - FFS whether UEs needs to be informed that SCell has 12 PRB or 20 PRB transmission bandwidth for 3MHz or 5MHz CBW, respectively - Issue 1-7: Sync raster applicability for Scell with < 5MHz CBW for CA_n100-n101 - FFS whether any restrictions on ARFCN for SSB locations for SCell SSB need to be defined - Option 1: Require less than 5 MHz SCell to be associated with the new sync raster points for less than 5MHz, similar as PCell. - Other options are not precluded

The above is WF in last meeting [2], there are two issues left to be discussed. For the first issue, according to the UE features introduced for support less than 5 MHz channel bandwidth in Rel-18 [3], they are all per band and optional with capability signalling. From our understanding, support CA/DC with 3MHz CBW in the one band and 5MHz or 10MHz CBW in the other band should be based on Rel-18 UE capability of supporting less than 5 MHz channel bandwidth, which is listed as the following table. However, it specified that this feature is applicable only to single-carrier operation. Hence, it is necessary to update Rel-18 UE capability signalling to enable less than 5 MHz CBW operation for CA/DC.

Proposal 1: It is necessary to update Rel-18 UE capability signalling to enable less than 5 MHz CBW operation for CA/DC.

support12PRB-CORESET0-r18 Indicates whether the UE supports reception of 12 PRB CORESET0 with an associated SS/PBCH block that is located according to Table 5.4.3.1-2 in TS 38.101-1 [2]. A UE supporting this feature shall also indicate support of support-3MHz-ChannelBW-Symmetric-r18. This feature is supported for 15kHz SCS only. This feature is only applicable to single-carrier operation. This feature is not applicable to UEs indicating supportOfRedCap-r17 or supportOfERedCap-r18. NOTE: The UE supporting this capability supports configuration of 12 PRB BWP operation.

support3MHz-ChannelBW-Asymmetric-r18 Indicates whether the UE supports 3 MHz channel bandwidth in uplink with larger than 3 MHz channel BW in DL, including short RACH preamble formats with 15kHz SCS, and long PRACH formats with 1.25kHz SCS. This feature is supported for 15kHz SCS only. It is applicable only to single-carrier operation and applies to bands where the UE indicates support for asymmetricBandwidthCombinationSet with 3 MHz UL according to clause 5.3.6 of TS 38.101-1 [2]. This feature is not applicable to UEs indicating supportOfRedCap-r17 or supportOfERedCap-r18. NOTE 1: The UE supporting this feature supports configuration of 15 PRB UL BWP operation. NOTE 2: If the UE indicates support in asymmetricBandwidthCombinationSet for a 3MHz UL in a band according to clause 5.3.6 of 38.101-1 [2], this feature shall be indicated for the band.

support3MHz-ChannelBW-Symmetric-r18 Indicates whether the UE supports 3 MHz symmetric channel bandwidth in DL and UL, including the following functional components: - Reception of 12 PRB PBCH based on RB-level puncturing; - Short RACH preamble formats with 15kHz SCS, and long PRACH formats with 1.25kHz SCS; - Reception of 15 PRB CORESET0. This feature is supported for 15kHz SCS only. It is applicable only to single-carrier operation and when an associated SS/PBCH block is located according to Table 5.4.3.3-2 in TS 38.101-1 [2]. This feature is not applicable to UEs indicating supportOfRedCap-r17 or supportOfERedCap-r18. NOTE: The UE supporting this capability supports configuration of 15 PRB BWP operation in DL and UL.

support5MHz-ChannelBW-20PRB-CORESET0-r18 Indicates whether the UE supports short RACH preamble formats with 15kHz SCS, and long PRACH formats with 1.25kHz SCS, and the reception of 20 PRB CORESET0. This feature is supported for 15 kHz SCS only. This feature is only applicable when an associated SS/PBCH block is located in band n100 at GSCN 41638 of Table 5.4.3.1-3 in TS 38.101-1 [2]. This feature is only applicable to single-carrier operation. This feature is not applicable to UEs indicating supportOfRedCap-r17 or supportOfERedCap-r18. NOTE: The UE supporting this feature supports configuration of 20 PRB BWP operation.

support12PRB-CORESET0-GSCN-41637-r18 Indicates whether the UE supports reception of 12 PRB CORESET0 with an associated SS/PBCH block located at GSCN 41637. A UE supporting this feature shall also indicate support of support3MHz-ChannelBW-Symmetric-r18. This feature is supported for 15 kHz SCS only. This feature is only applicable when an associated SS/PBCH block is located in band n100 at GSCN 41637 of Table 5.4.3.1-3 in TS 38.101-1 [2]. NOTE: The UE supporting this FG supports configuration of 12 PRB BWP operation. This feature is only applicable to single-carrier operation. This feature is not applicable to UEs indicating supportOfRedCap-r17 or supportOfERedCap-r18.

The transmission bandwidth of SCell is indicated in carrierBandwidth through SCellConfig > ServingCellConfigCommon > DownlinkConfigCommon > FrequencyInfoDL > SCS-SpecificCarrier > carrierBandwidth. According to the description of carrierBandwidth in TS 38.331, which is copied in the following table, it means that the width of carrier can be any number of PRBs. So, for 12 PRB or 20 PRB transmission bandwidth for 3MHz or 5MHz CBW, it will be indicated by carrierBandwidth for SCell.

SCS-SpecificCarrier field descriptions

carrierBandwidth Width of this carrier in number of PRBs (using the subcarrierSpacing defined for this carrier) (see TS 38.211 [16], clause 4.4.2).

Observation 1: CarrierBandwidth means the width of carrier can be any number of PRBs, and 12 PRB or 20 PRB transmission bandwidth for 3MHz or 5MHz CBW will be indicated by carrierBandwidth for SCell.

For the second issue, one of the motivations of new sync raster points is for UE to distinguish 12RB PBCH and 20RB PBCH, which means new sync raster is associated with PBCH bandwidth. If UE is not required to be in the new sync raster points as usual in the case of CA and SCell with less than 5MHz, it may use the frequencies same as legacy sync raster points, and result in UE cannot identify PBCH bandwidth and cannot read SSB. Hence, it’s proposed to require less than 5 MHz SCell to be associated with the new sync raster points for less than 5MHz, similar as PCell.

Proposal 2: It’s proposed to require less than 5 MHz SCell to be associated with the new sync raster points for less than 5MHz, similar as PCell.

# 3 Conclusion

In this contribution, we shared some views on power boosting and/or MPR reduction and the proposals are made as follows:

Proposal 1: It is necessary to update Rel-18 UE capability signalling to enable less than 5 MHz CBW operation for CA/DC.

Observation 1: CarrierBandwidth means the width of carrier can be any number of PRBs, and 12 PRB or 20 PRB transmission bandwidth for 3MHz or 5MHz CBW will be indicated by carrierBandwidth for SCell.

Proposal 2: It’s proposed to require less than 5 MHz SCell to be associated with the new sync raster points for less than 5MHz, similar as PCell.

# 4 Reference

RP-240832, New WID: WI resulting from discussion on NR channel BW less than 5MHz for TN, vivo

R4-2410602, WF on UE RF requirements for Rel-19 less than 5MHz, Intel

R4-2312572, Updated RAN1 UE features list for Rel-18 NR after RAN1#115, AT&T, NTT DOCOMO, INC.