# 3GPP TSG- RAN WG4 Meeting # 112	R4-2411855

# Maastricht, Netherlands, 19th–23rd Aug, 2024

# Source: 	ZTE Corporation, Sanechips

# Title: 	Discussion on system parameters for NTN less than 5MHz

# Agenda item:	8.8.3.1

# Document for:	Approval

# 1 Introduction

In RAN#104 meeting, a new Work Item [1] was approved on enhanced requirements and conductive test methodology for NR NTN and IoT NTN with the following RAN4 objective. In this contribution, we want to share some initial views about less than 5MHz for NTN from system parameters perspective.

Less than 5MHz for NTN - Specify the system parameters for channel bandwidth of less than 5 MHz for NR-NTN (NR based Non-Terrestrial Networks) in FR1-NTN bands - Consider the NR-NTN bands n255, n256, and n254 - Add less than 5MHz channel bandwidth - 3MHz with the RAN1 design in Rel-18 - Investigate and if necessary, specify the additional synchronization (sync) raster(s) for the additional channel bandwidth - Leverage the work on less than 5MHz for TN - Note: no RAN1 impact is expected - Specify the necessary SAN RF core requirements for NR-NTN in FR1-NTN bands - Specify the necessary NR-NTN UE RF core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Focus on power class 3 (PC3) for Tx RF requirements - Specify the necessary RRM core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Specify the necessary signaling to support the above objectives

# 2 Discussion

  - Less than 5MHz channel bandwidth was introduced in FR1 during Rel-18, and the following system parameters are defined for 3MHz channel bandwidth.

- Maximum transmission bandwidth NRB for 3MHz channel bandwidth is 15 RBs.

- Minimum guard band for 3MHz channel bandwidth is 142.5 kHz.

- Reuse existing channel spacing formula for 3MHz channel bandwidth.

- Reuse existing 100kHz channel raster for 3MHz channel bandwidth.

- Sync raster for 3MHz channel bandwidth is N * 600 kHz + M * 50 kHz + 300 kHz, N = 1:1665, M ϵ {1,3,5}.

For less than 5 MHz channel bandwidth for NR-NTN in FR1-NTN bands, the first four parameters can be reused. Whether the sync raster design defined for less than 5MHz for TN can be reused, we need to consider large doppler shift caused by NTN. Doppler shift can be up to 48kHz in 2GHz frequency caused by LEO at an altitude of 600km, and sync raster for NR-NTN less than 5MHz should leave enough margin for this, otherwise legacy UE not supporting 3MHz channel bandwidth may still try to read PBCH with 12 PRBs transmission bandwidth. Regarding the offset between legacy sync raster for above 3MHz CBW and new sync raster for 3MHz CBW, which is shown in Figure 2.1, it can be observed that the minimum offset between legacy and new sync raster is 100 kHz and it is larger than 48kHz doppler shift. Hence, it is proposed to reuse sync raster defined for less than 5MHz for TN.

Figure 2.1: The offset between legacy sync raster for above 3MHz CBW and new sync raster for 3MHz CBW

Proposal 1: Define the following system parameters for less than 5 MHz CBW for NR-NTN in FR1-NTN bands.

- Maximum transmission bandwidth NRB for 3MHz channel bandwidth is 15 RBs.

- Minimum guard band for 3MHz channel bandwidth is 142.5 kHz.

- Reuse existing channel spacing formula for 3MHz channel bandwidth.

- Reuse existing 100kHz channel raster for 3MHz channel bandwidth.

- Sync raster for 3MHz channel bandwidth is N * 600 kHz + M * 50 kHz + 300 kHz, N = 1:1665, M ϵ {1,3,5}, and applicable SS raster entries can be defined as Table 2.1 for 3 MHz channel bandwidth in bands n256, n256, n254.

Table 2.1: Applicable SS raster entries per operating band for 3 MHz channel bandwidth

| NR operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n256 | 15 kHz | Case A | 37492 – <1> – 37629 |
| n255 | 15 kHz | Case A | 34267 – <1> – 34424 |
| n254 | 15 kHz | Case A | 39060 – <1> – 39129 |
| NOTE 1: SS Block pattern is defined in clause 4.1 in TS 38.213 [7]. |  |  |  |

In addition, the UE capability of supporting 3MHz channel bandwidth in Rel-18 is per band and optional with capability signalling, we prefer to use current approach to define 3MHz channel bandwidth is optional for NR-NTN in FR1-NTN bands.

Proposal 2: We prefer to use current approach to define 3MHz channel bandwidth is optional for NR-NTN in FR1-NTN bands.

# 3 Conclusion

In this contribution, we shared some views on system parameters for NTN less than 5MHz and the proposals are made as follows:

Proposal 1: Define the following system parameters for less than 5 MHz CBW for NR-NTN in FR1-NTN bands.

- Maximum transmission bandwidth NRB for 3MHz channel bandwidth is 15 RBs.

- Minimum guard band for 3MHz channel bandwidth is 142.5 kHz.

- Reuse existing channel spacing formula for 3MHz channel bandwidth.

- Reuse existing 100kHz channel raster for 3MHz channel bandwidth.

- Sync raster for 3MHz channel bandwidth is N * 600 kHz + M * 50 kHz + 300 kHz, N = 1:1665, M ϵ {1,3,5}, and applicable SS raster entries can be defined as Table 2.1 for 3 MHz channel bandwidth in bands n256, n256, n254.

Table 2.1: Applicable SS raster entries per operating band for 3 MHz channel bandwidth

| NR operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n256 | 15 kHz | Case A | 37492 – <1> – 37629 |
| n255 | 15 kHz | Case A | 34267 – <1> – 34424 |
| n254 | 15 kHz | Case A | 39060 – <1> – 39129 |
| NOTE 1: SS Block pattern is defined in clause 4.1 in TS 38.213 [7]. |  |  |  |

Proposal 2: We prefer to use current approach to define 3MHz channel bandwidth is optional for NR-NTN in FR1-NTN bands.

# 4 Reference

- RP-240857, New WID: Enhanced requirements and test methodology for NR and IoT NTN, Huawei