# 3GPP TSG- RAN WG4 Meeting # 112	R4-2411856

# Maastricht, Netherlands, 19th–23rd Aug, 2024

# Source: 	ZTE Corporation, Sanechips

# Title: 	Discussion on UE RF requirements for NTN less than 5MHz

# Agenda item:	8.8.3.2

# Document for:	Approval

# 1 Introduction

In RAN#104 meeting, a new Work Item [1] was approved on enhanced requirements and conductive test methodology for NR NTN and IoT NTN with the following RAN4 objective. In this contribution, we want to share some initial views about less than 5MHz for NTN from UE RF requirements perspective.

Less than 5MHz for NTN - Specify the system parameters for channel bandwidth of less than 5 MHz for NR-NTN (NR based Non-Terrestrial Networks) in FR1-NTN bands - Consider the NR-NTN bands n255, n256, and n254 - Add less than 5MHz channel bandwidth - 3MHz with the RAN1 design in Rel-18 - Investigate and if necessary, specify the additional synchronization (sync) raster(s) for the additional channel bandwidth - Leverage the work on less than 5MHz for TN - Note: no RAN1 impact is expected - Specify the necessary SAN RF core requirements for NR-NTN in FR1-NTN bands - Specify the necessary NR-NTN UE RF core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Focus on power class 3 (PC3) for Tx RF requirements - Specify the necessary RRM core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Specify the necessary signaling to support the above objectives

# 2 Discussion

The WID is aimed at introducing less than 5MHz channel bandwidth in FR1-NTN bands during Rel-19, and the following UE RF requirements need to be defined for 3MHz channel bandwidth.

Table 2.1 UE RF requirements for 3MHz channel bandwidth in FR1-NTN bands

| NR-NTN UE Tx/Rx requirement | Proposals for 3 MHz channel bandwidth for NR-NTN in FR1-NTN bands |
| --- | --- |
| 6.2.1 UE maximum output power | No specification impact. |
| 6.2.2 MPR | NR MPR is channel bandwidth agnostic --> no specification impact. |
| 6.2.3 A-MPR | Considering the NR-NTN bands n255, n256, and n254, these bands need A-MPR study. - NS_01, NS_02N: 3 MHz shall be added to the list of considered channel BW for the NS. - NS_24, NS_03N, NS_04N, NS_05N: A-MPR shall be specified for 3 MHz channel bandwidth. - NS_100 is not dependent on the considered channel BW. |
| 6.2.4 Configured transmitted power | No specification impact. |
| 6.3.1 Minimum output power | It is channel bandwidth dependent, specification change is needed. Considering reuse -40 dBm and the corresponding measurement bandwidth can be 2.715 MHz. |
| 6.3.2 Transmit OFF power | Value - 50 dBm is channel bandwidth agnostic --> 3 MHz / 15 kHz SCS shall still be added in table 6.3.2-1. |
| 6.3.3 Transmit ON/OFF time mask | No specification impact. |
| 6.3.4 Power control | No specification impact. |
| 6.4.1 Frequency error | No specification impact. |
| 6.4.2 Transmit modulation quality | No specification impact. |
| 6.5.1 Occupied bandwidth | Specification impact, addition of 3 MHz into Table 6.5.1-1 |
| 6.5.2.2 Spectrum emission mask | Specification impact, considering reuse general SEM defined for 3 MHz for TN . |
| 6.5.2.3 Additional Spectrum emission mask | No specification impact. |
| 6.5.2.4.1 NR ACLR | Specification impact, addition of 3 MHz into Table 6.5.2.4.1-1. |
| 6.5.2.4.2 UTRA ACLR | Channel bandwidth agnostic for bands which is applicable --> no specification impact. |
| 6.5.3.1 General spurious emissions | Specification impact. Boundary between NR out of band and general spurious emission domain can be defined as 6 MHz. |
| 6.5.3.2 Spurious emissions for UE co-existence | No specification impact. |
| 6.5.3.3 Additional spurious emissions | Specification impact. Band n256, n255, n254 has additional spurious emissions, 3 MHz needs to be listed. |
| 6.5.4 Transmit intermodulation | Channel bandwidth agnostic --> no specification impact. |
| 7.3.2 Reference sensitivity | Channel bandwidth dependent, if SU is same as TN 3MHz then the scaling factor of 2.2dB from 5MHz REFSENS for 3MHz can be reused --> specification impact. |
| 7.4 Max input level | Specification impact, addition of 3 MHz into Table 7.4-1. |
| 7.5 Adjacent channel selectivity | Specification impact --> 33dB can be reused, and 3 MHz needs to be added into Table 7.5-1, Table 7.5-2 and Table 7.5-3. |
| 7.6.2 In-band blocking | Specification impact --> Define in-band blocking requirement similar to TN for 3MHz, and 3MHz needs to be added into Table 7.6.2-1. |
| 7.6.3 Out of band blocking | Specification impact --> Define out of band blocking requirement similar to TN for 3MHz, and 3MHz needs to be added into Table 7.6.3-1. |
| 7.6.4 Narrow band blocking | Specification impact --> Define narrow band blocking requirement similar to TN for 3MHz, and 3MHz needs to be added into Table 7.6.4-1. |
| 7.7 Spurious response | Specification impact --> Define spurious response requirement similar to TN for 3MHz, and 3 MHz needs to be added into Table 7.7-1. |
| 7.8 Intermodulation characteristics | Intermodulation characteristics requirement for NTN satellite UE is referred to TS 38.101-1 clause 7.8. --> no specification impact. |
| 7.9 Spurious emissions | Channel bandwidth agnostic --> no specification impact. |

Proposal 1: To use the proposals in Table 2.1 for UE RF requirements for 3 MHz channel bandwidth in FR1-NTN bands.

# 3 Conclusion

In this contribution, we shared some views on UE RF requirements for NTN less than 5MHz and the proposals are made as follows:

Proposal 1: To use the proposals in Table 2.1 for UE RF requirements for 3 MHz channel bandwidth in FR1-NTN bands.

# 4 Reference

- RP-240857, New WID: Enhanced requirements and test methodology for NR and IoT NTN, Huawei