# 3GPP TSG- RAN WG4 Meeting # 112	R4-2411857

# Maastricht, Netherlands, 19th–23rd Aug, 2024

# Source: 	ZTE Corporation, Sanechips

# Title: 	Discussion on SAN RF requirements for NTN less than 5MHz

# Agenda item:	8.8.3.3

# Document for:	Approval

# 1 Introduction

In RAN#104 meeting, a new Work Item [1] was approved on enhanced requirements and conductive test methodology for NR NTN and IoT NTN with the following RAN4 objective. In this contribution, we want to share some initial views about less than 5MHz for NTN from SAN RF requirements perspective.

Less than 5MHz for NTN - Specify the system parameters for channel bandwidth of less than 5 MHz for NR-NTN (NR based Non-Terrestrial Networks) in FR1-NTN bands - Consider the NR-NTN bands n255, n256, and n254 - Add less than 5MHz channel bandwidth - 3MHz with the RAN1 design in Rel-18 - Investigate and if necessary, specify the additional synchronization (sync) raster(s) for the additional channel bandwidth - Leverage the work on less than 5MHz for TN - Note: no RAN1 impact is expected - Specify the necessary SAN RF core requirements for NR-NTN in FR1-NTN bands - Specify the necessary NR-NTN UE RF core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Focus on power class 3 (PC3) for Tx RF requirements - Specify the necessary RRM core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Specify the necessary signaling to support the above objectives

# 2 Discussion

The WID is aimed at introducing less than 5MHz channel bandwidth in FR1-NTN bands during Rel-19, and the following SAN RF requirements need to be defined for 3MHz channel bandwidth.

Table 2.1 SAN RF requirements for 3MHz channel bandwidth in FR1-NTN bands

| NR BS RF Tx/Rx requirement | Proposal for 3MHz channel bandwidth in FR1-NTN bands |
| --- | --- |
| 6.2 Satellite Access Node output power | Channel bandwidth agnostic --> No specification impact. |
| 6.3.2 RE power control dynamic range | Channel bandwidth agnostic --> No specification impact. |
| 6.3.3 Total power dynamic range | Specification impact --> Minimum requirement calculated according to transmission bandwidth configuration in Table 6.3.3.2-1. |
| 6.4 Transmit ON/OFF power | Not applicable to SAN --> No specification impact. |
| 6.5 Transmitted signal quality | No specification impact. |
| 6.6.2 Occupied bandwidth | No specification impact. |
| 6.6.3 Adjacent Channel Leakage Power Ratio | Specification impact --> Same requirement limits as other channel bandwidth, but assume different adjacent channel carriers in Table 6.6.3.2-1 and Table 6.6.3.2-2. |
| 6.6.4 Out-of-band emissions | No specification impact. |
| 6.6.5 Transmitter spurious emissions | No specification impact |
| 6.7 Transmitter intermodulation | Not applicable for SAN --> No specification impact. |
| 7.2 Reference sensitivity level | Specification impact --> Add minimum requirements for 3 MHz in Table 7.2.2-1 and Table 7.2.2-2. Scaling from 5 MHz can be used if the same SU as TN 3 MHz is adopted. |
| 7.3 Dynamic range | Specification impact --> Add minimum requirements for 3 MHz in Table 7.3.2-1. Scaling from 5 MHz can be used if the same SU as TN 3 MHz is adopted. |
| 7.4.1 Adjacent Channel Selectivity (ACS) | Specification impact --> Define ACS requirement similar to TN for 3MHz in Table 7.4.1.2-1, and assume same interferer frequency offset values as TN 3 MHz in Table 7.4.1.2-2. |
| 7.4.2 In-band blocking | Not applicable for SAN --> No specification impact. |
| 7.5 Out-of-band blocking | No specification impact. |
| 7.6 Receiver spurious emissions | Not applicable for SAN --> No specification impact. |
| 7.7 Receiver intermodulation | Not applicable for SAN --> No specification impact. |
| 7.8 In-channel selectivity | Add minimum requirements for 3 MHz in Table 7.8.2-1, Table 7.8.2-2 and Table 7.8.2-3. Requirement limits depend on ICS FRC to be defined for 3MHz. |
| A.1 Fixed Reference Channels for RF Rx requirements (QPSK, R=1/3) | Specification impact --> FRC for 3MHz needs to be added in Table A.1-1. |
| A.2 Fixed Reference Channels for dynamic range (16QAM, R=2/3) | Specification impact --> FRC for 3MHz needs to be added in Table A.2-1. |

Proposal 1: To use the proposals in Table 2.1 for SAN RF requirements for 3 MHz channel bandwidth in FR1-NTN bands.

# 3 Conclusion

In this contribution, we shared some views on SAN RF requirements for NTN less than 5MHz and the proposals are made as follows:

Proposal 1: To use the proposals in Table 2.1 for SAN RF requirements for 3 MHz channel bandwidth in FR1-NTN bands.

# 4 Reference

- RP-240857, New WID: Enhanced requirements and test methodology for NR and IoT NTN, Huawei