# 3GPP TSG- RAN WG4 Meeting # 112	R4-2411858

# Maastricht, Netherlands, 19th–23rd Aug, 2024

# Source: 	ZTE Corporation, Sanechips

# Document for:	Approval

# Title: 	Discussion on coexistence study based on ITU regulations for NR NTN Ku band

# Agenda item:	8.9.2

# 1 Introduction

In RAN#104 meeting, a new Work Item [1] was approved on introduction of Ku Band for NR NTN for NR NTN with the following RAN4 objective. In this contribution, we want to share some initial views about NR NTN Ku band from coexistence study perspective.

The following assumptions are taken as the baseline for this work: - GSO and NGSO - NTN capable UE type 1 to 5 (38.101-5 Table 9.2.1.0-1) for above 10 GHz (no handheld devices) - FDD Mode - GNSS Receiver capability The objectives are: - Use the regulation requirements in ITU Regions 1, 2 and 3 to identify relevant adjacent band co-existence scenarios for NTN Ku Band covering the following frequency ranges, considering targeted deployment scenarios: - For ITU regions 1 and 3: - Downlink 10.70 – 12.75 GHz - Uplink 13.75-14.5 GHz - For ITU regions 2 excluding US: - Downlink 10.70 – 12.70 GHz - Uplink 13.75-14.5 GHz - For ITU regions 1 and 3 - Downlink 10.70 – 12.75 GHz - Uplink 12.75 - 13.25 GHz - For ITU regions 2 excluding US - Downlink 10.70 – 12.70 GHz - Uplink 12.70 - 13.25 GHz - Note: Appropriate TX/Rx separation for uplink downlink pairing will be required. - Conduct coexistence study. Reuse Ka band coexistence study assumptions as much as possible, with modifications where necessary, e.g. for frequency range and subcarrier spacing. Conduct new co-existence studies based on adjacent services . - Specify RF requirements for satellite access node and NTN VSAT types in 38.101-5 also considering existing regulations on antenna sizes for certain parts of the Ku band. - Priority 1: for all ITU regions, define Ku band #1a and #1b for the downlink block and uplink block B to enable the future possibility of supporting satellite services in 125 MHz (UL) and 250 MHz (DL) aggregated bandwidths based on existing channel bandwidths. Potential approaches for achieving this shall be studied, from which one shall be selected including but not limited to: - Ku band #1a and #1b as FR1-NTN supporting 20 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing - Ku band #1a and #1b as FR2-NTN supporting 50 MHz, 100 MHz, 200 MHz, 400 MHz (120kHz SCS only) channel bandwidths with 120 kHz subcarrier spacing - RAN4 will select which one of the above approaches for DL and UL will be pursued by RAN #106. - Priority 2: for all ITU regions excluding the US in region 2, using the approach in Priority 1, define Ku band #2a and #2b as appropriate for the downlink block and uplink block A to support satellite services in 125 MHz (UL) and 250 MHz (DL) aggregated bandwidths. - Specify RRM requirements Note: Support for NTN CA is not in the scope of this WID

# 2 Discussion

  - 2.1 Coexistence scenario

As indicated in the WID, RAN4 needs to consider the coexistence study between Ku band and the TN bands. The detailed spectrum status between Ku band and surrounding TN bands are shown as following.

Figure 2.1-1. Coexistence scenario for Ku band and other TN bands

Based on the above adjacent spectrum for NTN Ku band, then the coexistence scenario could be summarized as following.The aggressor and victim combinations are listed in Table 2.1-1.

Table 2.1-1: Coexistence scenario between Ku band NTN and TN

| No. | Combination | Aggressor | Victim |
| --- | --- | --- | --- |
| 1 | TN with NTN | NTN UL | TN UL |
| 2 | TN with NTN | TN UL | NTN UL |
| 3 | TN with NTN | NTN UL | TN DL |
| 4 | TN with NTN | TN DL | NTN UL |
| 5 | TN with NTN | TN DL | NTN DL |
| 6 | TN with NTN | NTN DL | TN DL |
| NOTE 1: For coexistence between Ku band and surrounding TN bands, this need more discussions since currently there are no 3GPP defined TN bands specified. |  |  |  |

The frequency and bandwidth are listed in Table 2.1-2.

Table 2.1-2: Proposed frequency and bandwidth for co-existence study

|  | Frequency2 | Bandwidth | Duplex mode | FRF |
| --- | --- | --- | --- | --- |
| TN Rural | 10 GHz | 100MHz | FFS | 1 |
| TN Urban macro | 10 GHz | 100MHz | FFS | 1 |
| GEO | 10 GHz | 100MHz | FDD | 1, 31 |
| LEO | 10 GHz | 100MHz | FDD | 1, 31 |
| NOTE 1: Only FRF=1 has been used in co-existence studies for simplification. NOTE 2: Study 10GHz frequency firstly, and other frequency is not excluded. |  |  |  |  |

  - 2.2 System parameters

    - 2.2.1	NTN satellite parameters

According to TR 38.821, the general satellite parameters for NTN Ka band are listed in the following Table 2.2.1-1 and Table 2.2.1-2. We can consider satellite parameters for Ka band in Table 2.2.1-1 and Table 2.2.1-1 as a starting point for Ku band NTN coexistence study.

Table 2.2.1-1: Set-1 satellite parameters for system level simulation

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| Satellite antenna pattern |  | Section 6.4.1 in [2] | Section 6.4.1 in [2] | Section 6.4.1 in [2] |
| Payload characteristics for DL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note 1) | Ku-band | 5 m | 0.5 m | 0.5 m |
| Satellite EIRP density |  | 40 dBW/MHz | 10 dBW/MHz | 4 dBW/MHz |
| Satellite Tx max Gain |  | 58.5 dBi | 38.5 dBi | 38.5 dBi |
| 3dB beamwidth |  | 0.1765 deg | 1.7647 deg | 1.7647 deg |
| Satellite beam diameter (Note 2) |  | 110 km | 40 km | 20 km |
| Payload characteristics for UL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note1) | Ku-band | 3.33 m | 0.33 m | 0.33 m |
| G/T |  | 28 dB K-1 | 13 dB K-1 | 13 dB K-1 |
| Satellite RX max Gain |  | 58.5 dBi | 38.5 dBi | 38.5 dBi |
| NOTE 1: This value is equivalent to the antenna diameter in Sec. 6.4.1 of [2]. NOTE 2: This beam size refers to the Nadir pointing of the satellite NOTE 3: All these satellite parameters are applied per beam. NOTE 4: The EIRP density values are considered identical for all frequency re-use factor options. NOTE 5: The EIRP density values are provided assuming the satellite HPA is operated with a back-off of [5] dB. |  |  |  |  |

Table 2.2.1-2: Set-2 satellite parameters for system level simulation

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| Satellite antenna pattern |  | Section 6.4.1 in [2] | Section 6.4.1 in [2] | Section 6.4.1 in [2] |
| Payload characteristics for DL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note 1) | Ku-band | 2 m | 0.2 m | 0.2 m |
| Satellite EIRP density |  | 32 dBW/MHz | 2 dBW/MHz | -4 dBW/MHz |
| Satellite Tx max Gain |  | 50.5 dBi | 30.5 dBi | 30.5 dBi |
| 3dB beamwidth |  | 0.4412 deg | 4.4127 deg | 4.4127 deg |
| Satellite beam diameter (Note 2) |  | 280 km | 90 km | 50 km |
| Payload characteristics for UL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note1) | Ku-band | 1.33 m | 0.13 m | 0.13 m |
| G/T |  | 20 dB K-1 | 5 dB K-1 | 5 dB K-1 |
| Satellite Rx max Gain |  | 50.5 dBi | 30.5 dBi | 30.5 dBi |
| NOTE 1: This value is equivalent to the antenna diameter in Sec. 6.4.1 of [2]. NOTE 2: This beam size refers to the Nadir pointing of the satellite NOTE 3: All these satellite parameters are applied per beam. NOTE 4: The EIRP density values are considered identical for all frequency re-use factor options. |  |  |  |  |

    - 2.2.2	NTN UE parameters

According to TR 38.821, NTN UE parameters for Ka band are given in Table 2.2.2-1. We can consider NTN UE parameters for Ka band in Table 2.2.2-1 as a starting point for Ku band NTN coexistence study.

Table 2.2.2-1: NTN UE characteristics for system level simulation

| Characteristics | VSAT (Note 2) |  |  |
| --- | --- | --- | --- |
| Frequency band | Ku band |  |  |
| Antenna type and configuration | Directional Section 6.4.1 of [2] with 60 cm equivalent aperture diameter of TR 38.821 |  |  |
| Polarisation | circular |  |  |
| Rx Antenna gain | 39.7 dBi |  |  |
| Antenna temperature | 150 K |  |  |
| Noise figure | 1.2 dB |  |  |
| Tx transmit power | 2 W (33 dBm) |  |  |
| Tx antenna gain | 43.2 dBi |  |  |
| NOTE 1: Moving platforms (e.g., aircrafts, vessels), building mounted devices. These values are provided for information. NOTE 2: VSAT terminal characteristics could be implemented with phased array antenna |  |  |  |

    - 2.2.3	TN parameters

According to TR 38.921, TN parameters for co-existence study are given in Table 2.2.3-1, 2.2.3-2 and 2.2.3-3.

Table 2.2.3-1: Simulation parameters of TN

| Parameters | Urban macro |
| --- | --- |
| Carrier frequency | 10 GHz |
| Channel bandwidth | 100 MHz |
| Scheduled channel bandwidth per UE (DL) | 98.28 MHz |
| Scheduled channel bandwidth per UE (UL) | 98.28 MHz (1 user, Note 4) 32.76 MHz (3 user) |
| The number of active UE (DL) (Note 1) | 1 |
| The number of active UE (UL) (Note 1) | 1 or 3 |
| Traffic model | Full buffer |
| DL power control | NO |
| UL power control | YES |
| BS max TX power in dBm | 43 (Note 3) |
| UE max TX power in dBm | 23 or 20 (Note 2) |
| UE min TX power in dBm | -33 |
| BS Noise figure in dB | 7 (@10 GHz) |
| UE Noise figure in dB | 9 or 13 (Note 5) |
| Handover margin | 3 dB |
| Note 1 Same as the number of BS beam(s); Note 2: 20 dBm as optional case where CLx-ile should be reduced by 3 dB; Note 3: BS max TX power is defined per polarization; Note 4: 1 user scheduling is a baseline assumption for coexistence evaluation; Note 5: NF of 9 dB is a baseline assumption and NF of 13 dB is as optional case. |  |

Table 2.2.3-2: Network layout model for TN

| Parameters |  | Values | Remark |
| --- | --- | --- | --- |
| Network layout |  | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around |  |
| Inter-site distance |  | 0.45 km (urban) (Note 1) 0.9 km (suburban) | Based on cell range: 0.3 km (urban) 0.6 km (suburban) |
| BS antenna height |  | 20 m (urban) 25 m (suburban) |  |
| UE location | Outdoor/indoor | Outdoor and indoor |  |
|  | Indoor UE ratio | 20% |  |
|  | Low/high penetration loss ratio | 50% low loss, 50% high loss |  |
|  | LOS/NLOS | LOS and NLOS |  |
|  | UE antenna height | Same as 3D-UMa in TR 36.873 |  |
| UE distribution (horizontal) |  | Uniform |  |
| Minimum BS - UE distance (2D) |  | 35 m |  |
| Channel model |  | UMa |  |
| Shadowing correlation |  | Between cells: 1.0 Between sites: 0.5 |  |
| Note: Results with 0.4 km ISD at 10 GHz can also be provided if simulation results show 5%-tile throughput cannot be achieved with 0.45 km ISD at 10 GHz. |  |  |  |

Table 2.2.3-3: ACLR/ACS for TN

|  |  | NR |
| --- | --- | --- |
| BS | ACLR | 37 dB |
|  | ACS | 40 dB |
| UE | ACLR | 24 dB |
|  | ACS | 31 dB |

    - 2.2.4	TN UE parameters

According to TR 38.921, UE operating in 10 - 10.5 GHz will most likely therefore have a conducted interface with an assumed isotropic radiation pattern antenna and no beamforming.

  - 2.3 Antenna and beamforming pattern modelling

    - 2.3.1	NTN satellite and UE antenna and beam forming pattern modelling

According to [2], Satellite and UE Antenna and beam forming pattern modelling of satellite could be referred to section 6.4.1 in TR 38.811.

Satellite antenna pattern

Parabolic antenna pattern for SAN:

Take the normalised SAN antenna pattern for Ka-Band coexistence studies and related half-power beam-width (2θ−3dB).

| Equation | Half-power beam-width 2θ−3dB Value | GEO constellation type @ 17GHz | LEO constellation type @ 17GHz |
| --- | --- | --- | --- |
| Fθ=2 J1uu | 2θ−3dB=2×Arcsin1.616×λπD | 0.1763 degrees | 1.7337 degrees |

where:

- Jix  is the Bessel function of first type and itℎ order with argument x

- is the angle in a θ;φ spherical coordinates system,

- u=πDλsin⁡(θ)

- D : Antenna diameter

- : Wavelength

Parabolic antenna pattern for NTN UE:

  - Use the following parabolic antenna model for NTN UE

Fθ=2 J1uu

With:

- Jix  is the Bessel function of first type and itℎ order with argument x

- is the angle in a θ;φ spherical coordinates system,

- u=πDλsin⁡(θ)

- D : Antenna diameter

- : Wavelength

Satellite and UE beam forming pattern

The following table is agreed for the beam layout definition for a single satellite simulation.

Table 2.3.1-1: Beam layout definition for single satellite simulation

| Beam layout definition | Baseline: Hexagonal mapping of the beam bore sight directions on UV plane defined in the satellite reference frame. Only the 3dB beam width parameters should be used. The beam diameter and beam spacing values can be computed directly from the 3 dB beam width assumptions and should be considered as informative. |  |  |
| --- | --- | --- | --- |
| Number of beams | Baseline: 7-beam layout (i.e. 6 co-frequency beams surrounding the central beam) |  |  |
| UV plane illustration (extracted from [19]) |  |  |  |
| UV plane convention | U axis is defined as the perpendicular line to the satellite-earth line on the orbital plane as illustrated here after: The straight line being orthogonal to UV plane is pointing towards the Earth centre. UV coordinates of the nadir of the reference satellite is (0,0) |  |  |
| Adjacent beam spacing on UV plane | Baseline: Adjacent beam spacing computation based on 3dB beam width of the satellite antenna pattern: ABS[rad] = sqrt(3) x sin(HPBW[degrees]/2) or ABS[rad] = sqrt(3) x sinr(HPBW[rad]/2) with ABS [degree]=180/pi x ABS[rad] and with HPBW the Half-Power BandWidth of the main lobe from the satellite antenna pattern. |  |  |
| Central beam bore sight direction definition | Baseline: Case 1: Central beam center is considered at nadir point Case 2: TBD° for GEO and LEO |  |  |
|  | Option 1: FRF=1 | Option 2: FRF=3 | Option 3: FRF=2 |
| Polarization re-use | Option 1: Disable Option 2: Enable Note: Polarization re-use should apply only if circular polarization for terminal antenna is considered |  |  |
| UEs outdoor/indoor distribution | 100% outdoor distribution for UEs |  |  |
| UE distribution | The cell area associated to a given beam is defined as the Voronoi cell associated with the corresponding beam centers. |  |  |
| UE configuration | Fixed and mobile VSAT for GSO Fixed VSAT for NGSO |  |  |
| UE orientation | Ideal Tracking serving beam |  |  |
| UE attachment | RSRP |  |  |
| NOTE 1: Typical impairment values (additional frequency error, SNR loss) due to the feeder link except for delay can be considered to be negligible. When available, specific values can be considered in the evaluation and should be reported. NOTE 2: For the calibration purpose, the ionospheric scintillation loss shall be considered equal to zero (i.e., the UEs are located between 20 and 60 degrees of latitude). |  |  |  |

Proposal 5: To consider NTN satellite and UE antenna and beam forming pattern modelling for Ka band as a starting point for Ku band NTN coexistence study.

    - 2.3.2	TN BS and UE antenna and beam forming pattern modelling

According to TR 38.921 clause 4.2.3, in Table 2.3.2-1, the parameters used by the parameterized array antenna model are described.

Table 2.3.2-1: Parameters of the parameterized array antenna model

| Parameter | Symbol | Unit |
| --- | --- | --- |
| Front to back ratio | Am | dB |
| Side lobe suppression | SLAv | dB |
| Horizontal HPBW | 3dB | Degrees |
| Vertical HPBW | 3dB | Degrees |
| Array element peak gain | GE,max | dBi |
| Number of radiating elements rows and columns | (M, N) | Integer |
| Horizontal element separation | dh | m |
| Vertical element separation | dv | m |
| Electrical down-tilt angle | etilt | Degrees |
| Electrical scan angle | escan | Degrees |

The parameterized antenna model is built around array antenna model where the element factor, array factor and linear phase progressing is characterized as described by equations in Table 2.3.2-2.

Table 2.3.2-2: Array antenna model details

| Description | Equation | Unit |
| --- | --- | --- |
| Peak normalized element radiation pattern | Aθ,φ=−min−−min12φφ3dB2,Am−min12θ−90θ3dB2,SLAv ,Am | dB |
| Peak gain normalized element radiation pattern | AEθ,φ=GE,max+Aθ,φ | dBi |
| Composite array radiation pattern | AAθ,φ=AEθ,φ+10log10m=1Mn=1Nwm,nvm,n2 , where vm,n=expj2πm−1dvλcosθ+n−1dℎλsinθsinφ wm,n=1MNexpj2πm−1dvλsinθetilt−n−1dℎλcosθetiltsinφescan | dBi |

In Table 2.3.2-3, base station array antenna parameters for different deployment scenarios is listed. Element parameters have been selected to produce correct element peak gain determined by calculating the directivity from a given geometry including beam widths. The element directivity can be calculated based on the pattern described by Table 2.3.2-3 in dBi as:

GE,max=DE,max−LE

, where the peak directivity DE,max is calculated from given values on 3dB, 3dB, dh and dv as:

DE,max=10log104πAlinθ,φmax−ππ0πAlinθ,φsinθdθdφ

, where Alin(,) is defined in linear scale as:

Alinθ,φ=10Aθ,φ10

Table 2.3.2-3: BS array antenna parameters

| Parameter | Macro Sub-urban | Macro Urban | Micro Urban | Small cell indoor |
| --- | --- | --- | --- | --- |
| Am (dB) | 30 | 30 | 30 | 30 |
| SLAv (dB) | 30 | 30 | 30 | 30 |
| 3dB (deg.) | 90 | 90 | 90 | 90 |
| 3dB (deg.) | 65 | 90 | 90 | 90 |
| GE,max (dBi) | 6.4 | 5.5 | 5.5 | 5.5 |
| LE (dB) | 2.0 | 2.0 | 2.0 | 2.0 |
| (M, N) | (16, 8) | (16, 8) | (8,8) | (4, 4) |
| Number of supported polarizations, P | 2 | 2 | 2 | 2 |
| dh (m) | 0.5 | 0.5 | 0.5 | 0.5 |
| dv (m) | 0.7 | 0.5 | 0.5 | 0.5 |
| Horizontal coverage range (deg.) | +/- 60 | +/- 60 | +/- 60 | N/A |
| Vertical coverage range (deg.) | 90 to 100 | 90 to 120 | 90 to 120 | N/A |
| Conducted power (before ohmic loss) per antenna element, Ptx (dBm) | 22 (Note 4) | 22 (Note 4) | 16 (Note 5) | 9 (Note 6) |
| Mechanical downtilt (deg.) | 6 | 10 | N/A | N/A (Note 7) |
| Note 1: MxN means there are M vertical and N horizontal elements Note 2: LE is included in GE,max Note 3: The vertical coverage range includes the mechanical downtilt. Note 4: The conducted power per element assumes 16x8x2 elements (i.e. power per H/V polarized element). Note 5: The conducted power per element assumes 8x8x2 elements (i.e. power per H/V polarized element). Note 6: The conducted power per element assumes 4x4x2 elements (i.e. power per H/V polarized element). Note 7: Boresight direction is perpendicular to the ceiling. |  |  |  |  |

Based on the array parameters sets fundamental characteristics such as peak EIRP can be derived. For radiated power considerations peak Equivalent Isotropic Radiated Power (EIRP) is calculated in logarithmical scale calculated as:

EIRP=Ptx+GE,max+20log10MN+10log10P

, where P, M, N, Ptx, GE,max is given by Table 2.3.2-3.

According to TR 38.921, UE operating in 10 - 10.5 GHz will most likely therefore have a conducted interface with an assumed isotropic radiation pattern antenna and no beamforming.

  - 2.4 ACIR model

- Based on [2], we can use the following ACIR model for Ka band as a starting point for Ku band NTN coexistence study.

- To use clause 5.1.1.4.1 and 5.1.1.4.2 as in 3GPP TR 36.942 as baseline for the ACLR modelling.

- To use 1 UE for both TN and NTN in the DL

- To use 10 UEs for NTN and 1 UE for TN in the UL

- To use flat ACLR modeling for co-existence study.

  - 2.5 Propagation model

Based on [2], the following table is to be used for calibration for Ka band. We can use the propagation model for Ka band as a starting point for Ku band NTN coexistence study.

| Link | Propagation model |
| --- | --- |
| TN BS to Fixed VSAT on roof | Free space path loss |
| TN BS to L-ESIM at 1.5 m | UMa as in 3GPP TR 38.803 |
| TN BS to TN UE | UMa as in 3GPP TR 38.803 |
| TN UE to Fixed VSAT on roof | UMa as in 3GPP TR 38.803 (BS is to be replaced with VSAT) |
| TN UE to L-ESIM | Umi |
| Satellite to TN BS/UE | 3GPP TR 38.821 |
| Satellite to VSAT/ESIM | 3GPP TR 38.821 |
| TN BS to Satellite | 3GPP TR 38.821 |
| Note1: For the propagation models which use the 3GPP TR 38.821 [3], to use same assumptions as in [3] and consider the atmospheric losses and the scintillation losses. Note2: TN BS height is 25m |  |

  - 2.6 Transmission power control model

    - 2.6.1	TN UL TPC

For uplink scenario, TPC model specified in Section 9.1 TR 36.942 is applied for TN with following parameters.

where:

-	Pmax = 23dBm,

-	Rmin = TBD dB,

-	CLx-ile and γ are set as following:

-	CLx-ile = 88 + 10*log10 (200/X) + 11 – Y, where X is UL transmission BW (MHz) and Y is the BS noise figure

-	γ = 1For uplink scenario,

    - 2.6.2	NTN UL TPC

For the coexistence study, the same TPC model of TN for NTN UL scenarios is adopted

    - 2.6.3	NTN and TN DL TPC

For downlink scenario, no power control scheme is applied.

  - 2.7 Received power model

The received power in downlink and uplink scenarios is defined as below:

RX_PWR = TX_PWR – Path loss + G_TX + G_RX

where:

-	RX_PWR is the received power

-	TX_PWR is the transmitted power

-	G_TX is the transmitter antenna gain (directional array gain)

-	G_RX is the receiver antenna gain (directional array gain).

  - 2.8 Performance metric

For TN, the average throughput loss and 5%-ile throughput loss should be less than 5%.

For NTN, the average throughput loss and 5%-ile throughput loss should be less than 5%.

  - 2.9 Throughput ~ SNR mapping

According to section 5.2.7 of TR 38.803, the throughput of a modem with link adaptation can be approximated by an attenuated and truncated form of the Shannon bound. (The Shannon bound represents the maximum theoretical throughput than can be achieved over an AWGN channel for a given SINR). The following equations approximate the throughput over a channel with a given SINR, when using link adaptation:

Tℎrougℎput SINR, bps/Hz =0                                  for SINR                                         ∝∙SSINR                     for SINRMIN≤SINR<SINRMAX ∝∙SSINRMAX               for SINR ≥ SINRMAX

Where:

S(SINR)   Shannon bound, S(SINR) =log2(1+SINR)  bps/Hz
			Attenuation factor, representing implementation losses
SINRMIN  	Minimum SINR of the code set, dB
SINRMAX  Maximum SINR of the code set, dB

The parameters α, SINRMIN and SINRMAX can be chosen to represent different modem implementations and link conditions. The parameters (SINR in dB instead of linear scale) proposed in table 2.9-1 represent a baseline case, which assumes:

-	1:1 antenna configurations

-	AWGN channel model

-	Link Adaptation (see table 2.9-1 for details of the highest and lowest rate codes)

-	No HARQ

Table 2.9-1: Parameters describing baseline Link Level performance for 5G NR

| Parameter | DL | UL | Notes |
| --- | --- | --- | --- |
| α, attenuation | 0.6 | 0.4 | Represents implementation losses |
| SINRMIN, dB | -10 | -10 | Based on QPSK, 1/8 rate (DL) & 1/5 rate (UL) |
| SINRMAX, dB | 30 | 22 | Based on 256QAM 0.93(DL) & 64QAM 0.93 (UL) |

# 3 Conclusion

In this contribution, we shared some views on coexistence study for NTN Ku band and the proposal is made as follows:

Proposal: It is proposed to use the simulation assumptions in this paper for Ku band NTN coexistence study.

# 4 Reference

- RP-241690, New WID on Introduction of Ku Band for NR NTN, Intelsat et al.

- R4-2309771, Simulation assumptions for NTN co-existence study in bands above 10GHz, Samsung

- TR 38.863

- TR 38.921

- TR 38.821

- TR 38.803

- TR 38.811

- TR 36.942