# 3GPP TSG- RAN WG4 Meeting # 112	R4-2411860

# Maastricht, Netherlands, 19th–23rd Aug, 2024

# Source: 	ZTE Corporation, Sanechips

# Title: 	Discussion on UE RF requirements for NR NTN Ku band

# Agenda item:	8.9.4

# Document for:	Approval

# 1 Introduction

In RAN#104 meeting, a new Work Item [1] was approved on introduction of Ku Band for NR NTN for NR NTN with the following RAN4 objective. In this contribution, we want to share some initial views about NR NTN Ku band from UE RF requirements perspective.

The following assumptions are taken as the baseline for this work: - GSO and NGSO - NTN capable UE type 1 to 5 (38.101-5 Table 9.2.1.0-1) for above 10 GHz (no handheld devices) - FDD Mode - GNSS Receiver capability The objectives are: - Use the regulation requirements in ITU Regions 1, 2 and 3 to identify relevant adjacent band co-existence scenarios for NTN Ku Band covering the following frequency ranges, considering targeted deployment scenarios: - For ITU regions 1 and 3: - Downlink 10.70 – 12.75 GHz - Uplink 13.75-14.5 GHz - For ITU regions 2 excluding US: - Downlink 10.70 – 12.70 GHz - Uplink 13.75-14.5 GHz - For ITU regions 1 and 3 - Downlink 10.70 – 12.75 GHz - Uplink 12.75 - 13.25 GHz - For ITU regions 2 excluding US - Downlink 10.70 – 12.70 GHz - Uplink 12.70 - 13.25 GHz - Note: Appropriate TX/Rx separation for uplink downlink pairing will be required. - Conduct coexistence study. Reuse Ka band coexistence study assumptions as much as possible, with modifications where necessary, e.g. for frequency range and subcarrier spacing. Conduct new co-existence studies based on adjacent services . - Specify RF requirements for satellite access node and NTN VSAT types in 38.101-5 also considering existing regulations on antenna sizes for certain parts of the Ku band. - Priority 1: for all ITU regions, define Ku band #1a and #1b for the downlink block and uplink block B to enable the future possibility of supporting satellite services in 125 MHz (UL) and 250 MHz (DL) aggregated bandwidths based on existing channel bandwidths. Potential approaches for achieving this shall be studied, from which one shall be selected including but not limited to: - Ku band #1a and #1b as FR1-NTN supporting 20 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing - Ku band #1a and #1b as FR2-NTN supporting 50 MHz, 100 MHz, 200 MHz, 400 MHz (120kHz SCS only) channel bandwidths with 120 kHz subcarrier spacing - RAN4 will select which one of the above approaches for DL and UL will be pursued by RAN #106. - Priority 2: for all ITU regions excluding the US in region 2, using the approach in Priority 1, define Ku band #2a and #2b as appropriate for the downlink block and uplink block A to support satellite services in 125 MHz (UL) and 250 MHz (DL) aggregated bandwidths. - Specify RRM requirements Note: Support for NTN CA is not in the scope of this WID

# 2 Discussion

The objective of this WID states that specify RF requirements for satellite access node and NTN VSAT types in 38.101-5 also considering existing regulations on antenna sizes for certain parts of the Ku band. And NTN VAST is a UE operating in FR2-NTN which could be a Fixed VSAT or a Mobile VSAT. Firstly, we need to discuss whether Fixed VSAT and Mobile VSAT can be supported in Ku band.

Proposal 1: We need to discuss whether Fixed VSAT and Mobile VSAT can be supported in Ku band.

The following UE RF requirements need to be defined for NTN Ku band.

Table 2.1 UE RF requirements for NTN Ku band

| NR-NTN UE Tx/Rx requirement | Proposals for NTN Ku band |
| --- | --- |
| UE maximum output power | Band specific --> Specification impact. Need further check whether and which the following requirements defined for FR2-NTN bands can be reused. - Minimum peak EIRP for Fixed VSAT - Maximum output power limits for Fixed VSAT - Minimum peak EIRP for Mobile VSAT - Maximum output power limits for Mobile VSAT |
| UE maximum output power reduction | Depend on the ACLR, SEM and EVM requirement and discussion could be postponed until other requirement is more clear. |
| UE additional maximum output power reduction | Band specific --> Specification impact. Depend on other coexistence requirement or regulatory requirement. Operators’ input are encouraged. |
| Configured transmitted power | Not band specific --> No specification impact. |
| Minimum output power | Not applicable to VSAT. |
| Transmit OFF power | Not band specific --> No specification impact. |
| Transmit ON/OFF time mask | Not band specific --> No specification impact. |
| Power control | Not band specific --> No specification impact. |
| Frequency error | Not band specific --> No specification impact. |
| Transmit modulation quality | Not band specific --> No specification impact. |
| Occupied bandwidth | Not band specific. If we divide Ku band to FR1, requirements for 20 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidth need to be defined. |
| Spectrum emission mask | Specification impact. Need to check whether general SEM defined for FR2-NTN can be reused and whether additional SEM is needed for NTN Ku band. |
| Adjacent channel leakage ratio | Specification impact. Depend on the outcome of coexistence study. |
| Spurious emissions | Specification impact. Need to check whether general SEM defined for FR2-NTN can be reused, and whether on-axis and off-axis needs to be defined for Ku band. |
| Reference sensitivity level | Band specific --> Specification impact. Need further study. |
| Max input level | If we divide Ku band to FR1, requirements for 20 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidth need to be defined. |
| Adjacent channel selectivity | Specification impact. Depend on the outcome of coexistence study. |
| Blocking characteristics | Need further study. If we divide Ku band to FR1, requirements for 20 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidth need to be defined. |
| Spurious response | Need further study. If we divide Ku band to FR1, requirements for 20 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidth need to be defined. |
| Intermodulation characteristics | NA |
| Spurious emissions | Not applicable to VSAT. |

Proposal 2: To use the proposals in Table 2.1 for UE RF requirements for NTN Ku band.

# 3 Conclusion

In this contribution, we shared some views on UE RF requirements for NTN Ku band and the proposals are made as follows:

Proposal 1: We need to discuss whether Fixed VSAT and Mobile VSAT can be supported in Ku band.

Proposal 2: To use the proposals in Table 2.1 for UE RF requirements for NTN Ku band.

# 4 Reference

- RP-241690, New WID on Introduction of Ku Band for NR NTN, Intelsat et al.