# 3GPP TSG- RAN WG4 Meeting # 112	R4-2411861

# Maastricht, Netherlands, 19th–23rd Aug, 2024

# Source: 	ZTE Corporation, Sanechips

# Title: 	Discussion on SAN RF requirements for NR NTN Ku band

# Agenda item:	8.9.5

# Document for:	Approval

# 1 Introduction

In RAN#104 meeting, a new Work Item [1] was approved on introduction of Ku Band for NR NTN for NR NTN with the following RAN4 objective. In this contribution, we want to share some initial views about NR NTN Ku band from SAN RF requirements perspective.

The following assumptions are taken as the baseline for this work: - GSO and NGSO - NTN capable UE type 1 to 5 (38.101-5 Table 9.2.1.0-1) for above 10 GHz (no handheld devices) - FDD Mode - GNSS Receiver capability The objectives are: - Use the regulation requirements in ITU Regions 1, 2 and 3 to identify relevant adjacent band co-existence scenarios for NTN Ku Band covering the following frequency ranges, considering targeted deployment scenarios: - For ITU regions 1 and 3: - Downlink 10.70 – 12.75 GHz - Uplink 13.75-14.5 GHz - For ITU regions 2 excluding US: - Downlink 10.70 – 12.70 GHz - Uplink 13.75-14.5 GHz - For ITU regions 1 and 3 - Downlink 10.70 – 12.75 GHz - Uplink 12.75 - 13.25 GHz - For ITU regions 2 excluding US - Downlink 10.70 – 12.70 GHz - Uplink 12.70 - 13.25 GHz - Note: Appropriate TX/Rx separation for uplink downlink pairing will be required. - Conduct coexistence study. Reuse Ka band coexistence study assumptions as much as possible, with modifications where necessary, e.g. for frequency range and subcarrier spacing. Conduct new co-existence studies based on adjacent services . - Specify RF requirements for satellite access node and NTN VSAT types in 38.101-5 also considering existing regulations on antenna sizes for certain parts of the Ku band. - Priority 1: for all ITU regions, define Ku band #1a and #1b for the downlink block and uplink block B to enable the future possibility of supporting satellite services in 125 MHz (UL) and 250 MHz (DL) aggregated bandwidths based on existing channel bandwidths. Potential approaches for achieving this shall be studied, from which one shall be selected including but not limited to: - Ku band #1a and #1b as FR1-NTN supporting 20 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing - Ku band #1a and #1b as FR2-NTN supporting 50 MHz, 100 MHz, 200 MHz, 400 MHz (120kHz SCS only) channel bandwidths with 120 kHz subcarrier spacing - RAN4 will select which one of the above approaches for DL and UL will be pursued by RAN #106. - Priority 2: for all ITU regions excluding the US in region 2, using the approach in Priority 1, define Ku band #2a and #2b as appropriate for the downlink block and uplink block A to support satellite services in 125 MHz (UL) and 250 MHz (DL) aggregated bandwidths. - Specify RRM requirements Note: Support for NTN CA is not in the scope of this WID

# 2 Discussion

  - 2.1 SAN classes

Regarding SAN classes, there are three SAN classes defined in TS 38.108: GEO satellite, LEO 600 km satellite and LEO 1200 km satellite. Considering the assumption of this WID, it takes GSO and NGSO as a baseline. So, we propose that GEO, LEO-600, and LEO-1200 should be allowed for NTN Ku band.

Proposal 1: We propose that GEO, LEO-600, and LEO-1200 should be allowed for NTN Ku band.

  - 2.2 SAN RF requirements

We make the following proposals for SAN RF requirements for NTN Ku band.

Table 2.2 SAN RF requirements for NTN Ku band

| NR BS RF Tx/Rx requirement | Proposal for NTN Ku band |
| --- | --- |
| SAN output power | Need to discuss which SAN type can be supported. |
| Output power dynamics | Not band specific --> No specification impact. |
| Transmit ON/OFF power | Not applicable. |
| Transmitted signal quality | Not band specific --> No specification impact. |
| Occupied bandwidth | Not band specific --> No specification impact. |
| Adjacent Channel Leakage Power Ratio (ACLR) | It depends on the outcome of coexistence study. |
| Out-of-band emissions | It depends on the outcome of coexistence study and ITU recommendation. |
| Transmitter spurious emissions | To reuse the same spurious emission requirement defined in TS 38.108. |
| Transmitter intermodulation | Not applicable. |
| Reference sensitivity level | Declaration basis according to the noise figure of SAN, antenna array configuration, the target SNR, FRC. |
| Dynamic range | Not band specific. If we divide Ku band to FR1, requirements for 20 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidth need to be defined. |
| Adjacent channel selectivity | It depends on the outcome of coexistence study. |
| In-band blocking | It depends on the outcome of coexistence study. |
| Out-of-band blocking | Needs further check whether out-of-band blocking requirements can be reused. |
| Receiver spurious emissions | Not applicable. |
| Receiver intermodulation | Not applicable. |
| In-channel selectivity | Not band specific. If we divide Ku band to FR1, requirements for 20 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidth need to be defined. |

Proposal 3: We make the proposals as Table 2.2 for SAN RF requirements for NTN Ku band.

# 3 Conclusion

In this contribution, we shared some views on SAN RF requirements for NTN Ku band and the proposals are made as follows:

Proposal 1: We propose that GEO, LEO-600, and LEO-1200 should be allowed for NTN Ku band.

Proposal 2: We make the proposals as Table 2.2 for SAN RF requirements for NTN Ku band.

# 4 Reference

- RP-241690, New WID on Introduction of Ku Band for NR NTN, Intelsat et al.