# 3GPP TSG- RAN WG4 Meeting # 112	R4-2411862

# Maastricht, Netherlands, 19th–23rd Aug, 2024

# Source: 	ZTE Corporation, Sanechips

# Title: 	Discussion on UE RF requirements for NR NTN phase3

# Agenda item:	8.25.2.1

# Document for:	Approval

# 1 Introduction

In RAN#102 meeting, a new Work Item [1] was approved on Non-Terrestrial Networks (NTN) for NR Phase 3 with the following RAN4 objective. In this contribution, we want to share some further views from RedCap UE RF perspective.

- Support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands [RAN4, RAN1] - For full-duplex FDD RedCap and eRedCap UEs, define the RF and RRM requirements [RAN4] - For HD-FDD RedCap UEs and eRedCap UEs, check whether any essential changes are needed for their support (i.e. focusing on HD collision rules) by end of Q2/2024 [RAN1] - Depending on feasibility assessment above, define the RF and RRM requirements [RAN4] - Notes for this objective: - GNSS (Global Navigation Satellite Systems) capabilities and simultaneous GNSS and NR-NTN operation is supported in RedCap/eRedCap UE.

# 2 Discussion

  - 2.1 HD-FDD refsens for 2 Rx

- Issue 1-2: HD-FDD refsens for 2 Rx Agreement: [0.5] dB tightening for n254 and n256, FFS for n255. n256 to be confirmed in RAN4#112. Note: RAN4#111 maintenance agreement on n256 refsens in R4-2408787 was not considered for this agreement.

The above is way forward in last meeting [2], HD-FDD RedCap REFSENS for band n255 needs further discussion. For NTN bands n255, REFSENS aligns with NR bands n24 as their DL operating bands are overlapping. Regarding NR HD-FDD RedCap REFSENS for band n24, it was agreed to be no tightening for 5MHz channel bandwidth as the following table, which is based on the principle of a band with higher duplexer insertion loss having bigger tightening value. Considering RF front-end implementation is expected to be the same for band n255 and n24, HD-FDD RedCap REFSENS for band n255 should align with n24 instead of 0.5dB tighter than n24.

Proposal 1: Considering RF front-end implementation is expected to be the same for band n255 and n24, HD-FDD RedCap REFSENS for band n255 should align with n24 instead of 0.5dB tighter than n24.

Table 2.1: NR HD-FDD 5MHz REFSENS tightening for different NR bands

| FD-FDD 5MHz REFSENS | HD-FDD REFSENS Tightening | Bands |
| --- | --- | --- |
| ≤ -100 dBm | 0 dB | n1, n18, n24, n70 |
| > -100 dBm and ≤ -99 dBm | 0.5 dB | n30, n65, n66, n74 |
| > -99 dBm | 0.8 dB | n2, n3, n5, n7, n8, n12, n13, n14, n20, n25, n26, n28, n71, n85 |

  - 2.2 Maximum output power

- Issue 1-5: Maximum output power options for further discussion: Option 1: Consider increase in PC3 power level for NTN RedCap UE which only supports HD-FDD operation for a band. Option 2: PC3 power level is the same for non-RedCap NR NTN UE and (e)RedCap NR NTN UE

Based on discussion in last meeting, we see the motivation of this issue is to increase high level of power class tolerance as the single-band 1Rx Refsens difference caused by using FD-FDD and HD-FDD. However, the power class tolerance is designed to address implementation issues, for example, change of output power with temperature, change of output power with battery voltage, impact of higher power class in terms of EMC issues and SAR. It doesn’t make sense to increase power class tolerance based on better REFSENS for HD-FDD. Hence, PC3 power level is the same for non-RedCap NR NTN UE and (e)RedCap NR NTN UE.

Proposal 2: PC3 power level is the same for non-RedCap NR NTN UE and (e)RedCap NR NTN UE.

# 3 Conclusion

In this contribution, we shared some views on UE RF requirements for NR NTN phase3 and the proposals are made as follows:

Proposal 1: Considering RF front-end implementation is expected to be the same for band n255 and n24, HD-FDD RedCap REFSENS for band n255 should align with n24 instead of 0.5dB tighter than n24.

Proposal 2: PC3 power level is the same for non-RedCap NR NTN UE and (e)RedCap NR NTN UE.

# 4 Reference

- RP-234078, New WID: Non-Terrestrial Networks (NTN) for NR Phase 3, Huawei

- R4-2410574, WF on Rel-19 phase 3 NR-NTN UE RF requirements, Qualcomm Inc.