3GPP TSG-RAN WG4 Meeting#112		R4-2411865

Maastricht, the Netherlands, Aug 19th -23rd, 2024

Agenda item:	8.20.2.1

Source: 	Spreadtrum Communications

Title: 	Discussion on deployment scenarios and spectrum usage for ambient IoT

Document for:	Discussion

Introduction

In RAN4#111, a WF [1] was approved, in which there were remaining open issues as listed. This contribution continues to be discussed as follow.

- Topic 2-1: Deployment scenario Issue 2-1-1: deployment scenarios for D1T1 Option 1-1: Legacy NR gNB are outdoor macro gNB while AIoT reader/CW/devices are all indoors. Legacy NR UE is only allowed outdoors. Option 1-2: Legacy NR gNB are outdoor macro gNB while AIoT reader/CW/devices are all indoors. Legacy NR UE is indoor accessing to outdoor NR marco gNB Option 2-1: Legacy NR gNB are co-located with AIoT reader and CW. All of NR and AIoT BS/UE/Reader/Device/CW are indoors. AIoT reader /CW and Legacy gNB share same hardware Option 2-2: Legacy NR gNB are co-located with AIoT reader and CW. All of NR and AIoT BS/UE/Reader/Device/CW are indoors. AIoT reader /CW and Legacy NR gNB do not share same hardware. (less limitation on the power boosting) Agreement in RAN4#111: - Consider only adjacent RB/channel co-existence evaluation for in-band deployment scenario for NR and AIOT - Encourage companies to provide the simulation results for option 1-1 and 1-2 - FFS on co-site scenario (option 2-1 and 2-2)

- Topic 2-2: Spectrum usage Issue 2-2-1: Spectrum usage for R2D in D1T1 Agreement in RAN4#111: - Use FDD DL as starting point for co-existence evaluation for R2D in D1T1 - FFS on FDD UL spectrum. Issue 2-2-2: Spectrum usage for CW transmission in D1T1 for the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering For the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering, and for topology 1, the following cases for CW transmission are studied. · Case 1-1: CW is transmitted from inside the topology, transmitted in DL spectrum · Case 1-2: CW is transmitted from inside the topology, transmitted in UL spectrum · Case 1-4: CW is transmitted from outside the topology, transmitted in UL spectrum Agreement in RAN4#111: - Use inside topology as starting point for co-existence evaluation (case 1-1, case 1-2) for calibration. - Further discuss the difference of outside topology (case 1-4) from co-existence study perspective. Issue 2-2-4: Spectrum usage for R2D in D2T2 Agreement: - Use FDD UL spectrum for R2D in D2T2. Issue 2-2-5: Spectrum usage for CW transmission in D2T2 for the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering For the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering, and for topology 2, the following cases for CW transmission are studied. · Case 2-2: CW is transmitted from inside the topology (i.e., intermediate UE), transmitted in UL spectrum · Case 2-3: CW is transmitted from outside the topology, transmitted in DL spectrum · Case 2-4: CW is transmitted from outside the topology, transmitted in UL spectrum Agreement: - Use case 2-2 as starting point for co-existence evaluation for calibration. - FFS on case 2-3 - Further discuss the difference of outside topology (case2-4) from co-existence study perspective.

Discussion

Deployment scenarios for D1T1

The difference of option 2-1 and option 2-2 is whether A-IoT reader/CW and Legacy gNB share same hardware or not. If A-IoT reader/CW and Legacy gNB share some hardware, which can save cost to a certain extent but power boosting will be limited (e.g., 6dB). If A-IoT reader/CW and Legacy gNB do not share some hardware, power boosting will not be limited. According to previous DL co-existence simulation results, in order to co-exist with NR under the spectrum deployment of in-band NR, power boosting should be at least 12.5dB. Obviously, based on the above analysis, option 2-2 is worth studying and evaluating compared with option 2-1.

Proposal 1: Option 2-2 needs to be prioritized for evaluation considering the feasibility.

Spectrum usage

Spectrum usage for CW transmission in D1T1 for the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering

Figure1: D1T1-A1, CW in UL spectrum                                   Figure 2: D1T1-A2, CW in UL spectrum                                                            Figure3: D1T1-B, CW in UL spectrum

According to the agreement in RAN4#111, case 1-1 and case 1-2 as a starting point for calibration purpose. As to outside topology, it is could be achieved that CW2D in UL spectrum and D2R in UL spectrum to meet the spectrum regulation. The interference of intra A-IoT system will be mitigated due to CW2D transmitted by outside topo, from the interference perspective, case 1-4 will be better or not worse than case 1-2. However, from the co-existence perspective, outside topo is different inside topo, specific simulation results needs to be proven  that inside topo result could reused for outside topo or not.

Observation 1: From the interference perspective, case 1-4 will be better or not worse than case 1-2.

Proposal 2: Case 1-4 should be included in the co-existence case.

Spectrum usage for CW transmission in D2T2 for the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering

Similar with D1T1, the interference of intra A-IoT system will be mitigated due to CW2D transmitted by outside topo, from the interference perspective, case 1-4 will be better or not worse than case 1-2. However, from the co-existence perspective, outside topo is different inside topo, specific simulation results needs to be proven  that inside topo result could reused for outside topo or not.

Proposal 3: Case 2-4 should be included in the co-existence case.

Conclusions

In this paper, we provide our views on deployment scenarios and spectrum usage, we have made the following proposals.

Proposal 1: Option 2-2 needs to be prioritized for evaluation considering the feasibility.

Observation 1: From the interference perspective, case 1-4 will be better or not worse than case 1-2.

Proposal 2: Case 1-4 should be included in the co-existence case.

Proposal 3: Case 2-4 should be included in the co-existence case.

Reference

- R4-2410567, Way forward on co-existence evaluation for ambient IoT, CMCC. TSG-RAN4#111 meeting