3GPP TSG-RAN WG4 Meeting#112		R4-2411866

Maastricht, the Netherlands, Aug 19th -23rd, 2024

Agenda item:	8.20.2.2

Source: 	Spreadtrum Communications

Title: 	Discussion on co-existence evaluation for Ambient IoT and NR/LTE

Document for:	Discussion

Introduction

In RAN4#111, a WF [1] was approved, in which there were remaining open issues as listed. This contribution continues to be discussed as follow.

- Topic 2-4: Evaluation methodology Issue 2-4-1: Evaluation methodology Agreement in RAN4#111: - Use the Monte-Carlo method as baseline for co-existence evaluation, i.e. Section 5.3 in TR38.803 - FFS on whether RAN4 needs to perform link level simulation Issue 2-4-2: Performance metric for AIOT Agreement in RAN4#111: - Use SINR for calibration purpose - FFS on performance metric for co-existence evaluation and requirements definition. Issue 2-4-3: SINR definition for D2R Agreement in RAN4#111: Do not consider CW interference for calibration purpose for D1T1-A2 and D2T2-A2 FFS on how to consider CW cancellation capability in formal simulation Issue 2-4-4: SINR definition for R2D Agreement in RAN4#111: SINR for R2D for calibration purposes - signal power of device to the noise and interference within 10MHz - Assume interference NR BW is 10MHz - FFS on BB LPF Issue 3-2-7: Multi-operator scenario Agreement in RAN4#111: FFS on co-existence between AIOT system and adjacent operator NR system

- 3 Evaluation parameters - Topic 4-1: Adjacent RB TX and Rx characteristics - Topic 4-2: General parameters and layout - Topic 4-3: Parameters for AIOT BS/intermediate UE and device - Topic 4-4: Parameters for legacy NR

Discussion

Evaluation methodology

- Evaluation methodology

In RAN4#111 meeting, Monte-Carlo method was used as a baseline for co-existence evaluation, that is, Section 5.3 in TR 38.803 was agreed as evaluation methodology. About on RAN4 whether needs to perform link level simulation or not needs further study. Based on previous simulation results, in some cases, guard RBs are needed. With regard to distance and worst cases, in our understanding, RAN4 needs to perform link level simulation for some cases, especially for worst cases, to drive guard RBs.

Proposal 1: RAN4 needs to perform link level simulation for some worst cases.

- Performance metric for A-IoT

As is known to all, the performance of the A-IoT system is worse and its coverage is much smaller than the legacy NR system. For NR system, use 5% throughput loss as a performance metric for legacy. Therefore, it is reasonable to use 10% throughput loss [BLER] for A-IoT system, including reader, device and intermediate UE.

Proposal 2: For A-IoT system, use 10% throughput loss as a performance metric.

- SINR definition for D2R

For calibration purpose for D1T1-A2 and D2T2-A2, RAN4 does not consider CW interference. But in formal simulation, we need to CW cancellation capability. According to RAN1 LLS parameters, companies to report for scenario A2/A1/B for BS and intermediate UE. We can follow RAN1’s CW cancellation capability. Remaining CW interference = CW Tx Power (dBm)+CW TX antenna gain-Cabel,connector,combiner,body losses,(dB)+Receiver antenna gain (dBi)- Cabel,connector,combiner,body losses,(dB)-CW cancellation(dB).

Proposal 3: Refer to RAN1’s CW cancellation capability. Use formula as follow in formal simulation.

- Remaining CW interference = CW Tx Power (dBm)+CW TX antenna gain(dBi)-Cabel,connector,combiner,body losses,(dB)+Receiver antenna gain (dBi)- Cabel,connector,combiner,body losses,(dB)-CW cancellation capability(dB)

- SINR definition for R2D

For calibration purpose, we assume no BB LPF. However, RAN1 uses BB LPF to do LLS. Digital baseband low pass filter (e.g., Butterworth 3rd or 5rd ) brings no additional power consumption, and the size and cost are controllable. In our understanding, we need to consider BB LPF in formal simulation.

Proposal 4: We need to consider BB LPF in formal simulation.

Evaluation cases

- Evaluation cases for D1T1 for device2b between NR and AIoT

In RAN4#110-bis meeting, device1 and device2a has been reached an agreement, now we need to further study device2b. Based on the analysis of spectrum usage, adopt Table 1 as the proposed scenarios for in-band co-existence in formal simulation.

Table 1: A-IoT co-existence scenarios with in-band NR deployment for D1T1

| Deployment scenario and topology | spectrum | aggressor | victim |
| --- | --- | --- | --- |
| D1T1-C | R2D: DL D2R: UL | Device | NR UL |
|  |  | NR UL | Reader |
|  |  | Reader | NR DL |
|  |  | NR DL | device |
|  |  | Device | NR DL |
|  |  | NR DL | Reader |

Proposal 5: Adopt Table 1 as the in-band co-existence evaluation scenarios for D1T1-C.

- Evaluation cases for D2T2 between NR and AIoT

In RAN4#110-bis meeting, device1 and device2a has been reached an agreement, now we need to further study device2b. Based on the analysis of spectrum usage, adopt Table 2 as the proposed scenarios for in-band co-existence in formal simulation.

Table 2: A-IoT co-existence scenarios with in-band NR deployment for D2T2-C

| Deployment scenario and topology | spectrum | aggressor | victim |
| --- | --- | --- | --- |
| D1T1-C | R2D: UL D2R: UL | Device | NR UL |
|  |  | NR UL | Reader |
|  |  | Reader | NR UL |
|  |  | NR UL | device |

Proposal 6: Adopt Table 2 as the in-band co-existence evaluation scenarios for D2T2-C.

- Multi A-IoT operators scenario

To improve A-IoT system capacity and spectrum efficiency, FDMA should be studied for R2D transmission, especially for multiple A-IoT operators’ coexistence scenario. FDMA divides different frequency resource units in the frequency domain which are used for data transmission by different devices. Ideally, the frequency resource units should not overlap with each other, and a guard band may be needed between adjacent frequency resource units.

However, due to relatively large RF filter (Matching Network) bandwidth in front of RF envelop detection, e.g., 10MHz, reader may have to reserve at least 10MHz system bandwidth for A-IoT R2D transmission to avoid any interference falling into the RF envelop detector, i.e., Bsys,R2D = 10MHz. Specifically, within the wide system bandwidth, only one R2D signal can be transmitted at one instance; otherwise, if multiple R2D signals are simultaneously transmitted by FDMA within the wide system bandwidth, all others R2D signals will be absorbed by the RF envelop detector, which will greatly affect the performance. Considering that the spectrum allocated for A-IoT systems may be limited, if the total spectrum is less than twice of system bandwidth Bsys,R2D (as shown in Figure1), FDMA may not be feasible without any improvements.

To increase system deployment flexibility and save the spectrum allocated for A-IoT system, FDMA divides different frequency resource units for different operators in the frequency domain which are used for different devices deployed in different operators. That is to say, the frequency resource can divide into two R2D signal channel, as shown in Figure 2. Two R2D signal channels are allocated at the edge of the A-IoT system bandwidth and for different operators, at the same time defining the system bandwidth Gap in the overlapped frequency resource, guaranteeing that there is no signals transmitted in the system bandwidth Gap, FDMA can still be achieved without introducing any interference between these two R2D channels.

Figure2: one possible FDM candidate solution for multiple A-IoT operators scenario

Proposal 7: Multiple A-IoT operators co-existence scenario should be investigated and capture figure1 in TR.

Proposal 8: How operators can co-ordinated needs to be further studied.

Evaluation parameters

- Layout for D1T1

For scenario 1-1 and 1-2, according to the agreement in RAN4#111 meeting, assumptions for D1T1 reached a conclusion, but about legacy indoor UE distribution for formal simulation for scenario 1-2 needs to further study. According to the offline discussion as follow.

• for formal simulation: X% legacy indoor UE and FFS for detailed distribution • option 1: uniform distribution in the sector (no penetration loss between legacy UE and A-IoT system) • option 2: uniform distribution in the same factory/office as A-IoT system (no penetration loss) • option 3: uniform distribution in the sector (penetration loss should be considered based on the distribution)

In our understanding, legacy indoor UE should be uniform distributed in the same factory/office, but we need to adjust the percent of indoor UE. Generally speaking, there are more than 10 NR UE in a sector, 80% NR UE are distributed indoors, and 20% NR UE are distributed outdoors. Of course, these indoors including ordinary indoor and factory indoor. However, in A-IoT coexistence simulation, ordinary indoor is ignorable. In order to better meet the actual deployment situation, we should adjust the percentage of indoor UEs in the factory/office. The percentage can be calculated according to the area of the factory and the area of the sector. A sector area is 108250 m2, a factory area is 7200 m2, the potation of factory area is 5%, and outdoor NR is 20%. Thus the ratio of indoor factory UE to outdoor UE is 1:4 that is 20%. Based on the above analysis, we think that X% legacy indoor UE should be 20%, outdoor UE is 80%.

Proposal 9: Support option2, 20% legacy indoor UE and 80% legacy outdoor UE in formal simulation.

Conclusions

In this paper, we provide our views on co-existence between A-IoT and NR/UE, we have made the following observations and proposals.

Proposal 1: RAN4 needs to perform link level simulation for some worst cases.

Proposal 2: For A-IoT system, use 10% throughput loss as a performance metric.

Proposal 3: Refer to RAN1’s CW cancellation capability. Use formula as follow in formal simulation.

- Remaining CW interference = CW Tx Power (dBm)+CW TX antenna gain(dBi)-Cabel,connector,combiner,body losses,(dB)+Receiver antenna gain (dBi)- Cabel,connector,combiner,body losses,(dB)-CW cancellation capability(dB)

Proposal 4: We need to consider BB LPF in formal simulation.

Proposal 5: Adopt Table 1 as the in-band co-existence evaluation scenarios for D1T1-C.

Proposal 6: Adopt Table 2 as the in-band co-existence evaluation scenarios for D2T2-C.

Proposal 7: Multiple A-IoT operators co-existence scenario should be investigated and capture figure1 in TR.

Proposal 8: How operators can co-ordinated needs to be further studied.

Proposal 9: Support option2, 20% legacy indoor UE and 80% legacy outdoor UE in formal simulation.

Reference

- R4-2410567, WF on co-existence evaluation for ambient IoT, CMCC, INC.TSG-RAN WG4#111 meeting