3GPP TSG-RAN WG4 Meeting#112		R4-2411868

Maastricht, the Netherlands, Aug 19th -23th, 2024

Agenda item:	8.20.3.3

Source: 	Spreadtrum Communications

Title: 	Discussion on RF requirements impact for intermediate node (UE)

Document for:	Discussion

Introduction

In RAN4#111 meeting, a WF [1] was approved, in which there were remaining open issues as listed. This contribution continues to be discussed as follow.

Discussion

TX requirements

Agreement: - Use the following table as starting point for RF requirements impact study. The table is for information. RF Requirement for A-IoT intermediate UE- TX part. TX requirement; Maximum output power. Output power dynamics. Transmit ON/OFF power. Transmitted signal quality. Transmission times. Occupied bandwidth. Tx intermodulation. ACLR. Operating band unwanted emissions. Transmitter spurious emissions.

According to the WF [1], we provide some initial views about RF requirements for A-IoT intermediate UE TX part in Table 1.

Proposal 1: Some initial views about TX RF requirements for A-IoT intermediate UE in Table1.

Table1: some initial views about TX RF requirements for A-IoT intermediate UE

| RF Requirement for A-IoT intermediate UE- TX part |  |  |
| --- | --- | --- |
| TX requirement | Maximum output power | Refer to RAN1’s LLS result, take a large value between RAN1’s LLS result and legacy UE. |
|  | Output power dynamics | Based on the DL co-existence simulation results, in order to coexist with NR under the spectrum deployment of in-band NR, power boosting for A-IoT system could improve the interference caused by NR UE |
|  | Minimum output power | The legacy UE requirement as a starting point |
|  | Transmit OFF power | The legacy UE requirement as a starting point(e.g., -50dBm) |
|  | Transmit ON/OFF time mask | The legacy UE requirement as a starting point |
|  | Transmitted signal quality | The legacy UE of frequency error as a starting point. |
|  | Transmission times | The definition needs further clarification |
|  | Occupied bandwidth | Wait for RAN1s’s progress on R2D |
|  | Tx intermodulation | The legacy UE requirement as a starting point |
|  | ACLR | Depends on co-existence study |
|  | Operating band unwanted emissions | Depends on co-existence study and regulatory requirements |
|  | Transmitter spurious emissions | Depends on co-existence study and regulatory requirements |

RX requirements

Agreement: - Use the following table as starting point for RF requirements impact study. The table is for information. RF Requirement for A-IoT intermediate UE- RX part. RX requirement; Reference sensitivity. Maximum input power. ICS. ACS. In-band blocking. Out-of-band blocking. Receiver intermodulation. Rx spurious emission.

According to the WF [1], we provide some initial views about RF requirements for A-IoT intermediate UE RX part in Table 2.

Proposal 2: Some initial views about RX RF requirements for A-IoT intermediate UE in Table2.

Table 2: some initial views about RX RF requirements for A-IoT intermediate UE

| RF Requirement for A-IoT intermediate UE- RX part |  |  |
| --- | --- | --- |
| RX requirement | Reference sensitivity | The legacy UE requirement as a starting point, but need to consider CW interference cancellation and R2D waveform |
|  | Maximum input power | The legacy UE requirement as a starting point |
|  | ICS | Depends on co-existence study |
|  | ACS | Depends on co-existence study |
|  | In-band blocking | Depends on co-existence study |
|  | Out-of-band blocking | Depends on co-existence study |
|  | Receiver intermodulation | The legacy UE requirement as a starting point |
|  | Rx spurious emission | The legacy UE requirement as a starting point |

CW for D2T2

Agreement: - To further investigate output power, emission requirements for CW node - FFS for other requirements.

- Output power

According to our RAN1’s link budget for coverage results [2], D2R is a bottleneck, especially for device1 and device2a for D2T2-A2. When CW output power is 23dBm, it may not be able to meet the minimum coverage target of 10 meters. If we increase the output power by 3dB, the coverage is increased by about 1 meter when the coverage is less than 10 meters. Thus, in order to increase the coverage target, a higher CW output power level can be considered (e.g., 26dBm or 29dBm).

Proposal 3: Higher CW output power can be considered (e.g., 26dBm or 29dBm).

- Emission requirements

According to RAN1’s agreement, for multiple unmodulated single-tone transmitted by one CW node, other number of tones (i.e. >2) is deprioritized. That is to say, CW is most likely one single-tone or two unmodulated single-tone. When CW is two unmodulated single-tone, two unmodulated single-tone will produce intermodulation products, such as IMD3, IMD5 or even IMD7. These intermodulation products will be backscattered by device and then affect the reception on the reader side. Thus, these unwanted emissions need to be suppressed. These unwanted emission may be in-band emission, may be ACLR and so on.

Proposal 4: Unwanted emissions caused by intermodulation products of CW need to be further studied.

- Other requirements

For Deployment scenario 2 with topology 2, the following scenarios are used for evaluation of coverage and co-existence.

From the perspective of D2R for reader, it will suffer from self-interference generated by CW2D, R2D and co-frequency interference caused by other devices with FDM. As we all know, the ability of interference cancellation is critical to sensitivity and coverage. Interference suppression technology is mainly divided into three aspects, self-interference suppression in spatial domain, analog domain and digital domain respectively. Self-interference suppression in spatial domain refers to improving the isolation of transmitter and receiver channels and suppressing self-interference signals by antenna arrangement. The main methods include separation of transmission and reception, electromagnetic wave isolation and antenna pattern diversity. For example, the above D2T2-A1 is the interference suppression in the spatial domain. Analog domain self-interference suppression is to use the transmitter baseband or RF self-interference signal as a reference source to generate a copy of the analog self-interference signal at the RF front end of the receiver and cancel it. Self-interference suppression in digital domain is to estimate and suppress the multipath channel and nonlinear characteristics of self-interference signal in digital domain of receiver by using digital self-interference signal as reference source. Self-interference suppression depends on implementation, but the degradation of sensitivity needs further discussion.

Proposal 5: CW for D2T2 interference cancellation ability and MSD need to be further studied.

Conclusions

In this paper, we provide our views on RF requirements impact of intermediate node (UE), we have made the following proposals.

Proposal 1: Some initial views about TX RF requirements for A-IoT intermediate UE in Table1.

Proposal 2: Some initial views about RX RF requirements for A-IoT intermediate UE in Table2.

Proposal 3: Higher CW output power can be considered (e.g., 26dBm or 29dBm).

Proposal 4: Unwanted emissions caused by intermodulation products of CW need to be further studied.

Proposal 5: CW and the intermodulation products of CW for D2T2 interference cancellation ability need to be further studied.

Reference

- R4-2410597, WF on impacts of A-IoT  on RF requirements, Huawei, TSG RAN WG4 meeting #111

- R4-2405910, Discussion on evaluation assumption and results for Ambient IoT, Spreadtrum, TSG RAN WG1 meeting#117