3GPP TSG-RAN4 Meeting #112	R4-2411869

# Maastricht, Netherlands, 19th – 23rd August, 2024

Source:	LG Electronics

Title:	HPUE for inter-band UL CA and EN-DC

Agenda item:	8.1.1.1.2

Document for:	Discussion

Introduction

In this paper, we provide our views on HPUE for inter-band UL CA and EN-DC.

Discussion

In last RAN4 meeting, the following WF was approved [1]. This paper discusses the requirement of HPUE for inter-band UL CA and EN-DC.

1.1 Duty cycle solution for SAR compliance - Duty cycle solution is considered for both intra-band CA and inter-band CA/EN-DC 3.1 Requirements between handheld UE and FWA - <WF>: - For PC2 and PC1.5 2-band inter-band UL CA with 2Tx or 3Tx, - only define one set of UE RF requirements for both handheld UE and FWA - For PC2 and PC1.5 2-band inter-band EN-DC with 2Tx or 3Tx, - only define one set of UE RF requirements for both handheld UE and FWA. 3.2 Whether to collect configurations to derive general requirements? - <WF>: - General requirements can be specified including considerations of different duplex mode configurations: - For PC2, - FDD+FDD, FDD+TDD, TDD+TDD with 2Tx and 3Tx - For PC1.5, - [FDD+FDD], FDD+TDD, TDD+TDD with 2Tx and 3Tx 3.3 3UL CC with 3Tx - <WF>: RAN4 to further check whether there is demand for 3 ULCC with 3Tx scenario (such as CA_nXA-nY(2A) and CA_ nXA-nYB), and whether it is desirable to be introduced in Rel-19. 3.4 Release independence - <WF>: Discuss the release independence for 3Tx band combination for handheld UE and FWA at later stage of this WI - For FWA, only discuss the configurations that were not introduced in Rel-18 - 4.4 MSD and SAR compliance Way forward: - If higherPowerLimit-R17/higherPowerLimtMRDC-R17 capability is adopted for increasing UE transmission power, then taking same methodology as for higherPowerLimit-R17/ higherPowerLimtMRDC-R17 for SAR compliance and MSD. More specific, - For a given band combination, no additional MSD requirements are specified in addition to the MSD requirements of its legacy power class(es) - No new duty-cycle solution/mechanism for SAR compliance is considered - P-MPR can be used for SAR mitigation

PC1.5 UE for 2- band NR inter-band uplink CA with 2Tx and/or 3Tx for handheld and FWA

For PC1.5 UE supporting NR inter-band UL CA with 2Tx,

PC2+PC2 for 2-band NR bands is new power combination.

- Case 1 : PC2 NR FDD + PC2 NR TDD

- Case 2 : PC2 NR TDD + PC2 NR TDD

If case1 and case2 above are introduced in Rel-19, the existing duty cycle solution should be modified.

Proposal 1: Modify the existing PC2 duty cycle solution if PC2 FDD/TDD + PC2 TDD with 2Tx is introduced.

For PC1.5 UE supporting NR inter-band UL CA with 3Tx,

In Rel-18, the following combination of PC for NR bands is supported.

- Case 1 : PC3 NR FDD + PC1.5 NR TDD (UL-MIMO or Tx Diversity)

Whether to support the following combinations in Rel-19 need to be decided.

- Case 2 : PC2 NR FDD + PC1.5 NR TDD (UL-MIMO or Tx Diversity)

- Case 3 : PC2 NR FDD + PC2 NR TDD (UL-MIMO or Tx Diversity)

- Case 4 : PC3 NR TDD + PC1.5 NR TDD (UL-MIMO or Tx Diversity)

- Case 5 : PC2 NR TDD + PC1.5 NR TDD (UL-MIMO or Tx Diversity)

- Case 6 : PC2 NR TDD + PC2 NR TDD (UL-MIMO or Tx Diversity)

Among the cases, the total maximum output power can be higher than 29dBm in case 1, case 2, case 4 and case 5. The difference from 29dBm is 0.97dB for case 1 and case 4 and 1.76dB for case 2 and case 5.

- Case 1 & Case 4 : 23dBm + 29dBm = 29.97 dBm

- Case 2 & Case 5 : 26dBm + 29dBm  = 30.76 dBm

- Case 3 & Case 6 : 26dBm + 26dBm  = 29 dBm

If higherPowerLimit is indicated, PPower class, CA is replaced with 29.97dBm for case 1 and case 4 and 30.76dBm for case 2 and case 5.

The existing duty cycle solution for PC1.5 needs be updated to reduce the duty cycle by the boosted delta power for SAR compliance.

Proposal 2: For PC1.5 UE supporting NR inter-band UL CA with 3Tx,  consider

- PC2 NR FDD + PC2/PC1.5 NR TDD (UL-MIMO or Tx Diversity)

PC1.5 and PC2 for two band EN-DC with 2Tx and/or 3Tx for handheld and FWA

For PC2 UE supporting two band EN-DC with 2Tx,

In Rel-18, the following combination of PC for E-UTRA band and PC for NR band is supported.

- Case 1 : PC3 E-UTRA FDD + PC3 NR TDD

- Case 2 : PC3 E-UTRA FDD + PC2 NR TDD

- Case 3 : PC3 E-UTRA TDD + PC3 NR TDD

- Case 4 : PC3 E-UTRA TDD + PC2 NR TDD

Whether to support the following combinations in Rel-19 need to be decided.

- Case 5 : PC3 E-UTRA FDD + PC3 NR FDD

- Case 6 : PC3 E-UTRA FDD + PC2 NR FDD

- Case 7 : PC2 E-UTRA TDD + PC3 NR TDD

For SAR compliance,

In Case 5 and Case 6, the existing E-UTRA FDD + NR TDD duty cycle solution can be reused with a new capability, for example, maxUplinkDutyCycle-interBandENDC-FDD-PC2 which comprises of maxUplinkDutyCycle-FDD-EN-DC1 and  maxUplinkDutyCycle-FDD-EN-DC2,  instead of maxUplinkDutyCycle-FDD-TDD-EN-DC1 and  maxUplinkDutyCycle-FDD-TDD-EN-DC2 which are applicable only for inter-band FDD+TDD EN-DC.

In Case 7, the existing E-UTRA TDD + NR TDD duty cycle solution needs to be modified, because it is based on PC3 E-UTRA TDD. PC2 E-UTRA TDD based duty cycle solution is needed.

For example, Table 2.1 shows the percentage of maximum E-UTRA/NR uplink transmission for Case 7.

Table 2.1: Percentage of maximum E-UTRA/NR uplink transmission in PC2 EN-DC (Case 7)

| E-UTRA TDD Uplink-downlink Configuration (X) | Percentage of maximum E-UTRA/NR uplink transmission during evaluation period (%) |  | Supported EN-DC power class (Z:actual NR % of uplink transmission, D:threshold) |  |
| --- | --- | --- | --- | --- |
|  | E-UTRA (26dBm) | NR (23dBm) |  |  |
|  | Y=0~50 | Z=0~100, D=100-2*Y | Z <= D | Z>D |
| 0 | 70 | N/A | PC2 | PC3 |
| 1 | 50 | N/A |  |  |
| 2 | 30 | 40 |  |  |
| 3 | 40 | 20 |  |  |
| 4 | 30 | 40 |  |  |
| 5 | 20 | 60 |  |  |
| 6 | 60 | N/A |  |  |

- X : E-UTRA TDD Uplink-downlink Configuration

- Y : maximum percentage of E-UTRA uplink transmission during any evaluation period of time

- Z : actual percentage of NR uplink transmission during any evaluation period of time

- D : threshold of NR uplink transmission during any evaluation period of time

Here, E-UTRA TDD uplink-downlink configuration ‘0’, ‘1’ and ‘6’ are not applicable for EN-DC uplink transmission with maximum output power for SAR compliance. A minimum percentage of 20% for maximum NR uplink transmission during evaluation period can be assumed as threshold when the field of UE capability maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is absent.

For example,

When E-UTRA TDD Uplink-downlink Configuration (X) = 3 and threshold of NR uplink transmission = 20,

- Z > 20,        PC3 EN-DC is applied

- Z ≤ 20,      PC2 EN-DC is applied

Table 2.2 shows the percentage of maximum E-UTRA/NR uplink transmission for Case 3. Threshold of 30% was specified with the minimum percentage of maximum NR uplink transmission for PC3 E-UTRA TDD+PC3 NR TDD

when the field of UE capability maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is absent.

Table 2.2: Percentage of maximum E-UTRA/NR uplink transmission in PC2 EN-DC (Case 3)

| E-UTRA TDD Uplink-downlink Configuration (X) | Percentage of maximum E-UTRA/NR uplink transmission during evaluation period (%) |  | Supported EN-DC power class (Z:actual NR % of uplink transmission, D:threshold) |  |
| --- | --- | --- | --- | --- |
|  | E-UTRA (23dBm) | NR (23dBm) |  |  |
|  | Y=0~100 | Z=0~100, D=100-Y | Z <= D | Z>D |
| 0 | 70 | 30 | PC2 | PC3 |
| 1 | 50 | 50 |  |  |
| 2 | 30 | 70 |  |  |
| 3 | 40 | 60 |  |  |
| 4 | 30 | 70 |  |  |
| 5 | 20 | 80 |  |  |
| 6 | 60 | 40 |  |  |

Proposal 3: For PC2 UE supporting two band EN-DC with 2Tx,  consider

- PC3 E-UTRA FDD + PC3/PC2 NR FDD

- PC2 E-UTRA TDD + PC3 NR TDD

Proposal 4: For PC3 E-UTRA FDD + PC3/PC2 NR FDD,  consider

- a new capability, for example, maxUplinkDutyCycle-interBandENDC-FDD-PC2 which comprises of maxUplinkDutyCycle-FDD-EN-DC1 and maxUplinkDutyCycle-FDD-EN-DC2

Proposal 5: For PC2 E-UTRA TDD + PC3 NR TDD,  consider

- New duty cycle solution considering the possible E-UTRA TDD uplink-downlink configurations and the percentage of maximum E-UTRA/NR uplink transmission

For PC1.5 UE supporting two band EN-DC with 2Tx,

In Rel-18, it is not supported.

Whether to support the following combination in Rel-19 needs to be decided.

- Case 1 : PC2 E-UTRA TDD + PC2 NR TDD

For SAR compliance,

In Case 1, the existing E-UTRA TDD + NR TDD duty cycle solution needs to be modified, because it is based on PC3 E-UTRA TDD. PC2 E-UTRA TDD based duty cycle solution is needed.

For example, Table 2.3 shows the percentage of maximum E-UTRA/NR uplink transmission for Case 1.

Table 2.3: Percentage of maximum E-UTRA/NR uplink transmission in PC1.5 EN-DC (26dBm(E-UTRA TDD)+26dBm(NR TDD))

| E-UTRA TDD Uplink-downlink Configuration (X) | Percentage of maximum E-UTRA/NR uplink transmission during evaluation period (%) |  |  | Supported EN-DC power class (Z:actual NR % of uplink transmission, D(D1,D2) :threshold) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | E-UTRA (26dBm) | NR (26dBm) (Z=Z1 or Z2) |  |  |  |  |
|  | Y=0~50 | Z1=0~50, D1=50-Y | Z2=0~50, D2=25-Y = D1-25 | Z > D1 | D2<=Z<=D1 or D1>=Z | Z < D2 |
| 0 | 70 | N/A | N/A | N/A | N/A | N/A |
| 1 | 50 | N/A | N/A | N/A | N/A | N/A |
| 2 | 30 | 20 | N/A | PC3 | PC2 | N/A |
| 3 | 40 | 10 | N/A | PC3 | PC2 | N/A |
| 4 | 30 | 20 | N/A | PC3 | PC2 | N/A |
| 5 | 20 | 30 | 5 | PC3 | PC2 | PC1.5 |
| 6 | 60 | N/A | N/A | N/A | N/A | N/A |

Here, E-UTRA TDD uplink-downlink configuration ‘0’, ‘1’ and ‘6’ are not applicable for EN-DC uplink transmission with maximum output power for SAR compliance.

- X : E-UTRA TDD Uplink-downlink Configuration

- Y : maximum percentage of E-UTRA uplink transmission during any evaluation period of time

- Z : actual percentage of NR uplink transmission during any evaluation period of time

- D : threshold of NR uplink transmission during any evaluation period of time

For example,

When E-UTRA TDD Uplink-downlink Configuration (X) = 5 and threshold of NR uplink transmission = 30 (D1=30, D2=5),

- Z > 30,        PC3 EN-DC is applied

- 5 < Z ≤ 30, PC2 EN-DC is applied

- Z ≤ 5,         PC1.5 EN-DC is applied

Proposal 6: For PC1.5 UE supporting two band EN-DC with 2Tx,  consider

- PC2 E-UTRA TDD + PC2 NR TDD

Proposal 7: For PC2 E-UTRA TDD + PC2 NR TDD,  consider

- New duty cycle solution considering the possible E-UTRA TDD uplink-downlink configurations and the percentage of maximum E-UTRA/NR uplink transmission

For PC2 UE supporting two band EN-DC with 3Tx,

In Rel-18, the following combination of PC for E-UTRA band and PC for NR band is supported.

- Case 1 : PC3 E-UTRA FDD + PC3 NR TDD (UL-MIMO or Tx diversity)

- Case 2 : PC3 E-UTRA FDD + PC2 NR TDD (UL-MIMO or Tx diversity)

- Case 3 : PC3 E-UTRA TDD + PC3 NR TDD (UL-MIMO or Tx diversity)

- Case 4 : PC3 E-UTRA TDD + PC2 NR TDD (UL-MIMO or Tx diversity)

Whether to support the following combinations in Rel-19 need to be decided.

- Case 5 : PC3 E-UTRA FDD + PC3 NR FDD (UL-MIMO or Tx diversity)

- Case 6 : PC3 E-UTRA FDD + PC2 NR FDD (UL-MIMO or Tx diversity)

- Case 7 : PC2 E-UTRA TDD + PC3 NR TDD (UL-MIMO or Tx diversity)

For SAR compliance,

In Case 5 and Case 6, the existing E-UTRA FDD + NR TDD duty cycle solution can be reused with a new capability, for example, maxUplinkDutyCycle-interBandENDC-FDD-PC2 which comprises of maxUplinkDutyCycle-FDD-EN-DC1 and  maxUplinkDutyCycle-FDD-EN-DC2,  instead of maxUplinkDutyCycle-FDD-TDD-EN-DC1 and  maxUplinkDutyCycle-FDD-TDD-EN-DC2 which are applicable only for inter-band FDD+TDD EN-DC.

In Case 7 , the existing E-UTRA TDD + NR TDD duty cycle solution needs to be modified, because it is based on PC3 E-UTRA TDD. PC2 E-UTRA TDD based duty cycle solution is needed. Table 2.1 can be considered.

Proposal 8: For PC2 UE supporting two band EN-DC with 3Tx,  consider

- PC3 E-UTRA FDD + PC3/PC2 NR FDD (UL-MIMO or Tx diversity)

- PC2 E-UTRA TDD + PC3 NR TDD (UL-MIMO or Tx diversity)

Proposal 9: For PC3 E-UTRA FDD + PC3/PC2 NR FDD (UL-MIMO or Tx diversity),  consider

- a new capability, for example, maxUplinkDutyCycle-interBandENDC-FDD-PC2 which comprises of maxUplinkDutyCycle-FDD-EN-DC1 and maxUplinkDutyCycle-FDD-EN-DC2

Proposal 10: For PC2 E-UTRA TDD + PC3 NR TDD (UL-MIMO or Tx diversity),  consider

- New duty cycle solution considering the possible E-UTRA TDD uplink-downlink configurations and the percentage of maximum E-UTRA/NR uplink transmission

For PC1.5 UE supporting two band EN-DC with 3Tx,

In Rel-18, it is not supported.

Whether to support the following combinations in Rel-19 needs to be decided.

- Case 1 : PC3 E-UTRA FDD + PC1.5 NR TDD (UL-MIMO or Tx diversity)

- Case 2 : PC3 E-UTRA TDD + PC1.5 NR TDD (UL-MIMO or Tx diversity)

- Case 3 : PC2 E-UTRA TDD + PC1.5 NR TDD (UL-MIMO or Tx diversity)

- Case 4 : PC2 E-UTRA TDD + PC2 NR TDD (UL-MIMO or Tx diversity)

For SAR compliance,

In Case 1, the following formula can be assumed between the LTE UL duty cycle, NR UL duty cycle and the overall duty threshold.

-   DutyLTE x( PLTE/ P29) + DutyNR x(PNR/ P29)  ≤ Duty threshold

-   0.25xDutyLTE + DutyNR  ≤ Duty threshold

DutyLTE is from 0 to 100%.

DutyNR is from 0 to 25%.

PLTE, PNR, P29 represent the maximum linear power (mW) of LTE, NR, and EN-DC PC1.5 respectively; DutyLTE, DutyNR represent the maximum uplink percentage of LTE, NR respectively. Duty threshold represents the maximum UL duty cycle that can maintain the PC1.5 power class for FDD+TDD EN-DC.

The LTE reference configuration can be DutyLTE=70%. The default value of NR UL duty capability can be 7.5% i.e. "DutyLTE=70% and PLTE=23dBm", corresponding to default DutyNR=7.5%, PNR=29dBm.

For better flexibility, two LTE configurations are supported with LTE reference configuration of DutyLTE=70% and 40%.

The LTE reference configuration can be DutyLTE=40%. The default value of NR UL duty capability can be 15% i.e. "DutyLTE=40% and PLTE=23dBm", corresponding to default DutyNR=15%, PNR=29dBm.

It means that Duty threshold is scaled with 0.25.

In Case 2, the corresponding duty cycle solution needs to be specified.

Table 2.4 shows this example for the percentage of maximum E-UTRA/NR UL transmission. The possible E-UTRA TDD uplink-downlink configurations and the percentage need to be considered when specifying duty cycle solution.

Table 2.4: Percentage of maximum E-UTRA/NR uplink transmission in PC1.5 EN-DC (23dBm(E-UTRA TDD)+2x26dBm(NR TDD))

| E-UTRA TDD Uplink-downlink Configuration (X) | Percentage of maximum E-UTRA/NR uplink transmission during evaluation period (%) |  |  | Supported EN-DC power class (Z:actual NR % of uplink transmission, D(D1,D2) :threshold) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | E-UTRA (23dBm) | NR (29dBm) (Z=Z1 or Z2) |  |  |  |  |
|  | Y=0~100 | Z1=0~25, D1=(100-Y)/4 =25-Y/4 | Z2=0~25 D2=(50-Y)/4 =D1-12.5 | Z > D1 | D2<=Z<=D1 or D1>=Z | Z < D2 |
| 0 | 70 | 7.5 | N/A | N/A | N/A | N/A |
| 1 | 50 | 12.5 | N/A | N/A | N/A | N/A |
| 2 | 30 | 17.5 | 5 | PC3 | PC2 | N/A |
| 3 | 40 | 15 | 2.5 | PC3 | PC2 | N/A |
| 4 | 30 | 17.5 | 5 | PC3 | PC2 | N/A |
| 5 | 20 | 20 | 7.5 | PC3 | PC2 | PC1.5 |
| 6 | 60 | 10 | N/A | N/A | N/A | N/A |

For example,

When E-UTRA TDD Uplink-downlink Configuration (X) = 5 and threshold of NR uplink transmission = 20 (D1=20, D2=7.5),

- Z > 20,            PC3 EN-DC is applied

- 7.5 < Z ≤ 20, PC2 EN-DC is applied

- Z ≤ 7.5,         PC1.5 EN-DC is applied

In Case 3, the corresponding duty cycle solution needs to be specified.

Table 2.5 shows this example for the percentage of maximum E-UTRA/NR UL transmission. The possible E-UTRA TDD uplink-downlink configurations and the percentage need to be considered when specifying SAR solution.

Table 2.5: Percentage of maximum E-UTRA/NR uplink transmission in PC1.5 EN-DC (26dBm(E-UTRA TDD)+2x26dBm(NR TDD))

| E-UTRA TDD Uplink-downlink Configuration (X) | Percentage of maximum E-UTRA/NR uplink transmission during evaluation period (%) |  |  | Supported EN-DC power class (Z:actual NR % of uplink transmission, D(D1,D2) :threshold) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | E-UTRA (26dBm) | NR (29dBm) (Z=Z1 or Z2) |  |  |  |  |
|  | Y=0~50 | Z1=0~25, D1=(50-Y)/2 | Z2=0~25 D2=(25-Y)/2 =Z1-12.5 | Z > D1 | D2<=Z<=D1 or D1>=Z | Z < D2 |
| 0 | 70 | N/A | N/A | N/A | N/A | N/A |
| 1 | 50 | N/A | N/A | N/A | N/A | N/A |
| 2 | 30 | 10 | N/A | PC3 | PC2 | N/A |
| 3 | 40 | 5 | N/A | PC3 | PC2 | N/A |
| 4 | 30 | 10 | N/A | PC3 | PC2 | N/A |
| 5 | 20 | 15 | 2.5 | PC3 | PC2 | PC1.5 |
| 6 | 60 | N/A | N/A | N/A | N/A | N/A |

For example,

When E-UTRA TDD Uplink-downlink Configuration (X) = 5 and threshold of NR uplink transmission = 15 (D1=15, D2=2.5),

- Z > 15,           PC3 EN-DC is applied

- 2.5 < Z ≤ 15, PC2 EN-DC is applied

- Z ≤ 2.5,         PC1.5 EN-DC is applied

In Case 4, the corresponding duty cycle solution needs to be specified.

Table 2.6 shows this example for the percentage of maximum E-UTRA/NR UL transmission. The possible E-UTRA TDD uplink-downlink configurations and the percentage need to be considered when specifying duty cycle solution.

Table 2.6: Percentage of maximum E-UTRA/NR uplink transmission in PC1.5 EN-DC (26dBm(E-UTRA TDD)+2x23dBm(NR TDD))

| E-UTRA TDD Uplink-downlink Configuration (X) | Percentage of maximum E-UTRA/NR uplink transmission during evaluation period (%) |  |  | Supported EN-DC power class (Z:actual NR % of uplink transmission, D(D1,D2) :threshold) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | E-UTRA (26dBm) | NR (26dBm) (Z=Z1 or Z2) |  |  |  |  |
|  | Y=0~50 | Z1=0~50, D1=50-Y | Z2=0~50 D2=25-Y = D1-25 | Z > D1 | D2<=Z<=D1 or D1>=Z | Z < D2 |
| 0 | 70 | N/A | N/A | N/A | N/A | N/A |
| 1 | 50 | N/A | N/A | N/A | N/A | N/A |
| 2 | 30 | 20 | N/A | PC3 | PC2 | N/A |
| 3 | 40 | 10 | N/A | PC3 | PC2 | N/A |
| 4 | 30 | 20 | N/A | PC3 | PC2 | N/A |
| 5 | 20 | 30 | 5 | PC3 | PC2 | PC1.5 |
| 6 | 60 | N/A | N/A | N/A | N/A | N/A |

For example,

When E-UTRA TDD Uplink-downlink Configuration (X) = 5 and threshold of NR uplink transmission = 30 (D1=30, D2=5),

- Z > 30,        PC3 EN-DC is applied

- 5 < Z ≤ 30, PC2 EN-DC is applied

- Z ≤ 5,         PC1.5 EN-DC is applied

Proposal 11: For PC1.5 UE supporting two band EN-DC with 3Tx,  consider

- PC3 E-UTRA FDD/TDD + PC1.5 NR TDD (UL-MIMO or Tx diversity)

- PC2 E-UTRA TDD + PC2/PC1.5 NR TDD (UL-MIMO or Tx diversity)

Proposal 12: For PC3 E-UTRA FDD/TDD + PC1.5 NR TDD (UL-MIMO or Tx diversity), consider

- New duty cycle solution considering the possible E-UTRA TDD uplink-downlink configurations and the percentage of maximum E-UTRA/NR uplink transmission

Proposal 13: For PC2 E-UTRA TDD + PC2/PC1.5 NR TDD (UL-MIMO or Tx diversity), consider

- New duty cycle solution considering the possible E-UTRA TDD uplink-downlink configurations and the percentage of maximum E-UTRA/NR uplink transmission

Conclusion

In this contribution, we proposed our view on HPUE for inter-band CA and EN-DC. The proposals are as follows.

For PC1.5 UE supporting NR inter-band UL CA with 2Tx,

Proposal 1: Modify the existing PC2 duty cycle solution if PC2 FDD/TDD + PC2 TDD with 2Tx is introduced.

For PC1.5 UE supporting NR inter-band UL CA with 3Tx,

Proposal 2: For PC1.5 UE supporting NR inter-band UL CA with 3Tx,  consider

- PC2 NR FDD + PC2/PC1.5 NR TDD (UL-MIMO or Tx Diversity)

For PC2 UE supporting two band EN-DC with 2Tx,

Proposal 3: For PC2 UE supporting two band EN-DC with 2Tx,  consider

- PC3 E-UTRA FDD + PC3/PC2 NR FDD

- PC2 E-UTRA TDD + PC3 NR TDD

Proposal 4: For PC3 E-UTRA FDD + PC3/PC2 NR FDD,  consider

- a new capability, for example, maxUplinkDutyCycle-interBandENDC-FDD-PC2 which comprises of maxUplinkDutyCycle-FDD-EN-DC1 and maxUplinkDutyCycle-FDD-EN-DC2

Proposal 5: For PC2 E-UTRA TDD + PC3 NR TDD,  consider

- New duty cycle solution considering the possible E-UTRA TDD uplink-downlink configurations and the percentage of maximum E-UTRA/NR uplink transmission

For PC1.5 UE supporting two band EN-DC with 2Tx,

Proposal 6: For PC1.5 UE supporting two band EN-DC with 2Tx,  consider

- PC2 E-UTRA TDD + PC2 NR TDD

Proposal 7: For PC2 E-UTRA TDD + PC2 NR TDD,  considere

- New duty cycle solution considering the possible E-UTRA TDD uplink-downlink configurations and the percentage of maximum E-UTRA/NR uplink transmission

For PC2 UE supporting two band EN-DC with 3Tx,

Proposal 8: For PC2 UE supporting two band EN-DC with 3Tx,  consider

- PC3 E-UTRA FDD + PC3/PC2 NR FDD (UL-MIMO or Tx diversity)

- PC2 E-UTRA TDD + PC3 NR TDD (UL-MIMO or Tx diversity)

Proposal 9: For PC3 E-UTRA FDD + PC3/PC2 NR FDD (UL-MIMO or Tx diversity),  consider

- a new capability, for example, maxUplinkDutyCycle-interBandENDC-FDD-PC2 which comprises of maxUplinkDutyCycle-FDD-EN-DC1 and maxUplinkDutyCycle-FDD-EN-DC2

Proposal 10: For PC2 E-UTRA TDD + PC3 NR TDD (UL-MIMO or Tx diversity),  consider

- New duty cycle solution considering the possible E-UTRA TDD uplink-downlink configurations and the percentage of maximum E-UTRA/NR uplink transmission

For PC1.5 UE supporting two band EN-DC with 3Tx,

Proposal 11: For PC1.5 UE supporting two band EN-DC with 3Tx,  consider

- PC3 E-UTRA FDD/TDD + PC1.5 NR TDD (UL-MIMO or Tx diversity)

- PC2 E-UTRA TDD + PC2/PC1.5 NR TDD (UL-MIMO or Tx diversity)

Proposal 12: For PC3 E-UTRA FDD/TDD + PC1.5 NR TDD (UL-MIMO or Tx diversity), consider

- New duty cycle solution considering the possible E-UTRA TDD uplink-downlink configurations and the percentage of maximum E-UTRA/NR uplink transmission

Proposal 13: For PC2 E-UTRA TDD + PC2/PC1.5 NR TDD (UL-MIMO or Tx diversity), consider

- New duty cycle solution considering the possible E-UTRA TDD uplink-downlink configurations and the percentage of maximum E-UTRA/NR uplink transmission

- References

[1] R4-2410750, “WF on higher power UE”, Samsung

[2] RP-240828, “New WID : UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, Huawei, ATT