3GPP TSG-RAN4 Meeting #112	R4-2411870

# Maastricht, Netherlands, 19th – 23rd August, 2024

Source:	LG Electronics

Title:	HPUE for increasing high power limit

Agenda item:	8.1.1.1.3

Document for:	Discussion

Introduction

In this paper, we provide our views on HPUE for increasing high power limit.

Discussion

In last RAN4 meeting, the following WF was approved [1]. This paper discusses the requirement of HPUE for increasing high power limit.

- 4.1 The methodology Online agreement: Use the higherPowerLimit-R17 and higherPowerLimtMRDC-R17 capability as the starting point. - 4.2 Whether increasing power limit applicability requires ΔPPowerClass,CA /ΔPPowerClass,EN-DC =0 Adhoc agreement: The high-power limit feature (higherPowerLimit-R17/higherPowerLimtMRDC-R17) only applies with ΔPPowerClass,CA /ΔPPowerClass,EN-DC = 0 dB, which is aligned with the Rel-17 agreement - 4.3 Scenarios to be considered in Rel-19 Way forward: FFS - 4.4 MSD and SAR compliance Way forward: - If higherPowerLimit-R17/higherPowerLimtMRDC-R17 capability is adopted for increasing UE transmission power, then taking same methodology as for higherPowerLimit-R17/ higherPowerLimtMRDC-R17 for SAR compliance and MSD. More specific, - For a given band combination, no additional MSD requirements are specified in addition to the MSD requirements of its legacy power class(es) - No new duty-cycle solution/mechanism for SAR compliance is considered - P-MPR can be used for SAR mitigation

Scenarios to be considered in Rel-19

For PC1.5 UE supporting NR inter-band UL CA with 2Tx,

The possible combination of PC for NR bands is as follows.

- Case 1 : PC2 NR FDD + PC2 NR TDD

- Case 2 : PC2 NR TDD + PC2 NR TDD

The increasing high power limit does not apply to both cases.

For PC1.5 UE supporting NR inter-band UL CA with 3Tx,

In Rel-18, the following combination of PC for NR bands is supported.

- Case 1 : PC3 NR FDD + PC1.5 NR TDD (UL-MIMO or Tx Diversity)

Whether to support the following combinations in Rel-19 need to be decided.

- Case 2 : PC2 NR FDD + PC1.5 NR TDD (UL-MIMO or Tx Diversity)

- Case 3 : PC2 NR FDD + PC2 NR TDD (UL-MIMO or Tx Diversity)

- Case 4 : PC3 NR TDD + PC1.5 NR TDD (UL-MIMO or Tx Diversity)

- Case 5 : PC2 NR TDD + PC1.5 NR TDD (UL-MIMO or Tx Diversity)

- Case 6 : PC2 NR TDD + PC2 NR TDD (UL-MIMO or Tx Diversity)

Among the cases, the total maximum output power can be higher than 29dBm in case 1, case 2, case 4 and case 5. The difference from 29dBm is 0.97dB for case 1 and case 4 and 1.76dB for case 2 and case 5.

- Case 1 & Case 4 : 23dBm + 29dBm = 29.97 dBm

- Case 2 & Case 5 : 26dBm + 29dBm  = 30.76 dBm

With capability of higher power limit, PPower class, CA is replaced with 29.97dBm for case 1 and case 4 and 30.76dBm for case 2 and case 5.

For PC2 UE supporting two band EN-DC with 2Tx,

In Rel-18, the following combination of PC for E-UTRA band and PC for NR band is supported.

- Case 1 : PC3 E-UTRA FDD + PC3 NR TDD

- Case 2 : PC3 E-UTRA FDD + PC2 NR TDD

- Case 3 : PC3 E-UTRA TDD + PC3 NR TDD

- Case 4 : PC3 E-UTRA TDD + PC2 NR TDD

Whether to support the following combinations in Rel-19 need to be decided.

- Case 5 : PC3 E-UTRA FDD + PC3 NR FDD

- Case 6 : PC3 E-UTRA FDD + PC2 NR FDD

- Case 7 : PC2 E-UTRA TDD + PC3 NR TDD

Among the cases, the total maximum output power can be higher than 26dBm in case 2, case 4, case 6 and case 7. The difference from 26dBm is 1.76dB for the cases.

- Case 2 & Case 4 & Case6 & Case 7 : 23dBm + 26dBm = 27.76 dBm

With capability of higher power limit, PPower class, EN-DC is replaced with 27.76dBm for the cases.

The capability of higher power limit needs to be differentiated from Rel-17 capability.

For PC1.5 UE supporting two band EN-DC with 2Tx,

Whether to support the following combination in Rel-19 need to be decided.

- Case 1 : PC2 E-UTRA TDD + PC2 NR TDD

The increasing high power limit does not apply to the case.

For PC2 UE supporting two band EN-DC with 3Tx,

In Rel-18, the following combination of PC for E-UTRA band and PC for NR band is supported.

- Case 1 : PC3 E-UTRA FDD + PC3 NR TDD (UL-MIMO or Tx diversity)

- Case 2 : PC3 E-UTRA FDD + PC2 NR TDD (UL-MIMO or Tx diversity)

- Case 3 : PC3 E-UTRA TDD + PC3 NR TDD (UL-MIMO or Tx diversity)

- Case 4 : PC3 E-UTRA TDD + PC2 NR TDD (UL-MIMO or Tx diversity)

Whether to support the following combinations in Rel-19 need to be decided.

- Case 5 : PC3 E-UTRA FDD + PC3 NR FDD (UL-MIMO or Tx diversity)

- Case 6 : PC3 E-UTRA FDD + PC2 NR FDD (UL-MIMO or Tx diversity)

- Case 7 : PC2 E-UTRA TDD + PC3 NR TDD (UL-MIMO or Tx diversity)

Among the cases, the total maximum output power can be higher than 26dBm in case 2, case 4, case 6 and case 7. The difference from 26dBm is 1.76dB for the cases.

- Case 2 & Case 4 & Case6 & Case 7 : 23dBm + 26dBm = 27.76 dBm

With capability of higher power limit, PPower class, EN-DC is replaced with 27.76dBm for the cases.

The capability of higher power limit needs to be differentiated from Rel-17 capability.

For PC1.5 UE supporting two band EN-DC with 3Tx,

Whether to support the following combinations in Rel-19 need to be decided.

- Case 1 : PC3 E-UTRA FDD + PC1.5 NR TDD (UL-MIMO or Tx diversity)

- Case 2 : PC3 E-UTRA TDD + PC1.5 NR TDD (UL-MIMO or Tx diversity)

- Case 3 : PC2 E-UTRA TDD + PC1.5 NR TDD (UL-MIMO or Tx diversity)

- Case 4 : PC2 E-UTRA TDD + PC2 NR TDD (UL-MIMO or Tx diversity)

Among the cases, the total maximum output power can be higher than 29dBm in case 1, case 2, and case 3. The difference from 29dBm is 0.97dB for case 1 and case 2 and 1.76dB for case 3.

- Case 1 & Case 2 : 23dBm + 29dBm = 29.97 dBm

- Case 3 :                 26dBm + 29dBm  = 30.76 dBm

With capability of higher power limit, PPower class, EN-DC is replaced with 29.97dBm for case 1 and case 2 and 30.76dBm for case 3.

The capability of higher power limit needs to be differentiated from Rel-17 capability.

Table 2-1 shows a list of applicable inter-band CA scenarios for increasing UE transmit power limit.

Table 2-1: A list of applicable inter-band CA scenarios for increasing UE transmission power limit

| CA power class | Power class configuration Band A + Band B |  | 2Tx | 3Tx |
| --- | --- | --- | --- | --- |
| PC2 | PC2 TDD | PC3 TDD | R18 | R18 |
|  |  | PC3 FDD | R18 | R18 |
|  | PC2 FDD | PC3 TDD | R19 ? | R19 ? |
|  |  | PC3 FDD | R19 ? | R19 ? |
| PC1.5 | PC1.5 TDD | PC3 TDD | N/A | R19 ? |
|  |  | PC3 FDD | N/A | R18 |
|  |  | PC2 TDD | N/A | R19 ? |
|  |  | PC2 FDD | N/A | R19 ? |

Table 2-2 shows a list of applicable inter-band EN-DC scenarios for increasing UE transmit power limit.

Table 2-2: A list of applicable inter-band CA scenarios for increasing UE transmission power limit

| CA power class | Power class configuration Band A(E-UTRA) + Band B(NR) |  | 2Tx | 3Tx |
| --- | --- | --- | --- | --- |
| PC2 | PC3 TDD | PC2 TDD | R18 | R18 |
|  | PC3 FDD | PC2 TDD | R18 | R18 |
|  |  | PC2 FDD | R19 ? | R19 ? |
|  | PC2 TDD | PC3 TDD | R19 ? | R19 ? |
| PC1.5 | PC3 TDD | PC1.5 TDD | N/A | R19 ? |
|  | PC3 FDD |  | N/A | R19 ? |
|  | PC2 TDD |  | N/A | R19 ? |
|  | PC2 FDD |  | N/A | R19 ? |

Based on the scenarios,

Proposal 1: Decide which inter-band CA combinations are applicable in Rel-19 from lists in Table 2-1.

Proposal 2: Decide which inter-band EN-DC combinations are applicable in Rel-19 from lists in Table 2-2.

In the increasing high power limit, for SAR compliance, the duty cycle needs to be reduced by the boosted delta power. The capability of higher power limit needs to be differentiated from Rel-17 capability.

The capability of higher power limit needs to be defined, for example, with ’higherPowerLimit-R19/higherPowerLimtMRDC-R19’ to differentiate from Rel-17 capability.

Proposal 3: Define Rel-19 capability of higher power limit for inter-band CA and inter-band EN-DC.

Proposal 4: Consider new duty cycle solution if Rel-19 capability of higher power limit is adopted.

Conclusion

In this contribution, we proposed our view on HPUE for increasing high power limit. The proposals are as follows.

Proposal 1: Decide which inter-band CA combinations are applicable in Rel-19 from lists in Table 2-1.

Proposal 2: Decide which inter-band EN-DC combinations are applicable in Rel-19 from lists in Table 2-2.

Proposal 3: Define Rel-19 capability of higher power limit for inter-band CA and inter-band EN-DC.

Proposal 4: Consider new duty cycle solution if Rel-19 capability of higher power limit is adopted.

- References

[1] R4-2410750, “WF on higher power UE”, Samsung

[2] RP-240828, “New WID : UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, Huawei, ATT