TSG-RAN Working Group 4 (Radio) meeting #112	R4-2411872

Maastricht, EU, 19th to 23rd Augusty 2024

Source:	Ericsson

Title:	On general aspects related to BS RF evolution WI

Agenda item:	8.11.1

Document for:	Approval

# Introduction

At the last RAN4 meeting (RAN4#111 in Fukuoka) the work part of Rel-19 BS RF evolution work item NR_BS_RF_req_evo-Core progressed according to the workplan agreed in [1]. The WI consists of multiple work packages with different objectives and time schedules. In addition to running activities the discussion related to BS conformance aspects starts at this meeting.

With multiple running activities with different objectives touching on different sections both in BS RF core specification and BS RF conformance specifications as well as technical reports with different time schedules it is essential to carefully plan the activities for coming meetings. A set of draft CRs for TS 38.104, TS 38.141-2, TR 37.941 and TR 38.xyz (New external technical report dedicated for technical background information for the new requirement required to protect FSS UL applicable for band n104) will be needed to collect input information to all relevant technical specifications and technical reports.

Since the finalization dates for FSS UL protection for band n104 is different than the finalization date of Rel-19. There will be a need for at least two separate drops of updates to TS 38.104 and TS 38.141-2.

In this contribution this contribution we provide an overview of the work and some proposals to further simulate progress.

# Discussion

This work item was created based on multiple work packages with different time schedules. The work packages will touch on both BS RF core specification (TS 38.104) and BS Conformance test specification (TS 38.141-2). From the workplan the following work items can be identified, as listed in Table 2-1.

Table 2-1: Work item objectives

| Work package | Description |
| --- | --- |
| EEIRP | Introduction of protection of FSS UL for band n104 in TS 38.104 and TS 38.141-2. |
| COLOC | Study of co-location requirement concept improvement. |
| TSR | Test Scope Reduction for co-location requirements. |
| TRP | TRP test method improvements. |
| SPEC | BS specification structure improvement. A check point is defined in December to check if time is available. |

The body of the work plan can be found in [1]. Together with some further refinements the workplan is visualised in Table 2-2.

For BS co-location requirement two work packages have been identified (COLOC and TSR). It may be worth to consider consolidating those packages into one common work package for BS co-location requirement improvements or evolution. It would be beneficial for RAN4 to study how co-location requirements can be evolved to support higher frequencies and larger antenna structures to enable BS type 1-O products on the market. Rather than putting effort into fixing test scope for Rel-15 concept, put more time into looking into spectrum in the upper part of FR1 and future spectrum above FR1.

Table 2-2: Work plan

| Meeting | BS RF Core | BS Conformance |
| --- | --- | --- |
| RAN4#110-bis (April 2024) | EEIRP COLOC TR 38.xyz skeleton for EEIRP |  |
| RAN4#111 (May 2024) | EEIRP COLOC Endorse TR 38.xyz skeleton for EEIRP |  |
| RAN#104 (June 2024) | WID Update |  |
| RAN4#112 (August 2024) | COLOC | EEIRP TSR TRP |
| RAN#105 (September 2024) | WID Update |  |
| RAN4#112-bis (October 2024) | COLOC | EEIRP TSR TRP |
| RAN4#113 (November 2024) | COLOC Agree CR for EEIRP to TS 38.104 Agree CR for TR 38.xyz | Agree CR for EEIRP to TS 38.141-2 TSR TRP |
| RAN#106 (December 2024) | WID Update |  |
| RAN4#114 (February 2025) | COLOC SPEC | TSR TRP SPEC |
| RAN#107 (March 2025) |  |  |
| RAN4#114bis (April 2025) | COLOC SPEC | TSR TRP SPEC |
| RAN4#115 (May 2025) | COLOC SPEC | TSR TRP SPEC |
| RAN#108 (June 2025) |  |  |
| RAN4#116 (August 2025) | Finalize COLOC SPEC | TSR TRP SPEC |
| RAN#109 (September 2025) |  |  |
| RAN4#116-bis (October 2025) | SPEC | COLOC TSR TRP SPEC |
| RAN4#117 (November 2025) | Agree CR for SPEC to TS 38.104 | Agree CR for COLOC to TS 37.941 Agree CR for TSR to TS 38.141-2 Agree CR for TRP to TS 38.141-2 Agree CR for SPEC to TS 38.141-2 |
| RAN#110 (December 2025) |  |  |

For work related to EEIRP the corresponding input to the discussion is captured in a companion contribution [3].

At the RAN plenary meeting in December 2024 (RAN#106) there is a checkpoint to consider initiating specification structure improvements in the case time is available. According to the plan the EEIRP work package is supposed to end in end of 2024.

In the following sections information to TSR (in section 2.1), TRP (in section 2.2) and SPEC (in section 2.3) will be presented.

2.1		Co-location requirement overview

The co-location requirements are defined based on a concept of a co-location reference antenna mimicking a specific co-location scenario. In a contribution presented at a previous meeting [5], some technical issues and test challenges were identified with relation to the current requirement concept.

The technical background and challenges related to using current test specification and adding support for upper parts of FR1 (e.g., band n79 and n104) and above 7125 MHz can be summarized as:

Requirement issues:

- Co-location scenario: Before adopting the co-location concept for more bands above 2.5 GHz, RAN4 need to re-evaluate the BS-to-BS co-location scenario used as foundation for the co-location requirements defined in Rel-15 for BS type 1-O. There are many relevant co-location scenarios, such as side-by-side (used as baseline for Rel-15), on top of each other, in different sites pointing towards each other (used as baseline before Rel-15), or any other relevant scenario. The scenario considered for Rel-15 may not be representative for many deployed networks now (end of Rel-18). The antenna arrays used at higher frequencies are typically electrically larger than what was assumed for Rel-15.

- Co-location type: The Rel-15 co-location scenario consider an AAS co-located with a non-AAS with a passive single column antenna. With a single column antenna, the array factor in the horizontal domain is not captured. During the introduction of AAS, AAS co-located with non-AAS was a reasonable scenario to consider, but now with new bands above 2.5 GHz also scenarios with AAS to AAS may be highly relevant. Furthermore, for AAS, a statistical approach would be needed to capture relevant dynamic coupling effects due to beam forming.

- Network type: The Rel-15 co-location scenario is focused on wide area deployment. Its relevance for other BS classes like medium range BS and local area BS, etc. is not clear. Eventually, new co-location antennas or scenario may be needed for those BS classes to make specifications complete.

- Isolation assumption: All requirement levels are derived based on an assumed port-to-port isolation of 30 dB. When 30 dB was assumed, the isolation between a single column antenna and a AAS with few branches was considered. Now RAN4 consider higher frequencies and much larger array structures. The isolation value to assume above 2.5 GHz with more than 32 transceiver branches and considering array excitations should be further studied. To find a relevant isolation a statistical analysis including beamforming aspects on both victim and aggressor would be required. As coupling given same physical separation would be frequency dependent, the requirement levels need to be adjusted accordingly.

- Emission relaxation: To maintain degradation level used for non-AAS BS, RAN4 should reconsider the AAS BS co-location spurious emission and not include the 9 dB relaxation part of requirement derivation background. The current requirement levels for AAS BS may not guarantee the same level of degradation as for non-AAS BS.

Conformance test issues:

- Logistical issues in laboratory: A band specific Co-location Test Antenna (CLTA) is required per declared supported co-location band. This will create a logistical problem during conformance testing since a product is typically declared to support many co-location situations. This will result in that many CLTAs are needed per tested product, which creates a logistical challenge for test laboratories. The CLTA is also based on a commercial antenna selected based on specified guidance. This means that different labs can use different types of CLTA for the same test.

- CLTA availability: For bands defined above 2.5 GHz it is very difficult to find commercially available single column BS antennas or multi-columns with similar characteristics as specified in TS 38.141-2, subclause 4.12. Therefore, the availability and access to relevant CLTAs are very limited for bands above 2.5 GHz.

- Complex test setup: By experience, it can be concluded that the test setup for transmitter intermodulation is very complex. A large PA capable of very high output power is required to generate the interfering signal clean enough to be injected by the CLTA, while TRP unwanted emissions is measured and evaluated. The TRP level for unwanted emission can be measured in a far-field test environment or in a reverberation chamber test environment, as described in TS 38.141-2.

- Test chamber restrictions: The CLTA puts additional burden on the OTA test environments, such as volume restrictions and additional weight on the positioner used in the test chamber. It is therefore not suitable to use a passive BS antenna with multiple columns, even though current specification allows it. In a reverberation chamber the high output power, implicitly a consequence of mounting the CLTA side-by-side, is a major concern.

- Large test scope: Due to the very complex test setup required to test transmitter intermodulation (TX IMD), careful review of the test scope is required to minimize total test time. The test is based on measuring TRP at the same time an interferer signal is injected using the CLTA. Further evaluation of the test scope in TS 38.141-2, subclause 6.8.4 to capture only relevant cases of e.g., carrier, bandwidth, multi-band and carrier aggregation configurations.

- Test equipment considerations: It should be noted that many co-location requirements either require very high OTA chamber dynamic range (co-location spurious emission) or require generation of very high power in the chamber as CLTA and existing co-location concept would imply blocking interferer level 30 dB higher than existing conducted requirements.

For upper FR1 and the frequency range between 7 to 24 GHz, the co-location requirement concept needs to be evolved. It would be suitable to define requirement such that the need for a specified dedicated co-location reference antenna can be avoided. For such approach RAN4 need to establish relevant parameter assumptions required to derive requirement levels based on OTA parameters such as field-strength or power density at a given test points. In [5], some basic technical background for such approach was presented. Further elaborations and studies may be needed to fully define an evolved requirement concept for co-location requirements.

To progress the work related to improving specification, the work can be divided into two categories:

- An improvement co-location requirement concept (COLOC): With this work the goal is to resolve all issues listed above and define a requirement concept that can be adopted above 2.5 GHz.

- Test scope reduction (TSR): The test scope reduction aims to find a test coverage large enough to capture the intention with the requirement in a time efficient manner. This work would be relevant for the current Rel-15 requirement concept based on the defined co-location reference antenna concept. The test scope reduction work may be relevant for NR BS type 1-O base stations operating below 2.5 GHz. But it is not a viable long-term solution for frequencies above 2.5 GHz.

Proposal 1: In the next WID revision refine objectives for BS co-location requirements to focus on large AAS base station operating at frequencies in the upper part and above FR1.

2.2		TRP test method improvement

For NR BS type 1-O and NR BS type 2-O, Total Radiated Power (TRP) is used as figure of merit for BS output power and unwanted emission requirements. TRP can be measured using either Reverberation Chamber (RC) or far-field based test environments in Anechoic Chamber (AC) by averaging over spatially discrete EIRP samples. Currently, the specification describes a collection of different methods to be used under different circumstances as alternative methods.

It has been noticed that the different EIRP averaging methods described as different alternatives produce very different measurement results, which is not acceptable for characterises related to licence condition and regulatory requirements such as output power and unwanted emission requirements. Also, the description on what method apply for a specific situation is not very clear. 

The technical background for measuring BS output power, ACLR and spurious emission is very different. It is proposed to improve the description of test method alternatives in TS 38.141-2, Annex I with respect to:

- General test method description improvement and applicability overview improvement: Describe the situations when a specific test method can be used, e.g., for a certain requirement specifically what methods that may apply and what additional information (antenna geometry, physical size of the antenna, etc.) that may be needed to be declared by the BS manufacturer. Add missing declarations to manufacturers declarations in TS 38.141-2, subclause 4.6. There is a great potential for improvements of Annex I to increase readability.

- Reducing overlapping methods: Some test method alternatives are overlapping or describes a non-relevant special case, e.g., ULA for AAS BS is not very relevant. Overlapping and non-relevant cases can be removed in TS 38.141-2, Annex I.

- Pre-scan description improvement: Improve the description of the pre-scan procedure essential for spurious emission measurements. Depending on emission type (harmonic, image, LO-leakage, etc.) the beamforming will be ranging from full antenna gain to sub-array gain. The pre-scan procedure that is a quick search method in the frequency domain to identify emission, can be improved by adding more information related to special frequencies where high beam forming is expected.

- Enhanced measurement uncertainty evaluation: Improve Measurement Uncertainty (MU) evaluation for TRP methods in anechoic camber. Currently, a common MU evaluation is adopted for all alternative test approaches in TS 38.141-2, Annex I. It is not clear if measurement uncertainty contributions assumed by the common MU evaluation approach is applicable for all test methods.

With better guidance and a reduced set of alternatives the description in TS 38.141-2, Annex I can be significantly improved resolving issues raised by regulatory bodies and test laboratories.

In current WID [6] the following statement is captured:

Investigate and if possible, simplify BS TRP test methods for FR1 and FR2 by improving the applicability and not removing any of the existing TRP test methods.

With the above listed challenges, it seems that the WID needs to be revised.

Proposal 2: In the next WID revision refine objective to allow for improvements of TRP test method description, including aspects such as reducing test method overlap, pre-scan description improvement, enhancement of measurement uncertainty evaluation.

2.3		Specification structure improvement

Since the introduction of NR in Rel-15 the number of network nodes have increased with the introduction of new type e.g., Repeater, IAB and NCR. The majority of the requirements for the new network nodes was originally derived for the BS and re-used for other nodes.

This means that now when a new operation band is defined, the number of required CRs to include the band for all specifications (BS RF core specification and conformance test specification), all nodes using the same requirement baseline (BS, Repeaters, IAB, NCR) have increased enormously.

With the current specification structure, it can be noticed that many requirements are defined per band. For some requirements this would result in very large tables with a dedicated row per band (see additional spurious emission requirements in subclause 6.6.5.2.3 and co-location with other base stations in sub-clause 6.6.5.2.4).

The work related to introduction of a new band and regular maintenance work can be significantly reduced by adopting a different approach for coexistence spurious emission requirements, co-location requirements and other situations where requirement levels are repeated.

If requirement levels are repeated for many bands, it is better to define a baseline requirement that applies for all bands and list all deviations in a separate list for bands where different requirement levels may apply. For the future it would be worth to consider this approach for both coexistence spurious emission and co-location spurious emission requirements.

To resolve issue with duplication of requirements it is proposed to make the BS RF specifications (TS 38.104, TS 38.141-1 and TS 38.141-2) the main source for network node requirements. This would mean that in other node specifications where BS requirements is used (e.g., for IAB TS 38.175, TS 38.176-1 and TS 38.176-2, etc.) a reference is added to the BS RF specification and the requirement tables removed to avoid having duplicate information in multiple specifications. This approach works in the case the same requirement level is adopted. If for any reason the requirement levels are different this approach cannot be adopted, hence separate tables will be required. Adopting this approach would decrease the number of CRs needed to introduce new bands and handle general maintenance.

The approach using references will break the concept of having self-contained specifications in RAN4 for IAB and NCR and repeaters. But it seems like reasonable approach to appoint the BS RF specifications as self-contained core specifications for network nods since the BS is the central core network node and other nodes is used as a complement to improve network flexibility, coverage and/or performance.

The work on other network node specifications can be split up in two parts:

- Define baseline requirements for co-existence spurious emission and co-location requirements to reduce size of tables in BS RF specifications.

- Remove requirement duplicates in other network node specifications and add reference to BS specifications.

There will also be regular maintenance based on feedback from regulatory forum such as ECC and others. The work needs to be coordinated since similar parts of the specifications will be impacted.

The work in this work package will be to very high degree focusing on editorial changes spanning over many specifications and therefore the need for a separate work item should be further discussed.

Proposal 3: At the first RAN plenary after EEIRP finalization update WID to add objective for BS specification improvement.

# Conclusion

The work related to BS RF evolution progress well with focus on EEIRP work package. In this contribution we would like to remind RAN4 on all other objective also included in the work item. According to the workplan work related to BS co-location requirement improvements (COLOC and TSR) and TRP emission test method improvements are scheduled to ramp up as the work related to EEIRP is expected to ramp down end of 2024.

In this contribution some essential background for the work related to COLOC, TRP and SPEC is collected as reference for future work.

In this contribution we propose following:

Proposal 1: In the next WID revision refine objectives for BS co-location requirements to focus on large AAS base station operating at frequencies in the upper part and above FR1.

Proposal 2: In the next WID revision refine objective to allow for improvements of TRP test method description, including aspects such as reducing test method overlap, pre-scan description improvement, enhancement of measurement uncertainty evaluation.

Proposal 3: At the first RAN plenary after EEIRP finalization update WID to add objective for BS specification improvement.

# References

[1]	R4-2405624, “Work plan for Rel-19 BS RF evolution”, ZTE

[2] 	R4-2409955, “Way Forward for [111][311] NR_BS_RF”, ZTE

[3]	R4-2413219, “Introduction of OTA spatial emission above the horizon requirement for BS operating in band n104”, Ericsson

[4]	R4-2402475, “NR BS RF enhancements in Rel-19”, Ericsson

[5]	R4-2412912, “On the topic of BS RF evolution related to co-location requirements”, Ericsson

[6]	RP-240829, “New WID: NR base station (BS) RF requirement evolution for FR1/FR2 and testing”, ZTE