TSG-RAN Working Group 4 (Radio) meeting #112	R4-2411873

Maastricht, EU, 19th to 23rd Augusty 2024

Source:	Ericsson

Title:	On the topic of additional information requested by ITU-R WP 5D

Agenda item:	8.2.5

Document for:	Approval

# Introduction

At the last RAN4 meeting (RAN4#111 in Fukuoka) parameter for relevant for future IMT systems was extensively discussed. The first of three scheduled response LS to ITU-R WP 5D was approved in [1]. This included a workplan for sending information to ITU-R WP 5D and parameters relevant for the frequency range 4400 to 4800 MHz.

Progress and details related to the other frequency ranges 7125 to 8400 MHz and 14800 to 15350 MHz can be found in a way-forward contribution [2].

Technical background information will be captured in a technical report (TR 38.922). A combined text proposal collected all input from last meeting in [3].

In addition to parameter assumptions ITU-R WP 5D have also requested information related to some additional topics. In this contribution we will provide information related to modelling of antenna gain outside the wanted carrier and simulation results considering impact due to considered beamforming scheme assumption.

# Discussion

In the LS received by RAN4, ITU-R WP 5D [5] is asking for information on specific questions:

- The particular type of AAS implementation and associated model for IMT base stations, i.e., AAS composed of an array-of-sub-arrays or AAS composed of single-element arrays with its technical justification.

- If providing specific technical AAS implementation parameters for the array-of-sub-arrays case, WP 5D would welcome:

- The pre-set sub array tilt is understood as a parameter (a pre-tilt within the elements combined to form a sub array) that is used to focus the main beam below the boresight where a base station is intended to serve its UEs, similarly to the mechanical down-tilt. Noticing that the mechanical down-tilt of the BS antenna is environment (urban/suburban/rural) dependent, WP 5D welcomes further confirmation whether the preset sub-array down-tilt varies with the environment or not, what would be the range of values for this parameter and the associated environments and if multiple set of configurable preset sub-array down-tilt per BS is foreseen in the future.

- Information of advancements in MIMO architectures that could be applicable for future systems.

- Whether AAS at IMT user equipment is likely or not. If AAS at IMT user equipment is considered as a possibility WP 5D welcomes information relating to the structure of the AAS at the IMT user equipment, associated frequency bands, and the associated parameters of the AAS.

- IMT base station (inclusive of its AAS-related parameters) and IMT user equipment technical parameters for analysing adjacent band compatibility of terrestrial IMT systems with other co-primary services in the ITU Radio Regulations.

WP 5D also seeks information about how the AAS performs in adjacent bands i.e., does its performance drops down to that of a single element or still some array features remain. If the latter is true how far in frequency should one assume the continuity of array features even with some degradation?

- Considering the system-level simulation work typically conducted in WP 5D for sharing and compatibility studies using 3D modelling of antenna pattern, WP 5D is seeking information on whether for implementation of IMT BS multiuser spatial beamforming techniques, such as zero-forcing (ZF) or minimum mean-square error (MMSE) based schemes, would be necessary to improve the assessment of interference and the accuracy of studies. If considered necessary, guidance on the process of deriving the necessary beamforming weights for the IMT AAS BS to compute its radiation pattern is welcomed.

Answers to questions a, b and c will be provided per frequency range. In this contribution we will provide technical input relevant for developing answers to question d and e.

  - Modelling of out-of-carrier array antenna gain

The reason for the question is the need to properly model interference from AAS base station to system operating in adjacent spectrum situations. Adjacent sharing situations will be highly relevant for all three frequency regions (4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz) identified ITU-R WP 5D.

As indicated in previous contribution [4] the array antenna model defined in 3GPP can be used to model the array gain for situations where the full array factor is not expected, like moving away outside the carrier. One of the challenges moving forward is to define a model for the correlation factor with parameters such as frequency offset and carrier bandwidth as input parameters. Another challenge will be to define relevant parameter values to be shared with ITU-R WP 5D that can be used for sharing studies.

The array antenna model adopted by 3GPP in TR 38.803 creates the composite pattern as a multiplication between the element factor (or sub-array element factor) and array factor. The array antenna model was first introduced in TR 37.840 in the early days of AAS for BS. Based on the antenna model, the average radiation pattern produced by an array antenna was evaluated and an analytical expression for the average radiation pattern was established.

The composite average radiation gain pattern can be expressed in logarithmical scale as:

AAθ,φ=Asubθ,φ+10log101+ρm=1Mn=1Nwm,nvm,n2-1

, where r is the correlation factor defined in the interval 0≤ρ≤1 and other parameters are defined in TR 38.803. For r equal to 0 only the element or sub-array produce gain (peak gain equal to GE,max + 10log10(Msub) dBi), while for r equal to 1 the composite array gain is produced (peak gain equal to GE,max + 10log10(Msub) + 10log10(MN) dBi).

Observation 1: Support for modelling array antenna gain adjacent carrier have been documented since Rel-13 in TS 37.840, while relevant parameters for the adjacent channel regions have not been defined.

Observation 2: The array antenna average gain varies between peak sub-array gain and peak full array gain (including array factor) depending on signal correlation properties.

Since the array factor gain relies on coherent combining of signals it is not expected that the array factor will operate outside the carrier with full strength in situations outside the carrier where the emission is created as the sum of correlated intermodulation distortion and uncorrelated wide band noise. The Inter-Modulation (IM) distortion is coming from non-linear PA characteristics and can be categorized in IM3, IM5, etc. It can be shown that distortions due to IM distortion will end up in different frequency regions mapping to adjacent carrier channels.

To model the array antenna gain pattern outside the carrier the correlation factor needs to be modelled as a function of frequency offset to the carrier centre frequency. To capture fundamental aspects of the gain drop outside the wanted signal a stepped roll-off model can be adopted for the correlation factor, as visualised in Figure 2.1-1. The roll-off is here assumed to be symmetrical around the carrier frequency.

# Figure 2.1-1: Correlation factor roll-off model

For the carrier (within the interval -0.5B≤f≤0.5B) full array factor is expected which means that the correlation factor is equal to 1. Moving away from the carrier edge (f>0.5B) the correlation will drop gradually due to the composition of the emission outside the carrier. The emission will be created as the sum of correlated noise due to intermodulation products and non-correlated noise due to wideband PA noise. The relation to the induvial noise levels will determine the out-of-carrier correlation properties in the adjacent emission regions. At a certain frequency offset to the carrier edge the emission noise will be dominated by wideband noise which means that the correlation is equal to 0.

Observation 3: The modelled array average gain characteristics will depend on the carrier bandwidth.

This model captures the effects of array factor roll-off due to correlation effects close to the carrier. Close the carrier the emission is dominated by intermodulation distortion products (3rd order and 5th order) and further away the un-correlated wide band PA noise is typically dominating.

Since the modelling of the correlation factor is dependent on the carrier bandwidth the break points for the correlation factor will depend on carrier bandwidth. As an example, the correlation factor model can be described in terms of normalized frequency offset as listed in Table 2.1-1.

Table 2.1-1: Array antenna correlation factor roll-off model

| Normalized frequency offset f/B | Correlation factor r(f) |
| --- | --- |
| < -2.5 | 0.00 |
| -2.5 to -1.5 | 0.25 |
| -1.5 to -0.5 | 0.50 |
| -0.5 to 0.0 | 1.00 |
| 0.0 to 0.5 | 1.00 |
| 0.5 to 1.5 | 0.50 |
| 1.5 to 2.5 | 0.25 |
| > 2.5 | 0.00 |

In this example, indicative values for the correlation factor have been listed. Further discussion is required to establish relevant correlation values for given frequency offsets. The example indicates one approach how to model the correlation factor roll-off for array antennas. A more thorough analysis is required to establish relevant parameter values for all relevant BS classes, BS types and supported frequency ranges (FR1 and FR2). For a given implementation and a certain carrier configuration the correlation roll-of parameter values can be established from evaluating transceiver array characteristics from simulation results or by analysing radiation pattern measurements.

Observe that this model only captures effects related to the array factor. Keep in mind that even though the array factor collapses the sub-array still produce gain far outside the carrier. Typically, the sub-array starts to collapse outside the operating band until the antenna is completely detuned far from the centre of the operating band.

Another aspect to consider when modelling BS emission outside the carrier is the suppression of emission outside the carrier in the adjacent channel regions. In these regions the emission is suppressed by the ACLR. If the emission outside the operation band is considered also the RF filter will suppress the emission even further.

The radiated emission from a base station to be studied in a sharing situation can be calculated as:

EIRPθ,φ,f=Ptxf+AAθ,φ,f

, where Ptx is the power fed to the antenna, f is the frequency offset between the carrier edge and the considered frequency. The frequency dependence for the gain pattern AA(q,j,f) comes from the correlation factor r(f) considered for a specific frequency offset f.

The power fed to the antenna can be expressed as a function of frequency offset as:

Ptxf=P-S(f)

, where is P is the carrier power, S(f) is the suppression achieved by ACLR1 (First adjacent channel), ACLR2 (Second adjacent channel), ACLR3 (Third adjacent channel) and additional RF filter suppression (if frequencies outside the operating band is considered) as function frequency offset. It is evident that the radiated power in the adjacent channel regions depends on the drop of antenna gain and the power fed to the antenna. The combined effect should be considered to properly model sharing situations.

Observation 4: When adjacent channel gain is modelled in sharing studies it is essential to use correct information for power fed to the array antenna in conjunction with information on array correlation properties. In a sharing situation both power and gain will affect the produced EIRP level.

Proposal 1: In TR 38.922 capture information on how to model adjacent channel EIRP using the array antenna model and capture relevant model for correlation factor in the adjacent channel regions.

In addition to antenna model described in TR 37.840, a model for the correlation factor capturing the expected array factor response due to moving away from the carrier is presented in this contribution. This model can be used as additional information to better model the array antenna gain response outside the carrier when sharing with other services is considered.

  - AAS BS beamforming assumption

In this contribution, we address some of the points presented in contribution 5D/68 (WRC-27) [5] regarding the modelling of active antenna systems (AAS) in terrestrial International Mobile Telecommunication (IMT) networks. Accurate modelling of AAS is important for analysing how IMT can coexist with other in-band or adjacent band services. The current approach for IMT-2020 and beyond mainly relies on the IMT AAS radiation pattern model described in Recommendation ITU-R M.2101 and the document 5D/716 (Annex 4.4 - WRC-23) for the extension of the model to support sub-array structures.

Item 4 in 5D/68 discusses how commercial IMT base stations (BSs) use digital beamforming techniques, such as zero-forcing (ZF) beamforming, maximum-ratio transmission (MRT), and minimum mean-squared error (MMSE) transmission. These techniques adjust the transmitted beams patterns to enhance the signal reception at the intended user equipment (UE) while reducing interference with other co-scheduled UEs. The current model in ITU-R M.2101 does not account for these adjustments, which could affect the accuracy of interference calculations.

However, our simulation studies captured in Annex in section 5 indicate that the differences introduced by ZF beamforming technique, compared to the model in ITU-R M.2101, are minor. Despite the different beamforming techniques, the simulation results consistently show minimal impact on the overall AAS BS gain towards a number of potential interfered-with receivers and on the BS antenna gain integrated over parts of or the entire angular sphere around the antenna.

Observation 5: Based on our simulation results, there is no need to change the beamforming assumptions in Recommendation ITU-R M.2101. The differences introduced by ZF beamforming technique, compared to the model in M.2101, are minor. The simulation results show minimal impact on the overall AAS BS gain towards a number of potential interfered-with receivers and on the BS antenna gain integrated over parts of or the entire sphere around the antenna.

Proposal 2: Based on outcome of beamforming modelling evaluation, the current modelling approach remains effective for its intended purposes. Modifying the existing beamforming model is not necessary, as the variations observed are insignificant and do not justify the complexity of updating the model. Capture simulation results and corresponding conclusion in TR 38.922.

# Conclusion

In this contribution we present a concept for modelling the out-of-carrier array antenna gain response and simulation results to evaluate the impact of different beam forming schemes. The intention with the information is to prepare technical information for TR 38.922 and response LS to ITU-R WP 5D with answers to additional questions.

The correlation factor model can be used as an addition to the 3GPP array antenna model. This would allow for modelling adjacent channel radiation pattern characteristics including realistic AAS BS array antenna behaviour in the frequency region outside the carrier.

In this contribution we observe following:

Observation 1: Support for modelling array antenna gain adjacent carrier have been documented since Rel-13 in TS 37.840, while relevant parameters for the adjacent channel regions have not been defined.

Observation 2: The array antenna average gain varies between peak sub-array gain and peak full array gain (including array factor) depending on signal correlation properties.

Observation 3: The modelled array average gain characteristics will depend on the carrier bandwidth.

Observation 4: When adjacent channel gain is modelled in sharing studies it is essential to use correct information for power fed to the array antenna in conjunction with information on array correlation properties. In a sharing situation both power and gain will affect the produced EIRP level.

Observation 5: Based on our simulation results, there is no need to change the beamforming assumptions in Recommendation ITU-R M.2101. The differences introduced by ZF beamforming technique, compared to the model in M.2101, are minor. The simulation results show minimal impact on the overall AAS BS gain towards a number of potential interfered-with receivers and on the BS antenna gain integrated over parts of or the entire sphere around the antenna.

Based on the presented information and observations we propose following to progress the work:

Proposal 1: In TR 38.922 capture information on how to model adjacent channel EIRP using the array antenna model and capture relevant model for correlation factor in the adjacent channel regions.

Proposal 2: Based on outcome of beamforming modelling evaluation, the current modelling approach remains effective for its intended purposes. Modifying the existing beamforming model is not necessary, as the variations observed are insignificant and do not justify the complexity of updating the model. Capture simulation results and corresponding conclusion in TR 38.922.

Proposal 3: Include information relevant for adjacent channel EIRP modelling and beamforming evaluation in scheduled LS responses to ITU-R WP 5D.

A draft text proposal TR 38.922 to add a sub-clause for adjacent channel array antenna modelling is provided at the end of this contribution.

# References

[1]	R4-2410576, “LS on Parameters for 4400 to 4800 MHz of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-27”, Ericsson

[2]	R4-2410575, “WF on IMT parameters study”, Ericsson

[3]	R4-2410763, “Combined TP to TR 38.922”, Ericsson

[4]	R4-2408083, “TP to TR 38.922: Addition of array antenna model description in clause 7”, Ericsson

[5]	R4-2400333, “Parameters of terrestrial component of IMT for sharing and compatibility studies in the frequency bands 4 400-4 800 MHz, 7 125-8 400 MHz and 14.8-15.35 GHz”, ITU-R WP 5D

# Annex

  - Introduction to zero-forcing beamforming

In modern wireless communication systems, effective management of inter-user interference is important to maximize the performance of IMT networks. Beamforming is a critical technique to achieve this goal by focusing signal transmissions towards the intended users while reducing interference with other co-scheduled users, and therefore improving signal quality.

Among the advanced techniques employed in Multiple-Input Multiple-Output (MIMO) systems, Zero-Forcing (ZF) beamforming is an appealing solution as it has a moderate computational complexity while achieving spatial multiplexing. This technique decouples the multi-user channel into multiple independent subchannels by nullifying unwanted signals in the direction of other users, ensuring that each user receives their intended signal with minimal interference.

For simplicity, a BS with a horizontal Uniform Linear Array (ULA) is considered for the rest of this section. Mathematically, the ZF precoder can be derived as the pseudo-inverse of the channel matrix:

WZF=HHHHH-1		(Eq. 1)

where H represents the channel matrix with its number of rows corresponding to the number of Rx antennas (i.e., UEs) and its number of columns corresponding to the number of Tx antennas (BS antenna ports), and HH is the conjugate transpose of H.

For simplicity, the channel matrix H can be generated reflecting only the Angles of Arrival (AoA) from each UE. This simplification is applicable because, geometrically speaking, typically only the direct path between the BS and UEs is considered in system-level simulations for sharing and compatibility studies.

# Figure 5.1-1: Depiction of horizontal angle of arrival (AoA) to an IMT antenna array

In this study, Monte Carlo simulations are used to compare the IMT BS gain towards a number of potential interfered-with receivers when using ZF beamforming and the current beamforming model described in Recommendation ITU-R M.2101.

It is noted that the applicability of ZF beamforming depends mainly on the angular separation between UEs and the number of antennas of the BS. Put simply, this is because as more antenna elements are stacked, the beamwidths of the BS beams narrow, therefore improving the BS beamforming angular resolution. This allows closer UEs to be paired for ZF beamforming. Consequently, the channel matrix H only includes the UEs that fulfil a minimum angular reparation requirement depending on the size of the BS antenna, so H may have two or three rows in each Monte Carlo snapshot depending in the geometry of the BS and the UEs. For the UEs that cannot be paired, the current beamforming model in Recommendation ITU-R M.2101 is used.

To determine a suitable minimum angular separation for pairing UEs in the horizontal domain, we use a ULA with 8 antenna elements spaced 0.5 wavelengths and the single element pattern model from Recommendation ITU-R M.2101 with parameters listed in Table 5.1-1.

Table 5.1-1: Antenna characteristics

| Parameter | Macro Urban |
| --- | --- |
| Element gain (dBi) | 5.5 |
| Horizontal/vertical 3 dB beamwidth of single element (degree) | 90º for H 90º for V |
| Horizontal/vertical front to back ratio (dB) | 30 for both H/V |

The previous parameters are taken from the antenna characteristics for IMT in 6 425-10 500 MHz for macro urban deployments in the document 5D/716 (Annex 4.4 - WRC23).

For this assessment, as shown in Figure 5.1-2, we assume only two co-channel UEs served by a BS (UE #1 always at the BS boresight and UE #2 located at a certain angular separation from UE #1).

# Figure 5.1-2: Scenario to determine a suitable minimum angular separation for pairing UEs in the horizontal domain

Figure 5.1-3 shows the radiation patterns towards UE #1 of the ULA. As example, Figure 5.1-3a shows the radiation pattern with no ZF beamforming, while Figures 5.1-3b, 5.1-3c, and 5.1-3d show the radiation patterns when a ZF precoder is applied to generate nulls towards the UE #2 at an angular separation of 20, 10, and 5 degrees from UE #1, respectively. As can be seen, the gain reduction towards UE #1 is 0.22, 0.68, and 4.72 dB, respectively. This reduction is due to the limited resolution provided by the ULA with 8 elements. Therefore, in the preceding example and for the rest of this study, a UE minimum angular separation of 10 degrees is deemed appropriate as the antenna gain reduction is under 0.7 dB (i.e., less than 15% reduction in the linear domain).

# Figure 5.1-3: Radiation patterns of a ULA (8 antenna elements and a spacing of 0.5λ) with and without ZF beamforming

  - Simulation parameters

The IMT terrestrial component relevant parameters are based on the agreed technical and operational characteristics in the document 5D/716 (Annex 4.4 - WRC23). Table 5.2-1 contains the deployment related parameters of BSs and UEs, and Table 5.2-2 contains the beamforming antenna characteristics.

Table 5.2-1: IMT deployment-related parameters

| Parameter | Macro Urban |
| --- | --- |
| Base station |  |
| Carrier frequency | 6 GHz |
| Channel bandwidth | 100 MHz |
| BS antenna height | 18 m |
| Cell radius | 400 m |
| Sectorization | 1 sector 1 |
| Frequency reuse | 1 |
| User Equipment |  |
| UE height | 1.5 m |
| UE density for terminal that are transmitting simultaneously | 3 UEs per sector |
| NOTE 1: This study considers only a single-entry scenario. |  |

Table 5.2-2: IMT base station beamforming antenna characteristics for IMT

| Parameter | Macro Urban |
| --- | --- |
| Antenna pattern | Refer to Recommendation ITU-R M.2101 Annex 1 |
| Element gain (incl. Ohmic loss) (dBi) | 5.5 |
| Horizontal/vertical 3 dB beamwidth of single element (degree) | 90º for H 90º for V |
| Horizontal/vertical front to back ratio (dB) | 30 for both H/V |
| Antenna polarization | Linear ±45º |
| Antenna array configuration (Row × Column) | 16 × 8 |
| Horizontal/Vertical radiating element spacing | 0.5 of wavelength for H 0.5 of wavelength for V |
| Array Ohmic loss (dB) | 2 |
| Base station maximum coverage angle in the horizontal plane (degrees) | ±60 |
| Base station vertical coverage range (degrees) | 90-120 1 |
| Mechanical down-tilt (degrees) | 10 |
| NOTE 1: The vertical coverage range includes the mechanical down-tilt. A minimum BS-UE distance along the ground of 35 m should be used for urban/suburban and rural macro environments. |  |

For the sake of completeness, additional results are provided with an AAS with a subarray configuration. The parameters for this AAS are taken from the WRC-23 studies 5D/1235 and 5D/1461 and are presented in the Table 5.2-3.

Table 5.2-3: IMT base station beamforming antenna characteristics for IMT with subarray configuration

| Parameter | Macro Urban |
| --- | --- |
| Antenna pattern | Refer to the extended AAS model in 5D/716 (Annex 4.4 - WRC23) |
| Element gain (incl. Ohmic loss) (dBi) | 6.4 |
| Horizontal/vertical 3 dB beamwidth of single element (degree) | 90º for H 65º for V |
| Horizontal/vertical front to back ratio (dB) | 30 for both H/V |
| Antenna polarization | Linear ±45º |
| Antenna array configuration (Row × Column) | 16 × 16 |
| Horizontal/Vertical radiating element spacing | 0.5 of wavelength for H 1.4 of wavelength for V |
| Number of element rows in sub-array | 2 |
| Vertical radiating element spacing in sub-array | 0.7 of wavelength |
| Pre-set sub-array down-tilt (degrees) | 3 |
| Array Ohmic loss (dB) | 2 |
| Base station maximum coverage angle in the horizontal plane (degrees) | ±60 |
| Base station vertical coverage range (degrees) | 90-100 1 |
| Mechanical down-tilt (degrees) | 6 |
| NOTE 1: The vertical coverage range includes the mechanical down-tilt. A minimum BS-UE distance along the ground of 35 m should be used for urban/suburban and rural macro environments. |  |

  - Deployment

The IMT network consists of a single site with a single sector. In each snapshot of the Monte Carlo simulation, 3 UEs are uniformly distributed within the sector, and the BS serves each UE with a dedicated beam. These beams can be generated by both the model in ITU-R M.2101 and as described in Section 5.1, assuming a UE minimum angular separation of 10 degrees.

To assess the interference to possible interfered-with receivers, two scenarios are considered, where these receivers are represented by sample points:

- Equidistant sample points towards the horizon around the IMT BS (Figure 5.3-1a)

- Equidistant sample points on a hemisphere above the IMT BS excluding the sample points towards the horizon (Figure 5.3-1b)

It is noted that the angular separation between sample points is approximately 15.5 degrees, with scenario 1 having 23 sample points and scenario 2 having 62 sample points.

# Figure 5.3-1: Equidistant sample points (a) towards the horizon around the BS, and (b) on a hemisphere above the BS

Is it noted that, given the UE distribution and BS vertical steering range, i.e., 90 to 120 degrees and the geometry of the deployment (Figure 5.3-2), ZF processing is limited to only the horizontal domain, while for elevation the same processing as in Recommendation ITU-R M.2101 is applied.

# Figure 5.3-2: Probability density function (PDF) of the elevation angle from the BS to the UEs

To validate the effectiveness of the ZF beamforming technique, for a BS without subarrays, Figure 5.3-3 shows the BS gain towards the co-scheduled UEs when serving a specific UE. As expected, ZF reduces the BS gain towards the co-scheduled UEs. Note that the gain reduction provided by ZF is not as large as one might expect from a beam null because the resolution of the beam patterns is 1 degree, so the null gain is usually rounded to the nearest integer angular direction. Note as well that all CDFs in Figure 5.3-3 are nearly identical because all UEs are deployed randomly, following the exact same distribution. Furthermore, it can be seen that the M.2101 and ZF models gains are the same in about 16% of cases, corresponding to situations where no UE can be paired for ZF beamforming because the angular separation between UEs is less than 10 degrees. The rest of the cases correspond to situations where either two UEs or all three UEs can be paired, depending on whether only two or all UEs fulfil the minimum angular separation requirement.

# Figure 5.3-3: BS gain towards the co-scheduled UEs when serving a specific UE with a BS without subarrays

Additionally, Figure 5.3-4 shows the BS gain towards the intended UEs. The antenna gain difference for all cases is less than 0.7 dB, as expected, with a minimum angular separation of 10 degrees. Again, all CDFs are nearly identical because all UEs are deployed randomly, following the exact same distribution.

# Figure 5.3-4: BS gain towards the intended UEs with a BS without subarrays

  - Simulation results

In this section the Monte Carlo simulations results are presented. For each sample point in the scenarios in Figure 5.1-3, 10.000 snapshots are simulated. The cumulative distribution functions (CDFs) curves of the IMT BS gain towards the sample points of the different scenarios are shown in Figures 5.4-1 to 5.4-4. The CDF results for scenario 1 using a BS without and with subarrays are shown in Figures 5.4-1 and 5.4-2 respectively. The CDF results for scenario 2 using a BS without and with subarrays are shown in Figures 5.4-3 and 5.4-4 respectively. For instance, since scenario 1 has 23 sample points, Figures 5.4-1a and 5.4-2a show 23 pairs of CDFs.

# Figure 5.4-1: Scenario 1 (without subarrays) CDFs: BS gain towards (a) each sample point, and (b) all sample points (composite)

# Figure 5.4-2: Scenario 1 (with subarrays) CDFs: BS gain towards (a) each sample point, and (b) all sample points (composite)

# Figure 5.4-3: Scenario 2 (without subarrays) CDFs: BS gain towards (a) each sample point, and (b) all sample points (composite)

# Figure 5.4-4: Scenario 2 (with subarrays) CDFs: BS gain towards (a) each sample point, and (b) all sample points (composite)

To assess the impact on long- and short-term statistics, the 80th percentile and 99.9th percentile are considered respectively. For Figures 5.4-1a, 5.4-2a, 5.4-3a, and 5.4-4a, Table 5.4-1 contains the summary of the average BS antenna gain difference between ITU-R M.2101 beamforming model and ZF beamforming model.

Table 5.4-1: Average BS antenna gain difference between ITU-R M.2101 beamforming model and ZF beamforming

| Scenario | Long-term (80th percentile) | Short-term (99.9th percentile) |
| --- | --- | --- |
| Scenario 1 (without subarrays) | 0.62 dB | 0.20 dB |
| Scenario 1 (with subarrays) | 0.08 dB | 0.01 dB |
| Scenario 2 (without subarrays) | 0.65 dB | 0.10 dB |
| Scenario 2 (with subarrays) | 0.05 dB | 0.05 dB |

Furthermore, the BS antenna gain integrated over parts of or the entire angular sphere around the antenna, i.e., the Total Integrated Gain (TIG), is also evaluated to provide a set of more consistent results by considering all angular directions. The TIG is calculated as follows:

TIG≈π2NθNφφ=0Nφ-1θ=0Nθ-1Gθ,φsinθ		(Eq. 2)

where θ and φ represent the elevation and azimuth angles, respectively, Nθ and Nφ are the number of samples on elevation and azimuth, respectively, and G is the antenna gain in linear units at the spherical coordinates (θ, φ).

For this assessment, the BS antenna gain is integrated over the whole sphere and over only the upper hemisphere by limiting the range of θ. The integrated gain is calculated for each individual beam for 10000 snapshots, resulting in three values per snapshot. Note that the integrated gain of the upper hemisphere of the AAS, where θ = 90 degrees is the AAS boresight direction including down-tilt, is not exactly the integrated above the horizon but includes most of it. This is depicted in Figure 5.4-5.

# Figure 5.4-5: Upper hemisphere: (a) upper hemisphere above horizon, and (b) upper hemisphere considered in this study

Figures 5.4-6 and 5.4-7 show the BS antenna gain integrated over the whole sphere and over only the upper hemisphere (Figure 5.4-5b), including the horizon, for the BS without and with subarrays, respectively. For simplicity, it is noted that for all the presented results, the integrated gain is always normalized by π⁄(2Nθ Nφ), as indicated in (Eq. 2), regardless of the number of samples considered.

# Figure 5.4-6: CDFs of the BS antenna gain without subarrays, integrated over (a) the upper hemisphere, including the horizon, and (b) the entire angular sphere around the antenna

# Figure 5.4-7: CDFs of the BS antenna gain with subarrays, integrated over (a) the upper hemisphere, including the horizon, and (b) the entire angular sphere around the antenna

As can be seen, in all cases considered, the TIG difference between the beamforming schemes is negligible.

  - Concluding remarks

Based on our simulation results, there is no need to change the beamforming assumptions in Recommendation ITU-R M.2101. The differences introduced by ZF beamforming technique, compared to the model in M.2101, are minor. The simulation results show minimal impact on the overall AAS BS gain towards a number of potential interfered-with receivers and on the BS antenna gain integrated over parts of or the entire angular sphere around the antenna. Therefore, the current modelling approach remains effective for its intended purposes. Modifying the existing beamforming model is not necessary, as the variations observed are insignificant and do not justify the complexity of updating the model.

DRAFT to TR 38.922:

## 7.2	Adjacent channel array antenna modelling

### 7.2.1	Overview

The array antenna model adopted by 3GPP in TR 38.803 creates the composite pattern as a multiplication between the element factor (or sub-array element factor) and array factor. The array antenna model was first introduced in TR 37.840 in the early days of AAS for BS.

The composite average radiation gain pattern can be expressed in logarithmical scale as:

AAθ,φ=Asubθ,φ+10log101+ρm=1Mn=1Nwm,nvm,n2-1

, where r is the correlation factor defined in the interval 0≤ρ≤1 and other parameters are defined in TR 38.803. For r equal to 0 only the element or sub-array produce gain (peak gain equal to GE,max + 10log10(Msub) dBi), while for r equal to 1 the composite array gain is produced (peak gain equal to GE,max + 10log10(Msub) + 10log10(MN) dBi).

### 7.2.2	Parameter values

From analysis of measurement results it can be concluded that the array correlation factor reduces as function of frequency offset. In Table 7.2.2-1, relevant parameter values for the array correlation factor as function of frequency offset is listed.

Table 7.2.2-1: Parameter values for correlation roll-off model

| Normalized frequency offset f/B | Correlation factor r(f) |
| --- | --- |
| < -2.5 | 0.00 |
| -2.5 to -1.5 | 0.25 |
| -1.5 to -0.5 | 0.50 |
| -0.5 to 0.0 | 1.00 |
| 0.0 to 0.5 | 1.00 |
| 0.5 to 1.5 | 0.50 |
| 1.5 to 2.5 | 0.25 |
| > 2.5 | 0.00 |

### 7.2.3	EIRP modelling

The radiated emission from a base station to be studied in a sharing situation can be calculated as:

EIRPθ,φ,f=Ptxf+AAθ,φ,f

, where Ptx is the power fed to the antenna, f is the frequency offset between the carrier edge and the considered frequency. The frequency dependence for the gain pattern AA(q,j,f) comes from the correlation factor r(f) considered for a specific frequency offset f.

The power fed to the antenna can be expressed as a function of frequency offset as:

Ptxf=P-S(f)

, where is P is the carrier power, S(f) is the suppression achieved by ACLR1 (First adjacent channel), ACLR2 (Second adjacent channel), ACLR3 (Third adjacent channel) and additional RF filter suppression (if frequencies outside the operating band is considered) as function frequency offset. It is evident that the radiated power in the adjacent channel regions depends on the drop of antenna gain and the power fed to the antenna. The combined effect should be considered to properly model sharing situations.