3GPP TSG-RAN4 Meeting #112	R4-2411876

# Maastricht, Netherlands, 19th – 23rd August, 2024

Source:	LG Electronics

Title:	ATG UE RF requirements for UL-MIMO

Agenda item:	8.10.2.3

Document for:	Discussion

Introduction

In this paper, we provide our views on ATG UE RF requirements for UL-MIMO based on the approved WF[1].

Discussion

Based on the approved WF[1], we would like to discuss ULFPTx mode for ATG UL-MIMO.

ULFPTx mode for UL-MIMO

In NR, ULFPTx mode is specified for PC2 and PC1.5 as follows.

- Single layer for 2 Tx port & 4 Tx port

  - PC2/PC1.5

- 2 layers for 4 Tx port

  - PC1.5

For ATG single layer UL MIMO operation and two antenna-ports, in case that R-MOP of 26dBm is declared for UL-MIMO, ULFPTx mode 1/2/Full in Table 2-1 can be supported depending on UE PA implementation.

.

Table 2.1: Example of ATG ULFPTx (R-MOP = 26dBm)

| ATG ULFPTx mode | PA1(dBm) | PA2(dBm) |
| --- | --- | --- |
| Mode2 | 26 | 23~25 |
| Mode-Full | 26 | 26 |
| Mode1 | 23~25 | 23~25 |

Proposal 1: Consider ULFPTx mode for ATG UL-MIMO.

Conclusion

In this contribution, we provided our view on ULFPTx mode for ATG UL-MIMO.

Proposal 1: Consider ULFPTx mode for ATG UL-MIMO.

- References

[1] R4-2410687, “WF on Rel-19 ATG UE requirements”, CMCC

[2] RP-240839, “New WID on Enhancements for Air-to-ground network for NR”, Moderator(Apple)