3GPP TSG-RAN4 Meeting #112	R4-2411877

# Maastricht, Netherlands, 19th – 23rd August, 2024

Source:	LG Electronics

Title:	UE RF requirements for Ku band NTN

Agenda item:	8.9.4

Document for:	Discussion

Introduction

In this paper, we provide our views on Ku band NR NTN.

Discussion

In RAN #104 meeting, WID for Ku band NR NTN was approved [1]. This paper discusses the related UE RF requirements.

Objective [1]

The following assumptions are taken as the baseline for this work: - GSO and NGSO - NTN capable UE type 1 to 5 (38.101-5 Table 9.2.1.0-1) for above 10 GHz (no handheld devices) - FDD Mode - GNSS Receiver capability The objectives are: - Use the regulation requirements in ITU Regions 1, 2 and 3 to identify relevant adjacent band co-existence scenarios for NTN Ku Band covering the following frequency ranges, considering targeted deployment scenarios: - For ITU regions 1 and 3: - Downlink 10.70 – 12.75 GHz - Uplink 13.75-14.5 GHz - For ITU regions 2 excluding US: - Downlink 10.70 – 12.70 GHz - Uplink 13.75-14.5 GHz - For ITU regions 1 and 3 - Downlink 10.70 – 12.75 GHz - Uplink 12.75 - 13.25 GHz - For ITU regions 2 excluding US - Downlink 10.70 – 12.70 GHz - Uplink 12.70 - 13.25 GHz - Note: Appropriate TX/Rx separation for uplink downlink pairing will be required. - Conduct coexistence study. Reuse Ka band coexistence study assumptions as much as possible, with modifications where necessary, e.g. for frequency range and subcarrier spacing. Conduct new co-existence studies based on adjacent services . - Specify RF requirements for satellite access node and NTN VSAT types in 38.101-5 also considering existing regulations on antenna sizes for certain parts of the Ku band. - Priority 1: for all ITU regions, define Ku band #1a and #1b for the downlink block and uplink block B to enable the future possibility of supporting satellite services in 125 MHz (UL) and 250 MHz (DL) aggregated bandwidths based on existing channel bandwidths. Potential approaches for achieving this shall be studied, from which one shall be selected including but not limited to: - Ku band #1a and #1b as FR1-NTN supporting 20 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing - Ku band #1a and #1b as FR2-NTN supporting 50 MHz, 100 MHz, 200 MHz, 400 MHz (120kHz SCS only) channel bandwidths with 120 kHz subcarrier spacing - RAN4 will select which one of the above approaches for DL and UL will be pursued by RAN #106. - Priority 2: for all ITU regions excluding the US in region 2, using the approach in Priority 1, define Ku band #2a and #2b as appropriate for the downlink block and uplink block A to support satellite services in 125 MHz (UL) and 250 MHz (DL) aggregated bandwidths. - Specify RRM requirements Note: Support for NTN CA is not in the scope of this WID

Figure 2-1 shows the targeted deployment scenarios for Ku band NTN.

Figure 2-1. Targeted deployment scenarios (Ku band)

For FR1-NTN vs FR2-NTN,

As seen in Figure 2-1, the targeted Ku band frequency is from 10.7GHz to 14.5GHz. About 3GHz is apart from FR1-NTN and from FR2-NTN.

Table 5.1-1: Definition of NTN frequency ranges

| Frequency range designation | Corresponding frequency range |
| --- | --- |
| FR1-NTN1 | 410 MHz – 7125 MHz |
| FR2-NTN2 | 17300 MHz – 30000 MHz |
| NOTE 1: [NTN bands within this frequency range are regarded as a FR1 band when references from other specifications.] NOTE 2: [NTN bands within this frequency range are regarded as a FR2 band when references from other specifications.] |  |

Considering the baseline for this work,

- GSO and NGSO

- NTN capable UE type 1 to 5 (38.101-5 Table 9.2.1.0-1) for above 10 GHz (no handheld devices)

- FDD Mode

- GNSS Receiver capability

UE maximum output power for Ku band should be specified as similar to that of VSAT which was specified with FR2-NTN in Rel-18. Considering it, approach #2 is preferred.

- Approach #1 : Ku band #1a and #1b as FR1-NTN supporting 20 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing

- Approach #2: Ku band #1a and #1b as FR2-NTN supporting 50 MHz, 100 MHz, 200 MHz, 400 MHz (120kHz SCS only) channel bandwidths with 120 kHz subcarrier spacing

Proposal 1: Consider  Ku band #1a and #1b as FR2-NTN.

For example, Ku band #1a/#1bcan be defined with  n509, and n508 respectively. Table 2-1 shows example of Ku band number.

Table 2-1: Example of NTN satellite bands in FR1-NTN

| NTN satellite operating band | Uplink (UL) operating band Satellite Access Node receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band Satellite Access Node transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n5121 | 27500 MHz - 30000 MHz | 17300 MHz - 20200 MHz | FDD |
| n5112 | 28350 MHz - 30000 MHz | 17300 MHz - 20200 MHz | FDD |
| n5103 | 27500 MHz - 28350 MHz | 17300 MHz - 20200 MHz | FDD |
| n5094 | 13750 MHz – 14500 MHz | 10700 MHz – 12750 MHz | FDD |
| n5085 | 13750 MHz – 14500 MHz | 10700 MHz – 12700 MHz | FDD |
| NOTE 1: This band is applicable in the countries subject to CEPT ECC Decision(05)01 and ECC Decision (13)01. NOTE 2: This band is applicable in the USA subject to FCC 47 CFR part 25. NOTE 3: This band is applicable for Earth Station operations in the USA subject to FCC 47 CFR part 25. FCC rules currently do not include ESIM operations in this band (47 CFR 25.202). NOTE 4: This band is for ITU regions 1 and 3. NOTE 5: This band is for ITU region 2 excluding US. |  |  |  |

For Ku band #2a/2b

In Ku band #2a/#2b, there is no frequency gap between downlink band and uplink band. To guarantee FDD operation in the Ku band #2a/#2b, some frequency gap needs to be considered.

Minimum and maximum of Tx-Rx carrier centre frequency separation can be calculated as follows.

- Minimum = (DL lowest edge frequency + minimum of supported CBW/2) - (UL highest edge frequency - minimum of supported CBW/2)

- Maximum = (DL highest edge frequency - minimum of supported CBW/2) - (UL lowest edge frequency + minimum of supported CBW/2)

If Tx-Rx carrier centre frequency separation is -50 MHz, it can make problem on FDD operation due to interference from transmission to receiver without frequency gap.

Figure 2-2 shows the absolute value of the Tx-Rx carrier centre frequency separation for #2a/#2b.

- 50 MHz for DL CBW of 50MHz and UL CBW of 50MHz in (a)

- 75 MHz for DL CBW of 100MHz and UL CBW of 50MHz in (b)

- (X+Y)/2 MHz for DL CBW of X MHz and UL CBW of Y MHz in (c)

Figure 2-2. Tx-Rx carrier frequency separation for Ku band #2a, and #2b

In general, when DL CBW is X MHz, UL CBW is Y MHz, and Tx-Rx carrier centre frequency separation is (X+Y)/2 MHz, Full Duplex FDD operation cannot be applicable due to interference from transmission to receiver. In this case, Half Duplex FDD operation can be applicable only.

Proposal 2 : Consider Half Duplex FDD operation in condition that Tx-Rx carrier centre frequency separation is (X+Y)/2 MHz. Here, X = DL CBW, Y = UL CBW.

For FDD operation, minimum Tx-Rx carrier centre frequency separation or minimum guard band can be considered.

Figure 2-3 shows Tx-Rx carrier centre frequency separation and guard band.

Figure 2-3. Tx-Rx carrier frequency separation & guard band for Ku band #2a, and #2b

If Tx-Rx carrier centre frequency separation is larger than or equal to the minimum separation, FDD operation is applicable. Else, half duplex FDD operation is applicable. Or,

If frequency gap between DL channel and UL channel is larger than or equal to the minimum guard band, FDD operation is applicable. Else, half duplex FDD operation is applicable.

Proposal 3 : Consider minimum Tx-Rx carrier centre frequency separation or minimum guard band for FDD operation with #2a, and #2b.

Proposal 4 : Consider UE capability on minimum Tx-Rx carrier centre frequency separation or minimum guard band for FDD operation with #2a, and #2b.

Conclusion

In this contribution, we proposed our view on Ku band NR NTN. The proposals are as follows.

Proposal 1: Consider  Ku band #1a and #1b as FR2-NTN.

Proposal 2 : Consider Half Duplex FDD operation in condition that Tx-Rx carrier centre frequency separation is (X+Y)/2 MHz. Here, X = DL CBW, Y = UL CBW.

Proposal 3 : Consider minimum Tx-Rx carrier centre frequency separation or minimum guard band for FDD operation with #2a, and #2b.

Proposal 4 : Consider UE capability on minimum Tx-Rx carrier centre frequency separation or minimum guard band for FDD operation with #2a, and #2b.

- References

[1] RP-241690, “New WID on Introduction of Ku Band for NR NTN”, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat