# 3GPP TSG-RAN WG4 Meeting # 112                                    R4-2411878

# Maastricht, Netherlands, Aug.19th – 23th, 2024

# Title: 	Discussion on PC3 n28 40MHz

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	7.20.3

# Document for:	Approval

# 1 Introduction

In RAN #104 meeting, a new WID on Power Class 2 and UE 40MHz Channel Bandwidth in NR band n28 was approved in [1], where the objectives are listed as below:

The objectives of this WI are to introduce requirements for UE Power Class 2 and 40MHz Channel Bandwidth for NR band n28 as optional features, including the following aspects: - Introduce requirements for PC2 and those associated with following bullets - Both 1Tx and 2Tx PC2 are supported - UL-MIMO is supported. - Specify A-MPR requirements for PC2 - Introduce 40MHz Channel bandwidth located in UL: 703 - 743MHz and DL: 758 - 798MHz for band n28 supporting PC3 and PC2. - Allow flexible channel allocations within the frequency range (UL: 703 - 743MHz and DL: 758 - 798MHz) for BW<40MHz. - Release independence of UE 40MHz Channel bandwidth to Rel-15 is anticipated. - No changes to the existing requirements (38.101-1 v18.5.0) (including A-MPR values and RB allocation regions) for power class 3 with existing channel bandwidths up to and including 30MHz to avoid impacts on existing power class 3 networks. In the event of new regulatory requirements, this assumption can be revisited.

It can be seen from the objectives, larger CBW(i.e. 40MHz) is introduced for both PC3 and PC2 in band n28 UL, but the current maximum supported channel bandwidth is only 30MHz in the UL. Thus due to new larger channel bandwidth is introduced, some RF requirements should be defined accordingly.

In this contribution, we give some initial discussions on the RF requirements of 40MHz for n28 PC3.

# 2	Discussion

In terms of the description of the WID, evolve duplexer implementation is desired from industry, i.e. single band duplexer implementation, would be used for studied. This is different with the existing specs in which dual duplexer architecture for UE is assumed. Also, it should be noted the existing requirements for power class 3 with existing channel bandwidths up to and including 30MHz are not changed.

1.	UE channel bandwidth in band 28

In current specification for band n28, NOTE 7 is applied to the supported channel bandwidths of 20MHz, 25MHz and 30MHz under the dual duplexer architecture assumption. Obliviously, the description in NOTE 7 should be updated to accommodate with 40MHz to incorporate all the supported channel bandwidths in the same NOTE together, where 40MHz Channel bandwidth is located in UL: 703 - 743MHz and DL: 758 - 798MHz according to the WID.

Proposal 1: To update the footnote 7 as:

| NR Band | SCS (kHz) | UE Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
| n28 | 15 | 34 | 5 | 10 | 15 | 207 | 257 | 307 |  | 407 |  |  |  |  |  |  |  |
|  | 30 |  |  | 10 | 15 | 207 | 257 | 307 |  | 407 |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 7: For the 20 MHz bandwidth, the minimum requirements are specified for NR UL carrier frequencies confined to either 713-723 MHz or 728-738 MHz. For the 25 MHz bandwidth, the minimum requirements are specified for NR UL carrier frequencies confined to either 715.5-720.5 MHz or 730.5-735.5 MHz. For the 30MHz bandwidth, the minimum requirements are specified for NR UL transmission bandwidth configuration confined to either 703-733 or 718-748 MHz. For the 40MHz bandwidth, the minimum requirements are specified for NR UL transmission bandwidth configuration confined to either 703-743 MHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

2.	∆MPR&A-MPR

The relative channel bandwidth of n28 30MHz and 40MHz are around 4.1% and 5.5%, respective. Both of them are larger than the 3% criteria of FDD bands. So same with 30MHz, it’s necessary to specify the ∆MPR requirements for n28 40MHz.

According to TR38.888, it mentioned that general MPR requirements in the current spec can be met for UL 30MHz channel bandwidth in band n28, Nevertheless, ∆MPR is still defined considering different implementations and accommodate the more pronounced PA memory effect to our understanding. For 40MHz, it is reasonable to hypothesize that the current general MPR specified in existing standards can indeed be met since the practical implementations typically exist margins for the existing general MPR values.

Furthermore, compared with 30MHz, it is anticipated that the 40MHz channel bandwidth would be not too much worse the general emission requirements. This expectation aligns with observations from band n71 given that band n71 falls within a comparable frequency band to band n28, wherein the introduction of additional channel bandwidths, namely 25 MHz, 30 MHz, and 35 MHz, did not significantly impact the emission requirements.

Therefore, we propose:

Proposal 2: ΔMPR= 0.5dB for PC3 n28 40MHz.

For A-MPR, there are two network signalling (i.e. NS_17 and NS_18) for band n28 in current specification, both of them are 6MHz DTV system, while the TV system has own standard based on 8MHz bandwidth in China. When 30MHz was added to band n28, there were no specific emission requirements for 30MHz in China, so no NS_xx defined in the current specification except NS_17 and NS_18.

Observation 1: NS_17 and NS_18 aren’t applicable to China.

Moreover, there is no A-MPR requirement defined for NS_17, instead the restrictions in LCRB, RB_start and the FOOB range result in MPR being enough for compliance. In terms of the contributions in the past, such as [3~5], non-zero A-MPR value for NS_17 would be needed when the single band duplexer architecture is used with the legacy assumptions for MPR/A-MPR simulations. However, non-zero A-MPR requirement for NS_17 will violate the objectives of “No changes to the existing requirements (38.101-1 v18.5.0) (including A-MPR values and RB allocation regions) for power class 3 with existing channel bandwidths up to and including 30MHz to avoid impacts on existing power class 3 networks.”

Observation 2: Non-zero A-MPR requirement for NS_17 is not feasible for PC3 n28 30MHz.

3. REFSEN

The REFSEN requirement for 40Mhz is another issue which should be discussed. For FDD band, the Tx noise falls into Rx frequency range are needed to be consider. In general, the Tx noise includes PA output noise, Tx chain noise, and etc. Except that, the C-IM3/5 or IM3 products from Tx may also fall into Rx range which would further worsen the REFSEN requirements. After calculation for the 40MHz channel bandwidth, the IM3 products will fall about 767.8425~781.3425MHz while DL is 758~798MHz, while no C-IM3/5 product of n28 UL would fall into n28 DL. Thus REFSEN for 40MHz should be studied.

Here we assume the same UL configurations and the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth, which is:

Table 1: Uplink configuration for reference sensitivity

| Operating band / SCS / Channel bandwidth / Duplex mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Duplex Mode |
| n28 | 15 | 25 | 251 | 251 | 251 |  | 251 | 251 |  |  |  |  |  | FDD |
|  | 30 |  | 101 | 101 | 101 |  | 101 | 101 |  |  |  |  |  |  |
| NOTE 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Proposal 3: For reference sensitivity, to reuse existing UL configurations for 40MHz.

# 3 Conclusion

In this contribution, we give some initial discussion on the RF requirements of 40MHz for n28 PC3. The conclusions are summarized in the following:

Observation 1: NS_17 and NS_18 aren’t applicable to China.

Observation 2: Non-zero A-MPR requirement for NS_17 is not feasible for PC3 n28 30MHz.

Proposal 1: To update the footnote 7 as:

| NR Band | SCS (kHz) | UE Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
| n28 | 15 | 34 | 5 | 10 | 15 | 207 | 257 | 307 |  | 407 |  |  |  |  |  |  |  |
|  | 30 |  |  | 10 | 15 | 207 | 257 | 307 |  | 407 |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 7: For the 20 MHz bandwidth, the minimum requirements are specified for NR UL carrier frequencies confined to either 713-723 MHz or 728-738 MHz. For the 25 MHz bandwidth, the minimum requirements are specified for NR UL carrier frequencies confined to either 715.5-720.5 MHz or 730.5-735.5 MHz. For the 30MHz bandwidth, the minimum requirements are specified for NR UL transmission bandwidth configuration confined to either 703-733 or 718-748 MHz. For the 40MHz bandwidth, the minimum requirements are specified for NR UL transmission bandwidth configuration confined to either 703-743 MHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Proposal 2: ΔMPR= 0.5dB for PC3 n28 40MHz.

Proposal 3: For reference sensitivity, to reuse existing UL configurations for 40MHz.

# 4	Reference

[1] RP-241660, New WID: Introduction of Power Class 2 and UE 40MHz Channel Bandwidth in NR band n28, CMCC, CBN

[2] TR38.888

[3] R4-2400170, On n28 with full band duplexer, Apple

[4] R4-2407163, PC2 n28 NS_17, Skyworks Solutions, Inc.

[5] R4-2407624, Views on b/n28 full-band duplex, Skyworks Solutions, Inc.