# 3GPP TSG-RAN WG4 Meeting # 112                                    R4-2411879

# Maastricht, Netherlands, Aug.19th – 23th, 2024

# Title: 	Discussion on PC2 n28 40MHz

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	7.20.2

# Document for:	Approval

# 1 Introduction

In RAN #104 meeting, a new WID on Power Class 2 and UE 40MHz Channel Bandwidth in NR band n28 was approved in [1], where the objectives are listed as below:

The objectives of this WI are to introduce requirements for UE Power Class 2 and 40MHz Channel Bandwidth for NR band n28 as optional features, including the following aspects: - Introduce requirements for PC2 and those associated with following bullets - Both 1Tx and 2Tx PC2 are supported - UL-MIMO is supported. - Specify A-MPR requirements for PC2 - Introduce 40MHz Channel bandwidth located in UL: 703 - 743MHz and DL: 758 - 798MHz for band n28 supporting PC3 and PC2. - Allow flexible channel allocations within the frequency range (UL: 703 - 743MHz and DL: 758 - 798MHz) for BW<40MHz. - Release independence of UE 40MHz Channel bandwidth to Rel-15 is anticipated. - No changes to the existing requirements (38.101-1 v18.5.0) (including A-MPR values and RB allocation regions) for power class 3 with existing channel bandwidths up to and including 30MHz to avoid impacts on existing power class 3 networks. In the event of new regulatory requirements, this assumption can be revisited.

It can be seen from the objectives, larger CBW(i.e. 40MHz) is introduced for both PC3 and PC2 in band n28 UL, but the current maximum supported channel bandwidth is only 30MHz in the UL. Thus due to new larger channel bandwidth is introduced, some RF requirements should be defined accordingly.

In this contribution, we give some initial discussions on the RF requirements of 40MHz for n28 PC2.

# 2	Discussion

In terms of the description of the WID, evolve duplexer implementation is desired from industry, i.e. single band duplexer implementation, would be used for studied. This is different with the existing specs in which dual duplexer architecture for UE is assumed. Also, it should be noted the existing requirements for power class 3 with existing channel bandwidths up to and including 30MHz are not changed.

For PC2 band n28 40MHz, the following requirements should be evolved:

1. ΔMPR&A-MPR

2. RSD

1. ΔMPR&A-MPR

Similar as PC3 40MHz, the ΔMPR requirements for PC2 n28 40MHz. In current spec, ΔMPR are defined for PC2 n40 and n71, and it use the same ΔMPR requirements as PC3. To our understanding, ∆MPR is used for different implementations and accommodating the more pronounced PA memory effect. Given that band n71 falls within a comparable frequency band to band n28, it is anticipated that the similar approach would be reused. Therefore we propose to use the same ∆MPR as PC3.

Proposal 1: To use the same ∆MPR as PC3 for PC2 n28 40MHz.

For A-MPR, there are two network signalling (i.e. NS_17 and NS_18) for band n28 in current spec. During the R18, the PC2 A-MPR for 5/10/15/20/25/30MHz have been agreed and captured in WF [5], and the PC2 A-MPR are applicable for both dual-duplexer and full-band duplexer implementations. Thus only 40MHz is left for PC2 A-MPR evaluation.

For NS_17, the evaluation results from companies in R18 shows A-MPR requirements are needed for PC2.

2. RSD

The RSD requirement for both 1Tx and 2Tx of PC2 n28 up to 30MHz have already been approved in the WF[6], where average values from companies were used. Thus only RSD for PC2 n28 40MHz for both 1Tx and 2Tx should be evaluated. However, due to the RSD requirements is the delta degradation value on top of the PC3 REFSEN, and there are no agreements on PC3 n28 40MHz REFSEN requirement so far, so the RSD requirements for PC2 n28 40MHz for both 1Tx and 2Tx should be postponed until the corresponding PC3 REFENS requirements are completed.

Proposal 2: The RSD requirements for PC2 n28 40MHz for both 1Tx and 2Tx should be postponed until the corresponding PC3 REFENS requirements are completed.

# 3 Conclusion

In this contribution, we give some initial discussion on the RF requirements of 40MHz for n28 PC3. The conclusions are summarized in the following:

Proposal 1: To use the same ∆MPR as PC3 for PC2 n28 40MHz.

Proposal 2: The RSD requirements for PC2 n28 40MHz for both 1Tx and 2Tx should be postponed until the corresponding PC3 REFENS requirements are completed.

# 4	Reference

[1] RP-241660, New WID: Introduction of Power Class 2 and UE 40MHz Channel Bandwidth in NR band n28, CMCC, CBN

[2] R4-2400170, On n28 with full band duplexer, Apple

[3] R4-2407163, PC2 n28 NS_17, Skyworks Solutions, Inc.

[4] R4-2407624, Views on b/n28 full-band duplex, Skyworks Solutions, Inc.

[5] R4-2310245, WF on PC2 A-MPR for band n8 and n28, Huawei, HiSilicon, Apple, China Unicom, RAN4#107

[6] R4-2306544, WF on FDD HPUE Basket WIs, China Unicom