# 3GPP TSG-RAN WG4 Meeting # 112                                    R4-2411880

# Maastricht, Netherlands, Aug.19th – 23th, 2024

# Source: 	ZTE Corporation, Sanechips

# Title: 	Further discussion on R19 Increasing UE transmission power limit

# Agenda item:	8.1.1.1.3

# Document for:	Approval

# 1 Introduction

In last meeting, some agreements/open issues are included in the WF[1]. In this contribution, we give some further discussions on R19 increasing UE transmission power limit.

# 2	Discussion

The methodology was agreed as below.

- 4.1 The methodology Online agreement: Use the higherPowerLimit-R17 and higherPowerLimtMRDC-R17 capability as the starting point. - 4.4 MSD and SAR compliance Way forward: - If higherPowerLimit-R17/higherPowerLimtMRDC-R17 capability is adopted for increasing UE transmission power, then taking same methodology as for higherPowerLimit-R17/ higherPowerLimtMRDC-R17 for SAR compliance and MSD. More specific, - For a given band combination, no additional MSD requirements are specified in addition to the MSD requirements of its legacy power class(es) - No new duty-cycle solution/mechanism for SAR compliance is considered - P-MPR can be used for SAR mitigation

However, in terms of the brief summary in our contribution [2], we think the above methodologies are not enough. Our intention is to adopt the existing R17/R18 methodologies as much as possible. Therefore, we have the following proposal:

Proposal 1: For R19 increasing UE transmission power limit, to use the following existing R17/R18 methodologies:

- Use the higherPowerLimit-R17 and higherPowerLimtMRDC-R17 capability

- No new power class was introduced

- A general way for the eligible band combination to enable increase higher power limit feature is used in the NOTE description in TS38.101-1/3.

- With the above methodologies, what we should do is to define the supported cases (a.k.a. eligible band combination) to our understanding. For the eligible band combination for 2Tx inter-band NR CA and ENDC, we propose to consider some additional eligible band combinations as shown in the following table 1.

- Table 1: Eligible HPUE 2Tx inter-band NR CA and ENDC

| eligible BC combination PC | band nX PC (1Tx) | band nY PC (1Tx) | enable high power limits |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | total power (dBm) | single carrier in each band | intra-band UL CA in one of the band |
| PC3 | PC3 | PC5 | 24.6 | R18 support | R18 support |
| PC2 | PC3 | PC2 | 27.8 | R17 support | Propose to be included in R19 |

- The reasons for including PC2 band combination with PC3+PC2 where intra-band UL CA in one of the band is that there are already several such PC2 band combination are included in TS38.101-1 v18.5.0, and also there are more such PC2 band combination included in the R18 big CR[4] in the basket WID HPUE_FR1_TDD_NR_CADC_SUL_R18, as shown in the following table.

Table 6.2A.1.3-1: UE Power Class for uplink inter-band CA (two bands) (Extracted from TS38.101-1)

| Uplink CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ... |  |  |  |  |  |  |  |  |
| CA_n3A-n41C |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| ... |  |  |  |  |  |  |  |  |
| CA_n3A-n79C |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| ... |  |  |  |  |  |  |  |  |
| CA_n8A-n79C |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| ... |  |  |  |  |  |  |  |  |
| CA_n28A-n41C |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| ... |  |  |  |  |  |  |  |  |
| CA_n28A-n79C |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| ... |  |  |  |  |  |  |  |  |
| CA_n39A-n41C |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| ... |  |  |  |  |  |  |  |  |
| CA_n40A-n79C |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| ... |  |  |  |  |  |  |  |  |
| CA_n41A-n79C |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n41C-n79A |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| ... |  |  |  |  |  |  |  |  |
| .. NOTE 7: The UE that supports a PC2 uplink CA configuration with single carrier for each individual band and a composite of supporting PC3 within an NR TDD or FDD band and PC2 within a second NR TDD band may signal a higherPowerLimit-r17 capability whereby the maximum output power indicated in the table may be exceeded in accordance with sub-clause 6.2A.4.1.3. The power classes referenced are according to the reported ue-PowerClassPerBandPerBC-r17 if indicated or ue-PowerClass otherwise. NOTE 8: The UE that supports a PC3 uplink CA configuration with a composite of supporting PC3 within an NR TDD or FDD band and PC5 within a second NR band listed in Table 6.2F.1-1 may signal a higherPowerLimit-r17 capability whereby the maximum output power indicated in the table may be exceeded in accordance with sub-clause 6.2A.4.1.3. The power classes referenced are according to the reported ue-PowerClassPerBandPerBC-r17 if indicated or ue-PowerClass otherwise. |  |  |  |  |  |  |  |  |

- However, due to Rel-18 WID have already been completed, thus we think it should be included in Rel-19.

- It should be noted that the following three UL cluster cases are not supported: CA_nX(2A)-nYA, CA_nXA-nY(2A) , and combinations with four UL CCs are not supported in terms of the WF [4].

- Based on the above considerations, we have the following proposals:

- Proposal 2. To consider the following additional eligible PC2 2Tx inter-band NR CA and ENDC to enable increasing higher power limit in Rel-19:

- Inter-band with intra-band UL contiguous CA in one of the NR band

- For PC2/PC1.5 3Tx inter-band NR CA and ENDC, we also propose to consider some additional eligible band combinations as shown in the following table 2.

- Table 2: Eligible HPUE 3Tx inter-band NR CA and ENDC

| Eligible BC combination PC | band nX PC (1Tx) | band nY PC (2Tx) | enable high power limits |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | total power (dBm) | single carrier in each band | intra-band UL contiguous CA in one of the band |
| PC2 | PC3 | PC2 | 27.8 | Propose to be included in R19 |  |
| PC1.5 | PC3 | PC1.5 | 30 | Propose to be included in R19 |  |
|  | PC2 | PC1.5 | 30.8 | Propose to be included in R19 |  |

- In table 2, except for some existing configurations in Rel-18, one additional eligible configuration of PC2+PC1.5 is added to enable more than 30dBm total maximum power.

- Therefore, we have the following proposal:

- Proposal 3. To consider the following additional eligible HPUE 3Tx inter-band NR CA and ENDC with up to 3CC in UL bands to enable increasing higher power limit in Rel-19:

- PC2 band combination of PC3+PC2

- PC1.5 band combination of PC3+PC1.5

- PC1.5 band combination of PC2+PC1.5

- Note: Only PC3 is considered for LTE FDD in EN-DC

- Note: For 3UL CC, only inter-band with intra-band UL contiguous CA in one of the NR band is considered.

# 3 Conclusion

In this contribution, we give some discussions on the R19 increasing UE transmission power limit. the proposals and conclusions are:

Proposal 1: For R19 increasing UE transmission power limit, to use the following existing R17/R18 methodologies:

- Use the higherPowerLimit-R17 and higherPowerLimtMRDC-R17 capability

- No new power class was introduced

- A general way for the eligible band combination to enable increase higher power limit feature is used in the NOTE description inTS38.101-1/3.

- Proposal 2. To consider the following additional eligible PC2 2Tx inter-band NR CA and ENDC to enable increasing higher power limit in Rel-19:

- Inter-band with intra-band UL contiguous CA in one of the NR band

- Proposal 3. To consider the following additional eligible HPUE 3Tx inter-band NR CA and ENDC with up to 3CC in UL bands to enable increasing higher power limit in Rel-19:

- PC2 band combination of PC3+PC2

- PC1.5 band combination of PC3+PC1.5

- PC1.5 band combination of PC2+PC1.5

- Note: Only PC3 is considered for LTE FDD in EN-DC

- Note: For 3UL CC, only inter-band with intra-band UL contiguous CA in one of the NR band is considered.

# 4	Reference

[1] R4-2410750, WF on high power UE, Samsung

[2] R4-2408353, on R19 Increasing UE transmission power limit, ZTE Corporation

[3] R4-2408936, Topic summary for [111][125] NR_ENDC_RF_Ph4_part2, Samsung(Moderator)

[4] R4-2410652, WF on two band DL band combination template,ZTE Corporation, Skyworks, Nokia, Huawei, HiSilicon