# 3GPP TSG-RAN WG4 Meeting # 112                                    R4-2411881

# Maastricht, Netherlands, Aug.19th – 23th, 2024

# Source: 	ZTE Corporation, Sanechips

# Title: 	Further discussion on R19 2Tx/3Tx PC2/1.5 Inter-band NR 		CA/ENDC

# Agenda item:	8.1.1.1.2

# Document for:	Approval

# 1 Introduction

In last meeting, some agreements/open issues are included in the WF[1]. In this contribution, we give some further discussions on 2Tx/3Tx PC2/1.5 Inter-band NR CA/ENDC.

# 2	Discussion

- For the duty cycle solution for SAR compliance, the agreement is shown below:

- 1. General aspect - 1.1 Duty cycle solution for SAR compliance Online agreement: Duty cycle solution is considered for both intra-band CA and inter-band CA/EN-DC

- Moreover, the agreements on different duplex mode configurations are:

Way forward: - General requirements can be specified including considerations of different duplex mode configurations: - For PC2, FDD+FDD, FDD+TDD, TDD+TDD with 2Tx and 3Tx - For PC1.5, [FDD+FDD,] FDD+TDD, TDD+TDD with 2Tx and 3Tx

- For the duty cycle based SAR solution, we think it should be based on the scenarios. it would be foreseen that the duty cycle scheme is different for the band combination between PC2 FDD+PC2/1.5 TDD and PC2 TDD+PC2/1.5 TDD, since there is no duty cycle scheme introduced for PC2 FDD single band, instead P-MPR scheme is used to meet the SAR requirements. Therefore, we think there is no need to define duty cycle scheme for the 2Tx PC1.5 NR inter-band UL CA including PC2 FDD band since it would not proper to assume the 100% duty cycle for PC2 FDD band, unlike PC3 FDD band. In the other words, duty-cycle solution should be considered for limited scenarios.

- Proposal 1: duty-cycle solution should be considered for limited scenarios of PC2/PC1.5 NR inter-band UL CA.

- Only consider P-MPR scheme for the band combination including PC2 FDD band, i.e. FDD+FDD, FDD+TDD

- Except P-MPR, duty cycle scheme is applied to the band combination including pure TDD bands, i.e. TDD+TDD

- For the 3UL CC with 3Tx, the agreements are:

- 3.3 3UL CC with 3Tx Way forward: RAN4 to further check whether there is demand for 3 ULCC with 3Tx scenario (such as CA_nXA-nY(2A) and CA_ nXA-nYB), and whether it is desirable to be introduced in Rel-19.

- In our understanding, it is difficult to check whether there is demand from operator for the time being since the demands from operators are usually changed as time goes by. Hence, it would be premature to exclude 2 bands 3UL CC with 3Tx scenarios.

- Actually, in the R19 HPUE basket WID[4][5], there are no restrictions for the UL CC number, instead there are limitation for the UL band number which limits to 2 bands. From this aspect, it may not appropriate to exclude 3UL CC for 3Tx scenario.

- However, there are some agreements for the valid UL configurations for PC3 band combination, which can be found in the WF for the TR template [6], as shown below:

# Annex B: Valid UL configurations For CA_nX-nY two band DL inter band combinations, the following UL configurations are applicable: - One band UL with one CC: nXA, nYA - One band UL with two CC: CA_nXB, CA_nXC, CA_nX(2A), CA_nYB, CA_nYC, CA_nY(2A) - two band UL with one CC per band: CA_nXA-nYA - two band UL with two CC in one band: CA_nXB-nYA, CA_nXC-nYA, CA_nXA-nYB, CA_nXA-nYC The following three UL cluster cases are not supported: CA_nX(2A)-nYA, CA_nXA-nY(2A) Combinations with four UL CCs are not supported.

- We can see that the 3UL CC with inter-band +intra-band non-contiguous UL CA (i.e.CA_nX(2A)-nYA, CA_nXA-nY(2A)) are not supported for R19 PC3 band combination, considering PC3 band combination is the fundamental band combination, so 3ULCC of CA_nXA-nY(2A) should be excluded from 3Tx scenario.

- Proposal 2: 3UL CC of CA_nXA-nY(2A) should be excluded from 3Tx scenario.

# 3 Conclusion

In this contribution, we give some further discussions on 2Tx/3Tx PC2/1.5 Inter-band NR CA/ENDC. The proposals and conclusions are:

- Proposal 1: duty-cycle solution should be considered for limited scenarios of PC2/PC1.5 NR inter-band UL CA.

- Only consider P-MPR scheme for the band combination including PC2 FDD band, i.e. FDD+FDD, FDD+TDD

- Except P-MPR, duty cycle scheme is applied to the band combination including pure TDD bands, i.e. TDD+TDD

- Proposal 2: 3UL CC of CA_nXA-nY(2A) should be excluded from 3Tx scenario.

# 4	Reference

[1] R4-2410750, WF on high power UE, Samsung

[2] R4-2408352, On R19 HUPE 2Tx/3Tx Inter-band NR CA/ENDC, ZTE Corporation

[3] R4-2408936, Topic summary for [111][125] NR_ENDC_RF_Ph4_part2, Samsung(Moderator)

[4] RP-241678, New WID: Rel-19 High power UE (power class 1.5 or 2) for Dual Connectivity (DC) combinations of LTE band(s) and NR band(s) RAN4 vice-chair (China Telecom)

[5] RP-241679, New WID: Rel-19 High power UE (power class 1.5 or 2) for NR intra-band Carrier Aggregation (CA) or NR inter-band CA/Dual connectivity (DC) band combinations with/without NR SUL (supplementary uplink), RAN4 vice-chair (China Telecom)

[6]R4-2410652, WF on two band DL band combination template,ZTE Corporation, Skyworks, Nokia, Huawei, HiSilicon