# 3GPP TSG-RAN WG4 Meeting # 112                                    R4-2411882

# Maastricht, Netherlands, Aug.19th – 23th, 2024

# Source: 	ZTE Corporation, Sanechips

# Title: 	Further discussion on R19 PC1.5 Intra-band UL CA

# Agenda item:	8.1.1.1.1

# Document for:	Approval

# 1 Introduction

In last meeting, some agreements/open issues are included in the WF[1]. In this contribution, we give some further discussions on PC1.5 intra-band contiguous and non-contiguous UL CA.

# 2	Discussion

- For the duty cycle solution for SAR compliance, the agreement was shown below:

- 1. General aspect 1.1 Duty cycle solution for SAR compliance Online agreement: Duty cycle solution is considered for both intra-band CA and inter-band CA/EN-DC

As discussed before, the duty cycle solution for SAR compliance is never used in the filed. Nevertheless, we think it is still needed to be considered for PC1.5 intra-band CA from the completeness of specification due to duty cycle solution is already defined for PC2 intra-band CA in the spec. Otherwise, it may cause specification improvements or maintenance problem in the future.

According to the following general descriptions for intra-band contiguous and non-contiguous CA in sub-clause 4.2:

d)	All the requirements for intra-band contiguous and non-contiguous CA apply under the assumption of the same slot format indicated by UL-DL-configuration-common in the PCell and SCells for NR SA.

The above text are described in power class agnostic manner, so it is nature to be applicable for PC1.5 intra-band contiguous and non-contiguous CA. Due to the component carries in the UL intra-band CA are synchronized, it is feasible to reuse the capability for single carrier case.

- Proposal 1: For duty cycle based SAR solution, reuse the PC1.5 single CC capability for PC1.5 TDD UL intra-band contiguous and non-contiguous CA.

PCMAX,C is the configured maximum output power for serving cell c, the agreement was shown below:

- 2.4 PCMAX,C Proposals: - Proposal 1: Define PCMAX,c limitation for each component carrier considering PC1.5 UE architecture of intra-band UL CA. (LGE) - For dualPA-architecture - PCMAX,C limitation for each component carrier is 26dBm - For TxD (dualTx) - PCMAX,CC1 = 26+10∗log10( LCRB1∗SCS1LCRB1∗SCS1+LCRB2∗SCS2)+3 - PCMAX,CC2 = 26+10∗log10( LCRB2∗SCS2LCRB1∗SCS1+LCRB2∗SCS2)+3 - Proposal 2: Changes to TS 38.101-1 for dual-PA architecture, inset new equation for PCMAX,c (Ericsson, more details refer to R4-2407721)

For PC1.5 TDD intra-band CA band combination, 2PA (i.e. 2*26dBm) shall be used due to curretly there is no commercial 29dBm single PA. Thus, the PCMAX,C limitation for each component carrier is 26dBm for dualPA-architecture.

For TxD(dualTx), in our understanding, equal maximum Tx power should be applied for each Tx. For each Tx support intra-band CA, the total of PCMAX,C limitation (i.e. PCMAX) should be 26dBm. The proposal 1 for TxD in the WF seems that equal PSD is assumed for each carrier, then the power scaling between the two CC in each Tx chain depends on the RB bandwidth scaling factor. However, as commented by company in last meeting, the unequal PSD exists in field.

For intra-band UL contiguous CA with TxD architecture, UL MIMO can be supported as well. When we look at configured transmitted power for intra-band UL contiguous CA with UL MIMO, the PCMAX,c of intra-band contiguous CA is applied, where PCMAX,c of single carrier is applied to intra-band CA. There are no PCMAX,C limitation for current intra-band UL CA with UL MIMO or TxD. Therefore, we think the same PCMAX,C limitation for each component carrier can also be applied for TxD.

Proposal 2.  For the PCMAX,C of PC1.5 UE architecture of intra-band UL CA:

- For dualPA-architecture

  - PCMAX,C limitation for each component carrier is 26dBm

- For TxD(dualTx)

  - PCMAX,C limitation for each component carrier is 29dBm

PCMAX is the total configured maximum output power, the agreement was shown below:

## 2.5 PCMAX Proposals: - Option 1: (Skyworks) - MOP for 2Tx architectures (TxD w/wo UL MIMO) is: 29 dBm - MOP for Dual-PA architectures (one PA/CC, 2LO) is: PCmax= 29 + 10*log(1/2*(1+Min(LCRB1*SCS1,LCRB2*SCS2)/Max(LCRB1*SCS1,LCRB2*SCS2))) dBm - Option 2: (Skyworks) MPR for Dual-PA architectures (one PA/CC, 2LO) is modified by: 10*log(1/2*(1+Min(LCRB1*SCS1,LCRB2*SCS2)/Max(LCRB1*SCS1,LCRB2*SCS2))) dB - Option 3: (Captured in last meeting’s WF) For R19 PC1.5 intra-band non-contiguous /contiguous ULCA with two 26dBm PAs and one PA per CC, the PCMAX is modified as follows to account for RB BW imbalances PCmax=10*log(10^(26/10) + 10^((26-10*log(LCRB1*SCS1/(LCRB2*SCS2)))/10)) - Option 4: For UEs indicating dualPA-Architecture (Architecture #1) the total configured output power PCMAX for the band combination is given by clause 6.2A.4.1.3 for inter-band CA in the current version of 38.101-1, the same applies for TxD (Ericsson)

It seems the equations in option 1/2/3 also assumed the equal PSD is applied to each carrier. We think the RF requirements should apply for any scenarios. As Pcmax means total configured maximum output power of each carrier(i.e. Pcmax,c), so it is straightforward that the Pcmax for 2Tx architecture is limited by 29dBm.

Proposal 3.  For PC1.5 UE architecture of intra-band UL CA, PCMAX limitation for 2Tx architecture is 29dBm.

# 3 Conclusion

In this contribution, we give further discussions on PC1.5 Intra-band contiguous and non-contiguous UL CA. The proposals and conclusions are:

Proposal 1: For duty cycle based SAR solution, reuse the PC1.5 single CC capability for PC1.5 TDD UL intra-band contiguous and non-contiguous CA.

Proposal 2.  For the PCMAX,C of PC1.5 UE architecture of intra-band UL CA:

- For dualPA-architecture

  - PCMAX,C limitation for each component carrier is 26dBm

- For TxD(dualTx)

  - PCMAX,C limitation for each component carrier is 29dBm

Proposal 3.  For PC1.5 UE architecture of intra-band UL CA, PCMAX limitation for 2Tx architecture is 29dBm.

# 4	Reference

[1] R4-2410750, WF on high power UE, Samsung

[2] R4-2408351, On R19 PC1.5 Intra-band contiguous and non-contiguous UL CA, ZTE

[3] R4-2408936, Topic summary for [111][125] NR_ENDC_RF_Ph4_part2, Samsung(Moderator)