# 3GPP TSG-RAN WG4 Meeting # 112                                    R4-2411883

# Maastricht, Netherlands, Aug.19th – 23th, 2024

# Title: 	Further discussion on 6Rx Reference sensitivity requirements

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	8.1.1.3.1

# Document for:	Approval

# 1 Introduction

In RAN #104 meeting, the WID of UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4 was revised in [1], wherein the objectives related to 6Rx for handheld UE and FWA UE are listed as below:

6Rx for handheld and FWA UE [RAN4, RAN1] - Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario - Example bands: n41, n77/n78, n79, n104 - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers - Specify the Rx requirements including reference sensitivity requirements for support 6Rx - Note: the specified requirements can be applicable to both handheld UE and FWA devices - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, depending on UE capability - RAN1 to study and specify, if necessary, SRS antenna switching to support 3T6R depending on UE capability. - Note: RAN1 discussion starts from RAN1#118bis (Oct. 2024) - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution.

In last meeting, some agreements are included in the approved WF[2].

In this contribution, we give some further discussions on the UE REFSEN requirements related to 6Rx.

# 2	Discussion

In WF[2], the agreements related to REFSEN (ΔRIB,6R) are extracted in the following:

# Topic 1: REFSENS (delta RIB,6R) ## Sub-topic 1-1: General considerations for specifying ΔRIB,6R value Issue 1-1-1: Whether band n104 should be included in the high band (n77, n78 and n79) category for 6Rx case Agreement: For ΔRIB,6R, include n77, n78, n79 and [n104] into the same group. Issue 1-1-2: Whether to use same ΔRIB,6R value for handheld UE and FWA Way Forward: Further discuss the following options. - Option 1: RAN4 needs to determine whether to define different ΔRIB,6R value for handheld UE and FWA separately - Option 2: Same value for handheld UE and FWA - Option 3: Different value for handheld UE and FWA Issue 1-1-3: Release independence of ΔRIB,6R value Way Forward: Follow RAN4 Chair guidance and defer the release independence discussions until the requirements are stable. ## Sub-topic 1-2: ΔRIB,6R values for handheld UE and FWA Issue 1-2-1: Proposed ΔRIB,6R values for handheld UE and FWA Way Forward: Further discuss the following options for the ΔRIB,6R values. Ultimately, the decision to specify separate or same requirements for handheld UE and FWA will be dependent on outcome of Issue 1-1-2. - Option 1: Consider range of ΔRIB,6R in brackets for each case below for further discussion. Operating bands; ΔRIB,6R for FWA; ΔRIB,6R for Handheld. n77, n78, n79, [n104]; [-3.0 ~ -3.3] dB; [-3.0 ~ -3.2] dB. n41; [-3.0 ~ -4.0] dB; [-3.0 ~ -4.0] dB. - Option 2: Consider average value of ΔRIB,6R from company proposals for further discussion. - Option 3: Other options are not precluded.

Similar with ΔRIB,4R, and ΔRIB,8R, ΔRIB,6R is also the reference sensitivity adjustment due to support for 6Rx antenna ports. Herein, the reference sensitivity adjustment means the reference sensitivity for 2Rx antenna ports are modified by ΔRIB,6R. This is applied for all the bands who support 6Rx antenna ports.

Regarding n104, some companies in last meeting mentioned that the frequency is much higher which may need more discussion considering more difficult implementations like RF loss, antenna mismatch, and etc.

In our understanding, the ΔRIB,xR is the reference sensitivity adjustment comparison of its own 2Rx reference sensitivity. Indeed, there exist implementation margins for a certain band in reality, not only for 2Rx, but also for 4/6/8Rx. Currently, the ΔRIB,4R, and ΔRIB,8R requirements are defined with different band groups. Due to band n104 have already been grouped together with n77/n78/n79 for 4Rx, so we think the same approach would be reused for 6Rx.

Proposal 1. For ΔRIB,6R, include n77, n78, n79 and n104 into the same group.

Regarding whether to use same ΔRIB,6R value for handheld UE and FWA, we understand the different restrictions are existed for handheld UE and FWA UE. However, due to the examples band in the WID belong to high band, it is anticipated that the issue caused by the antenna limitation/implementation is not as significant as in the sub-1GHz band. Therefore, it seems there is no need to define separated values for handheld UE and FWA UE.

Proposal 2. The same ΔRIB,6R value is used for handheld UE and FWA.

Regarding the values, compared to 4Rx and 8Rx, the ΔRIB,6R value should be in between ΔRIB,4R and ΔRIB,8R values due to 6Rx has less implementation challenges than 8Rx but more challenges than 4Rx. The ΔRIB,6R values would be:

Table 1: Range for ΔRIB,6R

| Operating band | ΔRIB,6R (dB) |
| --- | --- |
| n41 | (-4.3, -2.7) |
| n77, n78, n79, n104 | (-4.0, -2.2) |

According to the above, it is propose to adopt ΔRIB,6R = -3.5dB for band n41, and ΔRIB,6R = -3dB for n77/n78/n79/n104.

Proposal 3. ΔRIB,6R = -3.5dB for band n41, and ΔRIB,6R = -3dB for n77/n78/n79/n104.

In addition, it can be predicted that companies will propose different values in the Aug. Meeting, the situation may be the same with the previous meeting although the gap among all the values are small. In this case, we also think adopt the average value from company’s proposals is a good way.

Proposal 4. To use average value of ΔRIB,6R from company proposals if there are different values proposed by companies.

Moreover, another aspect of diversity characteristics should be discussed for 6Rx either. We summarize the diversity characteristics for 2Rx, 4Rx and 8Rx single carrier Rx requirements in the spec as below:

Table 2: diversity characteristics for 2Rx/4Rx/8Rx

|  | Single carrier REFSEN requirement | Other single carrier Rx requirements |
| --- | --- | --- |
| UE is equipped with 2Rx | 2Rx | 2Rx |
| UE is equipped with 4Rx | 2Rx, 4Rx | 4Rx, skip 2Rx |
| UE is equipped with 8Rx | 2Rx, 4Rx, 8Rx | 8Rx, skip both 2Rx and 4Rx |

When 6Rx is introduced, in order to reduce the test burden, the diversity characteristics aims for the requirements test verification shall be discussed. We think the similar method as 4Rx/8Rx could be reused, which are:

- For single carrier REFSEN requirements, additional requirement for four Rx antenna ports and six Rx antenna ports shall be verified in operating bands where the UE is equipped with six Rx antenna ports.

- For Rx requirements other than single carrier REFSEN, the UE shall be verified with six Rx antenna ports and skip both two and four Rx antenna ports requirements in operating bands where the UE is equipped with six Rx antenna ports.

We have the following proposal 5 for diversity characteristics:

Proposal 5. When the UE is equipped with six Rx antenna ports:

- For single carrier REFSEN requirements, additional requirement for four Rx antenna ports and six Rx antenna ports shall be verified in operating bands;

- For Rx requirements other than single carrier REFSEN, the UE shall be verified with six Rx antenna ports and skip both two and four Rx antenna ports requirements in operating bands.

# 3 Conclusion

In this contribution, we give some further discussion on the UE REFSEN requirements to support 6Rx, including REFSEN ∆TRxSRS values. The conclusions are:

Proposal 1. For ΔRIB,6R, include n77, n78, n79 and n104 into the same group.

Proposal 2. The same ΔRIB,6R value is used for handheld UE and FWA.

Proposal 3. ΔRIB,6R = -3.5dB for band n41, and ΔRIB,6R = -3dB for n77/n78/n79/n104.

Proposal 4. To use average value of ΔRIB,6R from company proposals if there are different values proposed by companies.

Proposal 5. When the UE is equipped with six Rx antenna ports

- For single carrier REFSEN requirements, additional requirement for four Rx antenna ports and six Rx antenna ports shall be verified in operating bands;

- For Rx requirements other than single carrier REFSEN, the UE shall be verified with six Rx antenna ports and skip both two and four Rx antenna ports requirements in operating bands.

# 4	Reference

[1]  RP-241656, UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, RAN4 chair (Huawei)

[2]  R4-2410751, WF on requirements for 6Rx, AT&T

[3]  R4-2405227 Discussion on 6Rx UE RF requirements, ZTE