# 3GPP TSG-RAN WG4 Meeting # 112                                    R4-2411884

# Maastricht, Netherlands, Aug.19th – 23th, 2024

# Title: 	Further discussion on 6Rx SRS antenna switching requirements

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	8.1.1.3.3

# Document for:	Approval

# 1 Introduction

In RAN #104 meeting, the WID of UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4 was revised in [1], wherein the objectives related to 6Rx for handheld UE and FWA UE are listed as below:

6Rx for handheld and FWA UE [RAN4, RAN1] - Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario - Example bands: n41, n77/n78, n79, n104 - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers - Specify the Rx requirements including reference sensitivity requirements for support 6Rx - Note: the specified requirements can be applicable to both handheld UE and FWA devices - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, depending on UE capability - RAN1 to study and specify, if necessary, SRS antenna switching to support 3T6R depending on UE capability. - Note: RAN1 discussion starts from RAN1#118bis (Oct. 2024) - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution.

In last meeting, some agreements are included in the approved WF[2].

In this contribution, we give some initial discussions on the SRS antenna switching related to 6Rx.

# 2	Discussion

In WF[2], the agreements related to SRS antenna switching are extracted in the following:

# Topic 2: SRS antenna switching and ΔTRxSRS ## Sub-topic 2-1: General considerations for SRS antenna switching and ΔTRxSRS Issue 2-1-1: SRS antenna switching configurations Way forward: RAN4 to focus on discussing ∆TRxSRS for 1tr6 and t2r6 until RAN1 concludes work on 3t6r and 4t6r. Pending RAN1 feedback, RAN4 can consider any necessary updates to the work scope in a Revised WID for the SRS antenna configurations subject to RAN Plenary approval. Issue 2-1-2: Whether to use different ∆TRxSRS based on operating frequency Way forward: RAN4 defines different ∆TRxSRS values based on operating frequency. Further discuss whether to introduce an additional breakpoint for bands whose FUL_high is higher than the FUL_low of n104. ## Sub-topic 2-2: ΔTRxSRS values Issue 2-2-1: Proposed ∆TRxSRS values Way forward: Further discuss the following options for the ∆TRxSRS requirements. - Option 1: Consider average value of ∆TRxSRS for further discussion. Operating bands; ΔTRxSRS requirement for t1r6; ΔTRxSRS requirement for t2r6; ΔTRxSRS requirement for t1r6-t2r6. Band n41, n77, n78; 4.0 dB; 3.4 dB; 4.4 dB. Band n79, [n104]; 5.3 dB; 4.5 dB; 5.7 dB. - Option 2: Consider range of ∆TRxSRS in brackets for each case below for further discussion. Operating bands; ΔTRxSRS requirement for t1r6; ΔTRxSRS requirement for t2r6; ΔTRxSRS requirement for t1r6-t2r6. Band n41, n77, n78; [2.8 ~ 5.0] dB; [2.3 ~ 4.0] dB; [3.3 ~ 5.5] dB. Band n79, [n104]; [3.6 ~ 6.0] dB; [3.0 ~ 5.5] dB; [4.2 ~ 6.5] dB. Note: Maximum level for t1r6 corrected for OPPO proposals per OPPO. - Option 3: Other options are not precluded.

Regarding the SRS antenna switching cases, it is clear that t4r6 is removed from the WID, and for t3r6, RAN1 will do some studies from RAN1 #118bis(Oct. 2024.). In other words, RAN4 should still postpone the discussion on t3r6 antenna switching in Aug. Meeting.

Observation: t4r8 is removed, and t3r6 will be under RAN1’s studied.

Therefore, the discussion in Aug. Meeting should only focus on the left two ones, i.e. t1r6, t2r6, it seems not only the pure t2r6 antenna switch is considered, but also the back compatibility antenna switch of t1r6, and their combinations should be considered. So the SRS IL requirements to be defined should consider the following SRS antenna switch scenarios.

1. t1r6

2. t2r6

3. t1r6-t2r6

For the ∆TRxSRS requirements, we have the same proposals in the last meeting, which are:

Proposal 1. Apply 4.5dB for n41/n77/n78 and 5.5dB for n79 for t1r6 ∆TRxSRS requirements.

Proposal 2. Apply 4dB for n41/n77/n78 and 5dB for n79 for t2r6 ∆TRxSRS requirements.

Proposal 3. Apply 5dB for n41/n77/n78 and 6dB for n79 for t1r6-t2r6 ∆TRxSRS requirements

For whether to introduce an additional breakpoint for bands whose FUL_high is higher than the FUL_low of n104, when we look at 4Rx, there are no such additional breakpoint for band n104. We try to recall the history but we found there are no discussions on the ∆TRxSRS requirement when n104 was introduced. It means the current values (breadpoint for n79) could be reused. Given the similar situation for 4Rx and 6Rx, if RAN4 decide to introduce an additional breakpoint for bands whose FUL_high is higher than the FUL_low of n104, then it is also needed to revisit the 4Rx ∆TRxSRS requirement for n104.

Proposal 4. If introduce an additional breakpoint for bands whose FUL_high is higher than the FUL_low of n104 for 6Rx, then it is also needed to revisit the ∆TRxSRS requirement for n104 4Rx.

However, companies proposed different ΔTRxSRS requirements for the above three SRS antenna switch scenarios in last meeting, which was summarized in the moderator’s summary, shown in table 1.

Table 1.  Averaged values based on proposals in moderator summary R4-2408937

|  | Operating Bands | ΔTRxSRS t1r6 (dB) | ΔTRxSRS t2r6 (dB) | ΔTRxSRS t1r6-t2r6 (dB) |
| --- | --- | --- | --- | --- |
| Xiaomi | Band n41, n77, n78 | 2.8 | 2.3 | 3.3 |
|  | Band n79, n104 | 3.6 | 3 | 4.2 |
| Spreadtrum | Band n41, n77, n78 | 3.5 | 3 | 3.5 |
|  | Band n79, n104 | 5 | 4.5 | 5 |
| vivo | Band n41, n77, n78 | 4 | 3.5 | 5.5 |
|  | Band n79 | 5 | 4 | 6.5 |
| ZTE | Band n41, n77, n78 | 4.5 | 4 | 5 |
|  | Band n79 | 5.5 | 5 | 6 |
| OPPO | Band n41, n77, n78 | 6 | 3 | 4.5 |
|  | Band n79 | 7.5 | 3.8 | 6 |
| Qualcomm | Band n41, n77, n78 | 4 | 4 | 4.5 |
|  | Band n79, n104 | 5.5 | 5.5 | 6 |
| Huawei | Band n41, n77, n78 | 4 | 4 | 4.5 |
|  | Band n79 | 5.5 | 5.5 | 6 |
| Ericsson | Band n41, n77, n78 | 3.5 | 3.5 | 4 |
|  | Band n79 | 5 | 5 | 5.5 |
| Average | Band n41, n77, n78 | 4.0 | 3.4 | 4.4 |
|  | Band n79, [n104] | 5.3 | 4.5 | 5.7 |

It is true that different RFFE architecture would cause different ∆TRxSRS values since the IL depends on which types of and how many switchers (such as SPDT, DPDT, SP4T, and etc). However, there are no discussion and agreements on the reference architectures and assumption, instead RAN4 discussed the value directly. So it is hard to say who is right or who is wrong although we think some values may not be reasonable.

Considering the above, using average value is a straightforward and fairness way.

Proposal 5. To use the average value of ∆TRxSRS if there are different values proposed by companies.

The values are based on the proposed requirements in the last meeting, and they will be updated if some original values are updated or more values are included.

# 3 Conclusion

In this contribution, we give some further discussion on the 6Rx SRS antenna switching, including REFSEN ∆TRxSRS values. The conclusions are:

Observation: t4r8 is removed, and t3r6 will be under RAN1’s studied.

Proposal 1. Apply 4.5dB for n41/n77/n78 and 5.5dB for n79 for t1r6 ∆TRxSRS requirements.

Proposal 2. Apply 4dB for n41/n77/n78 and 5dB for n79 for t2r6 ∆TRxSRS requirements.

Proposal 3. Apply 5dB for n41/n77/n78 and 6dB for n79 for t1r6-t2r6 ∆TRxSRS requirements.

Proposal 4. If introduce an additional breakpoint for bands whose FUL_high is higher than the FUL_low of n104 for 6Rx, then it is also needed to revisit the ∆TRxSRS requirement for n104 4Rx.

Proposal 5. To use the average value of ∆TRxSRS if there are different values proposed by companies.

# 4	Reference

[1]  RP-241656, UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, RAN4 chair (Huawei)

[2]  R4-2410751, WF on requirements for 6Rx, AT&T

[3]  R4-2405227, Discussion on 6Rx UE RF requirements, ZTE