# 3GPP TSG-RAN WG4 Meeting # 112                                    R4-2411886

# Maastricht, Netherlands, Aug.19th – 23th, 2024

# Title: 	View on Fragmented carrier

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	8.6.3

# Document for:	Approval

# 1 Introduction

In RAN #104 meeting, a new SID on NR FR1 DL Fragmented Carriers was approved in [1], where the objectives are listed as below:

The objective of this study is as follows: - Identify methods for reducing the number of UE Rx chains (e.g. from separate RF chains per CC to shared RF chains ) needed for single DL band with frequency span ≤ 100 MHz, containing two non-contiguous CCs within a CA combination for the inter-operator co-located scenario, considering: - Which RF requirements could be adjusted for the inter-operator co-located BS scenario, e.g. existing UE RF requirements such as ΔRIBNC, ACS and in-band blocking [RAN4]; - Other requirements are not precluded, if identified - The ability to semi-statically switch hardware resources (i.e. Rx chains) [RAN4, RAN2 – See note 2]; - Up to 6 dB DL received power spectral density imbalance between the two non-contiguous CCs [RAN4]; - Determine a reasonable level for the power spectral density difference between carriers of co-located adjacent channel operators for study [RAN4]; - Impacts on DL performance [RAN4] ; - Means for a UE to inform the network of appropriate CA configuration it can support with adjusted RF requirements [RAN4, RAN2 – See note 2].

In this contribution, we give some initial discussions on fragment carriers by reducing the Rx chain.

# 2	Discussion

In terms of the objectives in the SID, the scenario mainly focus on the intra-band non-contiguous DL CA with 2CC configured (i.e. DL CA_nX(2A)), where the frequency span for the DL band is restricted to 100MHz. However, there are no example CA combination mentioned in the SID. Noted that we interprated the frequency span≤100MHz is for the whole DL band, rather than for the outermost two CCs.

It is well known that the DL frequency range of almost all of the FDD band are smaller than 100MHz. However, there are few of TDD band such as band n39 and band n40 whose frequency range is also smaller than or equal to 100MHz. Although the bands operated as intra-band non-contiguous CA configurations meet the above conditions are all FDD bands in the latest TS38.101-1, the DL CA_n39(2A) and DL CA_n40(2A) are requested in the R19 basket WID[2] by operator. Therefore, both TDD and FDD intra-band DL contiguous CA should be included in this SID.

Proposal 1. Both TDD and FDD intra-band DL contiguous CA should be included.

For FDD band intra-band non-contiguous CA, the UL Tx noise caused by Tx chain/PA leakage and Tx spectrum spreading into DL Rx path shall be taken into account since the duplexer gap for some FDD bands are small, and such small duplexer gap cannot provide enough attenuation for the filters design. The UL Tx noise falls into DL Rx associated with its own Rx IIP2 will cause the REFSENS degradation. In the TS38.101-1, such degradation is given as ΔRIBNC requirements for FDD intra-band non-contiguous CA, including one UL PCC configuration and PCC+SCC UL configuration.

However, unlike FDD, there is no duplexer gap for TDD band, so there are no ΔRIBNC requirements defined for TDD intra-band non-contiguous CA.

Observation 1. No ΔRIBNC requirements defined for TDD intra-band non-contiguous CA.

In current TS38.101-1 spec, the Rx requirements for intra-band non-contiguous CA are derived based on the separate Rx chain assumption, which is one Rx chain corresponding to one CC handling. This approach comes from LTE. In our understanding, separate Rx chain means there are separate LO and filter for each CC handling. An example for intra-band non-contiguous CA reference transmitter architecture can be found in Fig1.

Fig 1. Example for intra-band non-contiguous CA reference transmitter architecture (Rx chain with separate LO)

In Fig 1, although we say it is separate Rx chain, some RF component such as external LNA would still be shared (i.e. partial separate RF chain or somehow partial share RF chain.).

This SID aims to reduce the UE Rx chain number such as from separate RF chains per CC to shared RF chains for the CCs handling. 	An example for intra-band non-contiguous CA reference transmitter architecture by reducing the UE Rx chain number can be found in Fig2.

Fig 2. Example for intra-band non-contiguous CA reference transmitter architecture (Rx chain with share LO)

In Fig 2, only 1 Rx chain is needed for CC1+CC2 handling if the conditions for the two CCs (such as PSD imbalance ≤ 6dB..) are met. Comparing with Fig 1, there is one Rx chain left which can support more other DL carriers. For the one Rx chain, there is only one LO which can do the down-conversion for both CC1 and CC2. It can be seen as share RF chain. Here we don’t consider the other RF components which may still separate.

In terms of the above example Rx chains in Fig 1 and Fig 2, we can see that the big difference is that the filter is shared for the two DL carriers in the share RF chain architecture in Fig 2, which means the shared filter cannot filter out each carrier anymore. the interference appeared in the gap is located in the pass band of the filter, and the filter cannot provide the attenuation in the gap between the two DL carriers.

The ΔRIBNC requirements for FDD intra-band NC CA would become larger since the Tx leakage from PCC to SCC would increase due to the attenuation of the shared filter is lower than the attenuation of the separated filter for SCC although reduce the Wgap can alleviate the interference from PCC. An illustration for ΔRIBNC requirements for FDD intra-band non-contiguous CA can be found in Fig 3.

Fig 3. Iillustration for ΔRIBNC requirements for FDD intra-band non-contiguous CA(worst case)

In current spec, the ΔRIBNC requirements are defined relate to Wgap, in some cases or a certain band combination, 0dB ΔRIBNC is defined. Due to share Rx chain, we think ΔRIBNC should be re-evaluated and reuse the existing test points is reasonable.

Proposal 2. Re-evaluated the ΔRIBNC requirements based on the existing test points.

For the maximum input level requirement, it mainly related to dynamic range of LNA. No matter UEs have two separate RF chains (LOs) or share RF chain, due to the front end LNA is still directly exposed to the duplexer, the dynamic range of LNA would be impacted by the total power received over the operating band for intra-band non-contiguous CA UEs. Therefore, we think there is no impact on maximum input level requirements by reducing the number of UE Rx chains.

Observation 2.There is no impact on maximum input level requirements by reducing the number of UE Rx chains.

By reducing the Rx chain, we think the mainly impacts are the requirements between the two DL carrier, a.k.a in-gap requirements. Except the ACS/IBB requirements, we think other Rx requirements such as Narrow band blocking should also need to be check since both in-gap and out-of-gap requirements are defined in the spec.

Observation 3. The in-gap Rx requirements should be checked when reducing the Rx chain.

Proposal 3. Except in-gap ACS/IBB, the in-gap narrow band blocking should also need to be checked when reducing the Rx chain.

For ACS and in-band-blocking, they are already identified in the SID. For the in-gap ACS and in-gap in-band-blocking requirements, it is challenge for a UE to handle high interference signal level falls into the passband of the shared RF filter.

For intra-band NC CA ACS, in-band-blocking and narrow-band blocking requirements testing, in case of larger gap width, there are for test cases for the two DL carriers: in-gap tests with positive and negative offsets and out-of-gap tests with positive and negative offsets, and all the DL non-contiguous carriers are tested simultaneously, i.e. all tha DL carriers are active. For each of the four tests, each downlink carrier satisfy the ACS/IBB requirements of the single carrier. Moreover, if the gap width is small, usually the in-gap tests can be ignored.

Moreover, the in-gap tests is applied only if the Wgap satisfy the equation conditions as specified in the spec, which is:

In our understanding, the existing conditions of the Wgap shall be also applied for the shared RF chain.

Observation 4. For the two DL carrier, each carrier satisfy the ACS/IBB/NBB requirements of the single carrier for in-gap and out-of-gap testing.

Proposal 4: For the in-band gap test of ACS, IBB and NBB requirements, the existing conditions of the Wgap shall be also applied for the shared RF chain.

ACS value is related to co-existence study, so we think it should be caution to modify the ACS dB level when reducing the Rx chain. Assuming that the ACS dB level is remained, then higher interference signal level would case more REFSEN degradation, as shown in Fig 4.

Fig 4. In-gap high interference cause high REFSEN degradation to maintain the same ACS level

In Fig 4, to keep the same ACS dB level, the interference signal level increase, the REFSEN degradation increase. This is similar for in-gap IBB requirements.

As mentioned in the SID, it is for inter-operator co-located BS scenario and check whether the existing Rx requirements can be adjusted. It is not clear for the ‘RF requirements adjust’. In our understanding, the in-gap interference level between inter-operators depends on the coordination between the operators, but it seems it is highly implementation dependence. Maybe the in-gap interference level from operator A to operator B can keep in a certain low level which will not cause additional degradation. But anyway, we think the existing ACS(i.e. value in Table 7.5-1 in TS38.101-1) level should be maintained.

Proposal 5. The existing ACS level should be maintained when reducing the Rx chain number.

Therefore, the adjustment for the RF requirements would be the test parameters like Pinterference, Pwanted and so on if high in-gap interference level is existed for inter-operator co-located BS scenario.

Moreover, it mentioned that up to 6 dB DL received power spectral density imbalance between the two non-contiguous CCs. Note that it is PSD imbalance, rather than power imbalance. The power imbalance for the two CC equals to PSD imbalance + 10log(BWCC1/BWCC2), specially, when BWCC1=BWCC2, them PSD imbalance equal to power imbalance. When the BWCC1>BWCC2, then the power imbalance would be larger than 6dB. In the other words, to keep the PSD imbalance, the power for the carrier who has larger channel bandwidth

Observation 5. With the PSD imbalance between two DL carriers up to 6dB, the power imbalance would be larger than 6dB considering the asymmetric carrier channel bandwidth.

# 3 Conclusion

In this contribution, we give some initial discussion on fragment carriers by reducing the Rx chain. The conclusions are summarized in the following:

Observation 1. No ΔRIBNC requirements defined for TDD intra-band non-contiguous CA.

Observation 2.There is no impact on maximum input level requirements by reducing the number of UE Rx chains.

Observation 3. The in-gap Rx requirements should be checked when reducing the Rx chain.

Observation 4. For the two DL carrier, each carrier satisfy the ACS/IBB/NBB requirements of the single carrier for in-gap and out-of-gap testing.

Observation 5. With the PSD imbalance between two DL carriers up to 6dB, the power imbalance would be larger than 6dB considering the asymmetric carrier channel bandwidth.

Proposal 1. Both TDD and FDD intra-band DL contiguous CA should be included.

Proposal 2. Re-evaluated the ΔRIBNC requirements based on the existing test points.

Proposal 3. Except in-gap ACS/IBB, the in-gap narrow band blocking should also need to be checked when reducing the Rx chain.

Proposal 4: For the in-band gap test of ACS, IBB and NBB requirements, the existing conditions of the Wgap shall be also applied for the shared RF chain.

Proposal 5. The existing ACS level should be maintained when reducing the Rx chain number.

# 4	Reference

[1]  RP-241669, New SID: Study on NR FR1 DL Fragmented Carriers, MediaTek Inc.

[2]  RP-241674, New WID: Rel-19 NR Carrier Aggregation (CA)/Dual Connectivity (DC) for x bands DL with y bands UL (x<7, y<3) and Supplementary Uplink (SUL) band combinations/CA band combinations with a single  SUL or two SUL cells, RAN4 chair (Huawei)