# 3GPP TSG-RAN WG4 Meeting # 112                                    R4-2411887

# Maastricht, Netherlands, Aug.19th – 23th, 2024

# Source: 	ZTE Corporation, Sanechips

# Title: 	On MSD requirements with intra-band contiguous UL CA

# Agenda item:	7.3.3

# Document for:	Approval

# 1 Introduction

In WF[1], the issue on the MSD requirements with intra-band contiguous UL CA was raised, see the proposals below:

Proposal 1: Companies are encouraged to consider the following options for handling the MSD requirements resulting from intra-band contiguous UL CA configured with non-contiguous allocations: Option 1: No change from TR 38.862 guidelines Option 2: Do not consider all MSD requirements resulting from intra-band contiguous UL CA configured with non-contiguous allocations. Option 3: Do not consider only the MSD requirements resulting from intra-band contiguous UL CA configured with 1RB+1RB allocations. Proposal 2: If Option 2 or Option 3 in Proposal 1 would be considered, which release to start taking effect? Option 1: From earliest release where such MSD requirements have been specified. Option 2: Rel-18 Option 3: Rel-19 Proposal 3: Is there a need to introduce cross-band MSD requirements resulting from intra-band contiguous UL CA configured with fully allocated maximum aggregated BW? Option 1: Yes Option 2: No

In last meeting, companies bring contributions [2~8] on the above issue but no consensus were reached. Due to the last meeting is the last meeting for R18 basket WID, so it seems it is not easy to overturn the R18 guidance and remove the non-contiguous allocation such as 1RB+1RB MSD test point for all the existing R18 configurations.

In this contribution, we give some further views .

# 2	Discussion

- In terms of the discussion in last meeting, some companies including operator think there is no need to consider intra-band non-contiguous allocation MSD test point, but some other companies hold the contrary view.

- The main reason for not considering the intra-band non-contiguous allocation such as 1RB+1RB MSD test point is that from the network scheduling point of view a network will not schedule another carrier and configure UL intra-band CA if one carrier is not fully utilized since it would impact the UL throughput [5]. In the other words, the non-contiguous allocation MSD test point would not be not a popular scenario in the filed, even it doesn't occur in operators’ networks.

- In [3], the background for the specification framework was established in Release 17 for intra-band ULCA for a TDD band and in the late part of Release 18 for the intra-band ULCA for FDD bands are given, where:

- 1: For a TDD band is operated as intra-band contiguous or non-contiguous ULCA,1RB+1RB allocation is used in the MSD test point assuming MPR requirement is applied to meet the emissions;

- 2: For a FDD band is operated as intra-band contiguous or non-contiguous ULCA, RBs equal to the equivalent BW REFSENS UL configuration is used assuming that the UE is at PCmax and shared with equal PSD amongst the two carriers.

- Consequently, the 1RB+1RB allocation MSD test point is only used in the TDD band as intra-band UL CA, and several RB+RB allocation MSD test point is only used in the FDD band as intra-band UL CA.

- In our view, it is debatable whether the scenario small RB configuration in each CC (even 1RB in each CC) in intra-band contiguous CA is meaningful in the filed. Nevertheless, out initial idea is to define MSD for practical scenarios in the spec, so we slight prefer not to consider the MSD for intra-band contiguous UL CA configured with non-contiguous allocations.

- Actually, this issue was somehow triggered by CA_n41C-n79A due to >55dB MSD is foreseen without MPR, and ~43dB MSD with MPR assumption, where such high MSD value cause deployment concerns although it is not the first combination whose MSD value is more than 45dB. Therefore, we also prefer to re-consider 1RB+1RB allocations, and it should only be for intra-band TDD UL CA only.

- Proposal 1: Re-considered 1RB+1RB RB allocation for intra-band TDD ULCA within an inter-band DL CA.

- For the guidelines in TR 38.862 guidelines, we think the existing guidelines in the TR are still valid since all of the guidelines are developed during R18 time phase, and due to there are no R19 FS_SimBC SID so far, if there are updates for the existing guidelines during R19, then the updated guidelines should be applied for the new introduced R19 band combination. In the other words, no CR to TR38.862 in R19.

- Proposal 2: if there are updates for the existing guidelines, they should be captured in the WF and applied for the new introduced R19 band combination.

Regarding the cross-band MSD requirements resulting from intra-band contiguous UL CA configured with fully allocated maximum aggregated BW, it is a new type of MSD and RAN4 never consider it in the past although there are several inter-band UL CA including intra-band CA configurations in the spec.

- Observation: In current spec, there are no cross band isolation MSD defined for intra-band contiguous UL CA configured with fully allocated maximum aggregated BW.

- As we discussed in [7], fully allocated maximum aggregated BW is a meaningful scenario in the field for one band supports intra-band UL CA, we think there might be needed define the MSD since the ACLR region caused by intra-band contiguous CA (maximum aggregated BW) is larger than single carrier, the ACLR region does not overlap with victim DL for single carrier does not imply the ACLR region will not overlap with victim DL for intra-band contiguous CA.

- Proposal 3: If the cross-band MSD requirements resulting from intra-band contiguous UL CA configured with fully allocated maximum aggregated BW is considered to be defined:

- - Only to define new cross band isolation MSD for ACLR1/ACLR2 interference source

- - To reuse cross band isolation MSD of single carrier for >ACLR2 interference source

- Moreover, if possible, guideline should be developped to avoid new calculation/evaluation efforts, such as scaling with the RB BW, or +x dB on top of the MSD of single carrier, or others.

# 3 Conclusion

In this contribution, we give some discussions on the MSD requirements with intra-band contiguous UL CA. The proposals and conclusions are:

- Proposal 1: Re-considered 1RB+1RB RB allocation for intra-band TDD ULCA within an inter-band DL CA.

- Proposal 2: if there are updates for the existing guidelines, they should be captured in the WF and applied for the new introduced R19 band combination.

- Observation: In current spec, there are no cross band isolation MSD defined for intra-band contiguous UL CA configured with fully allocated maximum aggregated BW.

- Proposal 3: If the cross-band MSD requirements resulting from intra-band contiguous UL CA configured with fully allocated maximum aggregated BW is considered to be defined:

- - Only to define new cross band isolation MSD for ACLR1/ACLR2 interference source

- - To reuse cross band isolation MSD of single carrier for >ACLR2 interference source

# 4	Reference

- [1] R4-2406676, WF on MSD requirements with intra-band contiguous UL CA, Apple

- [2] R4-2407082, On MSD requirements with intra-band contiguous UL CA, Apple

- [3] R4-2407372, On UL configuration for intra-band ULCA IMDs, Skyworks Solutions Inc.

[4] R4-2407622, Discussion on the MSD requirements of intra-band contiguous UL CA with non-contiguous RB allocation, Huawei, HiSilicon

[5] R4-2408731, Discussion on MSD requirements with intra-band contiguous UL CA, CMCC

[6] R4-2409319, Discussion on MSD test point trade-off for intra-band UL CA,Huawei, HiSilicon

[7] R4-2408357, On MSD requirements with intra-band contiguous UL CA, ZTE Corporation, Sanechips

[8] R4-2408853, MSD requirements with intra-band contiguous CA, Qualcomm France