# 3GPP TSG-RAN WG4 Meeting # 112                                    R4-2411888

# Maastricht, Netherlands, Aug.19th – 23th, 2024

Source:	ZTE Corporation, Sanechips

Title:	Views on power class indication for HPUE DL CA with single UL transmission

Agenda Item:	10.2

Document for:	Approval

- 1	Introduction

During the previous meetings, RAN4 spent nearly two years to discuss the power class indication for HPUE downlink CA with different UL configurations, also associated with the RAN2 LS issue on the relationship amongs the IEs of ue-PowerClassPerBandPerBC-r17 and the legacy IE ue-PowerClass/ue-PowerClass-v1610/ue-PowerClass-1700, powerClass/powerClass-v1610.

However,  there are no conclusions and the discussion was stopped in RAN4 if there is no further offline consensus

In last RAN planery, this issue was also raised and in the end, RAN task RAN4 to continue discusing this issue.

Way Forward -RAN tasks RAN4 to investigate how to enable HPUE maximum transmit power in downlink CA with a single configured UL -RAN task shall conclude by RAN#106 -RAN4 online meeting time should be limited to <0.1 TUs

- In this contribution, we provide some further discussions HPUE power class indication.

- 2	Discussion

### 2.2 Power class indication for different UL configurations

In the past discussions, there are following 7 cases are included:

| DL CA configuration | Case | UL configuration Cases |
| --- | --- | --- |
| intra-band CA | 1 | single carrier |
|  | #2 | intra-band UL CA |
| inter-band CA (2CC) | 3 | single carrier |
|  | #4 | inter band UL CA (2CC) |
| inter-band CA (3CC) | 5 | single carrier |
|  | 6 | intra-band UL CA only |
|  | 7 | inter+ intra UL CA (3CC) |

For cases 2/4/6/7, it was already agreed that power class (powerClass) shall limit the configured maximum output power (Pcmax,f,c) when higherPowerLimit-r17 is not indicated.

For cases 1/3/5, there are two cases for the single carrier in the UL:

1. single carrier UL configured

2. single carrier UL transmission (when the UL CC(s) is(are) deactivated )

Also how to reflect the above cases in the specification is quite controversial.

Here, case 1 is only discussed per RAN task.

It terms of the previous discussion for the DL only CA, it seems the common understanding was that it should allow UE to report higher power when UE have such ability. The tricky thing is whether it is mandatory or optional.

In our understanding, at the current stage it is too late to mandate the UE to meet maximum output power restricted by ue-PowerClass, especially when the power indicated by ue-PowerClass is higher than the power indicated by powerClass, since it may cause several problems such as UE implementation (likely NBC issue), RF requirements ambiguity, even test problems. Thus, we think for DL only CA including both single carrier UL configured and single carrier UL transmission, UE can optionally report higher power than than specified power classes.

Proposal 1: For HPUE DL CA with  single configured UL, it allows UE optionally report higher power than the band combination power classes.

One of the companion aspect is that whether to relate the above allownance with the MSD specification. There is no problem for the HPUE band combination without MSD issue. However, for some band combination, although there are no MSD defined for PC3 with some reasons such as negligible MSD value, MSD may need to be defined for the HPUE band combiantion due the UL power is increased.

There are many upcoming HPUE DL CA combinaitons requested and to be introduced in the specification, and for some HPUE DL CA with UL band is FDD band, we think the progress would be still slow since PC2 UL FDD band is  more complicated than TDD when derive the MSD requirements considering 1Tx and 2Tx architecture for PC2 FDD band.

Thus there exist some HPUE DL CA combiantions who have no MSD been defined but theMSD is defined for the corresponding PC3 DL CA.  Although it allows UE to unbind the UL power with the band combiantion power class, it would cause ambiguity since no MSD is defined for PC2. In this case, [2] propose to verify the MSD at the default power class output level.

It seems there are no other choices but to use the PC3 MSD for the test if there is no corresponding MSD defined for HPUE CA, unless UE should not report the power class by ue-PowerClassPerBandPerBC-r17 due to the RF requirements are not completed. In general, for single UL configured, the MSD value for PC3 combination would be larger than HPUE. It is no doubt that UE support HPUE DL CA can meet MSD values defined for PC2 as well if this UE pass PC3’s MSD.

Proposal 2: For a UE supports HPUE DL CA with single UL configured, if UE report UL higher power via ue-PowerClassPerBandPerBC-r17 than the band combination power classes, but the corresponding MSD is not defined , MSD of PC3 should be used as a reference.

The above proposal doesn’t mean the MSD for HPUE DL CA is not needed to be defined. Of course, it shall be needed to define the MSD requirements in the specification, and it belongs to basket WID HPUE band combiantion work.

- 3	Conclusion

In this paper, we provide some discussions on the issue on the ue-PowerClassPerBandPerBC-r17(R4 16-8). The conclusion are:

Proposal 1: For HPUE DL CA with  single configured UL, it allows UE optionally report higher power than the band combination power classes.

Proposal 2: For a UE supports HPUE DL CA with single UL configured, if UE report UL higher power via ue-PowerClassPerBandPerBC-r17 than the band combination power classes, but the corresponding MSD is not defined , MSD of PC3 should be used as a reference.

- 4	Reference

[1] RP-241625, Moderator's summary for offline discussion on CA Power Class Support, AT&T

[2] RP-241221, CA Power Class Support, Qualcomm Incorporated