# 3GPP TSG-RAN WG4 Meeting # 112                               R4-2411892

# Maastricht, Netherlands, 19th – 23rd August, 2024

# Title: 	Discussion on UE RF for type 4a and type 4b for NonCol_intraB_ENDC_NR_CA

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	8.5.2.1

# Document for:	Approval

# 1 Introduction

In RAN #103 meeting, A new work item on support of intra-band non-collocated EN-DC/NR-CA was approved[1].

The core part objectives of the new WID is as below.

The core part of the work item includes: - Specify the core requirements for type 4a/4b capable FWA UE that supports non-co-located scenarios for FR1 inter-band non-contiguous EN-DC with overlapping or partially overlapping bands and/or FR1 intra-band non-contiguous NR-CA for up to 4-layer DL MIMO per CC, where MRTD>CP is assumed. - RF power imbalance requirements of up to 25dB - RRM requirements including MTTD and MRTD requirements and other identified requirement such as scheduling restriction/availability, SCell operation related delay and interruption. - Work is limited to EN-DC/NR-CA for bands 42, n77/n78 - Discuss and if needed, specify the UE behavior when the UE supports type 4a/b but operates in co-located scenarios depending on whether network signaling is provided. R18 handling of UE behavior can be used as a reference.

This contribution shares our views about UE RF for type 4( or type 4a and type 4b).

- Discussion

Based on the conclusion in RAN4 #111 meeting, the following RF requirements need further discussion for type 4a/4b[2].

< Issue 2-1-2: How to capture the power imbalance and REFSENS requirements > Agreement: - Define the Type 4 receiver imbalance requirements in the same table as for Type 2. - FFS on the details of modification < Issue 2-1-3: Minimum DL frequency separation > Way Forward: - Continue further discussion on the following two options and conclude it in the next meeting. - Option 1. Center of BWanother relative to edge of BWwanted is assumed to be at least 80MHz+BWanother/2 away from the edge of the wanted CC. - Option 2. Max (5/2* BWanother, [50] MHz).

With regard to the specification implement for type 4a/4b, it is agreed in RAN4 # 112 to include RF requirements of type 4a/4b in the same table with type 2 in table 7.10A.2-1. Due to up to 4-layer DL MIMO(i.e. 4 antenna port) per CC is supported for type 4a/4b UE, the Rx requirements for four Rx ports are applicable for each component carrier. Therefore, the NOTE 4 in table 7.10A.2-1 should be updated to include type 4a/4b UE. As pointed by company last meeting, Note 3 also needs refinement as type 2 appears in the specification. Another issue is that the description in 7.10A is used for power imbalance of type 2 UE, and if type 4 also needs to be included in the chapter, the modification of the text in 7.10A is also necessary.

Proposal 1. To take the following modification of 7.10A into consideration for power imbalance of type 4 NRCA .

7.10A.1 General Power imbalance requirement is a measure of the receiver’s ability to receive a wanted signal in the presence of another signal with a power imbalance and a specific frequency offset from the wanted signal. Power imbalance requirement in this subclause is only applicable for a UE when A UE capable of intraBandNR-CA-non-collocated-r18 and is not provided with nonCollocatedTypeNR-CA-r18 and is configured with maxMIMO-Layers with value less than or equal to 2. Or a UE capable of [intraBandNR-CA-non-collocated-r19] and is not provided with [nonCollocatedTypeNR-CA-r19] and is configured with maxMIMO-Layers with value less than or equal to 4.

Table 7.10A.2-1: Power imbalance parameters for intra-band non-contiguous CA

| Test configurations | Carriers | Rx Power in transmission bandwidth configuration (dBm) | channel bandwidth | Center of BWanother Relative to edge of BWwanted |
| --- | --- | --- | --- | --- |
| 1 | Wanted carrier | REFSENS NOTE 4 + 1 | BWwanted ≤ BWanother | < max (5/2* BWanother, 50MHz |
|  | Another wanted carrier | Power of wanted carrier + 25 |  |  |
| 2 | Wanted carrier | REFSENS NOTE 4 + 1 | BWwanted > BWanother |  |
|  | Another wanted carrier | Power of wanted carrier + 25 – 10*log10(BWwanted /BWanother) |  |  |
| 3 | Wanted carrier | REFSENS NOTE 4 + 1 | NA | ≥ max (5/2* BWanother, 50MHz) |
|  | Another wanted carrier | Power of wanted carrier + 25 |  |  |
| NOTE 1: The transmitter shall be set to 24dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table 7.3.2-3 with PCMAX_L,f,c as defined in clause 6.2A.4. NOTE 2: BWwanted is the channel bandwidth of wanted carrier. BWanother is the channel bandwidth of another wanted carrier with 25 dB power imbalance. NOTE 3: It’s allowed to use one of test configurations to verify the RX power imbalance requirement. NOTE 4: REFSENS is the reference sensitivity level for two antenna port in Table 7.3.2-1b and/or for four antenna port in Table 7.3.2-1b modified by the ΔRIB,4R in Table 7.3.2-2. |  |  |  |  |

The minimum DL frequency separation is discussed for type 4a/4b UE and the following 2 options are reached.

< Issue 2-1-3: Minimum DL frequency separation > Way Forward: - Continue further discussion on the following two options and conclude it in the next meeting. - Option 1. Center of BWanother relative to edge of BWwanted is assumed to be at least 80MHz+BWanother/2 away from the edge of the wanted CC. - Option 2. Max (5/2* BWanother, [50] MHz).

The two options reached last meeting are not contradictory.

Option 1 is reached during Rel-18 NonCol_intraB_ENDC_NR_CA discussion. And an LS[3] is agreed to sent to RAN5 as below.

RAN4 has agreed to define NR-CA PDSCH demodulation requirements for Rel-18 WI on supporting intra-band non-colocated EN-DC/NR-CA deployment. PDSCH demodulation requirements are specified in TS 38.101-4 for UE indicating [intraBandNonColocatedCA-r18]. It was previously agreed [1] that RF conformance tests should introduce the following test condition in RAN5: - Center of BWanother relative to edge of BWwanted is assumed to be at least 80MHz+BWanother/2 away from the edge of the wanted CC in R18. The same agreement should be applied to NR-CA PDSCH demodulation requirement.

For Rel-18 NonCol_intraB_ENDC_NR_CA, there is not additional frequency separation limitation introduced in RAN4 specification for type 2 UE. The LS is intended to tell RAN5 the conformance test assumption.

For the same reason, it is not necessary to define additional minimum DL frequency separation in RAN4 specification for type 4a/4b UE as both type 4a/4b and type 2 use separate Rx for different CC receiving.

With regard to option 2, the 5/2* BWanother is the offset configuration for IBB case 2 offset. If additional minimum downlink frequency separation are defined for Type 4a/4b, then the additional minimum downlink frequency separation should also be defined for Type 2, as mentioned before—the implementation architecture for Type 2 and Type 4a/4b is identical. However, the current RAN4 specification does not define additional minimum downlink frequency separation for Type 2.

Proposal 2. Not to consider additional minimum DL frequency separation in RAN4 specification for type 4a/4b UE. The minimum separation applicable to type2 in RAN4 specification also applies to type4a/4b.

# 3 Conclusion

In this contribution, we give our viewpoint about requirements for Type 4a/4b for NonCol_intraB_ENDC_NR_CA.

The conclusions are:

Proposal 1. To take the following modification of 7.10A into consideration for power imbalance of type 4 NRCA .

7.10A.1 General Power imbalance requirement is a measure of the receiver’s ability to receive a wanted signal in the presence of another signal with a power imbalance and a specific frequency offset from the wanted signal. Power imbalance requirement in this subclause is only applicable for a UE when A UE capable of intraBandNR-CA-non-collocated-r18 and is not provided with nonCollocatedTypeNR-CA-r18 and is configured with maxMIMO-Layers with value less than or equal to 2. Or a UE capable of [intraBandNR-CA-non-collocated-r19] and is not provided with [nonCollocatedTypeNR-CA-r19] and is configured with maxMIMO-Layers with value less than or equal to 4.

Table 7.10A.2-1: Power imbalance parameters for intra-band non-contiguous CA

| Test configurations | Carriers | Rx Power in transmission bandwidth configuration (dBm) | channel bandwidth | Center of BWanother Relative to edge of BWwanted |
| --- | --- | --- | --- | --- |
| 1 | Wanted carrier | REFSENS NOTE 4 + 1 | BWwanted ≤ BWanother | < max (5/2* BWanother, 50MHz |
|  | Another wanted carrier | Power of wanted carrier + 25 |  |  |
| 2 | Wanted carrier | REFSENS NOTE 4 + 1 | BWwanted > BWanother |  |
|  | Another wanted carrier | Power of wanted carrier + 25 – 10*log10(BWwanted /BWanother) |  |  |
| 3 | Wanted carrier | REFSENS NOTE 4 + 1 | NA | ≥ max (5/2* BWanother, 50MHz) |
|  | Another wanted carrier | Power of wanted carrier + 25 |  |  |
| NOTE 1: The transmitter shall be set to 24dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table 7.3.2-3 with PCMAX_L,f,c as defined in clause 6.2A.4. NOTE 2: BWwanted is the channel bandwidth of wanted carrier. BWanother is the channel bandwidth of another wanted carrier with 25 dB power imbalance. NOTE 3: It’s allowed to use one of test configurations to verify the RX power imbalance requirement. NOTE 4: REFSENS is the reference sensitivity level for two antenna port in Table 7.3.2-1b and/or for four antenna port in Table 7.3.2-1b modified by the ΔRIB,4R in Table 7.3.2-2. |  |  |  |  |

Proposal 2. Not to consider additional minimum DL frequency separation in RAN4 specification for type 4a/4b UE. The minimum separation applicable to type2 in RAN4 specification also applies to type4a/4b.

# 4	Reference

- RP-240838, Rel-19 WID_Intra-band non-collocated EN-DC and NR-CA, Apple

- R4-2406627, WF on NonCol_intraB_ENDC_NR_CA, KDDI

- R4-2316951, LS on frequency separation for Type 2 UE NR-CA PDSCH demodulation requirements, TSG RAN WG4