# 3GPP TSG-RAN WG4 Meeting # 112                               R4-24xxxxx

# Maastricht, Netherlands, 19th – 23rd August, 2024

# Title: 	Discussion on UE Capability/UE behavior and network signaling for Type 4a Type 4b EN-DC/NR-CA

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	8.5.2.2

# Document for:	Approval

# 1 Introduction

In RAN #103 meeting, A new work item on support of intra-band non-collocated EN-DC/NR-CA was approved[1].

The core part objectives of the new WID is as below.

The core part of the work item includes: - Specify the core requirements for type 4a/4b capable FWA UE that supports non-co-located scenarios for FR1 inter-band non-contiguous EN-DC with overlapping or partially overlapping bands and/or FR1 intra-band non-contiguous NR-CA for up to 4-layer DL MIMO per CC, where MRTD>CP is assumed. - RF power imbalance requirements of up to 25dB - RRM requirements including MTTD and MRTD requirements and other identified requirement such as scheduling restriction/availability, SCell operation related delay and interruption. - Work is limited to EN-DC/NR-CA for bands 42, n77/n78 - Discuss and if needed, specify the UE behavior when the UE supports type 4a/b but operates in co-located scenarios depending on whether network signaling is provided. R18 handling of UE behavior can be used as a reference.

This contribution shares our views about UE Capability/UE behavior and network signaling type 4a and type 4b.

- Discussion

Some agreements were reached in RAN4 #111 meeting and some issues need further discussion [2].

< Issue 2-2-1: New UE Capabilities for Type 4a(EN-DC) and 4b(EN-DC/NR-CA) > Way Forward: - Continue further discussion on the following two options and conclude it in the next meeting. - Option 1. Add new UE capabilities for Type 4a and Type 4b support indication. - Option 2. Add one new UE capability for Type 4 (no distinction between Type 4a and 4b) support indication. < Issue 2-2-2: Whether to support Type 2 capability by UE having Type 4 capabilities > Agreement: - If UE reports the Type 4 capability, Type 2 capability shall be deemed as support by default regardless of whether UE indicates Type-2 capability or not. - If UE reports the Type 4 capability, it is not necessary for UE to report Type 2 capabilities. - If the issue is identified for these bullets, RAN4 will revisit them. < Issue 2-2-3: New BS Signalling to switch between Type 4a/4b and Type 1(collocated) > Way Forward: - First, conclude discussions of whether to add new UE capability(s) for Type 4. - And then, continue further discussion on BS signalling in the next meeting. < Issue 2-2-4: New BS Signalling to switch between Type 4a/4b and Type 2 > Way Forward: - First, conclude discussions of whether to add new UE capability(s) for Type 4. - And then, continue further discussion on BS signalling in the next meeting. < Issue 2-2-6: UE behavior between Type 1, Type 2 and Type 4a/4b with new BS signalling > Way Forward: - First, conclude discussions of whether to add new UE capability(s) and BS signalling for Type 4. - And then, continue further discussion on BS signalling/UE behavior in the next meeting.

With regard to the UE capability of Type 4a/4b, a conclusion is reached to introduce a new UE capability with 2 options which need further down-selection.

The difference between Type 4a and 4b is the number of Rx antenna as shown in table 2-1. As of now, it cannot be ruled out that some UEs will be implemented as Type 4a. If some UE implements Type 4a and does not implement Type 4b, the UE does not have the chance to report the capability of type 4a if Type 4a and Type 4b are merged into Type 4. Therefore two signals are needed to indicate the capabilities of both Type4a and Type4b. With regard to Type 2, 2 UE capability signaling are used, namely intraBandNR-CA-non-collocated-r18 and interBandMRDC-WithOverlapDL-Bands-r16. Similar to Rel-18 intra-band non-collocated EN-DC/NR-CA, it is preferred to introduce 2 separate signaling for 4a and 4b. If the signals indicating type 4a and type 4b are merged into one signal, it is required that the UE supports the ability of type 4a and type 4b which has not yet been agreed upon at present.

Table 2-1 UE types summary[3]

Proposal 1. To consider separate capability reporting siganling for type 4a and type 4b respectively.

With regard to the type 2 capability of a UE supporting type 4a/4b, a reasonable assumption is that the type 2 capability is supported by default if the type 4a/4b capability is reported.

For NW signalling for type switching between type 1 and type 4a/4b, the following lists the BS signaling agreed in Rel-18 specification[5].

nonCollocatedTypeMRDC This field is only present for a UE configured with maxMIMO-Layers with value less than or equal to 2 for all corresponding serving cells, in case of TDD-TDD inter-band (NG) EN-DC with overlapping or partially overlapping bands. If this field is present, the UE applies (NG)EN-DC MTTD/MRTD according to clause 7.5.3/7.6.3 in TS 38.133 [14] and inter-band RF requirements. If this field is absent, the UE applies (NG)EN-DC MTTD/MRTD according to clause 7.5.2/7.6.2 in TS 38.133 [14] and inter-band RF requirements when indicating support of interBandMRDC-WithOverlapDL-Bands-r16.

nonCollocatedTypeNR-CA This field is only present for a UE configured with maxMIMO-Layers with value less than or equal to 2 for all corresponding serving cells, in case of TDD-TDD intra-band NR-CA. If this field is present, the UE applies MRTD according to Table 7.6.4-1 in TS 38.133 [14] and UE RF requirements for intra-band NR-CA except for 7.10A in TS 38.101-1 [15]. If this field is absent, the UE applies MTTD/MRTD requirements according to Table 7.5.4-1/Table 7.6.4-2 in TS 38.133 [14] and UE RF requirements for intra-band non-collocated NR-CA including 7.10A in TS 38.101-1 [15] when indicating support of intraBandNR-CA-non-collocated-r18.

From the above, we can find the NW signaling of nonCollocated ENDC and nonCollocated NRCA are coupled with the maxMIMO-Layers as they are only present when a UE is configured with maxMIMO-Layers less than or equal to 2.

Observation 1. BS signaling for nonCollocated ENDC and nonCollocated NRCA only present when a UE is configured with maxMIMO-Layers less than or equal to 2.

With regard to type switching between type 1 and type 4a/4b, we reviewed the signaling for type switching between type 1 and type 2 and the relevant conclusions reached last meeting.

- RAN4 # 110b meeting[4] - UE having Type 4 capability support the following capabilities (Set 1) when operating in non-collocated scenarios. - Maximum 4 MIMO layers on each CC - Maximum power imbalance of 25dB - UE having Type 4 capability support the following capabilities (Set 2=Type 1 capability) when operating in collocated scenarios. - Maximum 8 MIMO layers on each CC - It may be optionally supported - Maximum PSD imbalance of 6dB

RAN4 # 111 meeting[2] < Issue 2-2-3: New BS Signalling to switch between Type 4a/4b and Type 1(collocated) > Way Forward: - First, conclude discussions of whether to add new UE capability(s) for Type 4. - And then, continue further discussion on BS signalling in the next meeting.

From the above, we can find that type 4a/4b UE may operate in Set 1 or Set 2. There is a chance to configure a type 2 capability UE in type 1 transmission (nonCollocatedTypeNR-CA) . For type 4a/4b UE, it is best for the network to have the opportunity to configure a UE as type 1 with one of these 2 sets of requirements which have been reached in RAN4 # 110b meeting. Similar to the type 2 UE discussion, new Rel-19 BS signaling is needed for type 4a/4b intra-band CA and inter-band ENDC to give NW a chance to configure/inform a UE in co-located operation.

Proposal 2. BS signaling is needed for type switching between type 4a/4b and type 1.

The type switching between type 4a/4b and type 2 is not reached yet.

< Issue 2-2-4: New BS Signalling to switch between Type 4a/4b and Type 2 > Way Forward: - First, conclude discussions of whether to add new UE capability(s) for Type 4. - And then, continue further discussion on BS signalling in the next meeting.

It is necessary to clarify the benefits of introducing the type switching between type 4a/4b and type 2. We assume the RF requirements are the same for type 2 and type 4a/4b UE, then the difference for type 2 and type 4 are the supported MIMO layer. For a UE report type 4 signaling, it means it can support up to 4 layer per cc for NRCA, which can also cover the requirement of Type 2. Hence, maybe it is enough to only consider type 4 UE requirements if a UE support both type 2 and type 4. From the above analysis, we do not see the benefit when a UE is reconfigured from type 4a/4b to type 2 as type 4a/4b can utilize more Rx than type 2. If it can be concluded that a type 4a/4b UE can support type 2 by default, and if the type switching from type 4a/4b does not provide benefit, the introducing of type switching from type 4a/4b is not necessary.

Observation 2. It is not expected that a UE reports both type 2 capability and type 4a/4b capability.

Proposal 3.Not to introduce type switching between type 4a/4b and type 2 if no benefit can be expected.

# 3 Conclusion

In this contribution, we give our viewpoint about requirements for type 4a/4b for NonCol_intraB_ENDC_NR_CA.

The conclusions are:

Observation 1. BS signaling for nonCollocated ENDC and nonCollocated NRCA only present when a UE is configured with maxMIMO-Layers less than or equal to 2.

Observation 2. It is not expected that a UE reports both type 2 capability and type 4a/4b capability.

Proposal 1. To consider separate capability reporting siganling for type 4a and type 4b respectively.

Proposal 2. BS signaling is needed for type switching between type 4a/4b and type 1.

Proposal 3.Not to introduce type switching between type 4a/4b and type 2 if no benefit can be expected.

# 4	Reference

- RP-240838, Rel-19 WID_Intra-band non-collocated EN-DC and NR-CA, Apple

- R4-2406627, WF on NonCol_intraB_ENDC_NR_CA, KDDI

- RP-240101, Motivation on supporting Intra-band non-collocated EN-DC/NR-CA deployment in Release 19, KDDI, etc.

- R4-2410686, WF on NonCol_intraB_ENDC_NR_CA, KDDI

- 3GPP TS 38.331 V18.2.0